TABLE OF CONTENTS

i2c.library/SetI2CDelay

i2c.library/SendI2C

i2c.library/ReceiveI2C

i2c.library/I2CErrText

i2c.library/GetI2COpponent

i2c.library/ShutDownI2C

i2c.library/BringBackI2C

This file describes the recommended use of i2c.library V39+ only.

Some functions have been added since V38, but more important, some functions

have become obsolete and are not recommended anymore. So, for your own

convenience, follow these guidelines and make your programs insist on V39.

i2c.library/SetI2CDelay                               i2c.library/SetI2CDelay

NAME

SetI2CDelay -- Set the delay for IC-bus clock-rate

SYNOPSIS

oldval = SetI2CDelay(ticks);

D0                  D0

ULONG SetI2CDelay(ULONG);

FUNCTION

Set the delay for the IC-bus clock, which must not exceed a

frequency of 100 KHz (or maybe less, with unfavourable hardware

configurations).

INPUTS

ticks - Number of idle reads from a CIA register, which will be

added to every HI and LO SCL pulse.  This is a slow

operation (about 1.5 s with OCS/ECS, slightly less with

AA chipset), regardless of CPU speed. Small values (0, 1,

2, 3, ...) will normally do.

A magic value I2CDELAY_READONLY will not change anything,

should you only want to read the old delay.

RESULTS

oldval - The old delay value, just in case you are curious.  You

don't have to restore it, you need not even look at it.

NOTES

Starting with i2c.library V40, the default delay value may be set

globally by an environment variable (I2CDELAY).  So for most programs

it's now reasonably safe to assume that the delay value has already

been set correctly, and they need not call SetI2CDelay at all.

Otherwise remember that the delay value affects all clients of the

library in the same way, not only the calling program.  And if you

do change it, never set it to a hardcoded value, but one that is

somehow adjustable by the end user.

SEE ALSO

BUGS

i2c.library/SendI2C                                       i2c.library/SendI2C

NAME

SendI2C -- Transmit bytes on the IC-bus

SYNOPSIS

error = SendI2C(addr, number, data);

D0             D0    D1      A1

ULONG SendI2C(UBYTE, UWORD, UBYTE []);

FUNCTION

Send data to a device connected to the IC-bus hardware.

INPUTS

addr - Address of device to receive the transmitted data.

number - Number of bytes to transmit from the array specified

by data.

data - Array, where the data to be transmitted is placed in.

RESULTS

error - may be considered as three UBYTE's: 0x00AABBCC, with

CC: Zero, if an error occurred.

BB: I/O error number (see i2c_library.h)

AA: Allocation error number (see i2c_library.h)

NOTES

The LSB in the addr byte will always be cleared to make a valid

IC-bus write address.  You can take advantage of this and address

a chip for both read and write using the same symbolic constant.

SEE ALSO

ReceiveI2C

I2CErrText

BUGS

i2c.library/ReceiveI2C                                 i2c.library/ReceiveI2C

NAME

ReceiveI2C -- Receive bytes from the IC-bus

SYNOPSIS

error = ReceiveI2C(addr, number, data);

D0                D0    D1      A1

ULONG ReceiveI2C(UBYTE, UWORD, UBYTE []);

FUNCTION

Receive data from a device connected to the IC-bus hardware.

INPUTS

addr - Address of device to receive data from.

number - Number of bytes to receive from the specified device.

data - Array, where to put the received data.

RESULTS

error - may be considered as three UBYTE's: 0x00AABBCC, with

CC: Zero, if an error occurred.

BB: I/O error number (see i2c_library.h)

AA: Allocation error number (see i2c_library.h)

NOTES

Be sure that the size of the data-array is the same or larger than

the requested number of bytes to receive.

The LSB in the addr byte will always be set to make a valid IC-bus

read address.

SEE ALSO

SendI2C

I2CErrText

BUGS

i2c.library/I2CErrText                                 i2c.library/I2CErrText

NAME

I2CErrText -- give a plaintext translation of an error code

SYNOPSIS

errtext = I2CErrText(errnum);

D0                   D0

STRPTR I2CErrText(ULONG);

FUNCTION

Gives a plaintext translation of an IC-bus I/O error, nice

information for the user.

INPUTS

errnum - the error code as reported by SendI2C or ReceiveI2C

RESULTS

errtext - STRPTR to a brief description (up to 20 chars) of the

supplied error code.

NOTES

Supplying random (i. e. illegal) error codes is safe and will only

return a "???" string.

If you want to report only error codes that are really errors, not

"OK", look at their low-order byte: it's zero with "real errors".

SEE ALSO

SendI2C

ReceiveI2C

BUGS

i2c.library/GetI2COpponent                         i2c.library/GetI2COpponent

NAME

GetI2COpponent -- Get name of task using our hardware

SYNOPSIS

name = GetI2COpponent();

D0

STRPTR GetI2COpponent(void);

FUNCTION

When hardware allocation via misc.resource fails, a STRPTR

description of the current resource owner is returned.  i2c.library

stores this name and lets you look at it with this function.

RESULTS

name - STRPTR to someone using the hardware that is needed

for the IC-bus interface (e.g. "serial.device"), or

NULL, if there is no name to return.

NOTES

This function is best used to provide additional user information

in situations where an error code returned by SendI2C/ReceiveI2C

indicates a resource conflict. (Check for ((err >> 8) & 0xff) ==

I2C_HARDW_BUSY.)

Note that even then, you might still receive a NULL pointer!  Check

for this before you try to process the string!

SEE ALSO

BUGS

i2c.library/ShutDownI2C                               i2c.library/ShutDownI2C

NAME

ShutDownI2C -- Release all allocated hardware immediately

SYNOPSIS

ShutDownI2C();

void ShutDownI2C(void);

FUNCTION

Returns the allocated hardware to the system, no matter how many

clients have the library open, and all subsequent I/O calls will

fail.  Not even AllocI2C (called implicitly when a new client opens

the library) will work, only BringBackI2C will return things to

normal.

INPUTS

RESULTS

NOTES

You should only call this function in direct response to a user

command.  Using it silently might cause major confusion.

DON'T misunderstand this function as a cleanup routine.  All you

have to do is close the library before you quit, and if you

were the last client, the allocated hardware will be returned

automatically.

DON'T call this function "just to be nice".  If your program

typically runs for a long time but only needs the IC-bus every now

and then, system-friendly behaviour would be:  Open the library

every time you need it, call the functions you need, and then close

it again.

Note that there are now many different implementations of

i2c.library, using a wide variety of hardware resources, so you

don't know what resources this call will return.  It might even

affect resources that aren't really needed elsewhere in the system

(zorro board or floppy disk unit for example).

SEE ALSO

BringBackI2C

BUGS

i2c.library/BringBackI2C                             i2c.library/BringBackI2C

NAME

BringBackI2C -- Undo the effect of ShutDownI2C

SYNOPSIS

err = BringBackI2C();

D0

BYTE BringBackI2C(void);

FUNCTION

Removes the lock set up by ShutDownI2C and then tries to allocate

the needed hardware again.

INPUTS

RESULTS

err - an error, if hardware allocation failed, zero if OK.

You need not look at this return value, because even if

BringBackI2C "fails", you don't have to call it again.

The lock will have been removed in any case, and that's

all that matters.

NOTES

You can translate the return value to plaintext using I2CErrText,

but you have to shift it first: call I2CErrText(err<<16).

SEE ALSO

ShutDownI2C

BUGS