VESA                                                         Super VGA Standard

Video Electronics Standards Association

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

2150 North First Street, Suite 360                        Phone: (408) 435-0333

San Jose, CA 95131-2020                                     Fax: (408) 435-8225

Super VGA BIOS Extension

Standard #VS911022

October 22, 1991

Document Version 1.0

VBE Version 1.2

PURPOSE

~~~~~~~

To standardize a common software interface to Super VGA video adapters in order

to provide simplified software application access to advanced VGA products.

SUMMARY

~~~~~~~

The standard provides a  set of functions which an application program can use

to A) obtain information about the capabilities and characteristics of a

specific Super VGA implementation and B) to control the operation of such

hardware in terms of video mode initialization and video memory access.  The

functions are provided as an extension to the VGA BIOS video services, accessed

through interrupt 10h.

VESA Super VGA Standard VS911022-2

Contents

~~~~~~~~

1.      Introduction ................................................. Page 3

2.      Goals and Objectives .........................................      4

2.1     Video environment information ........................      4

2.2     Programming support ..................................      4

2.3     Compatibility ........................................      5

2.4     Scope of standard ....................................      5

3.      Standard VGA BIOS ............................................      6

4.      Super VGA Mode Numbers .......................................      7

5.      CPU Video Memory Control .....................................      9

5.1     Hardware design consideration ........................      9

5.1.1   Limited to 64k/128k of CPU address space .....      9

5.1.2   Crossing CPU video memory window boundaries ..     10

5.1.3   Operating on data frolm different areas ......     10

5.1.4   Combining data from two different windows ....     10

5.2     Different types of hardware windows ..................     11

5.2.1   Single window systems ........................     11

5.2.2   Dual window systems ..........................     11

6.      Extended VGA BIOS ............................................     12

6.1     Status Information ...................................     12

6.2     00h - Return Super VGA Information ...................     12

6.3     01h - Return Super VGA mode information ..............     14

6.4     02h - Set Super VGA mode .............................     20

6.5     03h - Return Super VGA mode ..........................     20

6.6     04h - Save/restore Super VGA video state .............     21

6.7     05h - Super VGGA video memory window control .........     22

6.8     06h - Set/Get Logical Scan Line Length ...............     23

6.9     07h - Set/Get Display Start ..........................     24

6.10    08h - Set/Get DAC Palette Control ....................     25

7.      Application Example ..........................................     26

VESA Super VGA Standard VS911022-3

1.      Introduction

~~~~~~~~~~~~~~~~~~~~

This document contains a specification for a standardized interface to extended

VGA video modes and functions.  The specification consists of mechanisms for

supporting standard extended video modes and functions that have been approved

by the main VESA committee and non-standard video modes that an individual VGA

supplier may choose to add, in a uniform manner that application software can

utilize without having to understand the intricate details of the particular VGA

hardware.

The primary topics of this specification are definitions of extended VGA video

modes and the functions necessary for application software to understand the

characteristics of the video mode and manipulate the extended memory associated

with the video mode.

Readers of this document should already be familiar with programming VGAs at the

hardware level and Intel iAPX real mode assembly language.  Readers who are

unfamiliar with programming the VGA should first read one of the many VGA

programming tutorials before attempting to understand these extensions to the

standard VGA.

VESA Super VGA Standard VS911022-4

2.      Goals and Objectives

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The IBM VGA has become a defacto standard in the PC graphics world.  A multitude

of different VGA offerings exist in the marketplace, each one providing BIOS or

register compatibility with the IBM VGA.  More and more of these VGA compatible

products implements various supersets of the VGA standard.  These extensions

range from higher resolutions and more colors to improved performance and even

some graphics processing capabilities.  Intense competition has dramatically

improved the price/performance ratio, to the benefit of the end user.

However, several serious problems face a software developer who intends to take

advantage of these "Super VGA" environments.  Because there is no standard

hardware implementation, the developer is faced with widely disparate Super VGA

hardware architectures.  Lacking a common software interface, designing

applications for these environments is costly and technically difficult.  Except

for applications supported by OEM-specific display drivers, very few software

packages can take advantage of the power and capabilities of Super VGA products.

The purpose of the VESA VGA BIOS Extension is to remedy this situation.  Being a

common software interface to Super VGA graphics products, the primary objective

is to enable application and system software to adapt to and exploit the wide

range of features available in these VGA extensions.

Specifically, the VESA BIOS Extension attempts to address the following issues:

A) Return information about the video environment to the application, and B)

Assist the application in initializing and programming the hardware.

2.1             Video environment information

Today, an application has no standard mechanism to determine what Super VGA

hardware it is running on.  Only by knowing OEM-specific features can an

application determine the presence of a particular video board.  This often

involves reading and testing registers located at I/O addresses unique to each

OEM.  By not knowing what hardware an application is running on, few, if any, of

the extended features of the underlying hardware can be used.

The VESA BIOS Extension provides several functions to return information about

the video environment.  These functions return system level information as well

as video mode specific details.  Function 00h returns general system level

information, including an OEM identification string.  The function also returns

a pointer to the supported video modes.  Function 01h may be used by the

application to obtain information about each supported video mode.  Function 03h

returns the current video mode.

VESA Super VGA Standard VS911022-5

2.2             Programming support

Due to the fact that different Super VGA products have different hardware

implementations, application software has great difficulty in adapting to each

environment.  However, since each is based on the VGA hardware architecture,

differences are most common in video mode initialization and memory mapping.

The rest of the architecture is usually kept intact, including I/O mapped

registers, video buffer location in the CPU address space, DAC location and

function, etc.

The VESA BIOS Extension provides several functions to interface to the different

Super VGA hardware implementations.  The most important of these is Function

02h, Set Super VGA video mode.  This function isolates the application from the

tedious and complicated task of setting up a video mode.  Function 05h provides

an interface to the underlying memory mapping hardware.  Function 04h enables an

application to save and restore a Super VGA state without knowing anything of

the specific implementation.

2.3             Compatibility

A primary design objective of the VESA BIOS Extension is to preserve maximum

compatibility to the standard VGA environment.  In no way should the BIOS

extensions compromise compatibility or performance.  Another but related concern

is to minimiza the changes necessary to an existing VGA BIOS.  Ram, as well as

ROM-based implementations of the BIOS extension should be possible.

2.4             Scope of standard

The purpose of the VESA BIOS Extension is to provide support for extended VGA

environments.  Thus, the underlying hardware architecture is assumed to be a

VGA.  Graphics software that drives a Super VGA board will perform its graphics

output in generally the same way it drives a standard VGA, i.e. writing directly

to a VGA style frame buffer, manipulating graphics controller registers,

directly programming the palette, etc.  No significant graphics processing will

be done in hardware.  For this reason, the VESA BIOS Extension does not provide

any graphics output functions, such as BitBlt, line or circle drawing, etc.

An important constraint of the functionalities that can be placed into the VESA

BIOS Extension is that ROM space is severely limited in certain existing BIOS

implementations.

Outside the scope of this VESA BIOS Extension is the handling of different

monitors and monitor timings.  Such items are dealt with in other VESA fora.

The purpose of the VESA BIOS Extension is to provide a standardized software

interface to Super VGA graphics modes, independent of monitor and monitor timing

issues.

VESA Super VGA Standard VS911022-6

3.      Standard VGA BIOS

~~~~~~~~~~~~~~~~~~~~~~~~~

A primary design goal with the VESA BIOS Extension is to minimize the effects on

the standard VGA BIOS.  Standard VGA BIOS functions should need to be modified

as little as possible.  This is important since ROM, as well as RAM based

versions of the extensions, may be implemented.

However, two standard VGA BIOS functions are affected by the VESA extension.

These are Function 00h (Set video mode) and Function 0Fh (Read current video

state).  VESA-aware applications will not set the video mode using VGA BIOS

function 00h.  Nor will such applications use VGA BIOS function 0Fh.  VESA BIOS

functions 02h (Set Super VGA mode) and 03h (Get Super VGA mode) will be used

instead.

However, VESA-unaware applications (such as old Pop-Up programs and other TSRs,

or the CLS command of MS-DOS), might use VGA BIOS function 0Fh to get the

present video mode.  Later it may call VGA BIOS function 00h to

restore/reinitialize the old video mode.

To make such applications work, VESA recommends that whatever value returned by

VGA BIOS function 0Fh (it is up to the OEM to define this number) should be used

to reinitialize the video mode through VGA BIOS function 00h.  Thus, the BIOS

should keep track of the last Super VGA mode in effect.

It is recommended, but not mandatory, to support output functions (such as

TTY-output, scroll, set pixel, etc.) in Super VGA modes.  If the BIOS extension

doesn't support such output functions, bit D2 (Output functions supported) of

the ModeAttributes field (returned by VESA BIOS function 01h) should be clear.

VESA Super VGA Standard VS911022-7

4.      Super VGA mode numbers

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Standard VGA mode numbers are 7 bits wide and presently range from 00h to 13h.

OEMs have defined extended video modes in the range 14h to 7Fh.  Values in the

range 80h to FFh cannot be used, since VGA BIOS function 00h (Set video mode)

interprets bit 7 as a flag to clear/not clear video memory.

Due to the limitations of 7 bit mode numbers, VESA video mode numbers are 15

bits wide.  To initialize a Super VGA mode, its number is passed in the BX

register to VESA BIOS function 02h (Set Super VGA mode).

The format of VESA mode numbers is as follows:

D0-D8  = Mode number

If D8 == 0, this is not a VESA defined mode

If D8 == 1, this is a VESA defined mode

D9-D14 = Reserved by VESA for future expansion (= 0)

D15    = Reserved (= 0)

Thus, VESA mode numbers begin at 100h.  This mode numbering scheme implements

standard VGA mode numbers as well as OEM-defined mode numbers as subsets of the

VESA mode number.  That means that regular VGA modes may be initialized through

VESA BIOS function 02h (Set Super VGA mode), simply by placing the mode number

in BL and clearing the upper byte (BH).  OEM-defined modes may be initialized in

the same way.

To date, VESA has defined a 7-bit video mode number, 6Ah, for the 800x600,

16-color, 4-plane graphics mode.  The corresponding 15-bit mode number for this

mode is 102h.

The following VESA mode numbers have been defined:

GRAPHICS                                TEXT

15-bit   7-bit    Resolution   Colors   15-bit   7-bit    Columns   Rows

mode     mode                           mode     mode

number   number                         number   number

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~   ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

100h     -        640x400      256      108h     -        80        60

101h     -        640x480      256

109h     -        132       25

102h     6Ah      800x600      16       10Ah     -        132       43

103h     -        800x600      256      10Bh     -        132       50

10Ch     -        132       60

104h     -        1024x768     16

105h     -        1024x768     256

106h     -        1280x1024    16

107h     -        1280x1024    256

VESA Super VGA Standard VS911022-8

10Dh     -        320x200      32K   (1:5:5:5)

10Eh     -        320x200      64K   (5:6:5)

10Fh     -        320x200      16.8M (8:8:8)

110h     -        640x480      32K   (1:5:5:5)

111h     -        640x480      64K   (5:6:5)

112h     -        640x480      16.8M (8:8:8)

113h     -        800x600      32K   (1:5:5:5)

114h     -        800x600      64K   (5:6:5)

115h     -        800x600      16.8M (8:8:8)

116h     -        1024x768     32K   (1:5:5:5)

117h     -        1024x768     64K   (5:6:5)

118h     -        1024x768     16.8M (8:8:8)

119h     -        1280x1024    32K   (1:5:5:5)

11Ah     -        1280x1024    64K   (5:6:5)

11Bh     -        1280x1024    16.8M (8:8:8)

VESA Super VGA Standard VS911022-9

5.      CPU Video Memory Windows

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A standard VGA sub-system provides 256k bytes of memory and a corresponding

mechanism to address this memory.  Super VGAs and their modes require more than

the standard 256k bytes of memory but also require that the address space for

this memory be restricted to the standard address space for compatibility

reasons.  CPU video memory windows provide a means of accessing this extended

VGA memory within the standard CPU address space.

This chapter describes how several hardware implementations of CPU video memory

windows operate, their impact on application software design, and relates them

to the software model presented by the VESA VGA BIOS extensions.

The VESA CPU video memory windows functions have been designed to put the

performance insensitive, non-standard hardware functions into the BIOS while

putting the performance sensitive, standard hardware functions into the

application.  This provides portability among VGA systems together with the

performance that comes from accessing the hardware directly.  In particular, the

VESA BIOS is responsible for mapping video memory into the CPU address space

while the application is responsible for performing the actual memory read and

write operations.

This combination software and hardware interface is accomplished by informing

the application of the parameters that control the hardware mechanism of mapping

the video memory into the CPU address space and then letting the application

control the mapping within those parameters.

5.1             Hardware

5.1.1           Limited to 64k/128k of CPU address space

The first consideration in implementing extended video memory is to give access

to the memory to application software.

The standard VGA CPU address space for 16 color graphics modes is typically at

segment A000h for 64k.  This gives access to the 256k bytes of a standard VGA,

i.e. 64k per plane.  Access to the extended video memory is accomplished by

mapping portions of the video memory into the standard VGA CPU address space.

Every Super VGA hardware implementation provides a mechanism for software to

specify the offset from the start of video memory which is to be mapped to the

start of the CPU address space.  Providing both read and write access to the

mapped memory provides a necessary level of hardware support for an application

to manipulate the extended video memory.

VESA Super VGA Standard VS911022-10

5.1.2           Crossing CPU video memory window boundaries

The organization of most software algorithms which perform video operations

consists of a pair of nested loops: and outer loop over rows or scan lines and

an inner loop across the row or scan line.  The latter is the proverbial inner

loop, which is the bottle neck to high performance software.

If a target rectangle is large enough, or poorly located, part of the required

memory may be with within the video memory mapped into the CPU address space and

part of it may not be addressable by the CPU without changing the mapping.  It

is desirable that the test for remapping the video memory is located outside of

the inner loop.

This is typically accomplished by selecting the mapping offset of the start of

video memory to the start of the CPU address space so that at least one entire

row or scan line can be processed without changing the video memory mapping.

There are currently no Super VGAs that allow this offset to be specified on a

byte boundary and there is a wide range among Super VGAs in the ability to

position a desired video memory location at the start of the CPU address space.

The number of bytes between the closest two bytes in video memory that can be

placed on any single CPU address is defined as the granularity of the window

function.  Some Super VGA systems allow any 4k video memory boundary to be

mapped to the start of the CPY address space, while other Super VGA systems

allow any 64k video memory boundary to be mapped to the start of the CPU address

space.  These two example systems would have granularities of 4k and 64k,

respectively.  This concept is very similar to the bytes that can be accessed

with a 16 bit pointer in an Intel CPU before a segment register must be changed

(the granularity of the segment register or mapping here is 16 bytes).

Notes

~~~~~

If the granularity is equal to the length of the CPU address space, i.e. the

least significant address bit of the hardware mapping function is more

significant than the most significant bit of the CPU address, then the inner

loop will have to contain the test for crossing the end or beginning of the CPU

address space.  This is because if the length of the CPU address space (which is

the granularity in this case) is not evenly divisible by the length of a scan

line, then the scan line at the end of the CPU address will be in two different

video memory which cannot be mapped into the CPU address space simultaneously.

5.1.3           Operating on data from different areas

It is sometimes required or convenient to move or combine data from two

different areas of video memory.  One example of this is storing menus in the

video memory beyond the displayed memory because there is hardware support in

all VGAs for transferring 32 bits of video data with an 8 bit CPU read and

write.  Two separately mappable CPU video memory windows must be used if the

distance between the source and destination is larger than the size of the CPU

video memory window.

5.1.4           Combining data from two different windows

The above example of moving data from one CPU video memory window to another CPU

video memory only required read access to one window and only required write

access to the other window.  Sometimes it is convenient to have read access to

both windows and write access to one window.  An example of this would be a

raster operation where the resulting destination is the source data logically

combined with the original destination data.

VESA Super VGA Standard VS911022-11

5.2             Different types of hardware windows

Different hardware implementations of CPU video memory windows can be supported

by the VESA BIOS extension.  The information necessary for an application to

understand the type of hardware implementation is provided by the BIOS to the

application.  There are three basic types of hardware windowing implementations

and they are described below.

The types of windowing schemes described below do not include differences in

granularity.

Also note that is possible for a VGA to use a CPU address space of 128k starting

at segment A000h.

5.2.1           Single window systems

Some hardware implementations only provide a single window.  This single window

will be readable as well as writeable.  However, this causes a significant

performance degradation when moving data in video memory a distance that is

larger than the CPU address space.

5.2.2           Dual window systems

Many Super VGAs provide two windows to facilitate moving data within video

memory.  There are two separate methods of providing two windows.

5.2.2.1         Overlapping windows

Some hardware implementations distinguish window A and window B by determining

if the CPU is attempting to do a memory read or a memory write operation.  When

the two windows are distinguished by whether the CPU is trying to read or write

they can, and usually do, share the same CPU address space.  However, one window

will be read only and the other will be write only.

5.2.2.2         Non-overlapping windows

Another mechanism used by two window systems to distinguish window A and window

B is by looking at the CPU address within the total VGA CPU address space.  When

the two windows are distinguished by the CPU address within the VGA CPU address

space the windows cannot share the same address space, but they can each be both

read and written.

VESA Super VGA Standard VS911022-12

6.      Extended VGA BIOS

~~~~~~~~~~~~~~~~~~~~~~~~~

Several new BIOS calls have been defined to support Super VGA modes.  For

maximum compatibility with the standard VGA BIOS, these calls are grouped under

one function number.  This number is passed in the AH register to the INT 10h

handler.

The designated Super VGA extended function number is 4Fh.  This function number

is presently unused in most, if not all, VGA BIOS implementations.  A standard

VGA BIOS performs no action when function call 4Fh is made.  Super VGA Standard

VS900602 defines subfunctions 00h through 07h.  Subfunction numbers 08h through

0FFh are reserved for future use.

6.1             Status Information

Every function returns status information in the AX register.  The format of the

status word is as follows:

AL == 4Fh:      Function is supported

Al != 4Fh:      Function is not supported

AH == 00h:      Function call successful

AH == 01h:      Function call failed

Software should treat a non-zero value in the AH register as a general failure

condition.  In later versions of the VESA BIOS Extension new error codes might

be defined.

6.2             Function 00h - Return Super VGA Information

The purpose of this function is to provide information to the calling program

about the general capabilities of the Super VGA environment.  The function fills

an information block structure at the address specified by the caller.  The

information block size is 256 bytes.

Input:  AH = 4Fh        Super VGA support

AL = 00h        Return Super VGA information

ES:DI = Pointer to buffer

Output: AX = Status

(All other registers are preserved)

VESA Super VGA Standard VS911022-13

The information block has the following structure:

VgaInfoBlock    STRUC

VESASignature   db   'VESA'      ; 4 signature bytes

VESAVersion     dw   ?           ; VESA version number

OEMStringPtr    dd   ?           ; Pointer to OEM string

Capabilities    db   4 dup(?)    ; capabilities of the video environment

VideoModePtr    dd   ?           ; pointer to supported Super VGA modes

TotalMemory     dw   ?           ; Number of 64kb memory blocks on board

Reserved        db   236 dup(?)  ; Remainder of VgaInfoBlock

VgaInfoBlock    ENDS

The VESASignature field contains the characters 'VESA' if this is a valid block.

The VESAVersion is a binary field which specifies what level of the VESA

standard the Super VGA BIOS conforms to.  The higher byte specifies the major

version number.  The lower byte specifies the minor version number.  The current

VESA version number is 1.2.  Applications written to use the features of a

specific version of the VESA BIOS Extension, are guaranteed to work in later

versions.  The VESA BIOS Extension will be fully upwards compatible.

The OEMStringPtr is a far pointer to a null terminated OEM-defined string.  The

string may used to identify the video chip, video board, memory configuration,

etc. to hardware specific display drivers.  There are no restrictions on the

format of the string.

The Capabilities field describes what general features are supported in the

video environment.  The bits are defined as follows:

D0      = DAC is switchable

0 = DAC is fixed width, with 6-bits per primary color

1 = DAC width is switchable

D1-31   = Reserved

The VideoModePtr points to a list of supported Super VGA (VESA-defined as well

as OEM-specific) mode numbers.  Each mode number occupies one word (16 bits).

The list of mode numbers is terminated by a -1 (0FFFFh).  Please refer to

chapter 2 for a description of VESA mode numbers.  The pointer could point into

either ROM or RAM, depending on the specific implementation.  Either the list

would be a static string stored in ROM, or the list would be generated at

run-time in the information block (see above) in RAM.  It is the application's

responsibility to verify the current availability of any mode returned by this

Function through the Return Super VGA mode information (Function 1) call.  Some

of the returned modes may not be available due to the video board's current

memory and monitor configuration.

The TotalMemory field indicates the amount of memory installed on the VGA

board.  Its value represents the number of 64kb blocks of memory currently

installed.

VESA Super VGA Standard VS911022-14

6.3             Function 01h - Return Super VGA mode information

This function returns information about a specific Super VGA video mode that was

returned by Function 0.  The function fills a mode information block structure

at the address specified by the caller.  The mode information block size is

maximum 256 bytes.

Some information provided by this function is implicitly defined by the VESA

mode number.  However, some Super VGA implementations might support other video

modes than those defined by VESA.  To provide access to these modes, this

function also returns various other information about the mode.

Input:  AH = 4Fh        Super VGA support

AL = 01h        Return Super VGA mode information

CX = Super VGA video mode

(mode number must be one of those returned by Function 0)

ES:DI = Pointer to 256 byte buffer

Output: AX = Status

(All other registers are preserved)

The mode information block has the following structure:

ModeInfoBlock   STRUC

; mandatory information

ModeAttributes      dw  ?  ; mode attributes

WinAAttributes      db  ?  ; window A attributes

WinBAttributes      db  ?  ; window B attributes

WinGranularity      dw  ?  ; window granularity

WinSize             dw  ?  ; window size

WinASegment         dw  ?  ; window A start segment

WinBSegment         dw  ?  ; window B start segment

WinFuncPtr          dd  ?  ; pointer to windor function

BytesPerScanLine    dw  ?  ; bytes per scan line

; formerly optional information (now mandatory)

XResolution         dw  ?  ; horizontal resolution

YResolution         dw  ?  ; vertical resolution

XCharSize           db  ?  ; character cell width

YCharSize           db  ?  ; character cell height

NumberOfPlanes      db  ?  ; number of memory planes

BitsPerPixel        db  ?  ; bits per pixel

NumberOfBanks       db  ?  ; number of banks

MemoryModel         db  ?  ; memory model type

BankSize            db  ?  ; bank size in kb

NumberOfImagePages  db  ?  ; number of images

Reserved            db  1  ; reserved for page function

VESA Super VGA Standard VS911022-15

; new Direct Color fields

RedMaskSize         db  ?  ; size of direct color red mask in bits

RedFieldPosition    db  ?  ; bit position of LSB of red mask

GreenMaskSize       db  ?  ; size of direct color green mask in bits

GreenFieldPosition  db  ?  ; bit position of LSB of green mask

BlueMaskSize        db  ?  ; size of direct color blue mask in bits

BlueFieldPosition   db  ?  ; bit position of LSB of blue mask

RsvdMaskSize        db  ?  ; size of direct color reserved mask in bits

DirectColorModeInfo db  ?  ; Direct Color mode attributes

Reserved            db  216 dup(?)      ; remainder of ModeInfoBlock

ModeInfoBlock   ENDS

The ModeAttributes field describes certain important characteristics of the

video mode.  Bit D0 specifies whether this mode can be initialized in the

present video configuration.  This bit can be used to block access to a video

mode if it requires a certain monitor type, and that this monitor is presently

not connected.  Prior to Version 1.2 of the VESA BIOS Extension, it was not

required that the BIOS return valid information for the fields after

BytesPerScanline.  Bit D1 was used to signify if the optional information was

present.  Version 1.2 of the VBE requires that all fields of the ModeInfoBlock

contain valid data, except for the Direct Color fields, which are valid only if

MemoryModel field is set to a 6 (Direct Color) or 7 (YUV).  Bit D1 is now

reserved, and must be set to a 1.  Bit D2 indicates whether the BIOS has support

for output functions like TTY output, scroll, pixel output, etc. in this mode

(it is recommended, but not mandatory, that the BIOS have support for all output

functions).  If bit D2 is 1 then the BIOS must support all of the standard

output functions.

The field is defined as follows:

D0 = Mode supported in hardware

0 = Mode not supported in hardware

1 = Mode supported in hardware

D1 = 1 (Reserved)

D2 = Output functions supported by BIOS

0 = Output functions not supported by BIOS

1 = Output functions supported by BIOS

D3 = Monochrome/color mode (see note below)

0 = Monochrome mode

1 = Color mode

D4 = Mode type

0 = Text mode

1 = Graphics mode

D5-D15 = Reserved

VESA Super VGA Standard VS911022-16

Note: Monochrome modes have their CRTC address at 3B4h.  Color modes have their

CRTC address at 3D4h.  Monochrome modes have attributes in which only bit 3

(video) and bit 4 (intensity) of the attribute controller output are

significant.  Therefore, monochrome text modes have attributes of off, video,

high intensity, blink, etc.  Monochrome graphics modes are two plane graphics

modes and have attributes of off, video, high intensity, and blink.  Extended

two color modes that have their CRTC address at 3D4h are color modes with one

bit per pixel and one plane.  The standard VGA modes 06h and 11h would be

classified as color modes, while the standard VGA modes 07h and 0Fh would be

classified as monochrome modes.

The BytesPerScanline field specifies how many bytes each logical scanline

consists of.  The logical scanline could be equal to or larger then the

displayed scanline.

VESA Super VGA Standard VS911022-17

The WinAAttributes and WinBAttributes describe the characteristics of the CPU

windowing scheme such as whether the windows exist and are read/writeable, as

follows:

D0 = Window supported

0 = Window is not supported

1 = Window is supported

D1 = Window readable

0 = Window is not readable

1 = Window is readable

D2 = Window writeable

0 = Window is not writeable

1 = Window is writeable

D3-D7 = Reserved

If windowing is not supported (bit D0 = 0 for both Window A and Window B), then

an application can assume that the display memory buffer resides at the standard

CPU address appropriate for the MemoryModel of the mode.

WinGranularity specifies the smallest boundary, in KB, on which the window can

be placed in the video memory.  The value of this field is undefined if Bit D0

of the appropriate WinAttributes field is not set.

WinSize specifies the size of the window in KB.

WinASegment and WinBSegment address specify the segment addresses where the

windows are located in CPU address space.

WinFuncAddr specifies the address of the CPU video memory windowing function.

The windowing function can be invoked either through VESA BIOS function 05h, or

by calling the function directly.  A direct call will provide faster access to

the hardware paging registers than using Int 10h, and is intended to be used by

high performance applications.  If this field is Null, then Function 05h must be

used to set the memory window, if paging is supported.

The XResolution and YResolution specify the width and height of the video mode.

In graphics modes, this resolution is in units of pixels.  In text modes, this

resolution is in units of characters.  Note that text mode resolutions, in units

of pixels, can be obtained by multiplying XResolution and YResolution by the

cell width and height, if the extended information is present.

The XCharCellSize and YCharSellSize specify the size of the character cell in

pixels.

The NumberOfPlanes field specifies the number of memory planes available to

software in that mode.  For standard 16-color VGA graphics, this would be set to

4.  For standard packed pixel modes, the field would be set to 1.

The BitsPerPixel field specifies the total number of bits that define the color

of one pixel.  For example, a standard VGA 4 Plane 16-color graphics mode would

have a 4 in this field and a packed pixel 256-color graphics mode would specify

8 in this field.  The number of bits per pixel per plane can normally be derived

by dividing the BitsPerPixel field by the NumberOfPlanes field.

VESA Super VGA Standard VS911022-18

The MemoryModel field specifies the general type of memory organization used in

this mode.  The following models have been defined:

00h =           Text mode

01h =           CGA graphics

02h =           Hercules graphics

03h =           4-plane planar

04h =           Packed pixel

05h =           Non-chain 4, 256 color

06h =           Direct Color

07h =           YUV

08h-0Fh =       Reserved, to be defined by VESA

10h-FFh =       To be defined by OEM

In Version 1.1 and earlier of the VESA Super VGA BIOS Extension, OEM defined

Direct Color video modes with pixel formats 1:5:5:5, 8:8:8, and 8:8:8:8 were

described as a Packed Pixel model with 16, 24, and 32 bits per pixel,

respectively.  In Version 1.2 and later of the VESA Super VGA BIOS Extension, it

is recommended that Direct Color modes use the Direct Color MemoryModel and use

the MaskSize and FieldPosition fields of the ModeInfoBlock to describe the pixel

format.  BitsPerPixel is always defined to be the total memory size of the

pixel, in bits.

The NumberOfBanks field specifies the number of banks in which the scan lines

are grouped.  The remainder from dividing the scan line number by the number of

banks is the bank that contains the scan line and the quotient is the scan line

number within the bank.  For example, CGA graphics modes have two banks and

Hercules graphics mode has four banks.  For modes that don't have scanline banks

(such as VGA modes 0Dh-13h), this field should be set to 1.

The BankSize field specifies the size of a bank (group of scan lines) in units

of 1KB.  For CGA and Hercules graphics modes this is 8, as each bank is 8192

bytes in length.  For modes that don't have scanline banks (such as VGA modes

0Dh-13h), this field should be set to 0.

The NumberOfImagePages field specifies the number of additional complete display

images that will fit into the VGA's memory, at one time, in this mode.  The

application may load more than one image into the VGA's memory if this field is

non-zero, and flip the display between them.

The Reserved field has been defined to support a future VESA BIOS extension

feature and will always be set to one in this version.

The RedMaskSize, GreenMaskSize, BlueMaskSize, and RsvdMaskSize fields define the

size, in bits, of the red, green, and blue components of a direct color pixel.

A bit mask can be constructed from the MaskSize fields using simple shift

arithmetic.  For example, the MaskSize values for a Direct Color 5:6:5 mode

would be 5, 6, 5, and 0, for the red, green, blue, and reserved fields,

respectively.  Note that in the YUV MemoryModel, the red field is used for V,

the green field is used for Y, and the blue field is used for U.  The MaskSize

fields should be set to 0 in modes using a MemoryModel that does not have pixels

with component fields.

VESA Super VGA Standard VS911022-19

The RedFieldPosition, GreenFieldPosition, BlueFieldPosition, and

RsvdFieldPosition fields define the bit position within the direct color pixel

or YUV pixel of the least significant bit of the respective color component.  A

color value can be aligned with its pixel field by shifting the value left by

the FieldPosition.  For example, the FieldPosition values for a Direct Color

5:6:5 mode would be 11, 5, 0, and 0, for the red, green, blue, and reserved

fields, respectively.  Note that in the YUV MemoryModel, the red field is used

for V, the green field is used for Y, and the blue field is used for U.  The

FieldPosition fields should be set to 0 in modes using a MemoryModel that does

not have pixels with component fields.

The DirectColorModeInfo field describes important characteristics of direct

color modes.  Bit D0 specifies whether the color ramp of the DAC is fixed or

programmable.  If the color ramp is fixed, then it can not be changed.  If the

color ramp is programmable, it is assumed that the red, green, and blue lookup

tables can be loaded using a standard VGA DAC color registers BIOS call

(AX=1012h).  Bit D1 specifies whether the bits in the Rsvd field of the direct

color pixel can be used by the application or are reserved, and thus unusable.

D0 = Color ramp is fixed/programmable

0 = Color ramp is fixed

1 = Color ramp is programmable

D1 = Bits in Rsvd field are usable/reserved

0 = Bits in Rsvd field are reserved

1 = Bits in Rsvd field are usable by the application

Notes

~~~~~

Version 1.1 and later VESA BIOS extensions will zero out all unused fields in

the Mode Information Block, always returning exactly 256 bytes.  This

facilitates upward compatibility with future versions of the standard, as any

newly added fields will be designed such that values of zero will indicate

nominal defaults or non-implementation of optional features (for example, a

field containing a bit-mask of extended capabilities would reflect the absence

of all such capabilities).  Applications that wish to be backwards compatible to

Version 1.0 VESA BIOS extensions should pre-initialize the 256 byte buffer

before calling Return Super VGA mode information.

VESA Super VGA Standard VS911022-20

6.4             Function 02h - Set Super VGA video mode

This function initializes a video mode.  The BX register contains the mode to

set.  The format of VESA mode numbers is described in chapter 2.  If the mode

cannot be set, the BIOS should leave the video environment unchanged and return

a failure error code.

Input:  AH = 4Fh        Super VGA support

AL = 02h        Set Super VGA video mode

BX = Video mode

D0-D14 = Video mode

D15 = Clear memory flag

0 = Clear video memory

1 = Don't clear video memory

Output: AX = Status

(All other registers are preserved)

6.5             Function 03h - Return current video mode

This function returns the current video mode in BX.  The format of VESA video

mode numbers is described in chapter 2 of this document.

Input:  AH = 4Fh        Super VGA support

AL = 03h        Return current video mode

Output: AX = Status

BX = Current video mode

(All other registers are preserved)

Notes

~~~~~

In a standard VGA BIOS, function 0Fh (Read current video state) returns the

current video mode in the AL register.  In D7 of AL, it also returns the status

of the memory clear bit (D7 of 40:87).  This bit is set if the mode was set

without clearing memory.  In this Super VGA function, the memory clear bit will

not be returned in BX since the purpose of the function is to return the video

mode only.  If an application wants to obtain the memory clear bit, it should

call VGA BIOS function 0Fh.

VESA Super VGA Standard VS911022-21

6.6             Function 04h - Save/Restore Super VGA video state

These functions provide a mechanism to save and restore the Super VGA video

state.  The functions are a superset of the three subfunctions under standard

VGA BIOS function 1Ch (Save/restore video state).  The complete Super VGA video

state (except video memory) should be saveable/restoreable by setting the

requested states mask (in the CX register) to 000Fh.

Input:  AH = 4Fh        Super VGA support

AL = 04h        Save/restore Super VGA video state

DL = 00h        Return save/restore state buffer size

CX = Requested states

D0 = Save/restore video hardware state

D1 = Save/restore video BIOS data state

D2 = Save/restore video DAC state

D3 = Save/restore Super VGA state

Output: AX = Status

BX = Number of 64-byte blocks to hold the state buffer

(All other registers are preserved)

Input:  AH = 4Fh        Super VGA support

AL = 04h        Save/restore Super VGA video state

DL = 01h        Save Super VGA video state

CX = Requested states (see above)

ES:BX = Pointer to buffer

Output: AX = Status

(All other registers are preserved)

Input:  AH = 4Fh        Super VGA support

AL = 04h        Save/restore Super VGA video state

DL = 02h        Restore Super VGA video state

CX = Requested states (see above)

ES:BX = Pointer to buffer

Output: AX = Status

(All other registers are preserved)

Notes

~~~~~

Due to the goal of complete compatibility with the VGA environment, the standard

VGA BIOS function 1Ch (Save/restore VGA state) has not been extended to save the

Super VGA video state.  VGA BIOS compatibility requires that function 1Ch

returns a specific buffer size with specific contents, in which there is no room

for the Super VGA state.

VESA Super VGA Standard VS911022-22

6.7             Function 05h - CPU Video Memory Window Control

This function sets or gets the position of the specified window in the video

memory.  The function allows direct access to the hardware paging registers.  To

use this function properly, the software should use VESA BIOS Function 01h

(Return Super VGA mode information) to determine the size, location, and

granularity of the windows.

Input:  AH = 4Fh        Super VGA support

AL = 05h        Super VGA video memory window control

BH = 00h        Select Super VGA video memory window

BL = Window number

0 = Window A

1 = Window B

DX = Window position in video memory

(in window granularity units)

Output: AX = Status

(See notes below)

Input:  AH = 4Fh        Super VGA support

AL = 05h        Super VGA video memory window control

BH = 01h        Return Super VGA video memory window

BL = Window number

0 = Window A

1 = Window B

Output: AX = Status

DX = Window position in video memory

(in window granularity units)

(See notes below)

Notes

~~~~~

This function is also directly accessible through a far call from the

application.  The address of the BIOS function may be obtained by using VESA

BIOS Function 01h, Return Super VGA mode information.  A field in the

ModeInfoBlock contains the address of this function.  Note that this function

may be different among video modes in a particular BIOS implementation, so the

function pointer should be obtained after each set mode.

In the far call version, no status information is returned to the application.

Also, the AX and DX registers will be destroyed.  Therefore, if AX and/or DX

must be preserved, the application must do so priot to making the far call.

The application must load the input arguments in BH, BL, and DX (for set window)

but does not need to load either AH or AL in order to use the far call version

of this function.

VESA Super VGA Standard VS911022-23

6.8             Function 06h - Set/Get Logical Scan Line Length

This function sets or gets the length of a logical scan line.  This function

allows an application to set up a logical video memory buffer that is wider than

the displayed area.  Function 07h then allows the application to set the

starting position that is to be displayed.

Input:  AH = 4Fh        Super VGA support

AL = 06h        Logical Scan Line Length

BL = 00h        Select Scan Line Length

CX = Desired width in pixels

Output: AX = Status

BX = Bytes Per Scan Line

CX = Actual Pixels Per Scan Line

DX = Maximum Number of Scan Lines

Input:  AH = 4Fh        Super VGA support

AL = 06h        Logical Scan Line Length

BL = 01h        Return Scan Line Length

Output: AX = Status

BX = Bytes Per Scan Line

CX = Actual Pixels Per Scan Line

DX = Maximum Number of Scan Lines

Notes

~~~~~

The desired width in pixels may not be achieveable because of VGA hardware

considerations.  The next larger value will be selected thta will accommodate

the desired number of pixels, and the actual number of pixels will be returned

in CX.  BX returns a value that, when added to a pointer into video memory, will

point to the next scan line.  For example, in a mode 13h this would be 320, but

in mode 12h this would be 80.  DX returns the number of logical scan lines based

upon the new scan line length and the total memory installed and useable in this

display mode.  This function is also valid in text modes.  In text modes, the

application should find out the current character cell width through normal BIOS

functions, multiply that times the desired number of characters per line, and

pass the value in the CX register.

VESA Super VGA Standard VS911022-24

6.9             Function 07h - Set/Get Display Start

This function selects the pixel to be displayed in the upper left corner of the

display from the logical page.  This function can be used to pan and scroll

around logical screens that are larger than the displayed screen.  This function

can also be used to rapidly switch between two different displayed screens for

double buffered animation effects.

Input:  AH = 4Fh        Super VGA support

AL = 07h        Display Start Control

BH = 00h        Reserved and must be 0

BL = 00h        Select Display Start

CX = First Displayed Pixel in Scan Line

DX = First Displayed Scan Line

Output: AX = Status

Input:  AH = 4Fh        Super VGA support

AL = 07h        Display Start Control

BL = 01h        Return Display Start

Output: AX = Status

BH = 00h Reserved and will be 0

CX = First Displayed Pixel in Scan Line

DX = First Displayed Scan Line

Notes

~~~~~

This function is also valid in text modes.  In text modes, the application

should find out the current character cell width through normal BIOS functions,

multiply that times the desired starting character column, and pass that value

in the CX register.  It should also multiply the current character cell height

times the desired starting character row, and pass that value in the DX

register.

VESA Super VGA Standard VS911022-25

6.10            Function 08h - Set/Get DAC Palette Control

This function queries and selects the operating mode of the DAC palette.  Some

DACs are configurable to provide 6-bits, 8-bits, or more of color definition per

red, green, and blue primary color.  The DAC palette width is assumed to be

reset to standard VGA 6-bits per primary during a standard or VESA Set Super VGA

Mode (AX = 4F02h) call.

Input:  AH = 4Fh        Super VGA support

AL = 08h        Set/Get DAC Palette Control

BL = 00h        Set DAC palette width

BH = Desired number of bits of color per primary

(Standard VGA = 6)

Output: AX = Status

BH = Current number of bits of color per primary

(Standard VGA = 6)

Input:  AH = 4Fh        Super VGA support

AL = 08h        Set/Get DAC Palette Control

BL = 01h        Get DAC palette width

Output: AX = Status

BH = Current number of bits of color per primary

(Standard VGA = 6)

Notes

~~~~~

An application can find out if DAC switching is available by querying Bit D0 of

the Capabilities field of the VgaInfoBlock structure returned by VESA Return

Super VGA Information (AX = 4F00h).  The application can then attempt to set the

DAC palette width to the desired value.  If the Super VGA is not capable of

selecting the requested palette width, then the next lower value that the Super

VGA is capable of will be selected.  The resulting palette width is returned.

VESA Super VGA Standard VS911022-26

7.      Application Example

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following sequence illustrates how an application interface to the VESA BIOS

Extension.  The hypothetical application is VESA-aware and calls the VESA BIOS

functions.  However, the application is not limited to supporting just

VESA-defined video modes.  This it will inquire what video modes are available

before setting up the video mode.

1)      The application would first allocate a 256 byte buffer.  This buffer

will be used by the VESA BIOS to return information about the video

environment.  Some applications will statically allocate this buffer,

others will use system calls to temporarily obtain buffer space.

2)      The application would then call VESA BIOS function 00h (Return Super VGA

information).  If the AX register does not contain 004Fh on return from

the function call, the application can determine that the VESA BIOS

Extension is not present and handle such situation.

If no error code is passed in AX, the function call was successful.  The

buffer has been filled by the VESA BIOS Extension with various

information.  The application can verify that indeed this is a valid

VESA block by identifying the characters 'VESA' in the beginning of the

block.  The application can inspect the VESAVersion field to determine

whether the VESA BIOS Extension ha sufficient functionality.  The

application may use the OEMStringPtr to locate OEM-specific information.

Finally, the application can obtain a list of the supported Super VGA

modes by using the VideoModePtr.  This field points to a list of the

video modes supported by the video environment.

3)      The application would then create a new buffer and call the VESA BIOS

function 01h (Return Super VGA mode information) to obtain information

about the supported video modes.  Using the VideoModePtr obtained in

step 2) above, the application would call this function with a new mode

number until a suitable video mode is found.  If no appropriate video

mode is found, it is up to the application to handle this situation.

The Return Super VGA mode information function fills a buffer specified

by the application with information describing the features of the video

mode.  The data block contains all the information an application needs

to take advantage of the video mode.

The application would examine the ModeAttributes field.  To verify that

the mode indeed is supported, the application would inspect bit D0.  If

D0 is clear, then the mode is not supported by the hardware.  This might

happen is a specific mode requires a certain type of monitor but that

monitor is not present.

4)      After the application has selected a video mode, the next step is to

initialize the mode.  However, the application might first want to save

the present video mode.  When the application exits, this mode would be

restored.  To obtain the present video mode, the VESA BIOS function 03h

(Get Super VGA mode) would be used.  If a non-VESA (standard VGA or

OEM-specific) mode is in effect, only the lower byte in the mode number

is filled.  The upper byte is cleared.

5)      To initialize the video mode, the application would use VESA BIOS

function 02h (Set Super VGA mode).  The application has from this point

on full access to the VGA hardware and video memory.

VESA Super VGA Standard VS911022-27

6)      When the application is about to terminate, it would restore the prior

video mode.  The prior video mode, obtained in step 4) above could be

either a standard VGA mode, OEM-specific mode, or VESA-supported mode.

It would reinitialize the video mode by calling VESA BIOS function 02h

(Set Super VGA mode).  The application would then exit.