August 12, 1993

Installation instructions for the packet driver collection

(Currently only for PARnet & SLIP)

Document conventions

All numbers in this document are given in C-style

representation.  Decimal is expressed as 11, hexadecimal is

expressed as 0x0B, octal is expressed as 013.  All reference to

network hardware addresses (source, destination and multicast)

and demultiplexing information for the packet headers assumes

they are represented as they would be in a MAC-level packet

header being passed to the send_pkt() function.

Using the packet drivers

The packet driver must be installed prior to use.  Since

each packet driver takes only a few thousand bytes, this is best

done in your autoexec.bat.  Since the Ethernet boards typically

have jumpers on board, the packet driver must be informed of the

values of these jumpers (auto-configure is possible, but can

disturb other boards).  The first parameter is the software

interrupt used to communicate with the packet driver.  And

again, because each board is different, the rest of the

parameters will be different.

All parameters must be specified in C-style representation.

Decimal is expressed as 11, hexadecimal is expressed as 0x0B,

octal is expressed as 013.  Any numbers that the packet driver

prints will be in the same notation.

Before installing the packet driver, you must choose a software

interrupt number in the range between 0x60 and 0x80.  Some of

these interrupts are used for other purposes, so your first

choice may not work.  See Appendix A for the section of Ralf

Brown's interrupt list between 0x60 and 0x80.

Running a packet driver with no specifications will give a

usage message.  The parameters for each packet driver are

documented below.

Most drivers can also be used in a PROM boot environment, see

PROMBOOT.NOT for how to use -d and -n options for that purpose.

The -w switch is used for Windows.  Install the packet driver

before running MS-Windows.  This switch does not prevent Windows

from swapping your network application out of memory, it simply

detects when that has happened, and drops the packets on the

floor.

PARNET

usage: PARNET [-n] [-d] [-w] <packet_int_no> <int_level>

<io_addr> <netaddr>

The PARNET driver is an Ethernet adapter simulation for

PARnet-PC. PARNET-PC is a direct port from the Amiga's PARnet

PARnet which enables a multi-host network on one parallel line.

However, since not all parallel (printer) cards don't support

bi-directional pulled-up centronic cable, you must have a PARnet

INTERFACE. A description of this interface is given in the

PARPCXX.ZIP file in which XX represents the version number.

The parameters are as follows. The INT_LEVEL is the hardware IRQ

on which your parallel card is connected. In 99% of the cases

this is IRQ7 (enter: 7). The IO_ADDR is the portaddress of the

card, in standard configurations this is 0x378 (LPT1) or 0x278

(LPT2). The NETADDR is the PARnet address of this interface, this

is an unique number for each interface. You can choose any number

inbetween 0x01 and 0xfe (1 to 254 decimal). NOTE: When sending IP

packets, the driver uses the last octet in the IP addresses to

reach other hosts. So when you already have a fixed IP address for

your machine, let NETADDR be the last number. (e.g. when you have

the address 193.79.1.2, the NETADDR MUST be 2! ).

^                      ^

SLIP8250

usage: SLIP8250 [-n] [-d] [-w] packet_int_no [-h]

[driver_class] [int_no]

[io_addr] [baud_rate] [recv_buf_size]

The driver_class should be SLIP, KISS, AX.25, or a

number.

The SLIP8250 driver is not strictly an Ethernet adapter,

however some software packages (such as KA9Q's NET and NCSA

Telnet) support Serial Line IP (SLIP).  SLIP must be specially

supported because it doesn't use ARP and has no hardware

addresses prepended to its packets.  The PDS is not clear on

this, but the packet driver does the SLIP encoding.

The parameters are as follows.  The -h flag is included if you

wish to use hardware handshaking (the packet driver will then

suspend the transmission of characters while CTS is low).  The

driver_class is the class that is returned to a client of the

packet driver spec in the driver_info call.  The int_no is the

hardware interrupt number, defaults to 4 (COM1).  The io_addr

is the hardware I/O address, defaults to 0x3f8 (COM1).  The

baud_rate defaults to 4800 baud.  The recv_buf_size defaults to

3000.

Errorlevels

Some of the packet drivers return error codes.  Some of

these error codes indicate fatal errors, and some are merely

warnings.  For the moment, you must consult the source to see

what the errorcodes mean.  For example, pktchk returns 0 if a

packet driver exists at a given address, and 1 if not.  You

might use it in a batch file that only installs a packet driver

if one is not found.

rem only install the packet driver if there isn't one

rem already.

pktchk 0x7e

if errorlevel 0 goto gotit

ni5210 0x7e

:gotit

The "errorlevel" test is true if the errorlevel is less

than or equal to the parameter.

Appendix A

Interrupt usage in the range 0x60 through 0x80, from Ralf

Brown's interrupt list.

60 -- -- reserved for user interrupt

60 -- -- FTP Driver - PC/TCP Packet Driver Specification

60 01 FF FTP Driver - DRIVER INFO

60 02 -- FTP Driver - ACCESS TYPE

60 03 -- FTP Driver - RELEASE TYPE

60 04 -- FTP Driver - SEND PACKET

60 05 -- FTP Driver - TERMINATE DRIVER FOR HANDLE

60 06 -- FTP Driver - GET ADDRESS

60 07 -- FTP Driver - RESET INTERFACE

60 11 -- 10-NET - LOCK AND WAIT

60 12 -- 10-NET - LOCK

60 13 -- 10-NET - UNLOCK

60 20 -- FTP Driver - SET RECEIVE MODE

60 21 -- FTP Driver - GET RECEIVE MODE

60 24 -- FTP Driver - GET STATISTICS

61 -- -- reserved for user interrupt

62 -- -- reserved for user interrupt

63 -- -- reserved for user interrupt

64 -- -- reserved for user interrupt

65 -- -- reserved for user interrupt

66 -- -- reserved for user interrupt

67 -- -- LIM EMS

...

67 DE 00 Virtual Control Program Interface - INSTALLATION CHECK

...

68 01 -- APPC/PC

...

69 -- -- unused

6A -- -- unused

6B -- -- unused

6C -- -- system resume vector (CONVERTIBLE)

6C -- -- DOS 3.2 Realtime Clock update

6D -- -- VGA - internal

6E -- -- unused

6F -- -- Novell NetWare - PCOX API (3270 PC terminal interface)

6F 00 -- 10-NET - LOGIN

...

70 -- -- IRQ8 - AT/XT286/PS50+ - REAL-TIME CLOCK

71 -- -- IRQ9 - AT/XT286/PS50+ - LAN ADAPTER 1

72 -- -- IRQ10 - AT/XT286/PS50+ - RESERVED

73 -- -- IRQ11 - AT/XT286/PS50+ - RESERVED

74 -- -- IRQ12 - PS50+ - MOUSE INTERRUPT

75 -- -- IRQ13 - AT/XT286/PS50+ - 80287 ERROR

76 -- -- IRQ14 - AT/XT286/PS50+ - FIXED DISK

77 -- -- IRQ15 - AT/XT286/PS50+ - RESERVED

78 -- -- not used

79 -- -- not used

7A -- -- Novell NetWare - LOW-LEVEL API

7A -- -- AutoCAD Device Interface

7B -- -- not used

7C -- -- not used

7D -- -- not used

7E -- -- not used

7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE

7F -- -- HLLAPI (High-Level Language API)

80 -- -- reserved for BASIC