Sound Blaster AUDIODD Device Drivers

Version 2.0.1

CONTENT

=======

This release contains the following Sound Blaster device drivers:

SBDD     SYS     30266   9-17-92  3:37p

SBPDD    SYS     31456   9-17-92  2:33p

SBP2DD   SYS     33864   9-17-92  3:43p

SBPWD    SYS     23102   9-17-92  3:40p

SBWD     SYS     21946   9-17-92  3:13p

PURPOSE

=======

They are individually used for:

SBDD.SYS    Sound Blaster Wave+MIDI Dos Device Driver

SBPDD.SYS   SB PRO Wave+MIDI Dos Device Driver

SBP2DD.SYS  SB PRO OPL/3 Wave+MIDI Dos Device Driver

SBWD.SYS    Sound Blaster Wave-only (aka K12) Dos Device Driver

SBPWD.SYS   SB PRO Wave-only (aka K12) Dos Device Driver (also for PRO OPL/3)

AUDIODD

=======

AUDIODD device drivers are used by amongst others CD-ROM applications like

almanac, CD-ROM encyclopedia, and other IBM EDUQUEST supported products.

INSTALLING AUDIODD DEVICE DRIVERS

=================================

To use the driver, copy SBXXX.SYS (SBXXX.SYS being one of the drivers listed

above) into the C:\ root directory, and ensure the following line exist in

the CONFIG.SYS

DEVICE=C:\SBXXX.SYS audio [dma] int ioaddr bufsize

where

audio =   AUDIO device number (usually 1)

dma =     DMA channel used by the SB card (using 1) (not applicable to

non-SB PRO drivers)

int =     interrupt level used by the SB card (usually 5 or 7)

ioaddr =  i/o address used by the SB card (usually 220)

bufsize = i/o buffer size in kilobytes (usually 4)

and REBOOT the computer.

Hence,

DEVICE=C:\SBP2D2.SYS 1 1 7 220 4

This assumes that the Sound Blaster Pro OPL/3 card is set at DMA=1, IRQ=7,

I/O=220, buffer size of 4k, and addressed as device AUDIO1$.

Or,

DEVICE=C:\SBD2.SYS 1 5 220 4

This assumes that the Sound Blaster card is set at IRQ=5, I/O=220, buffer size

of 4k, and addressed as device AUDIO1$.  Notice that the DMA channel parameter

is missing.  This is because DMA channel is fixed at 1 for non-SB PRO cards.

########