CONTROL-SIM

Version 2.0

Users Manual

Copywrite 1992, Roger W. Thompson, All rights reserved

RTW Software

P.O. Box 710292

Houston, TX 77271

CONTROL-SIM Users Manual                Page 2

REGISTRATION

Please feel free to evaluate CONTROL-SIM and distribute copies of

CONTROL-SIM as long as all files are unaltered and distributed

together.  If you continue to use CONTROL-SIM you are obligated

to register this program by sending a U. S. check or money order

for $35 ($32 + $3 handling) to:

Roger W. Thompson

RTW Software

P.O. Box 710292

Houston, TX 77271

Texas residents should include  8.25% sales tax.

Upon registration, you will receive the latest version of

CONTROL-SIM, without the shareware screen.  In addition,

QuickBASIC source files for the main program and controller

subroutines, plus library files, will be included to allow

controller development.  You will also be notified of future

updates.  The registered version of CONTROL-SIM will be licensed

to you for single computer system use and it may not be

distributed in any manner.

Please send any comments or suggestions you may have with regard

to CONTROL-SIM. If feasible, they may be incorporated in future

updates.

DISCLAIMER

Although CONTROL-SIM has been tested extensively and is believed

to produce accurate results if used correctly, no guarantee is

provided.  RTW Software disclaims all warranties relating to this

software, whether express or implied, as to the merchantability

or fitness of this program for any purpose.  RTW Software also

assumes no liability for any damages, either direct or

inconsequential, which may result from use of this program.

CONTROL-SIM Users Manual                Page 3

1.0  INTRODUCTION

This program simulates operation of a simple plant control loop.  It

is intended to be used by engineers and students involved in

practical digital control system design and application.  Significant

uses include:

Tuning a controller

Specification (modeling) of a plant response

Evaluation of PID and other types of controllers

Simulation of a typical bounded controller output and input

Evaluation of effects of quantization and sample rate

Time response of LaPlace transforms

Conversion from LaPlace transforms to state equations

Development of control algorithms (registered users only)

The program is designed to be easy to use allowing the user to

concentrate on control system performance characteristics rather than

program commands and variables or extensive math.  Full featured

menu's and an initial (default) example permit the beginner to

experiment quickly, while the more advanced user can create,

evaluate, and save plant configuration files for future reference.

a.  CONTROL-SIM System Requirements:

* IBM PC, XT, AT or compatible

* EGA OR VGA color display

* IBM/Epson compatible printer

* MS-DOS 2.1 or higher

b.  Installation

For backup, make a working copy of the original distribution disc.

CONTROL-SIM can be run from either a floppy disc or from the hard

disc.  If hard disc operation is desired, create a new separate

directory and copy all CONTROL-SIM files to the new directory.

For example, at C:\ prompt enter:

MKDIR ctrl

COPY A:\*.* C:\ctrl

The original disc and the new directory should contain the following

files:

README       Initial instruct     PROCESS.PLT   Sample config file

CS.EXE       Program              TEMP.PLT      Sample config file

MANUAL.DOC   Documentation        SERVO.PLT     Sample config file

DX1.PRX      Utility file         EXAMPL1.PLT   Sample config file

CTRLBLK.EGX  Picture file         EXAMPL2.PLT   Sample config file

DAT1.PRN     Sample data file     COMPCKT.PLT   Sample config file

FZVALVE.PLT   Sample config file

FZSERVO.PLT   Sample config file

PIPDEL1.PLT   Sample config file

PIPDEL2.PLT   Sample config file

CONTROL-SIM Users Manual                Page 4

2.0  BASIC OPERATION

Begin CONTROL-SIM operation by entering at C:\ prompt:

CD \CTRL

CS

After introductory screens are displayed, the Main Menu will appear

with the following selections available:

1. DISPLAY DATA

2. DISPLAY BLOCK DIAGRAM DATA

3. PLOT DATA

4. MODIFY PLANT MODEL

5. MODIFY CONTROLLER

6. PLAYBACK DATA

7. CONFIGURATION FILES

8. EXIT

The first 3 selections will display time response data as follows:

DISPLAY DATA                - Meter and Bar Graph

DISPLAY BLOCK DIAGRAM DATA  - Meter

PLOT DATA                   - Chart Recorder traces

In all three, the user can enter setpoints, activate a load change,

change loop mode (open or closed), print or record data, and observe

elapsed time from setpoint initiation.  The DISPLAY DATA/DISPLAY

BLOCK DIAGRAM DATA selections also provide an IAE (Integrated

Absolute Error) indication to aid in tuning.  The PLOT display allows

user selection of up to 4 chart speeds depending on sample rate.

The MODIFY PLANT MODEL selection is concerned with plant

configuration, consisting of the plant model (LaPlace transform),

delay time (or system lag time), sample rate, and equation display

status.

The MODIFY CONTROLLER selection is concerned with controller

selection, control constants, quantization level, and range of

controller output.

The PLAYBACK DATA selection permits the user to view data text files

recorded previously.

CONFIGURATION FILES provides the means to save and retrieve

plant/controller configuration files and to designate an alternate

directory.

CONTROL-SIM Users Manual                Page 5

3.0 SYSTEM DESCRIPTION (Reference DISPLAY BLOCK DIAGRAM DATA)

The simulated system includes the CONTROLLER and the PLANT.  The

CONTROLLER includes data acquisition and control output functions,

controller function, loop mode function, and display of setpoint,

measurement, and control values.  The PLANT consists of the plant

transfer function, transport delay, and load change function.

The CONTROLLER simulates a typical hardware digital control system

where conditioned analog data (voltage) from a PLANT sensor is

sampled and converted to a digital value.  The resolution of the

digital value, or measurement, depends on the quantization level of

the analog-to-digital converter.  Typically, 12 bits is used,

resulting in 4096 resolvable steps.  The measured value is then

compared with the user input setpoint value within the controller

function.  The difference between setpoint value and measured value,

or error, is then used as input to the controller algorithm.  The

controller algorithm produces an output signal which is then

converted from digital to analog and sent to the PLANT actuator as a

voltage signal.  Again, the resolution of the digital-to-analog

converter is dependant on the quantization level, typically 12 bits.

If the output is allowed to swing both positive and negative about

zero, it is said to be bipolar and the resolution applies to the

complete range.  For example, 12 bits, or 4096 steps, would actually

be 2048 steps above zero and 2048 below zero.

The PLANT is simulated by a transfer function and a delay.  The

transfer function is assumed to represent all the elements of the

plant loop from actuator input to sensor output.  The transport delay

is the total deadtime of the plant loop which is the sum of the time

delays attributable to each element, including transport time between

elements.  The transfer function can be as simple as a single time

constant or as complex as a multiple of time constants up to 4th

order.  Typical plant hardware elements are actuators, control

valves, servo valves, electrical power controls, motors, heaters,

sensors, and signal conditioners.

The user can specify the plant transfer function as a LaPlace

transform.  As an example, the LaPlace transform of a simple

integration function is:

1           .5

G(s)  =  --------  =  --------

(2s + 1)     (s + .5)

where  .5 = the reciprocal of the time constant, 2 seconds

this is equivalent to the following RC network:

R = 2 megohm

o-----^\/^\/----------------o

|

___

INPUT                          ___ C = 1 microfarad         OUTPUT

|

|

o----------------------------o

CONTROL-SIM Users Manual                Page 6

CONTROL-SIM converts the LaPlace transform into state equations (also

called state space or state variables) for ease of computation.  A

further conversion is necessary from the continuous form of state

equations to discrete form state equations, to account for the

effects of sampled data.  The resulting difference equations, which

are updated each scan cycle, are simple multiply and add/subtract

operations which the computer performs handily.

4.0 DETAILED OPERATION

4.1 DISPLAY DATA Selection

Displays

Process - the 3 basic control system variables are displayed in

both bar graph and digital form.  The (S)etpoint value, or

reference value, is the desired value set by the user.  The

(C)ontrol value (also called manipulated variable) is the value

output by the Controller.  The (M)easure value (also called

controlled variable) is the Plant output value.  This is the

variable which the control system is attempting to control.

Elapsed Time - is displayed in seconds.  The time is reset to 0

when a setpoint or load change is set, to reflect the response

time of the plant.

Mode indicator - shows the loop status (see Mode change below).

Integrated Absolute Error (IAE) - is displayed to give a measure of

relative performance of the same upset conditions (setpoint or

load change) with different tuning constants or other

controller/plant settings.  It is reset to 0 when a setpoint or

load change is activated.  IAE is a measurement of the area

existing between the setpoint value and the measured value

response.

User Controls

Setpoints - Enter setpoint value (0 - 100) and depress the S key to

activate the new setpoint.  All system inputs and outputs are

represented as 0 to 100% full scale.  An audible beep is sounded

to signify acceptance of the new value.

In event an incorrect number is entered, the X key can be used to

erase the entry, prior to activating the setpoint.  If a

numerical key is held depressed, the new value will become 100.

AUTOSET is a feature that, when activated by the A key, causes

the last 2 setpoints entered to be reset.  Each time the S key is

depressed, the setpoints alternate.  This simplifies repeated

execution as when evaluating the effects of tuning changes.

Load transient - the L key causes the Control value (input to Plant

transfer function) to change by 50% full scale for 1 sample.  The

load change alternates polarity each time L is depressed.  This

allows evaluation of the control response to a plant upset.

CONTROL-SIM Users Manual                Page 7

Often times a system tuned for best response to setpoint change

will not fare as well during a load transient as a system tuned

for best load response.  The converse is also true.

Mode change - the M key changes the loop mode from Closed Loop to

Open Loop or vice versa.  The default Closed Loop mode causes the

measurement of plant output to feedback to the Controller

algorithm, thereby allowing corrections to made in the next

Controller output.  The Open Loop mode bypasses the Controller

algorithm and connects the setpoint value directly to the

Controller output.  Open Loop allows the Plant response to be

evaluated without any controlling influence.  For example, if the

simple integration transfer function illustrated previously is

implemented in Open Loop, the resulting time response will be

indicated as 63.2% (approx. 1 - 1/e) of the setpoint change at 2

seconds.

Print/Record - the P key is used to start or stop data recording.

Initially, when P is depressed, the user is requested to select

File or Printer.

If File is selected, the user is then requested to enter the file

name (without extension), after which all data is logged to the

disc file.  The extension .PRN is automatically appended.  If the

file already exists, the user is given opportunity to change the

name or reuse the same name.  Files are limited to approx.  900

records, or samples, to conserve disc space and to aid in data

review.  If longer runs are desired, reselect and enter another

file name (e.g. DAT1, DAT2, etc).

If Printer is selected, data printing begins immediately.  In

both cases, a header is recorded followed by elapsed time,

setpoint, control, and measure values for each sample point.

Logging continues until the P key is once again depressed or the

T key is used to terminate the display.

Terminate - the T key is used to terminate the display (or plot)

operations and return to the initial selection menu.  All

variables are retained and if one of the display operations is

again selected, all values will resume from the point of

termination.

4.2 DISPLAY BLOCK DIAGRAM DATA Selection

Same as DISPLAY DATA except that data is displayed in meter form

only and related to the CONTROL-SIM Block Diagram, as test points.

See 3.0 System Description for block diagram discussion.

4.3 PLOT DATA Selection

Chart Display

Data is presented in a strip chart type display, except that the

"pens" move and the "paper" remains still.  When the right edge

of the screen is reached, the "pens", or vertical bar,  continue

from the left edge, erasing old traces and writing new traces as

CONTROL-SIM Users Manual                Page 8

it proceeds.  Grid markers are also written to provide

appropriate time scaling.

User Controls

User controls are the same as DISPLAY DATA with the addition of:

Chart Speed - can be changed to better depict the area of interest

for a given sample rate.  Speed is increased by depressing the

"+" key and decreased with the "-" key. Four possible settings

are available.  For each setting, the elapsed time between grid

marks (vertical dots) is displayed in seconds.  Depending on

sample time selected, some speed positions may not be effective,

resulting in grid marks too dense, or too spread out, to be

useful.  If the Space bar is depressed, all updates are suspended

until the Space bar is pressed again.

4.4 MODIFY PLANT MODEL selection

After selection, a window is opened containing a display of the

current Plant Transfer Function in factored form.  In addition to

the transfer function, the current transport delay (deadtime) is

indicated as Td and the current sample period is indicated as Ts.

The user is asked "Is the plant correctly defined?" and a "yes"

response results in computation of state equations and return to

the main menu.

If "no", or any key other than "y", is entered, the PLANT MODEL

MODIFICATION menu is presented.

Modify Transfer Function

When selected, the format of the transfer function is displayed

as follows:

b0(s + b1)(s + b2)(s + b3)

Gp(s) =  ---------------------------------

1(s + a1)(s + a2)(s + a3)(s + a4)

and the user is asked to enter the nominator term, b0.  Term b0

must be a non-zero constant.  After each term is entered, the

next in sequence is requested, proceeding through the nominator

terms, b0 to b3, and then the denominator terms, a1 to a4.  The

b(i) terms are the "zeros" and the a(i) terms are the "poles" of

the plant transfer function, which can be fourth order.

Entering "0" for a term results is the operator "s" only, as in

the case of a "pole" at zero value.  The operator "s" is

equivalent to jw (j omega = j2(pi)f).  Depressing the Enter key

without a constant, terminates entry of the b(i) terms, jumping

to the a(i) terms.  Depressing Enter again without constants

terminates all entry and the previous display of the actual

transfer function is presented again for review.  The program

checks for a(i) values that match b(i) values and if present,

both terms are negated, since they cancel each other.

CONTROL-SIM Users Manual                Page 9

At this point, if the user is satisfied with the result, he can

answer "yes" to compute state equations, or he may enter "no" to

re-enter the PLANT MODEL MODIFICATION menu.

Modify Delay Time

When selected, a value for delay time in seconds can be entered.

Note, that the program only allows delay time that is a multiple

of sample time.  In the event a delay less than sample time is

entered, the resulting delay will be 0.  When satisfied with

value, depress "E" to exit to previous menu.

Modify Sample Time

When selected, a value for sample period in seconds can be

entered.  The sample period should be less than 1/2 the plant

time constant to insure data integrity and maintain control.

Note, that although sample time is really a Controller parameter,

it is included here because it affects the discrete state

equation computation. When satisfied with value, depress "E" to

exit to previous menu.

Display Equations

This selection allows the user to display the resulting plant

continuous and discrete state equations, in matrix form.  In both

cases, state variables, input variables, and output variables are

shown.  Depress "C" to change the display status (ON/OFF) and "E"

to exit.

Exit

Exit from PLANT MODEL MODIFICATION returns to the current

transfer function display and subsequent computation of state

equations, if the user is satisfied with its definition.

4.5 MODIFY CONTROLLER

The menu of changes to the Controller includes the following

selections:

Select Controller

Three controller choices are currently available: PID, FUZZY, &

PIP.

(a) The PID (Proportional-Integral-Derivative) Controller is the

default controller.  It is a general purpose, widely used

technique that allows the user to adjust control system

performance to accommodate various plant characteristics.

The transfer function of the PID controller is:

Gc(s) = Kp + Ki/s + Kds

The 3 constants are user specified and affect proportional gain,

reset rate (inverse integral time), and derivative time

CONTROL-SIM Users Manual                Page 10

respectively.

If Ki and Kd are set to zero, the controller output is

proportional to the error between setpoint and measurement.  The

amount of output can be changed with the proportional gain (Kp)

factor.  This factor is also called Proportional Band which is

100/Kp.  The proportional band is the band over which the control

output is allowed to swing.  With Kp = .1, the proportional band

is 10% of the 100% control output range.  A disadvantage of

proportional control is that an offset results between the

setpoint and the measured value.

To overcome offset error, Integral, or Automatic Reset, action is

used to slowly shift the controller output as long as an error

exists.  The Ki factor is the number of repeats per second, or

the number of repeats of proportional action required to equal

the integral action.  The inverse of Ki is the reset time

constant, which is the amount of time it takes the integral

control to produce the same action as proportional control.

Since the reset action can build to a large number in certain

situations, resulting in excessive overshoots, the reset windup

action is limited to the range of the control output.  This

feature is also called anti-reset-windup.

The fast action of the proportional control and the slow action

of integral control are adequate for many applications, but the

reset action may result in too much overshoot.  By adding

derivative control, also called rate action, to the controller,

it is possible to decrease the effect of sudden change in the

measurement value by anticipating the effect on controller

output.  The Derivative time constant (Kd) is the time in seconds

for the proportional controller to produce a signal equal to the

rate signal.

(b) The FUZZY Controller is a rule based algorithm which responds to

classes of current error and past error.  The current error

between setpoint and measurement is classified as positive big,

positive small, zero, negative small, or negative big.  With this

fuzzy relationship established for both current and past errors,

a rule will fire resulting in a controller output signal which is

similarly classified.  If the change in setpoint is larger than

the big classification, the class definitions are scaled up by an

order of magnitude.  When the errors become less than the big

classification, the class definitions are scaled back down.  This

controller allows the user to determine constants from subjective

evaluation of the plant response, including non-linear action.

The user can define the class sizes in the Modify Control

Constants menu selection.  Kp is used to specify positive &

negative big, Ki is used to specify positive & negative small,

and Kd is used to specify zero.

(c) The PIP Controller is a predictive PI controller.  In this design

the derivative portion of the PID controller is replaced with

another integration function.  The derivative function has no

CONTROL-SIM Users Manual                Page 11

effect on long delays.  This controller is especially suited for

plants with long deadtimes with respect to the plant's dominant

time constant.  The predictor portion integrates the difference

in control outputs between present and delayed values.  The

significance of the predictor portion is determined by the users

estimate of L, the total deadtime.  The resulting response is

considerably faster than the PI controller alone.

For this controller, the user has 3 adjustments available.  The

Kp and Ki constants are similar to the PID controller, but the Kd

constant should be used to specify deadtime in seconds.

After controller selection is complete, enter E to return to the

MODIFY CONTROLLER menu.

Modify Control Constants

This selection allows the user to enter values for the 3

controller constants.  After depressing the P, I, or D key, the

value for the Kp, Ki, or Kd (or equivalent) constant may be

entered.  Depressing E returns to the Main menu.

Modify Quantization Level

This selection permits the user to change the resolution of the

simulated analog-to-digital and digital-to-analog converters.

The default selection is 12 bits which is fairly standard in

hardware.  This results in an full scale continuous input signal

being quantized into 4096 steps (or approx. .0244 %FS per step).

As less bits are used, control becomes less smooth.  A maximum of

128 bits is permitted.  Depressing B allows entry of Number of

bits with resulting step size indicated.  Depressing E returns to

the Main menu.

Change Output Range

The default controller output signal is UNIPOLAR, ranging from 0

to 100 %FS.  While this range is suitable for many plant devices

such as valves which move from closed to open, some devices such

as servo motors benefit from a BIPOLAR control signal, which can

move from -100 to 100 %FS.  Note that when bipolar output is

specified though, that the selected resolution (say 12 bits)

applies across the complete range (200 %FS) resulting in a step

size of .0488 %FS.  Depress C to change the range selection and E

to return to the Main menu.

4.6 PLAYBACK

If data has been recorded under the display menu selections, it can

be reviewed with the PLAYBACK selection.  All files having the .PRN

extension are displayed with size, date, and time shown.  The

highlight bar can be moved up and down to select the desired file.

The data is displayed in column format identical to printer output,

with Elapsed Time, Setpoint, Control, and Measure values presented.

The arrow up and down keys, PgUp and PgDn keys, plus Home and End

CONTROL-SIM Users Manual                Page 12

keys can be used to scroll to the desired section of the file.

Note that the recorded, or printed, data includes more decimal

places than the display screens indicate, to provide better detail

of the control action.  The header at the beginning of the file

contains the name of the configuration file (.PLT) in use when the

data was recorded as well as the mode (open or closed loop) and the

date.

The Esc key is used to terminate the data file playback and return

to the Main menu or select another data file for review.

4.7 CONFIGURATION FILES

The CONFIGURATION FILES selection results in choices of RETRIEVE,

SAVE, DIRECTORY, or EXIT.

Retrieve

When selected, Retrieve presents a display of all configuration

files (.PLT), including size, date, and time.  The highlight bar

can be moved up and down to select the desired previously saved

configuration.

A retrieved configuration file contains all the user specified

characteristics of the MODIFY PLANT MODEL and MODIFY CONTROLLER

selections.  Upon retrieval, the state equations are computed for

the saved transfer function with return to the Main menu.

Save

This selection is used to save all the configuration

characteristics of the current PLANT MODEL and CONTROLLER.

The user is requested to specify the file name, without an

extension.  In the event file already exists, the user is

prompted to choose another name or rewrite the file.

Directory

This selection displays the current directory, including disc

drive and subdirectories.  A change in directory is accomplished

by depressing "D" and entering desired new directory.  "E" is

depressed to return to the CONFIGURATION FILES menu.  After a

directory has been changed, all file actions, including data

recording and playback will use the current directory until

program termination.  The default directory is where CONTROL-SIM

is located.

Exit

Returns selection to the Main menu.

5.0  APPLICATIONS

A few examples of CONTROL-SIM use are included as follows:

CONTROL-SIM Users Manual                Page 13

Tuning

Although there are only 3 adjustments to make, selecting the

proper values to obtain optimum performance can be tedious.

Several techniques have been devised to at least get the

controller into the 'ballpark'.  At that point it may still come

down to fine corrections by trial and error to get the desired

result.

One of the most popular and oldest techniques for tuning a PID

controller is called the Zeigler and Nichols Method.  First the

integral (Ki) and derivative (Kd) constants are set to zero.

Then the proportional gain factor is increased until the loop

just becomes unstable and a small setpoint change results in

sustained oscillation.  The period of the oscillation (Pu), in

seconds, is then noted along with the value of Kp (Ku).  The

constants are then set according to the following formulas

depending on which type of control is desired (or maybe works

best):

Proportional only    Kp = .5 Ku

PI control           Kp = .45 Ku, Ki = 1/(.8 Pu)

PID control          Kp = .6 Ku, Ki = 1/(.5 Pu), Kd = .125 Pu

Open Loop Characteristics

To determine the time response of a transfer function, enter the

transfer function and select DISPLAY & PRINT DATA.  Enter a file

name, or just depress to output to printer, and then select Open

Loop by depressing M.  To determine a unit step response, enter 1

then depress S.  Depress T to terminate when measurement value

has settled.  Note that some transfer functions are very unstable

and the result will be quickly out of range.  Examine the

printout, or select Playback of the file, to see the time versus

measurement values.

Simulation of a Text Book Example

The closed loop response of a transfer function can be determined

by setting the PID controller proportional gain to 1 (Kp = 1, Ki

= 0, Kd = 0) and examining the response to a setpoint change.

For example, assume the following servo system G(s) is entered.

.32

G(s) = -------

s(s + .6)

Since the closed loop transfer function with unity feedback is

G(s)           .32

F(s) = ------------ = -------------

1 + G(s)H(s)   s^2 + .6s + .32

with the characteristic form

s^2 + 2(zeta)(omega-n)s + (omega-n )^2

CONTROL-SIM Users Manual                Page 14

omega-n, the natural frequency, is equal to 2(pi)f, or f = .09 hz

and zeta, the damping ratio, is equal to .53.  The damping ratio

of a second order system is related to per cent overshoot by a

standard curve (or calculation) which in this case is 15%.

With the output range set to bipolar and quantization level set

at 128 bits, CONTROL-SIM will respond to a setpoint change with a

response which indicates 15% overshoot and approximately 11

seconds settling time.  Select EXAMPL1.PLT.

This technique then allows evaluation of many control systems

text book examples and problems.

Reset Controller and Plant

Sometimes after modifying the plant or controller, the

measurement or control signals may become saturated due to

excessive internal calculations.  It is easy to set up control

loops that are not controllable.

To reset the controller and plant, enter the Controller Selection

menu and reselect the controller.  This will return states to

initial conditions.

Example Configuration Files

Name        Type Order K  Poles  Zeros  Td  Ts  Kp  Ki  Kd   Bits Out

PROCESS.PLT  PID 2nd   1    -1         .2   .1  2.8 .25  2.5   12 Uni

-.2

TEMP.PLT     PID 1st .001 -.001         0   10  50  1.3   0    12 Uni

SERVO.PLT    PID 3rd  .3     0    -1    0   .1   1  .5   .5    12  Bi

-.2

-1.2

EXAMPL1.PLT  PID 2nd .32     0          0   .25  1    0   0   128  Bi

-.6

EXAMPL2.PLT  PID 2nd  10    -1          0    .1 .8   1.2 .4   128  Bi

-.5

COMPCKT.PLT  PID 4th 63.5    0   -.015  0   .01  3   1.08  1   12  Bi

-.012

-2

-20

FZVALVE.PLT  FZ  1st   2    -1          0   .25  .2  .05 .005  12 Uni

FZSERVO.PLT  FZ  2nd  .32    0          0    .1   1   .2  .02  12  Bi

-.6

PIPDEL1.PLT  PIP 1st   1    -1          5   .25   1   1    5   12 Uni

PIPDEL2.PLT  PIP 4th  64    -1         10    .1   1  1.5  11   12 Uni

-2

-4

-8

CONTROL-SIM Users Manual                Page 15

6.0  REFERENCES

The following are references used by the author which the user

should refer to for better understanding of control systems.

Claggett, E. H., "Keep a Notebook of Digital Control Algorithms,

Control Engineering", Cahners Publishing, Denver, CO, October

1980

Del Toro, V. and Parker, S. R., "Principles of Control Systems

Engineering, McGraw-Hill, New York, NY 1960

De Silva, C. W. and Aronson, M. H., "Process Control, A

Professional Course", Measurements and Data Corporation,

Pittsburg, PA, 1976

Franklin, G. F. and Powell, J. D., "Digital Control of Dynamic

Systems", Addison-Wesley, Reading, MA, 1980

Grabbe, E. M., Ramo S., and Woolridge D. E., "Handbook of

Automation, Computation, and Control, Vol 3, Systems and

Components", John Wiley & Sons, New York, NY, 1961

Haaglund, T., "A Predictive PI Controller for Processes with Long

Dead Times, IEEE Control Systems Magazine", IEEE, New York, NY,

February 1992

Hafer, C. R., "Electronics Engineering for Professional

Engineers' Examinations", McGraw-Hill, New York, NY, 1989

Kuo, B. C., "Digital Control Systems", Holt, Rhinehart and

Winston, New York, NY, 1980

CONTROL-SIM Users Manual                Page 16

APPENDIX 1  ERROR MESSAGES

Bad File Name or Number   Enter correctly.

Data Overflow             Program automatically terminates due to

excessively large calculated values.

Re-enter program and check plant and

controller settings.  Settings may be

unrealistic.

Delay storage area        Choice of delay time and sample rate

exceeded                  requires more memory than allocated.

Decrease delay or increase sample time.

Disc Full                 Change drive path or exit program and

make room.

File Already Exists       Determine whether to overwrite.

File Not Found            Non-existent file name.

Invalid                   Value entered not allowed.

Printer Not Ready         Turn on printer or check paper.

Redo From Start           Entry out of sequence.  Enter correctly

or exit and re-enter.