PARK: A configurable hard drive parking utility.

Some time ago, I put together an XT computer out of spare parts I had lying

around, and ended up with a rather weird disk configuration. I had this 12 Meg

hard drive, which had 6 heads and 240 cylinders. The closest drive table entry

on the XT controller was 6 heads and 400 cylinders.

I was able to use the drive by doing a partial format of only the first 240

cylinders, and establishing a primary DOS partition (using FDISK) which was 240

cylinders in size. Although the BIOS thought it was a 400 cylinder drive, it

had no reason to access to the cylinders which were not physically present.

This worked perfectly, until I tried to park the drive.... (Insert the sound

of a head actuator banging against the stop about 160 times!!!)

Since I had run into other situations (particularily on XT's) where I didn't

always have a park utility which put the heads away where I wanted, I decided

to solve the problem once and for all, and wrote PARK.COM described below:

The source code to PARK is provided in the file PARK.C, and may be

re-compiled using my MICRO-C compiler (See enclosed CATALOG file). It MUST

be complied in TINY model, which allows it to easily write out an updated

copy of itself (See '-s' option).

The program may be freely used and distributed, provided that my copyright

notices are not removed or altered.

In its simplest form, the PARK command consists of one decimal operand for

each drive in the system, which is the cylinder number at which to park the

drive. If you specify a cylinder number of 0 (zero), the highest cylinder

number for that drive as reported by BIOS is used:

PARK 0          ; Park drive C at highest known cylinder

PARK 240        ; Park drive C at cylinder 240

PARK 240 0      ; Park drive C at 240, drive D at highest

PARK 240 400    ; Park drive C at 240, drive D at 400

If the BIOS does not acknowledge the existance of a drive, it will be

ignored. The last two examples above would be equivalent to "PARK 240" on

a single drive system.

PARK.COM supports a '-s' option, which causes it to write a new PARK.COM

which defaults to the remaining options. Executing PARK with no arguments

will PARK at these default settings:

PARK -s 240 0   ; Save the settings

PARK            ; Park drive C at 240, drive D at highest

You may see the currently defined default settings by executing PARK

with the '-s' option, and no other arguments:

PARK -s         ; Display the default settings

The PARK.COM program supplied in this archive is defaulted to TWO drives,

both of which are parked at the highest cylinder number indicated by BIOS.

This is equivalent to:

PARK -s 0 0     ; Default to drive C+D, highest cylinder

PARK will display a brief help summary if you give it an operand of

'?' or '-?'.

PARK ?          ; Display usage info