The modes provided with TWEAK range from the obscure to the useful.

This file has some hints and some vital information.

First of all, a small warning:

All mode files with an 's' at the and of the name (for example

800x600s.16) use special dot clocks available on my Chips &

Technologies 80c451 SVGA card.  Feel free to try them on your card, but

don't expect them to work on a vanilla VGA.

Modes with a 'c' at the end of the name are chained modes, to

distinguish them from unchained modes of the same resolution.

Then, some info about modes of particular interest:

256x256c.256 - The chained Mode Q:

o  256x256 in 256 colors

o  exactly 1 page, uses every single byte of the video segment

o  1-to-1 linear pixel-to-address mapping, like mode 13h

o  keep (x,y) coordinates in a single word, and use that word as

an offset.  (Both x and y fit in a byte!)  No more MULs!

o  I named it 'Mode Q' for 256-cubed (256x256x256 = 256^3).

256x256.256 - The unchained Mode Q:

o  256x256 in 256 colors

o  exactly 4 pages

o  4-to-1 planar mapping, like Mode X

The 256-column modes have been provided in several other vertical

resolutions: 224 and 240 lines

400x600.256 - new, more stable version

o  400x600 in 256 colors (unchained)

o  1 page + 22144 bytes (55.36 lines)

o  4-to-1 planar mapping

o  good multisync monitor is still required

400x300.256 - New square aspect ratio mode

o  400x300 in 256 colors (unchained)

o  That means a square aspect ratio if stretched to fill the screen!

o  2 pages + 22144 bytes (55.36 lines)

o  4-to-1 planar mapping

o  NOT based on the above 400x600 mode!  Uses the lowest vertical

frequency, and should thus prove much more stable than the

600-line mode.

360x360.256

o  Interesting mode, which should work on most VGAs.

360x270.256

o  New square aspect ratio mode

Known problems:

---------------

o  The 400x-modes won't work on LCDs and similar equipment

o  They also fail (won't synchronize) on cheap, basic VGA monitors

like the Samsung I used to have.  I think your monitor must

support SVGA resolutions of 800x600x16 and up for the 400x-modes

to work.  Your card can probably be plain, vanilla VGA though!

o  Don't expect the modes to line up perfectly with your monitor

screen's edges!  You might have to adjust the size and position of

the image with your monitor's control knobs.

I have yet to see a VGA setup which doesn't support Mode Q.  Let me

know if you have trouble.

Disclaimer:

-----------

I can't guarantee that any of the above modes will be compatible with

your VGA card.  I will accept no responsibility for damages arising

from the use or abuse of the files included in this archive.

If any of the above modes don't work on your configuration, send me

an email stating the name/chipset of your (S)VGA card, and monitor

type.  I'm especially interested in any changes you could make to get

them to work!  Also mail me comments and suggestions to the TWEAK

archive.  I will probably release a new version by the end of the

summer.  Current ideas include:

o  A tutorial on tweaking in general, with focus on unchained, 256

color modes.  I ought to explain the significance of all timing

registers, and their relevance for the resulting resolution.

o  A program which takes as input a description of a screen mode

(i.e. resolution, colors) and outputs a list of suggested

registers settings.

o  A method of linking TWEAKed 256-color modes to Themie Gouthas' XLIB.

o  Context sensitive help on most registers and their individual bits

within the TWEAK utility.

o  Support for assembler and Pascal output.

Regards from

Robert Schmidt of Ztiff Zox Softwear, Norway

e-mail: robert@stud.unit.no