FILE: AIRNAV.DOC

AIR NAVIAGTION - FLIGHT PLANNING PROGRAM

1. INTRODUCTION

The AIRNAV program is written to prepare a flight log for

flight over a predetermined route of up to twenty points (for

most light aircraft this provides waypoints close enough

together, and enough waypoints for one leg without refueling.

The log gives the geographic coordinates of the point, the

distance between points, the true course, and the estimated time

between points.  It prints space on the log for entering actual

time between points.  All flight planning is done on the basis of

one estimated speed for the entire series of legs. The effects of

wind and changes of speed with changes of altitute are not taken

into consideration in this program.

The effect of wind and altitude need to be considered in the

intial selection of an estimated groundspeed but since they

change substantially over any single 300 to 400 mile trip segment

the pilot will have to make these revisions manually. An

additional program module could be added to compute effect over

the planned flight segement.

Waypoint data is contained in a separate sequential file which is

loaded into memory at the start of the program.  For flight

planning in various regions, such as the Pacfic Northwest and

Alaska, the complete file of data for the United States can be

broken down into regions, enabling the program to run faster.

2.  PROGRAM STRUCTURE

The program is arranged in four modules.  These are:

Calculate flight

Calculate intermediate points

Edit waypoint lists

Load new file

a. The flight cacluation prompts for load of a waypoint data

file and for input of the starting waypoint and up to 19

additional waypoints.  Each waypoint is in the file in a string

39 characters long which inlcude eight letters for the name

(additional letters are chopped off on input), four letters as

the aviation "identifier", the lattitude and longitude (which can

be plus or minus for north and south lattitude, and east and west

longitude to make the program work world wide.  The program

calcualtes the course and distance between each set of waypoints

by spherical trigonometry and computes the estimated time between

points based on the estimated groundspeed.  Output is in the form

of a flight log.  Courses are given as true but the deviation for

each waypoint is printed out so the pilot can make the mental

calculation.

b.  Calculate Points - This module will calculate the great

circle course and distance between any two given points.  It will

calculates the geographic coordinate of points at equal distances

along a great circle route, and will also calculate the longitude

and distance from the start at which the great circle course

intercepts any given lattitude, or the lattitude and distance

from the start at which it intercepts any given longitude.

c.  The edit module will add to, change, or list the points

in the waypoint file

d.  The new file module will load a new waypoint file into memory.

3.  DATA FILES

Data files consist of 39 characters.  The program was

originally wiritten for the Actrix computer which would hold only

500 waypoints in string space, so files of waypoints were set up

on a regional geographic basis.

Data files can be written modified and rearranged using any

text editor, or can be compiled and sorted from a data base.

Alternatively there is a uility program "SORTED.BAS" for use in

building and sorting waypoint files.  It uses the Shell-Metz sort

procedure but still runs slowly.  This is the format for the

waypoint string.

"SAMPLE FORMAT FOR WAYPOINT ENTRY"

"LOPEZISL S31- 048-29.2 0122-56.3 -21 WA"

"________ ____ ________ _________ ___ __"

The first item is eight characters containing the name of the

waypoint.  For simplicity spaces and periods in the waypoints are

eliminated and names run together.  Also, prefaces such as "Fort"

and "Saint" are abbreviated FT and ST without the period.  When

entering the names of waypoints you can enter the full name (more

than eight characters.  The input routine will chop off all but

the first eight.

The second item is the aviation "identifier" - four characters.

As a convention of this program intersections are writen with the

first three letters of the intersection name followed by an "I"

as the fourth letter.

The magnetic variation is given so it can be printed out on the

flight plan and the two letter abbreviation for the state,

province, our country is the last item.

4. CALCULATE FLIGHT

a.  Load basic, load "AIRNAV-B.BAS" and run

b.  You can select the "Load file" option first, then read

through the list of waypoints with the "Edit" option for

familiarization, or go directly to the "Calculate Flight" module.

c.  Enter up to twenty waypoints in succession.  As the

waypoints are entered they are listed in sequence so you you

where you are.  You can start over by typing "START".  If you

don't remember what waypoints are on the list you can type

"LIST".  The program will switch to the "Edit List" module so you

can see if your selected waypoint is on the list, or enter it if

it isn't.  When you have entered all the waypoints enter "Done".

You will then be prompted for the estimated groundspeed.  When

this is entered  the program will commence retrieving the

waypoints from memory and calculating the program.

d.  If the waypoint you have entered does not exist you will

be promted to re-enter another waypoint, or a different spelling.

At this point you can again go back and look  at the list to

check to spelling, or to add another waypoint.

e.  When calculations are complete for each leg the computer

will print out the flight log.  At the conclusion of the printing

it will as if you want to add another leg.  You can input

additional legs, one at a time.

5.  CALCULATE INTERMEDIATE POINTS

In this portion of the program you select the starting and

ending points, and a leg distance.  The program will calculate

the geographic coordinates of the points at the end of each leg

segement.  You can also compute the lattitude at which the course

intercepts any given longitude, or the longitude at which it

intercepts any given degree of latittude.

# estimated groundspeed.  When

this i