UPDATE.DOC                                               Jan 1989

NOTE:   INFORMATION CONTAINED IN THIS UPDATE FILE SUPERCEEDS  THE

MSFORMS.DOC MANUAL.

******     MSFORMS V3.10 UPDATE    ******

This release has added the long awaited and much needed  position

independence  of the entries in the QUAN report.  In order to  do

this  two things are required: 1) the USECODE must be set to  'Y'

in  the MSFORMS configuration file, and 2) the CODE entry in  the

NAMES.xx  file must be assigned.  The program now uses that  CODE

entry,  so  an explanation of how to set it up  correctly  is  in

order.

If  USECODE  is set to 'N' in the configuration  file,  then  the

program  works  as  before.  If set to 'Y' then  the  first  four

characters  of the compound name in the QUAN report are read  and

used to match the entry with the entry in the NAMES.xx file.  The

two  MUST match.  If they do not, then the program  assumes  that

QUAN  entry is of no interst, and ignores it.  The NAMES.xx  file

MUST   have  the  internal  standards  first,  followed  by   the

surrogates,  followed  by the analyte list (this is the  same  as

before!).  The order of the analytes in the NAMES.xx file, is the

order that they will appear on the report.

The  fact  that  the unwanted entries are  ignored  in  the  QUAN

report, means that now special NAMES.xx files can be set up  that

are only a subset of the entire QUAN report.  Example: you run  a

625 analysis, the TCA report contains all the BNA compounds,  but

you  are  asked  only to report the  acids.   Solution:  set  the

USECODE to 'Y', enter the QUAN filename (QUAN 5.BN) and for  NAME

LIST enter 'AC'.  An acids only report will be generated.

The  main data entry screen has two new entries.  The first  I've

already  mentioned: The NAME LIST entry.  You will be  asked  for

this  ONLY  if you have USECODE set to 'Y'.  This allows  you  to

specify the two letter extention of the NAMES.xx file you want to

use.   It can be different from the two letter extention  of  the

QUAN file.

The  second  new entry is PROTOCOL.  This allows you  to  specify

what protocol was used for analysis. IE: 624 or 8270 or  608/8080

etc.

The  USECODE is NOT used for specifying different NAMES.xx  files

if  you are generating EPA CLP forms.  It will still be  used  to

match the QUAN entries with the NAMES.xx entries.

The  CODE is NOT used to sort the entries, only match  them  with

the  NAMES.xx  entries.  As I stated earlier, the  order  of  the

entries in the NAMES.xx file is the order they will appear in the

printed report.

1

The  CODE entries in the NAMES.xx file MUST match the first  FOUR

characters the Finnigan library name.  The match HAS TO BE EXACT.

Also make sure that they are UNIQUE for each entry.  You can  use

only  letters,  or  only  numbers  or  a  combination  of   both.

As  additional  demos,  I have included  QUAN7.VO  and  QUAN8.BN.

These  two  files contain only hits (the non hits have  been  U'd

out)  and  they are not in order.  I have also  included  a  file

NAMES.ACW to demonstrate the subset of a full BNA analysis.

A final caveat.  The program will not handle duplicate entries in

the QUAN report properly.  It will use the first entry it  finds.

Example:   suppose  that  the last surrogate is  not  found,  you

manually  find it and append it to the end of the QUAN  list,  if

you  do not delete the first 'zero' entry for the surrogate,  the

program will find the first entry and stop looking.

Also  the SPIKE program has not yet been updated to handle  using

the  CODE.   Consequently,  QUAN reports must  contain  the  'NOT

FOUND' entries for the MS/MSD report generator to work properly.

The  MSFORMS.DOC manual states that a maximum of 160  entries  in

the QUAN report can be handled.  This number has been  downgraded

to a maximum of 120 entries with this release.

*****************************************************************

****    MSFORMS V2.80 UPDATE    ****

This release fixes two bugs that were present in the 2.70 release

- both  minor.   In addition support for EPA CLP format forms has

started. This release only supports FORM I.  Future releases will

add additional forms.  The menus reflect the additions.

Here then, is an explanation of the new additions:

Menu  item 4, EPA CLP REPORTS MENU.  This will display a menu  of

EPA  CLP  forms.   At  this  release  only  the  first  entry  is

functional,  the  HSL  REPORT  - FORM I.   The  other  forms  are

currently under development.

2

Selecting the HSL FORM I REPORT will display the header screen:

ORGANICS ANALYSIS DATA SHEET                   EPA SAMPLE NUMBER

QUAN FILENAME:...............                    .............

Lab Name:.........................   Contract:...........

Lab Code:.......  Case No:......  SAS No:.......  SDG:......

Matrix: (soil/water) ......       Lab Sample ID:.............

Sample wt/vol:..... (g/ml)....    Lab File ID:...........

Level:(low/med)......             Date Received:........

% Moisture: not dec.... dec....   Date Extracted:........

Extraction:(SepF/Cont/Sonc)....   Date Analysed:........

GPC Cleanup:(Y/N)...  pH:.....    CONC UNITS: .....

Column:(pack/cap)......           (ug/l or ug/Kg)

Dilution Factor:  Sample:........ Surrogates:........

If  you have conpleted the EPA HEADER in the configuration  menu,

then  the  lab name, contract, lab code, case no., sas  no.,  sdg

no., level, extraction, GPC and column will be filled in from the

file  information.   Otherwise  you  will  need  to  supply   the

information.  Hitting return will accept any existing data.  Some

items (matrix, sample (g/ml), level, extraction, GCP, column  and

conc  units) have specific required entries.  To speed  entry  in

these  fields,  it is only necessary to enter  one  letter.   The

accepted letters are:

FIELD             LETTER         DISPLAY

--------------------------------------------

MATRIX:             S            SOIL

W            WATER

SAMPLE (g/ml):      G            G

M            ML

LEVEL:              L            LOW

M            MED

EXTRACTION:         F            SEPF

C            CONT

S            SONC

GPC:                Y            Y

N            N

COLUMN:             P            PACK

C            CAP

CONC UNITS:         L            UG/L

K            UG/KG

Once you conplete the entries, the QUAN report is read in and the

EDIT  screen  is displayed.  The entry results can be  edited  at

this  time.  Hitting ESC will print the form.  An EPA CLP FORM  I

is  printed.   This matches the CLP format report in  most  ways.

The  few  descrepiencies are: fixed 34 compounds  per  page,  the

report  footer does not say 1/87 Rev and the SV page 2  does  not

have   the  diphenylamine note.  The header  form   number   says

only 1 instead of 1A or 1B or 1C etc.

3

CONFIGURE EPA HEADER

--------------------

This  section  accepts data that remains fairly constant  in  the

header  of  the  EPA reports.  It will create a  file  for  later

retrieval during program execution.  The display wiil be:

LAB NAME: .........................    CONTRACT: ...........

LAB CODE: ....... CASE NO: ...... SAS NO: ....... SDG NO: ......

LEVEL:(low/med) ......    VOLATILES COLUMN: (pack/cap) ........

SEMIVOLATILES:

EXTRACTION: (SepF/Cont/Sonc) ....   GPC CLEANUP: (Y/N) ...

The  same  single letter entries mentioned  above,  also  applies

here.  There is no entry validation performed here.

*********************

Finally,  an  addition  (improvement ??)  in  the  reporting   of

surrogate  summaries.   Currently,  when  you  print a  surrogate

summary report, the SURRHOLD file is archived in the form  of  an

SR999999 file.  Now, in addition to archiving the SURRHOLD   file

an  additional  file  is  created  -  SRxx9999.WKS.   As the  WKS

extention signifies, this file is a LOTUS 123 worksheet file  and

can be read by any version of LOTUS 123 from version 1.0 on.  The

four digits  in  the  SRxx9999  name correspond to the last  four

digits in the SR999999 name.    If your QA/QC officer uses  LOTUS

for control charting your surrogate results, this should make  it

a lot easier.

Declan                                                   Dec 88

NOTE:  This program has not been tested with the INCOS 50.  If

you have an INCOS 50 and this program does not parse the

QUAN report correctly, I would appreciate it very much

if you could send me some ASCII files of the INCOS 50

QUAN reports.  Then I could work on the parser.

Thank-you.

4

########################################################