Mite-Size

A Set of Merges to Minimize Code Size

for RBBS 17.3

by Ken Goosens (data line 703-978-6360)

What These Merges Do

These merges allow you select what functions to eliminate

from RBBS in order to reduce the size of the compiled program

and reduce the amount of memory needed.

WARNING!!!  These merges must be applied to the UNMODIFIED

source code to RBBS 17.3.  The merges substitute code for

code in RBBS, and therefore may destroy other changes you

have made to RBBS.   If you have fixes, you may want first to

reduce the code, then add in the fixes.

History of Changes

01-10-90. Lit code rewritten to conform to "periodless" base code.

END SUB written into line containing omitted subroutine

for uniformity, and to save a few extra ASCII bytes.

Most subroutine descriptions and 'SUBTITLEs' removed

when subs .lit-ted out.

08-06-89.  Problem fixed when omitting fossil driver.

07-30.     Incorporated changes made in 17.2B.

05-22.     More variables added.  Recommend using BLED 2.2,

since 2.1 had a bug that would cause erroneous warnings.

12-27-88.  Corrections to doc.  Should only use BLED 2.1.

Variables added.  More code removeable.

12-13.     Two variables left out of 12-11 release of SETLIT.INC:

DOORRTN, and ASMFREESP.  Added.

Three LIT files added:  ASMANSI (omits ANSI.OBJ) by using only

Basic screen writes.   CONFERENCE (no conferences).  And

DOORS (no doors).

Documentation (this file) corrected:  base should use is

17.2A, and QB 4.5 added as possible compiler.

01-09   Updated all .LIT files to recognize line numbers at the beginning

of every subroutine.  (These were made available in the Base dated

01/09/88.

01-09   Corrected SUB4LIT.MRG to correctly identify NOVIEW.LIT.  Also

created the file (NOVIEW.LIT) since it was not in the archive.

01-09   Added reference to DOORS.LIT to RBBSLIT.MRG.

01-09   Added reference to DOORS3.LIT to SUB3LIT.MRG.

01-09   Corrected CORVUS.LIT to use BRKFNAME as was being done in the

original code.

01-09   Created SUB5HDR.LIT so that we are consistant.

01-09   All SUBxHDR.LIT files were updated to identify that it is part

of a MITE SIZE program list.

01-09   Standardized headings of each LIT file.

01-15   Line should have been omitted for PCNET.  Doors code reduction

corrected.

01-16   Four new LIT variables added.

Why Code Reduction May be Needed

In the absence of an overlay linker or optimizing compiler for

BASIC, RBBS has gotten steadily larger in code size as new functions

have been added, and will continue to do so rather than not add

new functions.  The increase in code size causes a problem on

systems that wish to run 2 nodes within 640K of memory, or

otherwise has only a restricted amount of memory to use.

These merges allow sysops with a supported BASIC compiler to

reduce code size.  This is achieved by eliminating the code

for unused functions, redundant code (typically doing the same

thing in both BASIC and assembler), and functions which the

sysop is willing to sacrifice.

RBBS can then continue to grow while still supporting sysops

with limited memory.

How to Apply the Merges

1st, you need a copy of the unmodifed source code for 17.2A.

Put this in a separate subdirectory (e.g. 172A)

2nd, in the subdirectory where you normally recompile RBBS,

copy all the files in this application.

3rd, modify the file MAKELIT.BAT by putting in the drive/path

for the original code (the first parm to BLED).  The lines

you should modify all begin with "*$", which is the default

for a BLED metacommand.   The lines beginning with "* " are

BLED comments, which are ignored in a merge.

4th, get BLED version 2.2 or higher.  The latest is 2.2.

Earlier versions lack the necessary features.

Either put BLED in with the merges or in your

path.  The new BLED functions are documented in BLED.DOC.

5th, use your favorite text editor to modify the file

SETLIT.INC.   Directions are right in the file.   Here is where

you specify what options use are using and which you don't

want.   This is the fundamental "driver" used.   The typical

way you remove code is by setting a variable to the value

"OFF", e.g. BAUD450 to "OFF" to save code by not supporting

changing 300 baud to 450, or LIBRARY to "OFF" if not using the

Library section.

6th, execute the MAKELIT.BAT, which will generate the code

needed to support your application (a new RBBS-PC.BAS,

RBBSSUB2.BAS, and RBBSSUB3.BAS).  The version will say

"17.2A Mite-Sized".  You should get no BLED warnings.

7th, recompile.  Hint:  QB 3 produces the largest code.

QB 2.01 is smaller, and QB 1.02 is the smallest.   QB 4.5

works but produces the largest code of all.

Also when removing certain routines it is possible to

further reduce the size of the resulting .EXE file by

removing the associated .OBJ from the link step. i.e.

If you specify MULTNODE = NETBIOS then you can remove

BDRIVEC2.OBJ, PCNET.OBJ, RBBSML.OBJ, 10NET.OBJ and

RBBSDV.OBJ from the link parameters.

Never use the /E option in linking.  You should

get no errors when recompiling.

8th, reconfigure RBBS for the reduced code, if needed.  For example,

disable the commands for functions you remove, such as taking out

the 'A' command if you turn questionnaires off.

What the Files Mean

SETLIT.INC - An file shared by all merges, which sets the

options the sysop wants and does not want.  Basically

defines BLED metavariables.

RBBSLIT.MRG, SUB1LIT.MRG,SUB2LIT.MRG, SUB3LIT.MRG, SUB4LIT.MRG,

SUB5LIT.MRG.

The fundamental BLED merges for rbbs-pc.bas, rbbssub1.bas,

rbbssub2.bas, rbbssub3.bas, rbbssub4.bas and rbbssub5.bas

respectively.  Each reads in (includes) SETLIT.INC to set

the metavariables, then uses the values to determines what

merges to process in the run.

*.LIT.  Files that reduce the code.

You are welcome to enhance these merges with further code reduction.

Please report any problems to Ken Goosens, data # 703-978-6360.

Realize that there is no way I can test the many different

environments or all the combinations of choices.