MARNAV

C & E ASSOCIATES

119 PLACID DRIVE

FORT MYERS, FLA.

33919

MARNAV 1.0 DOCUMENTATION

SECTION I

INTRODUCTION

There is an increasing need for software which will convert

Latitude/Longitude  data  to  LORAN-C  delay   coordinates   and

vice-versa  for  the  purpose of adding LORAN-C data to the many

charts and maps, both land and marine which are published  with-

out  this  information.  In  addition  to  the  LORAN-Geographic

conversion, MARNAV also provides  a  means  of  calculating  all

delays  for a given geographic location thus allowing a shift to

another slave in the event of technical difficulty or as a means

of double checking a position.

All  LORAN-C  Chains  for which data is currently available

have been included in MARNAV. The chain data used  is  based  on

information  from  the  UNITED STATES COAST GUARD, Department of

Transportation as published in COMDTINST-M16562.3 (Old CG-462).

The following constants were used for all algorithms in the

MARNAV Program. Signal propagation velocity used is the velocity

of light in free space 299,792,458 meters/sec.., sea water  con-

ductivity  5.0  mhos/meter,  0.01745329251993889 radians/degree.

All calculations and data in double precision. Calculations nec-

essarily assume an all water path.

In  addition to LORAN-C Geographic conversions, MARNAV also

provides the ability to navigation  from  one  pair  of  LORAN-C

delays  to  another  pair  within  the  same  chain.  In  cruise

planning, waypoint data is generally listed during the  planning

stage  and  since  these data are already available it is conve-

nient  to  use  them  for  calculating  heading   and   distance

information. In addition it is  also possible to calculate head-

ing and distance information from Latitude/Longitude data over a

rhumb  line  course.  These  programs  do  not parallel programs

available within typical LORAN-C  units  available  today  since

those  computer  navigation  features  calculate  from  measured

delays at present position.

Since the Navigation solutions are available it only seemed

logical  to include a Time/Speed/Distance function. The simplest

data entry scheme appeared to be the query for each entry with a

calculated result for the one not entered.

MARNAV                   Documentation                   Page- 2

MARNAV has a data base of thirteen files, one for  each  of  the

thirteen chains world wide. The Shareware version is distributed

with only one of these, the Great Lakes Chain (GRI  8970)  which

will  let  you  try all of the programs features. In addition to

these data files, one file is maintained to indicate the  Active

Chain  (The chain you are currently using). When you first bring

up MARNAV, it will read this active chain file and then the data

for  the  indicated  chain. It is necessary that the program and

all of the data files be kept in the same  directory  since  the

program  looks for its data files in whatever directory you were

in when you first brought up the program!

The first screen gives the title and copyright data as well as a

message  indicating what Active GRI data has been loaded. At the

bottom of the screen is the main  menu.  You  will  be  able  to

return  to  this  menu from virtually anywhere in the program by

pressing the <HOME> key. In some cases you may also use the  <M>

key  as  indicated  on the screen at that time. Each of the menu

selections will be discussed below. Use the arrow keys  to  move

around  the  menu selections. When the blinking cursor is at the

end of any menu selection, the <RETURN>  key  will  select  that

menu choice.

Section II

LORAN >> LAT/LON-

As  the  menu  suggests,  when you make this choice, the program

will convert Loran delay data  to  Latitude/Longitude  position.

You  may  enter  delay data for any two secondaries. The program

will not only calculate and report your Latitude/Longitude posi-

tion  but will also calculate the remaining secondary delays for

that position. This may at times be very useful since  secondar-

ies do fail and it may be necessary to choose another.

A  number  of  comments  are in order at this point. Since Loran

positioning depends on the intersection of two Hyperbolic  delay

lines,  there  are two valid intersections with the same delays.

These are usually far enough apart so they do  not  confuse  the

user but the computer does not know which one you are interested

in and so may well calculate the other position. To avoid  this,

it  is  necessary  to  provide  the computer with an approximate

latitude and longitude position. Generally anything within a few

hundred  miles  is  close enough. (2 to 3 degrees) Those are the

numbers appearing at the top right of this screen. They  may  be

changed by pressing <END> in the conversion routines or by using

the utilities and that will be discussed in a later section.

Notice that the Latitude/Longitude  data  is  printed  in  three

formats: Degrees and decimal Degrees; Degrees, Minutes and deci-

mal Minutes; Degrees, Minutes, Seconds and decimal Seconds. This

is  done since different charts and maps have grids given in any

one of these three formats depending on the scale.

MARNAV                   Documentation                   Page- 3

In  normal  geographic  position  data, West Longitude and South

Latitude are negative values. In order  to  avoid  the  inconve-

nience of worrying about the sign of these numbers, the quadrant

data near the top center of the  screen  defines  the  quadrants

which  are defined with positive values. For example, the United

States is in the North latitude quadrant and the West  Longitude

quadrant. Consequently, these Lat/Lon numbers are given as posi-

tive values. The quadrants selected may also be changed  in  the

same way as are the approximate latitude and longitude by press-

ing the <END> key in the conversion routines.

If the entered delays are unrealistically far from the  Approxi-

mate Lat/Lon entries you will get a message:

NO USEFUL SOLUTION-CHECK INPUT DATA".

Press any  key  to

return to the Main Menu.

We  will  defer  discussion  of the "INCLUDED CORRECTION" column

until the Utilities are discussed in a later section.

Section III

LAT/LON >> LORAN

This menu choice lets you convert Lat/Lon data to Loran  delays.

The  Quadrant data and the Approximate Lat/Lon have already been

discussed in the previous section. Also discussed in  that  sec-

tion  were  the three formats used for Lat/Lon data output. Data

may also be input in any of these three formats and the  compute

will calculate the remaining two forms. This is a useful feature

if you need to convert from one format  to  another  when  using

different scale charts.

If you enter data as follows:

42.62 it will be accepted as format 1

Format 2 may be entered in any of these ways:

43/45.2 or 43:45.2 or 43,45.2 or 43 45.2

It  does not matter if you type a , or a / or a space or a colon

as the delimiter, on the screen each of these forms would appear

as 43/45.2.

Format  3  allows  the  same  delimiters  and  the data might be

entered as follows:

43/50/34.6 or 43:50:34.6 or 43,50,34.6 or 43 50 34.6

As  before,  no  matter  how you enter it, on the screen it will

look like the first example 43/50/34.6.

Once the Lat/Lon data is entered the Approximate Lat/Lon data is

temporarily  changed to conform and the calculation is completed

for all of the secondaries in the chain.

MARNAV                   Documentation                   Page- 4

CAUTION!  JUST BECAUSE THE COMPUTER TELLS YOU WHAT THE DELAY FOR

THAT POSITION WILL BE, IT MAY BE THAT YOU ARE SO FAR FROM A SEC-

ONDARY STATION THAT YOU MAY NOT BE ABLE TO RECEIVE IT!

As  mentioned  in  the previous section, the INCLUDED CORRECTION

column will be discussed later.

Section IV

BEARING/DISTANCE

The Bearing/Distance option  will  calculate  the  heading  from

point  A to point B, it will give the distance in Nautical miles

and the reciprocal heading. (The  heading  from  B  to  A).  The

points  may  be specified in either Lat/Lon or Loran delays. One

may defined in one way while the second is defined in the other.

If,  when the cursor is at the LOP1 (Delay 1 or Line of Position

1), you press the <ENTER>  key  the  program  assumes  you  will

specify  the position in Lat/Lon coordinates. The cursor is then

positioned on the ENTER LATITUDE line.

If you enter either position in Loran delays the  Lat/Lon  posi-

tion  is  calculated;  If,  however,  you  enter the position in

Lat/Lon coordinates, the Loran delays will  not  be  calculated.

This section should require no further discussion.

Section V

SPEED/TIME/DISTANCE

Little  comment  is  required  for  this  choice.  The  computer

calculates the relationship between these three variables. Enter

any  two  and  it will calculate the third. Simply press <ENTER>

for the one you want the solution for. If  the  distance  is  in

Nautical  miles, the speed given will be knots, or conversely if

speed is entered in knots, calculated distance will be in nauti-

cal  miles.  IF speed is miles per hour distance will be statute

miles.

If time is being calculated, the result is given in  both  Hours

and decimal hours as well as hours and minutes. If time is being

entered the same formats as discussed in Section III for degrees

and  minutes, etc. hold for time, which may be entered in hours,

minutes and seconds using the same delimiters of "/"  ","    ":"

or space.

Section VI

GRI UTILITIES

The GRI Utilities allow you to enter data for a new Loran Chain,

let you edit all of the data for Loran Chains you  already  have

in  files,  provide  an additional way to change the Initial GRI

(The GRI which comes up when you load the program), let you  add

correction  factors  for  each secondary and will print the data

file for any Loran Chain.

MARNAV                   Documentation                   Page- 5

When  you  select the EDIT option you have the choice of editing

the data currently in memory, reading a new GRI file for edit or

starting  over  to  enter a new Loran Chain. Editing is straight

forward. You can move around the data  screen  using  the  arrow

keys. If you press the <ENTER> key the program accepts the entry

as it appears on the screen. To edit a data  entry,  the  entire

entry  must  be  retyped.  When you have finished correcting the

data field, press <END> to return to the menu bar.  If  you  are

satisfied  with the field as it exists, move the cursor to WRITE

FILE-, press <ENTER> and the data will be written to disk.

If you selected the CHANGE GRI- option from the  EDIT  menu  the

new  data will replace the data you have been using. You can now

exit Edit and work in the new GRI or, you may want to edit  that

data  and  write  it  to the files. If, at that time you want to

change back to the GRI you had previously, you will have to  use

the CHANGE GRI- option again and call for the desired Chain.

If  you  are  entering  data for a new Loran Chain, latitude and

longitude data must be entered in the degree and decimal  degree

mode only. Double check all of your entries carefully because an

error here produces erroneous output. When all the data has been

entered, write the data to disk using the WRITE FILE- option.

When you want to change the Initial GRI, select that option from

the Utilities menu bar. The cursor will move to the  upper  left

portion  of  the screen. You then enter the new GRI designation,

along with the Latitude and  Longitude  quadrants  you  will  be

working  in.  (See Section II). When you press <ENTER> after the

longitude quadrant selection, the new GRI  file  will  be  read.

After that whenever you start the program, the data for the Ini-

tial GRI will be loaded automatically.

SECTION VII

CORRECTION FACTOR

Loran calculations and their accuracy depend on the  propagation

velocity  of radio waves. Unfortunately the propagation velocity

is not the same over all types of terrain so the delay time to a

given  point  may  be  slightly  different than it is to another

point whose distance from the secondary station is  exactly  the

same.  While the errors incurred are not large, they can lead to

position errors of one quarter to one third of a mile.

If you get the exact geographic location of  some  fixed  object

such  as  a bridge, a building or a point of land you can calcu-

late the theoretical Loran delays for that  point.  If  you  now

measure  the actual delays to that point using a Loran, the dif-

ference between the calculated delays and  the  measured  delays

can be entered as correction factors.

MARNAV                   Documentation                   Page- 6

These correction factors will be approximately correct  for  all

Loran data in the general area where they were determined. These

correction factors will have a negative value because the propa-

gation   velocity   of  radio  waves  is  decreased  by  surface

anomalies. These correction factors may be entered for each sec-

ondary  using  the  editor in the GRI UTILITIES mode. It is this

propagation velocity problem that results in the Absolute  Posi-

tion  Error  in Lorans even though Repeatable Accuracy may be as

little as 50 feet. The correction factors help to  resolve  this

difficulty when precise positioning is important.

Do  not  use  the chart position of markers or bouys from marine

charts since their geographic position  is  often  significantly

different  from  their  position on the chart. Land based points

whose position is determined by surveys are  more  suitable  for

calculating correction factors.

Section VIII

HARD COPY-OFF or ON

It  is  often convenient to have a hard copy of your conversions

or, more often, the calculations of heading and  distance.  This

is especially true when you are planning a boat cruise since you

can plan your entire trip in the comfort of your home or  office

and  have a printout of the results. The HARD COPY OFF/ON option

of the main menu toggles the printer on  and  off  whenever  the

cursor is positioned at this choice and you press <ENTER>. After

each of your conversions or calculations, the results are  auto-

matically printed.

MARNAV                   Documentation                   Page- 7

PROGRAM REGISTRATION

The  distribution of MARNAV.EXE by shareware vendors  and  other

distributors of user supported software  is  hereby  authorized.

MARNAV.EXE  is  copyrighted  software and is not placed  in  the

public domain but rather is entered in the Shareware  market  so

that potential users may insure that it meets their needs.

The registration fee  for the program  is a only  $35.00.   Reg-

istered users will automatically receive  software  support  and

notice of any program enhancements. Any future new versions will

be made available to registered users for a nominal fee.

PLEASE SHOW YOUR SUPPORT FOR THE SHAREWARE CONCEPT BY SUBMITTING

THE REGISTRATION FEE. WE ENCOURAGE YOU TO SHARE COPIES  OF  THIS

SOFTWARE  PLUS THE DOCUMENTATION FILE WITH OTHERS WHO MIGHT FIND

IT USEFUL.

______________________________________________________________

Registration Form

MARNAV.EXE

Name ____________________________________________

Street __________________________________________

City/State ______________________________________

Please enclose the $35.00 registration fee and mail to:

C & E Associates

119 Placid Drive

Fort Myers, FL

33919

Thank you for you support,

Ed Panner

#                                    MARNAV

C & E ASSOCIATES

119 PLACID DRIVE

FORT MYERS, FLA.

33919

MARNAV 1.0 DOCUMENTATION

SECTION I

INTRODUCTION

There is an increasing need for software which will convert

Latitude/Longitude  data  to  LORAN-C  delay   coordinates   and

vice-versa  for  the  purpose of adding LORAN-C data to the many

charts and maps, both land and marine which are published  with-

out  this  information.  In  addition  to  the  LORAN-Geographic

conversion, MARNAV also provides  a  means  of  calculating  all

delays  for a given geographic location thus allowing a shift to

another slave in the event of technical difficulty or as a means

of double checking a position.

All  LORAN-C  Chains  for which data is currently available

have been included in MARNAV. The chain data used  is  based  on

information  from  the  UNITED STATES COAST GUARD, Department of

Transportation as published in COMDTINST-M16562.3 (Old CG-462).

The following constants were used for all algorithms in the

MARNAV Program. Signal propagation velocity used is the velocity

of light in free space 299,792,458 meters/sec.., sea water  con-

ductivity  5.0  mhos/meter,  0.01745329251993889 radians/degree.

All calculations and data in double precision. Calculations nec-

essarily assume an all water path.

In  addition to LORAN-C Geographic conversions, MARNAV also

provides the ability to navigate from one pair of LORAN-C delays

to  another  pair  within  the  same  chain. In cruise planning,

waypoint data is generally listed during the planning stage  and

since  these  data are already available it is convenient to use

them for calculating heading and distance information. In  addi-

tion  it  is    also  possible to calculate heading and distance

information from  Latitude/Longitude  data  over  a  rhumb  line

course. These programs do not parallel programs available within

typical LORAN-C units available today since those computer navi-

gation  features  calculate  from  measured  delays  at  present

position.

Since the Navigation solutions are available it only seemed

logical  to include a Time/Speed/Distance function. The simplest

data entry scheme appeared to be the query for each entry with a

calculated result for the one not entered.

MARNAV                   Documentation                   Page- 2

MARNAV has a data base of thirteen files, one for  each  of  the

thirteen chains world wide. The Shareware version is distributed

with only one of these, the Great Lakes Chain (GRI  8970)  which

will  let  you  try all of the programs features. In addition to

these data files, one file is maintained to indicate the  Active

Chain  (The chain you are currently using). When you first bring

up MARNAV, it will read this active chain file and then the data

for  the  indicated  chain. It is necessary that the program and

all of the data files be kept in the same  directory  since  the

program  looks for its data files in whatever directory you were

in when you first brought up the program!

The first screen gives the title and copyright data as well as a

message  indicating what Active GRI data has been loaded. At the

bottom of the screen is the main  menu.  You  will  be  able  to

return  to  this  menu from virtually anywhere in the program by

pressing the <HOME> key. In some cases you may also use the  <M>

key  as  indicated  on the screen at that time. Each of the menu

selections will be discussed below. Use the arrow keys  to  move

around  the  menu selections. When the blinking cursor is at the

end of any menu selection, the <RETURN>  key  will  select  that

menu choice.

Section II

LORAN >> LAT/LON-

As  the  menu  suggests,  when you make this choice, the program

will convert Loran delay data  to  Latitude/Longitude  position.

You  may  enter  delay data for any two secondaries. The program

will not only calculate and report your Latitude/Longitude posi-

tion  but will also calculate the remaining secondary delays for

that position. This may at times be very useful since  secondar-

ies do fail and it may be necessary to choose another.

A  number  of  comments  are in order at this point. Since Loran

positioning depends on the intersection of two Hyperbolic  delay

lines,  there  are two valid intersections with the same delays.

These are usually far enough apart so they do  not  confuse  the

user but the computer does not know which one you are interested

in and so may well calculate the other position. To avoid  this,

it  is  necessary  to  provide  the computer with an approximate

latitude and longitude position. Generally anything within a few

hundred  miles  is  close enough. (2 to 3 degrees) Those are the

numbers appearing at the top right of this  screen.  The  Active

GRI  and  these  numbers may be changed by pressing <END> in the

conversion routines, from the master menu or by using the utili-

ties and that will be discussed in a later section.

Notice  that  the  Latitude/Longitude  data  is printed in three

formats: Degrees and decimal Degrees; Degrees, Minutes and deci-

mal Minutes; Degrees, Minutes, Seconds and decimal Seconds. This

is done since different charts and maps have grids given in  any

one of these three formats depending on the scale.

MARNAV                   Documentation                   Page- 3

In normal geographic position data,  West  Longitude  and  South

Latitude  are  negative  values.  In order to avoid the inconve-

nience of worrying about the sign of these numbers, the quadrant

data  near  the  top  center of the screen defines the quadrants

which are defined with positive values. For example, the  United

States  is in the North latitude quadrant and the West Longitude

quadrant. Consequently, these Lat/Lon numbers are given as posi-

tive  values.  The quadrants selected may also be changed in the

same way as are the approximate latitude and longitude by press-

ing the <END> key in the conversion routines.

If  the entered delays are unrealistically far from the Approxi-

mate Lat/Lon entries you will get a message:

NO USEFUL SOLUTION-CHECK INPUT DATA".

Press  any  key to

return to the Main Menu.

We will defer discussion of  the  "INCLUDED  CORRECTION"  column

until the Utilities are discussed in a later section.

Section III

LAT/LON >> LORAN

This  menu choice lets you convert Lat/Lon data to Loran delays.

The Quadrant data and the Approximate Lat/Lon have already  been

discussed  in  the previous section. Also discussed in that sec-

tion were the three formats used for Lat/Lon data  output.  Data

may  also be input in any of these three formats and the compute

will calculate the remaining two forms. This is a useful feature

if  you  need  to  convert from one format to another when using

different scale charts.

If you enter data as follows:

42.62 it will be accepted as format 1

Format 2 may be entered in any of these ways:

43/45.2 or 43:45.2 or 43,45.2 or 43 45.2

It does not matter if you type a , or a / or a space or a  colon

as the delimiter, on the screen each of these forms would appear

as 43/45.2.

Format 3 allows the  same  delimiters  and  the  data  might  be

entered as follows:

43/50/34.6 or 43:50:34.6 or 43,50,34.6 or 43 50 34.6

As before, no matter how you enter it, on  the  screen  it  will

look like the first example 43/50/34.6.

Once the Lat/Lon data is entered the Approximate Lat/Lon data is

temporarily changed to conform and the calculation is  completed

for all of the secondaries in the chain.

MARNAV                   Documentation                   Page- 4

CAUTION! JUST BECAUSE THE COMPUTER TELLS YOU WHAT THE DELAY  FOR

THAT POSITION WILL BE, IT MAY BE THAT YOU ARE SO FAR FROM A SEC-

ONDARY STATION THAT YOU MAY NOT BE ABLE TO RECEIVE IT!

As mentioned in the previous section,  the  INCLUDED  CORRECTION

column will be discussed later.

Section IV

BEARING/DISTANCE

The  Bearing/Distance  option  will  calculate  the heading from

point A to point B, it will give the distance in Nautical  miles

and  the  reciprocal  heading.  (The  heading  from B to A). The

points may be specified in either Lat/Lon or Loran  delays.  One

may defined in one way while the second is defined in the other.

If, when the cursor is at the LOP1 (Delay 1 or Line of  Position

1),  you  press  the  <ENTER>  key  the program assumes you will

specify the position in Lat/Lon coordinates. The cursor is  then

positioned on the ENTER LATITUDE line.

If  you  enter either position in Loran delays the Lat/Lon posi-

tion is calculated; If,  however,  you  enter  the  position  in

Lat/Lon  coordinates,  the  Loran delays will not be calculated.

This section should require no further discussion.

Section V

SPEED/TIME/DISTANCE

Little  comment  is  required  for  this  choice.  The  computer

calculates the relationship between these three variables. Enter

any two and it will calculate the third.  Simply  press  <ENTER>

for  the  one  you  want the solution for. If the distance is in

Nautical miles, the speed given will be knots, or conversely  if

speed is entered in knots, calculated distance will be in nauti-

cal miles. IF speed is miles per hour distance will  be  statute

miles.

If  time  is being calculated, the result is given in both Hours

and decimal hours as well as hours and minutes. If time is being

entered the same formats as discussed in Section III for degrees

and minutes, etc. hold for time, which may be entered in  hours,

minutes  and  seconds using the same delimiters of "/"  ","  ":"

or space.

Section VI

GRI UTILITIES

The GRI Utilities allow you to enter data for a new Loran Chain,

let  you  edit all of the data for Loran Chains you already have

in files, provide an additional way to change  the  Initial  GRI

(The  GRI which comes up when you load the program), let you add

correction factors for each secondary and will  print  the  data

file for any Loran Chain.

MARNAV                   Documentation                   Page- 5

When you select the EDIT option you have the choice  of  editing

the data currently in memory, reading a new GRI file for edit or

starting over to enter a new Loran Chain.  Editing  is  straight

forward.  You  can  move  around the data screen using the arrow

keys. If you press the <ENTER> key the program accepts the entry

as  it  appears  on the screen. To edit a data entry, the entire

entry must be retyped. When you  have  finished  correcting  the

data  field,  press  <END> to return to the menu bar. If you are

satisfied with the field as it exists, you must move the  cursor

to  WRITE  FILE-,  press <ENTER> and the data will be written to

disk.

If you selected the CHANGE GRI- option from the  EDIT  menu  the

new  data  will  replace the data you have been using so you can

edit that data and write it to the files. When you exit the EDIT

routines,  the  computer will automatically load the Initial GRI

file.

If you are entering data for a new  Loran  Chain,  latitude  and

longitude  data must be entered in the degree and decimal degree

mode only. Double check all of your entries carefully because an

error here produces erroneous output. When all the data has been

entered, write the data to disk using the WRITE FILE- option.

When you want to change the Initial GRI, select that option from

the  Utilities  menu bar. The cursor will move to the upper left

portion of the screen. You then enter the new  GRI  designation,

along  with  the  Latitude  and  Longitude quadrants you will be

working in. (See Section II). When you press <ENTER>  after  the

longitude  quadrant  selection,  the  new GRI file will be read.

After that whenever you start the program, the data for the Ini-

tial GRI will be loaded automatically.

SECTION VII

CORRECTION FACTOR

Loran  calculations and their accuracy depend on the propagation

velocity of radio waves. Unfortunately the propagation  velocity

is not the same over all types of terrain so the delay time to a

given point may be slightly different  than  it  is  to  another

point  whose  distance from the secondary station is exactly the

same. While the errors incurred are not large, they can lead  to

position errors of one quarter to one third of a mile.

If  you  get  the exact geographic location of some fixed object

such as a bridge, a building or a point of land you  can  calcu-

late  the  theoretical  Loran  delays for that point. If you now

measure the actual delays to that point using a Loran, the  dif-

ference  between  the  calculated delays and the measured delays

can be entered as correction factors.

MARNAV                   Documentation                   Page- 6

These  correction  factors will be approximately correct for all

Loran data in the general area where they were determined. These

correction factors will have a negative value because the propa-

gation  velocity  of  radio  waves  is  decreased   by   surface

anomalies. These correction factors may be entered for each sec-

ondary using the editor in the GRI UTILITIES mode.  It  is  this

propagation  velocity problem that results in the Absolute Posi-

tion Error in Lorans even though Repeatable Accuracy may  be  as

little  as  50 feet. The correction factors help to resolve this

difficulty when precise positioning is important.

Do not use the chart position of markers or  bouys  from  marine

charts  since  their  geographic position is often significantly

different from their position on the chart.  Land  based  points

whose  position  is  determined by surveys are more suitable for

calculating correction factors.

Section VIII

HARD COPY-OFF or ON

It is often convenient to have a hard copy of  your  conversions

or,  more  often, the calculations of heading and distance. This

is especially true when you are planning a boat cruise since you

can  plan your entire trip in the comfort of your home or office

and have a printout of the results. The HARD COPY OFF/ON  option

of  the  main  menu  toggles the printer on and off whenever the

cursor is positioned at this choice and you press <ENTER>. After

each  of your conversions or calculations, the results are auto-

matically printed.

MARNAV                   Documentation                   Page- 7

PROGRAM REGISTRATION

The    distribution of MARNAV.EXE by shareware vendors and other

distributors of user supported software  is  hereby  authorized.

MARNAV.EXE    is  copyrighted  software and is not placed in the

public domain but rather is entered in the Shareware  market  so

that potential users may insure that it meets their needs.

The registration fee  for the program  is a only  $35.00.   Reg-

istered users will automatically receive  software  support  and

notice of any program enhancements. Any future new versions will

be made available to registered users for a nominal fee.

PLEASE SHOW YOUR SUPPORT FOR THE SHAREWARE CONCEPT BY SUBMITTING

THE REGISTRATION FEE. WE ENCOURAGE YOU TO SHARE COPIES  OF  THIS

SOFTWARE  PLUS THE DOCUMENTATION FILE WITH OTHERS WHO MIGHT FIND

IT USEFUL.

______________________________________________________________

Registration Form

MARNAV.EXE

Name ____________________________________________

Street __________________________________________

City/State ______________________________________

Please enclose the $35.00 registration fee and mail to:

C & E Associates

119 Placid Drive

Fort Myers, FL

33919

Thank you for your support,

Ed Panner