;----------------------------------------------------------------------------

;This is not an documentation how to use the .mcm modules in your own

;software! Its an documentation about how to write modules for other

;card reader.

;----------------------------------------------------------------------------

TABLE OF CONTENTS

MCControlModule/General

MCControlModule/Module_Delay

MCControlModule/Module_Open

MCControlModule/Module_FrameOpen

MCControlModule/Module_FrameClose

MCControlModule/Module_ReadCommand

MCControlModule/Module_WriteCommand

MCControlModule/Module_PADOpen

MCControlModule/Module_PADClose

MCControlModule/Module_PADCommand

MCControlModule/Module_DirectFrame

MCControlModule/Module_DirectPage

;----------------------------------------------------------------------------

*General*

First the main structure:

RSRESET

Module_RTS                   rs.w 1 ;to avoid crash when user is starting module

Module_Version               rs.w 1 ;Currently 1

Module_ID                    rs.l 1 ;must be "MCCM"

Module_Flags                 rs.l 1 ;see below (unused flags must be zero)

Module_Exec_Base             rs.l 1 ;Filled by MCControl

Module_Dos_Base              rs.l 1 ;Filled by MCControl

Module_Intuition_Base        rs.l 1 ;Filled by MCControl

;--- Card Data

Module_DelayReadByte         rs.l 1 ;Filled by MCControl

Module_DelayReadBit          rs.l 1 ;Filled by MCControl

Module_DelayWriteByte        rs.l 1 ;Filled by MCControl

Module_DelayWriteBit         rs.l 1 ;Filled by MCControl

Module_Reserved1             rs.l 10 ;reserved (must be ZERO)

;--- Jump Table

Module_Delay                 rs.w 3 ;Filled by MCControl

Module_Open                  rs.w 3

Module_Close                 rs.w 3

Module_FrameOpen             rs.w 3

Module_FrameClose            rs.w 3

Module_ReadCommand           rs.w 3

Module_WriteCommand          rs.w 3

Module_PADOpen               rs.w 3

Module_PADClose              rs.w 3

Module_PADCommand            rs.w 3

Module_DirectFrame           rs.w 3

Module_DirectPage            rs.w 3

Module_Reserved2             rs.w 3*8 ;reserved

Module_SIZEOF                rs.b 0

;----------------------------------------------------------------------------

BITDEF Module,Delay,0         ;Tell MCControl you require timing data.

BITDEF Module,Device,1        ;Tell MCControl you require an Device/Unit

;selector

BITDEF Module,MultiPage,2     ;Tell MCControl you support MultiPage cards

BITDEF Module,DirectAccess,3  ;Tell MCControl you support DirectAccess support

BITDEF Module,MultiSlot,4     ;Tell MCControl you support more than one slot.

;----------------------------------------------------------------------------

Module_Error_NoError       = 0

Module_Error_OpenDevice    = 1

Module_Error_NotCompatible = 2 ;If hardware supports identification!

Module_Error_NoTimerDevice = 3 ;If you need the timer.device and opening fails

Module_Error_OutOfMemory   = 20

;----------------------------------------------------------------------------

Module_DirectFrame_Read    = 0

Module_DirectFrame_Write   = 1

;----------------------------------------------------------------------------

Module_DirectPage_Next     = 0

Module_DirectPage_Prev     = 1

;----------------------------------------------------------------------------

NOTES

The exec, dos and intuition  bases  are  for  free  usage.  This  saves

memory,  time and of cource code size. The card data is for free usage,

too.

Beside the arguments delivered for  each  function  youll  receive  an

pointer on the module itself in register A4!

Make sure to backup a2-a5 and d2-d7 just like the Amiga-OS is doing!

You must implement port and device allocation into Module_Open()! Dont

try  to  play  with  the hardware without owning the right to do so. If

possible try to use parallel.device or serial.device. (The user defined

device will be given during Module_Open().

Its not required to create an real  jumptable,  by  using  JMPs.  Feel

free, to use an BRA followed by an NOP. (See RamCard.mcm)

Its strongly required  to  implement  all  given  jumps.  (except  the

Module_Delay).

Implement the code as fast as possible, because its used for many times

during reading and writing memory cards. Try to avoid overhead by using

Module_Open, or if not possible Module_FrameOpen.

Feel free to use the Module_Delay function even  if  the  ModuleF_Delay

flags  isnt  set!  The  flag  only  deactivates the gadgets and avoids

filling out the timing sheet (Module_DelayXY)!

Your reader expects only a frame number and returns a  complete  frame?

Well,  this  isnt  a problem. Set the Module_Flag ModuleF_DirectAccess

and MCControl is using Module_DirectFrame insteed of  Module_FrameOpen,

Module_FrameClose,    Module_ReadCommand    and    Module_WriteCommand!

Module_DirectFrame is doing the same job for reading and writing.  Just

take a look on my RamCard.mcm as an DirectFrame example.

If you need help then ask me and Iam telling you what to  do.  If  you

dont  know how to code a driver then send me your information and Ill

do my very best.

;----------------------------------------------------------------------------

MCControlModule/Module_Delay

NAME

Module_Delay

SYNOPSIS

Module_Delay(Time);

D0

Module_Delay(ULONG);

FUNCTION

Standard MCControl timer for high speed delay

calculations.

INPUTS

Time faktor (use Module_Delay#? field for getting

values. The delay is on all systems nearly identicaly.

WARNING

Due multi tasking it is possible that this delay is

much longer, but this should be no problem.

;----------------------------------------------------------------------------

MCControlModule/Module_Open

NAME

Module_Open -- open and inits the module

SYNOPSIS

Error   = Module_Open(Device, Unit)

D0                    a0,     d0

(ULONG) = Module_Open(*UBYTE,ULONG)

FUNCTION

Open the required device and alloc additional memory

if needed.

INPUTS

Device: Pointer on device name (STRING IS READ ONLY!!!)

Unit:   Unit of the given device

RESULT

0 = No Error

1 = Open device error

2 = Not compatible

20 = Out of memory

Other results are currently not allowed!!

NOTES

Device and Unit are only valid if the Device-Flag within

the module structure (Module_Flags) is set.

SEE ALSO

Module_Close()

;----------------------------------------------------------------------------

MCControlModule/Module_Close

NAME

Module_Close -- free all data and close all devices.

SYNOPSIS

Module_Close()

FUNCTION

Close all devices and free all allocated memory .

NOTES

Module_Close must be save to be called more than

one time. This means you must detect an closing of

an just closed module. In this case you must return

without any action.

SEE ALSO

Module_Open()

;----------------------------------------------------------------------------

MCControlModule/Module_FrameOpen

NAME

Module_FrameOpen -- Prepare for frame read/write

SYNOPSIS

Error   = Module_FrameOpen(Slot)

D0                          D0

(ULONG) = Module_FrameOpen(UWORD);

INPUTS

Slot: (0-x) Number of slot to access!

RESULT

0 = No Error

1 = Slot not available

-1 = Other Error (you shouldnt need to use this)

Other results are currently not allowed!!

FUNCTION

No special function. Just do what is required to

access the given slot. Standard.mcm is using this

function to active the select line of the given

slot. RamCard.mcm is resetting data buffers only.

SEE ALSO

Module_FrameClose()

;----------------------------------------------------------------------------

MCControlModule/Module_FrameClose

NAME

Module_FrameClose -- frame read/write done

SYNOPSIS

Module_FrameClose()

FUNCTION

No special function. Just do what is required after

reading a frame. Standard.mcm is using this

function to deactive all select lines slot.

SEE ALSO

Module_FrameOpen()

;----------------------------------------------------------------------------

MCControlModule/Module_ReadCommand

NAME

Module_ReadCommand -- Send command and get answer

SYNOPSIS

Result = Module_ReadCommand(Command)

D0                          D0

BYTE   = Module_ReadCommand(BYTE)

FUNCTION

Command will be transfered to memory card. The

card answer will be returned to Result.

NOTES

There is no need to implement different routines

for Module_ReadCommand and Module_WriteCommand.

Internaly these commands are working equal. The

only difference is that the ReadCommand is used

when reading frames and the WriteCommand on writing

frames. This allows to use different delays for

reading and writing. If you hardware isnt using

any different delays use the same routine

for both commands.

SEE ALSO

Module_WriteCommand()

;----------------------------------------------------------------------------

MCControlModule/Module_WriteCommand

NAME

Module_WriteCommand -- Send command and get answer

SYNOPSIS

Result = Module_WriteCommand(Command)

D0                           D0

BYTE = Module_WriteCommand(BYTE)

FUNCTION

Command will be transfered to memory card. The

card answer will be returned to Result.

NOTES

There is no need to implement different routines

for Module_ReadCommand and Module_WriteCommand.

Internaly these commands are working equal. The

only difference is that the ReadCommand is used

when reading frames and the WriteCommand on writing

frames. This allows to use different delays for

reading and writing. If you hardware isnt using

any different delays use the same routine

for both commands.

SEE ALSO

Module_ReadCommand()

;----------------------------------------------------------------------------

MCControlModule/Module_PADOpen

NAME

Module_PADOpen -- Prepare for PAD emulation

SYNOPSIS

Error   = Module_PADOpen()

d0

(LONG)  = Module_PADOpen()

RESULT

0 = No Error

Other results are currently not allowed!!

FUNCTION

Again! There is no predefined function for this

function. Just do what is required or even nothing.

Standard.mcm is using this function to set all

IO ports to output.

SEE ALSO

Module_PADClose()

;----------------------------------------------------------------------------

MCControlModule/Module_PADClose

NAME

Module_PADClose -- PAD emulation done

SYNOPSIS

Module_PADClose()

FUNCTION

Again there is no predefined function for this

function. Just do what is required or even nothing.

Standard.mcm is using this function to reset all

IO ports to default.

SEE ALSO

Module_PADClose()

;----------------------------------------------------------------------------

MCControlModule/Module_PADCommand

NAME

Module_PADCommand -- Simulate PAD

SYNOPSIS

Module_PADCommand(Command,Answer)

D0,     D1

Module_PADCommand(BYTE   ,BYTE)

FUNCTION

Well, this is a little tricky. You must send the

Command on the command line, while simulating

an PSX Pad answer on the Dataline by using Answer.

The card will think its hearing the playstation is

talking to a card and vice versa.

SEE ALSO

Module_PADOpen(), Module_PADClose()

;----------------------------------------------------------------------------

MCControlModule/Module_DirectFrame

NAME

Module_DirectFrame -- Handle 1 frame at once

SYNOPSIS

Error   = Module_DirectFrame(Frame,Mode,Data)

D0                           D0,   D1,  A0

(LONG)  = Module_DirectFrame(WORD, LONG,*Byte)

INPUTS

Frame: Frame number to read or write!

Mode:  Module_DirectFrame_Read or Module_DirectFrame_Write

Data:  Pointer on Buffer!

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Read or write the given frame (128 bytes)!

NOTE

Data must be 128 bytes long!

This function requires the flag: ModuleF_DirectAccess!

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------

MCControlModule/Module_DirectPage

NAME

Module_DirectPage -- Handle page swapping

SYNOPSIS

Error   = Module_DirectPage(Offset,Mode)

D0                          D0,    D1

(LONG)  = Module_DirectPage(WORD, LONG)

INPUTS

Offset: Number of pages to jump!

Mode:   Module_DirectPage_Next or Module_DirectPage_Prev

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Change activ card page of an multipage MemoryCard!

NOTE

This function requires the flag: ModuleF_DirectAccess!

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------