Following are instructions to change the video

attributes of the clock display produced by the

monochrome clock program on this BBS, in case you

are concerned about the long term effects on the

phosphor on your screen.

There are two bytes to be concerned with.

The byte at 11E (in debug) controls the attributes

for all the numerals, while the one at 148 controls

the attribute for the colon. (:)

To set the display to normal (un-hilighted), change

the byte at 11E to 07, and the one at 148 to 87.

If you don't want the colon to flash, change the byte

at 148 to 07.

Example: To turn off the hi-lighting,

A>debug monoclok.com

-e 11e 07

-e 148 87

-w

Writing xx bytes

-q

###########################################################################