Screen Save Program for the Hercules Graphics Card

There is in circulation on electronic bulletin boards, magazines and

users groups a program called variously, Mono Blank, Monitor Blank,

MONBLK2, SCRNSAVE, and probably a few other things by now too.  The

program is designed to remove the display from the monochrome screen

after some period of inactivity.  This is meant to protect the monitor

from having an image burned into the phospher if something is left on

the screen for a long time.  The programs will work on IBM's monochrone

board, but are quite unsuitable for use with a monochrome graphics card.

This is because in graphics mode the 6845 CRT controller chip operates

with a different set of controlling parameters than it uses in a text

mode.  If one of these programs goes into effect while the board is in

graphics mode it will remove the display, but will not replace proper

parameters.  Quite severe monitor damage can result.

On this diskette is a file called HGC.COM.  It is an elaboration of the

HGC.EXE file that has been shipped with the Hercules Graphics Card model

GB101.  The program will blank the screen after there has not been any

activity at the keyboard for five minutes.  It will ONLY run properly on

the Hercules Graphics Card.  Do NOT use this program with a regular IBM

Monochrome Display/Printer Adapter Card, or any video adapter other than

the Hercules Graphics Card. If you are using any other version of this

program with a Hercules Graphics Card you should immediately get rid of

it.  And tell anyone else that you know who may be using it in the same

situation to do so as well.  This program was written by Hercules in

response to a number of their customers who were having unfortunate

experiences with other versions of this program.

To use the program simply type "HGC SAVE" while at the DOS level.  If you

want to incorporate this command in a regular start-up configuration for

the Hercules Card, you can specify "HGC FULL SAVE" or "HGC HALF SAVE".

After the program takes effect, and blanks the screen, the display may be

restored by striking any key.  (The shift key is recommended, because the

keystroke will be processed.)

#############################################################################