RS232 TEST

This program will test various RS232 hardware functions.  If it runs OK

then The Weak Link and The $25 Network should run on the hardware.

1) hook up the cable between machines

2) run  TMT.COM on one machine

3) run  RCV.COM on the other machine

4) the screen will display all errors

RCV runs continuosly until any key is pressed.  It prints "z" characters while

the other machine is dormant. TMT runs once and then exits to DOS.  Both

programs assume you are using COM1 to communicate. If not then enter the COM

number on the command line when you run the program:

A>TMT COM1

or

A>RCV COM2

or

A>TMT 3

The TMT program initiates the test.  The 1st test is to make sure the

COM number you specify is really installed.  Then each RS232 card that is

installed is printed out along with its hardware port address.  The total

number of ports is then printed. (To find out what ports are on a machine, run

RCV.)  The 2nd test is of the data channel.  If this test fails then the cable

may be bad, or it may not be a null modem cable, or it may be hooked up to the

wrong port.  MAKE SURE YOU DO NOT HAVE TWO CARDS INSTALLED AT THE SAME PORT

ADDRESS. Unfoutunately a program cannot test for this possibility.  But with

the proliferation of Multi I/O cards it is a common mistake to put two

different cards in a computer with the same RS232 port address. If your

computer has more RS232 connectors than the number of ports RCV shows you have

installed then it is possible you have 2 ports at one address.

After the data line test is the DSR-DTR line test.  If this test fails

then the cable you have is probably missing these two lines or they are not

crossed between the machines.

The last test is a string transfer from RCV to TMT.   The test string

will be printed to the TMT screen.  If this message appears then the test is

good and "CHECK OK" will be printed to both screens.  If you encounter errors

which you cannot decipher or fix then please call.

Don Jindra, Owner

Information Modes

P.O. Drawer F

Denton, Texas  76202

Ph. 817-387-3339