AceComm & Utilities (Tm)

Version  1.7

"Leading Software Technology"  for todays hi-speed modem

Evaluation copy 09/05/93

AceComm

AceComm & Utilities User Manual

Version 1.7

Copyright (c) 1990 - 1993 all rights reserved

by

MGP Programming Services & Michael G Phelps

���������Ŀ

���������Ŀ     �          (R)

�ĳ         �o    ���������������

AceComm Support Services          �   ���������Ŀ �Association of

Michael G Phelps                  �   �         ���  Shareware

3003 N. Central Ave Suite 121-171 ���ĳ    o    �  Professionals

Phoenix, AZ  85012  USA         �����ĳ    �    �����������������

�����������    MEMBER

Telephone   (602) 846-4563 (12 - 7pm MST 6 days)

AceComm BBS (602) 846-2940

CompuServe  75120,3306

CONTENTS

-----------------------------------------------------------------

Chapter 1  INTRODUCTION........................................2

PRESS TEXT.......................................2

FEATURES.........................................4

Chapter 2  QUICK START.........................................5

Chapter 3  TERMINAL AREA.......................................9

Pop Up Reference................................10

Chapter 4  PRM-data MENUS AREA................................12

General Controls................................13

Memory Manager..................................15

Internal Protocol Options.......................17

Dialing Area Options............................18

Terminal Area Options...........................19

Serial Port Setup...............................21

Port Select/Configure...........................22

UART Line Control...............................24

Modem Connect Strings...........................28

Modem Response Strings..........................30

Modem Commands..................................30

Modem Control...................................31

Default Video Emulation.........................32

Key Translation Sequence file (.KTS)............34

Keyboard MACRO Assignments......................35

Directories in Use Menu.........................44

Executable Utilities............................46

Auto Engage Sequence Triggers...................49

Auto Log On Sequence Triggers...................52

Mass Menu Attribute Changes.....................54

Interactive EMSI data...........................56

CONTENTS

-----------------------------------------------------------------

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS......................59

Internal, Defined-External Receiving Units......59

Internal, Defined-External Sending Units........63

Chapter 6  TELEPHONE DIRECTORY AREA...........................67

Chapter 7  INTERNAL FILE LISTER...............................74

Chapter 8  POINT MAIL FUNCTIONS...............................77

Chapter 9  MOUSE SUPPORT......................................79

Chapter 10 ACECOMM DATA FILES.................................80

ACE.PRM / .CTL..................................81

KTS files.......................................81

KBD files.......................................82

REQ/MLF,BCH,SCR,FON files.......................84

Chapter 11 WARRANTY...........................................86

Chapter 12 LICENSE............................................87

Chapter 13 ACECOMM User Support...............................90

Contacting the author...........................90

Chapter 14 Registering AceComm................................91

Page 1              "AceComm & Utilities"(Tm) v 1.7

Chapter 1  INTRODUCTION

PRESS TEXT

AceComm Support Services

Michael G Phelps

3003 N. Central Ave. Suite 121-171

Phoenix, AZ  85012  USA

Telephone   (602) 846-4563 (12 - 7pm MST 6 days)

AceComm BBS (602) 846-2940

CompuServe  75120,3306

AceComm & Utilities (Tm) -- Version  1.7

"Leading Software Technology"  for todays Hi-Speed Modem

For DOS IBM-PC/clone, monochrome or color video adapter

"AutoScriptGen" (Tm)

"Intelligent Auto Log On Technology" (Tm)

--Ease of Use--

o AceComm's basic setup provides a powerful and potently

complex communications interface, at introductory level.

--Very Configurational--

o Key Translation Sequence customizer (create custom terminal

emulation from keyboard)

o MACRO Language and Keyboard Re-Assignment provide custom

control at any user level. Users may easily re-configure

AceComm's keyboard to imitate other software they have

grown used to.

o Multiple (custom) KeyBoarD definition files are supported

o Custom Keyboards associated to Fon.Record can be loaded on

Dial/Connect

o Colors/Menus/Sounds customizer

--Multi Thread Functions--

o Multi-level MACROS provide Pop-up services inside all

windows

--Full Featured--

o Full mouse support using Microsoft mouse driver

Page 2              "AceComm & Utilities"(Tm) v 1.7

Chapter 1  INTRODUCTION

PRESS TEXT (continued)

specification

o Script Language with AutoScriptGen (Tm)

o Internal protocols Xmodem to 1k, Ymodem to 1k/G, Zmodem to

8k, ASCII text, EMSI/IEMSI

o Color Scroll-Back-Buffer, Screen-Tag, Screen-To-File, Screen

Blank

o Context sensitive help lines, Area sensitive Pop-up

Reference

o Filelister/Utility Area, unlimited external program support

o Unlimited phone directories, FON files can hold up to 5000

records each

Page 3              "AceComm & Utilities"(Tm) v 1.7

Chapter 1  INTRODUCTION

FEATURES

AceComm & Utilities (Tm)

Version 1.7

Copyright (c) 1990 - 1993 all rights reserved

by

MGP Programming Services & Michael G Phelps

Features list

* Auto Engaging Sequence Trigger support

* "AutoScriptGen" (Tm)

* Color Scroll Back Buffer

* Configurational PORT Address/IRQ, RX/TX Buffers

* Context Sensitive On Line Help

* EMSI protocol Point Mail System, FIDO File Requests

* EMS/XMS/DISK SWAP support

* FIFO (16550) UART support

* FON files to 2500 records (1000 in conventional memory)

* FON Import / Conversion / Merge functions

* Highly Configurational User Interface

* IEMSI protocol data handshaking

* "Intelligent Auto Log On Technology" (Tm)

* Internal Memory Manager (smart memory usage)

* Internal  Protocols  (Ascii,  Xmodem,  Xmodem1k,  Ymodem,

Ymodem1k YmodemG, Zmodem, Zmodem-ZedZap(to 8k)

* Internal Screen Blanker (screen saver)

* Key Translation Sequence customizer

(create custom keyboard emulation)

* Keyboard Re definition

* Macro Programing Language (supports stacking, data args)

* Mouse support for On line character I/O and all menus

* Multiple Phone Directory support

* Screen Tag, File Tag, Line Tag function

* Video Terminal Support for TTY, ANSI, AVATAR

Page 4              "AceComm & Utilities"(Tm) v 1.7

Chapter 2  QUICK START

QUICK START INSTRUCTIONS FOR AceComm & Utilities

Last revised 9/5/93

--Distribution DISK--

The AceComm package is distributed in a compressed format.  The

compression utility used with AceComm is the current version of

LHA (this writing LHA213).  AceComm package and sub-packages are

compressed as SelF eXtracting "SFX" files.  This means the

compressed file "ARCHIVE" uses an .EXE file extension.  This SFX

format allows you the user to simply type the name of an

archive.  The archive's SFX code will execute, and extract the

files to their un-compressed form.

--Electronic Distribution--

Many BBS systems will store the distributed SFX file inside an

archive envelope of their default archive.

The AceComm BBS provides the AceComm package in ARJ and ZIP

format. SFX files as described above are contained inside the

main package, and are SelF eXtracting.

FAST SETUP

Easiest way to AceComm is to just run INSTALL.BAT. The batch file

will run ACTL.EXE. ACTL.EXE copiles ACE.CTL into ACE.PRM.

ACTL creates subdirectories defined inside ACE.CTL. Directories

defined in the default setup are CAP, FON, FILES, KTS, SCR, and

MAIL. INSTALL.BAT copies FON files into FON, KTS files into KTS,

DOC files into a subdir DOC. INSTALL.BAT then runs ACE.EXE

QUICK SETUP STEPS

Step  1

- Un compress the AceComm package into it's own sub directory.

This example assumes the filename ACE170.EXE

ACE170 means version 1.70

.EXE means AceComm is distributed as a SelF eXtracting

archive (SFX)

Example

At the DOS prompt type:  md \ace

At the DOS prompt type:  copy ACE170.EXE \ACE

At the DOS prompt type:  cd \ace

At the DOS prompt type:  ACE170

Page 5              "AceComm & Utilities"(Tm) v 1.7

Chapter 2  QUICK START

The basic setup files will be ready in a sub directory ACE.

Other files that include SFX in the filename and end in .EXE

file extension, are also SelF eXtracting archives.  These

files are enhancements to the basic setup.

Step 2

- Read ACTL.DOC  (1 page)

- Run ACTL.EXE (complies ACE.CTL into ACE.PRM)

Example ACTL ace.ctl

ACTL.EXE need only be run when creating a new re-initialized

PRM file is desired.  All PRM-data is edit able from within

inside AceComm.

Note: AceComm will invoke ACTL.EXE directly when no ACE.PRM

file is found in the start up directory.

Step 3

- Run ACE.EXE

At the DOS prompt type:  ACE

- OPTIONAL -

Not required for QUICK START

After having become familiar with AceComm, you may decide to edit

ACE.CTL for some personal defaults.  This will allow future

compilations to use your custom setup instructions.

ACE.CTL can store personal and custom setup data for times when

a new ACE.PRM file is generated. This will be most used when

upgrading AceComm versions.

The following are enhancements to the AceComm package

a) FON_SFX.EXE

FON_SFX contains several sample .BBS, FON, and .NOD files.

It  is  suggested  that  FON_SFX  be  un-compressed  inside  a

sub directory to the AceComm directory

Example: C:\ACE\FON

Page 6              "AceComm & Utilities"(Tm) v 1.7

Chapter 2  QUICK START

At the DOS prompt type:  md \ace\fon

At the DOS prompt type:  copy FON_SFX.EXE \ace\fon

At the DOS prompt type:  cd \ace\fon

At the DOS prompt type:  FON_SFX

At the DOS prompt type:  cd \ace

NOTE: ACTL creates these directories if defined in ACE.CTL and

not found.

b) KTS_SFX.EXE

KTS_SFX contains the KTS files included in the default setup,

as well as the ascii versions.  Ascii version KTS files share

the filename with KTS files, but use .ASC extension.

It  is  suggested  that  KTS_SFX  be  un-compressed  inside  a

sub directory to the AceComm directory

Example: C:\ACE\KTS

At the DOS prompt type:  md \ace\kts

At the DOS prompt type:  copy KTS_SFX.EXE \ace\kts

At the DOS prompt type:  cd \ace\kts

At the DOS prompt type:  KTS_SFX

At the DOS prompt type:  cd \ace

(.KTS  files  are  the  Key  Translation  Sequence  files  for

terminals) This file contains ascii versions  of  .KTS  files.

ASC files are compiled into KTS files using KTS.EXE.

c) UTIL_SFX.EXE

UTIL_SFX contains several utilities that enhance the AceComm

package. Each utility has it's .DOC file.

It  is  suggested  that  UTIL_SFX  be  un-compressed  inside a

sub directory to the AceComm directory

Example: C:\ACE\UTIL

At the DOS prompt type:  md \ace\util

At the DOS prompt type:  copy UTIL_SFX.EXE \ace\util

At the DOS prompt type:  cd \ace\util

At the DOS prompt type:  UTIL_SFX

At the DOS prompt type:  cd \ace

CBF.EXE can compile direct FBL output into a FON  file.   'CBF

1_114.BBS'

Page 7              "AceComm & Utilities"(Tm) v 1.7

Chapter 2  QUICK START

IDX.EXE  creates  a  very small index file so that AceComm can

use a RAW NODELIST file for mailer mode/file requests.

FBL.EXE creates a .BBS and .NOD file from the raw nodelist for

a given NET HOST

KTS.EXE compiles a Key Translation Sequence file from ascii to

binary format.

ANSI.EXE filters ANSI and AVATAR video control sequence  codes

from files.  ANSI.EXE can also be used to view the files.

Custom colors, sounds, etc.

- See bottom of ACE.CTL file.

-  See  MASS  MENU  area  inside  PRM-data  editor (inside

AceComm)

Multiple .FON files

- Selectable from inside the FON LIST AREA

Page 8              "AceComm & Utilities"(Tm) v 1.7

Chapter 3  TERMINAL AREA

----------------------------------------------------------------

�Configure�Dialer�Files�Scroll_Back�Snapshot�Dnload�Upload�Shell

OK   AceComm Copyright (c) 1990-1993 by MGP Programming & Michae

AceComm (Tm) Version 1.60, made in USA

OK                         Evaluation copy 07/22/93

...

AceComm � 2,19200L,7E1 � Av/An � 08:06:48am � ALT-V =PopUp Ref,

----------------------------------------------------------------

AceComm begins presenting to the user it's TERMINAL Area.

If a mouse is detected, the top row of the screen will contain

a 1 line mouse menu.

TERMINAL is the area in which all other areas are accessible, and

where serial I/O takes place for the user.  Above is a sample of

the AceComm TERMINAL screen upon start up.  A status line is

present, when it's flag is SET(default).  If on line, the

'AceComm' would be replaced with 'CARRIER, and other information

particular to the current setup, and host.  Note the 'L' after

baud(19200) indicates LOCK BAUD is SET.  'AV/AN' indicates the

current active VIDEO EMULATION.

Hold down the ALT key for a Pop Up Reference to key-combos

available from the TERMINAL area.  Releasing the ALT key and the

POP-UP REFERENCE disappears.  The Pop Up Reference menu is called

using the <ViewKeyCombos_> MACRO.  ALT-V is assigned the

<ViewKeyCombos_> MACRO in AceComm's default keyboard setup

ACE_DEF.KBD file.  The default Keyboard is completely re

definable by re-defining the KEY-COMBO a different MACRO(S) (see

below)

Page 9              "AceComm & Utilities"(Tm) v 1.7

Chapter 3  TERMINAL AREA

Pop Up Reference

(TERMINAL screen below with POP UP REFERENCE)

��Current KBD MACRO(S) Assignments����������������������������

�>ALT-A Script Learn

� ALT-B Scroll Back Buffer

� ALT-C Capture File

� ALT-D Phone Dir/Dialer

� ALT-E <AutoAnswer> ON

� ALT-F File Lister/Utility

� ALT-G EMSI File Request

� ALT-H Hangup Modem

� ALT-I <LocalEcho> On <SendEMSI_ICI> <LocalEcho> Off

� ALT-J COMMAND SHELL

� ALT-L <RunProgram> list

� ALT-O Configuration Area

� ALT-P Configuration Area

� ALT-R Run Script File

� ALT-S Screen SnapShot

� ALT-T Screen TAG

� ALT-V Pop Up Key Reference

� ALT-X <EndAceComm> 0

� ALT-Y <AutoAnswer> Off

� ALT-Z <ClearWindow>

�  -----------------------------------------------------------

����>PgDn< Display  Template  Quit  Help �<>v^����������������

o The AceComm keyboard is macro driven

o The Popup Reference displays each key press that has been

assigned to a MACRO, or MACROS.

o Current .KBD file is displayed, AceComm supports unlimited

.KBD files. Current .KBD filename is always displayed at the

right side of the Terminal Area's status line.

AceComm's Keyboard is re definable. Each key press can be re

assigned a different function, or set of functions. To

accomplish keyboard re definition, AceComm uses descriptive

text symbols called MACROS. MACROS are enclosed inside of

angled brackets.

The Pop Up Reference menu displays the current .KBD file's

MACRO(S) assignments.  A .KBD file contains a copy of all

key-press MACRO(S) assignments.  AceComm supports unlimited .KBD

Page 10              "AceComm & Utilities"(Tm) v 1.7

Chapter 3  TERMINAL AREA

Pop Up Reference (continued)

files.  Each key press combination can be assigned a MACRO(S).

AceComm allows the user to re define the keyboard functions if so

desired.  AceComm provides a rich set of MACRO functions for all

of AceComm's internal functions.  A complete MACRO list with

descriptions is contained in the MACRO.DOC file.

- PgDn or <CR> will display the next key set at top of screen.

- Display  toggles the display from a text description and

the actual macro assignments

The key press is displayed to the left, with it's MACRO

assignment to the right.  The example below shows the MACRO for

AceComm's configurations area is assigned to ALT-P.

ALT-P <PrmDataMenus_>              <= MACRO assign

ALT-P Configuration Area           <= descriptive text

- Template allows you to print a list of keys that have MACRO

assignments.  The default filename is "PRN".  You may find it

handy to print to a FILE, (key.prn) or similar, this way you

can format the TEMPLATE, for a more custom print job.

Page 11              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

��D:\ACE\ACE.PRM�����������������ͻ

�>General Controls/Menus          �

� Serial Port/Modem  Setup        �

� KeyBoarD MACRO Assignments      �

� Directories in Use Menu         �

� Executables Menus               �

� Auto Engage Sequence Triggers   �

� Auto Log On Sequence Triggers   �

� Menus / Colors                  �

� Interactive EMSI data           �

� Point Mail Setup                �

� AutoSave is ON                  �

� Print ACEORDER.FRM              �

���� Select  Quit  Help �<>v^����ͼ

o MACRO for this menu is: <PrmDataMenus_>

(default assignment is to both ALT O & ALT P)

o All PRM-data changes are auto-saved when Auto-Save flag is

"ON"

o Each sub-menu contains context sensitive HELP

Virtually any data contained inside the ACE.PRM file is

accessible and configurational from the PRM-data area.  Above is

the menu you will see after pressing key-combo <PrmDataMenus_> is

assigned to from the TERMINAL area.

Notice the second to bottom line of text in the menu says

"AutoSave ON".  AutoSave can be toggled On/Off from inside the

"General Controls" menu.  When ON, changes made from inside the

PRM-data menus are saved to ACE.PRM automatically.  If AutoSave

has been toggled OFF, the "AutoSave ON" is replaced on the menu

with "Save Changes" and this menu item must be selected for

changes to be kept past exiting AceComm.  Note: If you toggle

"AutoSave" to Off, you will select the "Save Changes" option

before exiting AceComm, if you want the toggled option to remain

toggled.  Otherwise, AceComm will be back to "AutoSave" when

re-started.

Page 12              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

General Controls

��General Controls/Menus�����������������������ͻ

� Allow enhanced keyboard detection        Yes  �

� Allow mouse detection                    Yes  �

� Mouse horizontal sensitivity             6    �

� Mouse vertical sensitivity               4    �

� Auto Save Changes                        On   �

� Snow (vertical retrace) checking         Off  �

� Screen Saver timeout value (minutes)          �

� Keep Log file                            On   �

� All Sounds Override Switch <> Sounds are On   �

��Sub Menus������������������������������������ͺ

�>Control Memory Use                            �

� Internal Protocol Options                     �

� Dialing Options                               �

� Terminal Area Options                         �

���� Edit  Quit  Help �<>v^��������������������ͼ

o The controls are enable/disable flags, some of which can

also contain a value for timers.

o Sub menus contain more categorized controls

Most controls are a series of flags which tell AceComm when to

use some of the features, and when not to.  Flags with the ON/OFF

status are toggles, and will be toggled here in this menu.

Notice the "Screen Saver" flag description contains the number 5,

rather than an ON/OFF.  Selecting Screen Saver mode 0 = OFF.

- Allow enhanced keyboard detection        Yes

AceComm will detect and use enhanced BIOS keyboard functions.

Some systems may not fully support this mode properly. This

flag allows locking out AceComm's use of enhanced BIOS

keyboard functions.

- Allow mouse detection                    Yes

Setting this to "NO" will cause AceComm to act as if a mouse

is non existent, the mouse driver is never initialized, no

mouse menu is presented in the Terminal Area.

- Mouse horizontal sensitivity             6

Mouse sensitivity is already set in the mouse driver. This

Page 13              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

General Controls (continued)

value is internal to AceComm only

- Mouse vertical sensitivity               4

Mouse sensitivity is already set in the mouse driver. This

value is internal to AceComm only

- Auto Save Changes                        On

When "ON" AceComm will save PRM-data changes to the ACE.PRM

file.  When "OFF" the prompt inside the main PRM-data menu must

be selected for PRM-data changes to be recorded to file.  This

pertains to actual PRM-data below the menu system data.  Menu

area attributes are ALWAYS SAVED, with exception to "Mass Menu

Area" changes, these must be affected using the appropriate

selection from inside "Mass Menu".

- Snow (vertical retrace) checking         Off

AceComm uses direct video buffer writes for all it display

areas.  Most areas will be able to handle SNOW CHECKING

properly before making screen updates.

- Screen Saver timeout value (minutes)     5

Screen Blanking function.

Screen Saver can be set from 0 - 10.  Values above 0 are

minutes without a key press to AceComm blanking the screen.

Screen is restored with a key press.  Un-blank-key-press is

valid.  Use ALT alone for non-destructive key press to un-blank

the screen.

See also "Screen Saver if Data Carrier Detect" from Terminal

Area Options menu.

- Keep Log file                            On

Toggle AceComm use logging ON/OFF.

- All Sounds Override Switch <> Sounds are Off

Toggle, enable/disable all sounds. Master override.

Page 14              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Memory Manager

----------------------------------------------------------------

���� AceComm (Tm) Version 1.7 -- Copyright (c) 1990-1993 by Mich

�����������������������������������������������������������������

�����������������������������������������������������������������

�����D:\ACE\ACE.PRM�����������������ͻ���������������������������

������General Controls/Menus�����������������������ͻ������������

����� Allow enhanced keyboard detection        Yes  �re help from

����� Allow mouse detection                    Yes  �helps:

����� Mouse horizontal sensitivity             6    �846-4563 12-

�������Control Memory Use����������������������ͻ   �846-2940 24

������>XMS (kb) ok to use           64          �   � User ID 751

������ Scroll Back (kb) to use      64          �f  �DRESS 1:114/

������ Swap for <RunProgram>        On          �   �MM, ACECOMM

������ Swap for <CommandShell_>     On          �   �e ACECOMM ec

�������Current Memory Use����������������������ͺ   � it makes BA

������ XMS allocated                65536       ���ͺ

������ XMS free at load             2424832     �   � Support Ser

���Ⱥ� ACE.EXE load                 227040      �   � G Phelps

������ Scroll Back allocated        65536       �   � Central Ave

������ PRM file  allocated          36025       �   �, AZ  85012

����Ⱥ Rx buffer allocated          8192        ���ͼ������������

������ Tx buffer allocated          256         �����������������

��������� Select  Quit  Help �<>v^�������������ͼ����������������

Allow internal memory manager swap to XMS (only needs 64k at pre

-----------------------------------------------------------------

o With version 1.7, AceComm begins managing memory usage.

o AceComm can swap the scroll back buffer memory to XMS prior

to allocating large chunks of memory for other operations.

o EMS will be supported in the next release.

o XMS and EMS are supported for the SWAP functions.

- XMS (kb) ok to use

AceComm's internal memory manager can save overall memory

requirements by swapping some internal data to XMS memory when

about to perform another memory hungary task.

- Scroll Back (kb) to use

Up to 64 kb (64 x 1024) memory can be used for the back

buffer.

Future versions of AceComm will begin allowing multiple

64 kb memory be used for scroll back buffer.

Page 15              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Memory Manager (continued)

The first 64 kb memory is conventional memory.

- Swap for <RunProgram>                    On

XMS, EMS, DISK swapping for the AceComm executable will take

place when this flag is set "ON".  When SWAPPING is active,

AceComm 's SIO is deactivated while <RunProgram> MACRO is

active.

Notes: AceComm is currently using Ralf Brown's SWAP functions

for the SWAP service.  SWAP is the only code inside

AceComm executable that is not written entirely by MGP

Programming Services.  SWAP has proven to be the most

dependable code of this type that has been tested in our

executable files.  MGP Programming Services would like

to thank Ralf Brown for permission to use his SWAP code

in our products.

- Swap for <CommandShell_>              On

XMS, EMS, DISK swapping for the AceComm executable will take

place when this flag is set "ON".  When SWAPPING is active,

AceComm 's SIO is deactivated while <CommandShell_> MACRO is

active.

Notes: The Notes for <RunProgram> apply to this MACRO

Page 16              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Internal Protocol Options

��Internal Protocol Options���������������ͻ

�>Auto Engage                       On     �

� File Resume                       On     �

� Bell after internal protocol      3      �

� Time release from protocol        Off    �

���� Select  Quit �<>v^�������������������ͼ

- Auto Engage                              On

Activate/deactivate AutoD sequence triggers & Associated AutoM

MACROS.

- File Resume                              On

For internal Zmodem. Resume "ON" existing files, data is

appended to the file, "OFF", the file is created new, any

existing file of the same name is over written.

- Ring bell after internal protocol        3

Internal protocols will sound an alarm before returning to

Terminal Area.  A value of "0" disables any sound after

internal protocols.  A value of "1" plays a Victory tune.

Values from 2-30 become seconds to sound the internal bell.

See also <RingBell> 5 MACRO

- Time release from protocol             Off

Time slices for Windows/ OS/2, DV, and DOS are supported.

This flag applies to the DV and DOS time releases.

Releasing time to Windows/ OS/2 using the interrupt 2fh

function causes so many missed interrupts that AceComm

never releases int 2fh time slices during an internal

protocol function.

Time slices are otherwise given up generously.

Page 17              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Dialing Area Options

��Dialing Options������������������������������ͻ

�>Inter byte delay                         1    �

� Modem Response Timeout when dialing      60   �

� Seconds between re dials                 1    �

� Ring BELL on Dial/Connect                1    �

���� Select  Quit �<>v^������������������������ͼ

- Inter Byte Delay                         1

Add (value x 55 millisecond) delay between modem command

characters.  With a value of '0' modem command strings are sent

just as fast as the UART's Transmit Hold Register is Empty

(THRE).  (I've observed Supra modems behave much better using a

value of 1 for inter byte delay when in continuous re-dial)

- Modem Response Timeout when dialing      60

Master timeout when dialing.  Overrides your modem's own S7

register.

Notes: It may be best to set this value several seconds above

your modem's S7 register.

- Seconds between re dials                 1

AceComm will pause this many seconds between receiving a modem

response string, or timing out, and sending the next dial

string to the modem.  The variable can be toggled from 0

seconds to 90 seconds.

- Ring BELL on Dial/Connect                1

Seconds to ring BELL on successful detection of either DCD or a

connect message response from the modem.

Page 18              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Terminal Area Options

��Terminal Area Options��������������������������ͻ

�>Status line                              On     �

� Mouse menu                               Yes    �

� Screen Saver if Data Carrier Detect      No     �

� Terminal Area screen color (TTY mode)    7      �

� Drop DTR when quitting ACECOMM           No     �

� Log modem response while off line        Yes    �

� Strip high bit from incoming             Off    �

� Default Video Emulation                  Av/An  �

���� Edit  Quit  Help �<>v^����������������������ͼ

- Status line                              On

Toggle AceComm 's STATUS LINE On/Off

Notes: AutoScrGen (Tm) will assert the STATUS LINE, and restore

it on when through.  The RunScript function will do the

same.

- Mouse Menu                              Yes

Display 1 line mouse menu on Terminal Area screen

- Screen Saver if Data Carrier Detect      No

Screen Blanking service disabled during modem status DCD bit

detected to be ASSERTED.

- Terminal Area screen color (TTY mode)    7

This setting allows changing the Terminal Screen color that is

initially active when AceComm is started.  The color will

remain constant through TTY video emulation.

- Drop DTR when quitting AceComm            No

AceComm will or will not drop DTR when the <EndACECOMM> [n]

MACRO is run.  Most modems will by default, drop carrier along

with a drop in DTR.  DTR is a bit in the modem control register

of the UART.  Setting this to "YES" will cause AceComm to drop

DTR when ever <EndACECOMM> [n] MACRO is executed.

- Log modem response while off line        No

AceComm can log modem response strings that come in.  This flag

will tells AceComm if it should log the responses.  This is for

off line only.  This flag is most use full when you want to

know when your phone rings, AceComm will TIME STAMP PRM-data

Area MDM_RESPONSE_RING responses strings.  Many modem ROMS are

Page 19              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Terminal Area Options (continued)

now featuring caller ID.  Modems that have caller ID ability

will be able to present to the terminal screen the caller ID

info, if available.  This information will also be logged along

with the TIME STAMPED "RING".

- Strip high bit from incoming             Off

"Off" data received from the modem is un altered. "ON" data

is stripped of any bits above the decimal value 128.

- Default Video Emulation                  Av/An

This selection brings up another menu.  The menu displays the

available video terminal emulation modes.  Along with their

associated KTS files.  This setting affects the default video

terminal setting for AceComm.  Individual FON RECORDS contain

their own video terminal settings.

Page 20              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Serial Port Setup

����Serial Port/Modem  Setup����������ͻ

�>Port Number Select      1            �

� Line Control Setup      19200L,8N1   �

� Modem Connect Strings                �

� Modem Response Strings               �

� Modem Command Strings                �

� Modem Control Setup                  �

�� Select  Quit �<>v^�����������������ͼ

o The "Serial Port Setup Menu" contains sub-menus for all of

AceComm's serial port and serial port related configurations.

o Information presented to the right of menu selections are

current settings.  The 'L' appended to BAUD indicates LOCK BAUD

is SET "ON".

Page 21              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Port Select/Configure

����Serial Port Setup�����������������ķ

���Port          Address IRQ   Detected ������ķ

��>1             03F8h    4      Yes           �

�� 2             02F8h    3   In use           �

�� 3             03E8h    4      Yes           �

�� 4             02E8h    3       No           �

�� 5             0230h    4       No           �

Ӻ 6             0238h    4       No           �

� 7             0240h    4       No           �

� 8             0248h    4       No           �

�� Other stuff ������������������������������ĺ

� RBuf          8192                          �

� TBuf          256                           �

� FIFO trigger  14                            �

� Chain IRQ     Off                           �

� TX            Off                           �

� CTS/RTS       On                            �

� XON/XOFF      On                            �

�ĳInstall� Edit  Quit  Help �<>^v�����������Ľ

Port numbers are merely symbolic for identification.  Port

addresses and the Interrupt ReQuest line for the address is

configurational.

- Install option, AceComm will close the active port and install

it's SIO on the port selected.  MDM_INIT string is sent to

modem.

- Edit option, you will be prompted for port address and IRQ.

Address is a HEX number.  Entering a decimal value will fail.

- Rbuf/Tbuf are the buffer size used for serial I/O.  Normally

there won't be any advantage to raising the TBuf size.  RBuf

size will be adequate at about 1024 bytes for all normal use.

The larger value (default) will allow storage of incoming

during Back buffer, Tagger etc.  RBuf range is from 1k to 32k,

TBuf 128 bytes to 32k.  The Zmodem sending unit's window size

is equal to TBuf size.  Window size is not data-frame size.

Window size means "how much data do we let build up into the TX

buffer at any one time.  RBuf/TBuf can be changed at any time,

on line or not.  It is recommended that the HOST be still when

re-sizing RBuf/TBuf.  RBuf/TBuf changes take immediate affect.

Page 22              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Port Select/Configure (continued)

- FIFO trigger is the number of bytes buffered before a First In

First Out UART chip will alert the CPU by way of interrupt.

FIFO UART operation is automatically enabled when a FIFO UART

is detected.  AceComm always checks for FIFO UART presence

whenever installing onto a PORT.  FIFO trigger level changes

take immediate affect.  Trigger level of "1" affectively

disables the FIFO affect.

- Chain IRQ "share IRQ" will cause AceComm 's SIO ISR to pass on

the interrupt request to the Interrupt Service Routine which

was present on the IRQ when AceComm installed onto the port.

The Chain IRQ toggle takes immediate affect.  IRQ sharing is

not supported by -most- I/O cards.  Use the option at your own

risk.  Notes: IRQ sharing may appear to work properly on your

setup, even though your hardware was not designed for IRQ

sharing to take place.  There are some good technical points to

be made for degradation that may be caused in the I/O cards,

when forced to drive the IRQ line HIGH, while another is doing

it's best to pull the same IRQ line LOW.

- TX is a flag that tells AceComm to use interrupt driven

output.  Interrupt driven input is always on.  The TX toggle

takes immediate affect.  If you FAST computer/modem setup is

capable of overwhelming the other end when sending data,

toggling TX "OFF" may improve the situation.

- CTS/RTS hardware handshaking / FLOW control Hardware

handshaking may normally be left ON (default).  High speed

modems using fast data rates and typically locked port rates

will require software to use FLOW control.

- XON/XOFF software handshaking / FLOW control Many HOST software

will recognize XON/XOFF FLOW control.  You may need software

handshaking enabled for proper FLOW control while performing

ascii uploads.

Notes: AceComm uses FLOW control flags (above) primarily to

detect when not to send data to the modem.

AceComm will automatically use both forms of FLOW

control when it detects the receive buffer getting

full.

Page 23              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

UART Line Control

���� Serial Port Setup Menu ����������ͻ

� Port Number Select      1            �

��� UART Line Control ��������������ͻ �

��>Data Bits          8              � �

�� Parity             None           � �

�� Stop Bits          1              � �

�� Max/Lock Baud Rate 19200          � �

�� Lockbaud           On             � �

��� Select  Quit  Help �������������ͼͼ

The UART's line control register can be set for several different

data arrangements.  Most common being 8 data bits, no parity bit,

1 stop bit.  Most BBS systems will use the 8N1 configuration.

The UART uses the line control information when assembling

incoming bits into a byte.  Setting this information from the

PRM-data area, the format is stored in the default settings for

AceComm to use.  When you look at the FON directory records you

will see that each record stores it's personal line control

setting to be used when dialing the record.  Data format is saved

with each individual record in the record's .FON file.  When

using AceComm in HOST mode the default setting will be used.

AceComm retains the default line control register setting in the

ACE.PRM file.  FON record line control settings are stored in

their respective .FON files.

BAUD rate is also something that will normally be attached to a

FON record and used when dialing the record.  This is of more

concern to users of modems that do not allow LOCKING the port.

LOCK BAUD is almost always available as an option when using a

modem that can support CONNECT rates above 2400 baud.  When using

the LOCK BAUD option, AceComm will leave the port speed alone

when dialing.  The modem handles speed translation between

'connect' and 'computer'.  LOCK BAUD ON is generally preferable

with modems that support it.  The status displays will reflect

'L' appended to the baud rate when LOCK BAUD is set ON.  NOTE:

the 'L' is not appended in the Line Control Window's display

above.

Page 24              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

UART Line Control (continued)

���� Serial Port Setup Menu ����������ͻ

� Port Number Select      1            �

��� UART Line Control ��������������ͻ �

���� Data Bits ����������������������ͻ�

���>5                                 ��

��� 6                                 ��

��� 7                                 ��

��� 8                                 ��

���� Select  Quit �������������������ͼ�

���� Serial Port Setup Menu ����������ͻ

� Port Number Select      1            �

��� UART Line Control ��������������ͻ �

�� Data Bits          8              � �

���� Parity �������������������������ͻ�

���>None                              ��

��� Odd                               ��

��� Even                              ��

�Ⱥ Mark                              ��

� Space                             �

�� Select  Quit �������������������ͼ

���� Serial Port Setup Menu ����������ͻ

� Port Number Select      1            �

��� UART Line Control ��������������ͻ �

�� Data Bits          8              � �

�� Parity             None           � �

���� Stop Bits ����������������������ͻ�

���>1                                 ��

��� 2                                 ��

���� Select  Quit �������������������ͼ�

- Data Format

Stop bits, Parity, Data bits.  Each character that comes in the

serial port arrives as a series of BITS.  The BITS are

assembled in your serial port's UART's SERIAL DATA UNIT SDU).

AceComm's default setting of 8 data bits, no parity, and 1 stop

bit is the most popular arrangement, and that which most BBS

systems use.  You will change the Data Format when necessary

for connecting with systems that don't use the standard 8N1.

Page 25              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

UART Line Control (continued)

----------------------------------------------------------------

��� AceComm (Tm) Version 1.7 -- Copyright (c) 1990-1993 by Micha

����������������������������������������������������������������

����������������������������������������������������������������

�����D:\ACE\ACE.PRM�����������������ͻ��������������������������

���� General Controls/Menus          ���������������������������

��������Serial Port/Modem  Setup����������ͻTo Get more help fro

����� Port Number Select      1            �Michael Phelps:

�������UART Line Control����������������ͻ �OICE 602 846-4563 12

������ Data Bits          8              � �BS   602 846-2940 24

������ Parity             None           � �OMPUSERVE User ID 75

������ Stop Bits          1              � �IDONET ADDRESS 1:114

��������Baud Select����������ͻ          � �IDONET COMM, ACECOMM

����Ⱥ� 300                   �          �ͼdistribute ACECOMM e

���� Ⱥ 1200                  �#��������ͼNODES until it makes B

���� A� 2400                  �      �����-Write to:

���� P� 4800                  �      �����    AceComm Support Se

�����ͺ 9600                  �#����ͼ����    Michael G Phelps

�������>19200                 ������������    3003 N. Central Av

������� 38400                 ������������    Phoenix, AZ  85012

������� 57600                 ����������������������������������

������� 115200                ����������������������������������

�������� Select  Quit  Help �ͼ���������������������������������

----------------------------------------------------------------

o Connect speeds such as 12000, 14400 etc. are not actual

data rates that the UART (serial port)  can be set to.

o Modems capable of connecting at 9600 and above can usually be

used with a "Locked port"

o Setting the Baud rate to 19200 for a 9600 baud modem will

allow better performance when down loading data.

o Locking the port means selecting a baud rate higher than the

highest connect speed the modem is capable of, and telling the

communications software to lock it there.

o Using a locked port requires your modem be in full handshake

mode.

"Selecting your highest port speed when LOCKing the port"

Page 26              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

UART Line Control (continued)

Locking the port at 57600 with a high speed modem causes the

UART to assemble a character off the bit stream VERY quickly.

If you experience serial input errors with HIGH locked rates,

drop down to 38400 or even 19200.  The lower rate will cause

the UART to make characters available at a reduced speed.  For

instance a 12 MHZ AT will probably experience data overruns

using a locked port above 19200 when down loading TEXT files

with ZMODEM.

A common miss-conception is when CRC errors occur with HIGH

LOCKED PORT rates, that it is the UART unable to keep up.  This

is simply not true.  The UART/modem combo is capable of out

running the computer/software combo.  When using a TASK

switcher such as WINDOWS or DESQVIEW, a lower PORT RATE such as

19200 may very well be needed for efficient data transfers.

The idea is to set the port to a speed that the

computer/software combo is capable of responding to quickly

enough.

Page 27              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Modem Connect Strings

��D:\AC��Modem Connect Strings���ķ

�  Gene�>CONNECT                  �

�  Term� CONNECT 1200             �

�>����S� CONNECT 2400             ������ķ

� � Por� CONNECT 2400/ARQ         �      �

� � Lin� CONNECT 4800             �8N1   �

� �>Mod� CONNECT 9600             �      �

� � Mod� CONNECT 9600/ARQ         �      �

� � Mod� CONNECT 14400/ARQ        �      �

� � Mod� CONNECT 16800/ARQ        �      �

� �� Se� CONNECT 19200            ������Ľ

�  Prin� CONNECT 38400            �

�  Auto� CONNECT 57600            �

���� Se� CARRIER                  �

� CARRIER 9600             �

� CARRIER 12000            �

� CARRIER 14400            �

� CARRIER 16800            �

� CARRIER 19200            �

� CARRIER 28800            �

�� Edit  Quit ������������Ľ

o There is no order of sequence for these strings

o Connect strings will allow non-locked ports "auto adjust"

AceComm will detect a connection after dialing, a few seconds

quicker when able to match a connect string with the modem's

connect message.

High Speed modem users may configure the modem for more than 1

connect message to be returned from the modem.  AceComm need only

detect 1.  The connect string fields are freely configurational,

and follow no specific order.  Leave them, or edit them as needed

any way you see fit.

AceComm will fall out of re-dial on detection of CARRIER.  For

the LOCK BAUD users, this detection is right away.  For those who

use modems that don't allow port LOCKING, and want auto baud rate

adjustment to match the connect, CARRIER is checked only after

the dialing has timed out.  AceComm will be trying it's best to

get a CONNECT string so that the baud can be verified.

Page 28              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Modem Connect Strings (continued)

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

����� Serial Port Setup Menu ����������ͻ�

�� Port Number Select      1            ��

�� Line Control Setup      19200L,8N1   ��

�� Modem Connect Strings                ��

����Modem Response Strings�����������������������ͻ

���>MDM_RESPONSE_OK           OK                  �

��� MDM_RESPONSE_RING         RING                �

��� MDM_RESPONSE_NO_CARRIER   NO CARRIER          �

�Ⱥ MDM_RESPONSE_ERROR        ERROR               �

� MDM_RESPONSE_NO_DIAL_TONE NO DIAL TONE        �

� MDM_RESPONSE_BUSY         BUSY                �

� MDM_RESPONSE_NO_ANSWER    NO ANSWER           �

� MDM_RESPONSE_RINGING      RINGING             �

� MDM_RESPONSE_VOICE        VOICE               �

�� Edit  Quit ���������������������������������ͼ

o AceComm uses text response strings only

Modem response strings are normally UPPER CASE.  Response strings

are used by AceComm when dialing to more quickly respond to BUSY,

NO CARRIER etc.  HOSTMODE will be waiting for a RING code.(1) The

Response strings that can be matched while dialing are displayed

in the DIAL WINDOW as they come in.

(1)  HOSTMODE  also  uses the UART's modem status register RI bit

when detecting "RING".

Page 29              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Modem Commands

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

����� Serial Port Setup Menu ����������ͻ�

�� Port Number Select      1            ��

�� Line Control Setup      19200L,8N1   ��

�� Modem Connect Strings                ��

�� Modem Response Strings               ��

������Modem Command Strings�������������������������������ͻ

���>Init           ATZ^M~~~~ATES7=40                       �

��� Dial Prefix    ATEQ0X4DT                               �

�Ⱥ Dial Suffix                                            �

� Answer         ATA                                     �

� Hang up         +++~~~~~ATH0                           �

� Off hook        ATH1                                   �

� Abort          ^M                                      �

� Reset          ATZ                                     �

� Command Echo   ATE1                                    �

� Host Init      ATE                                     �

� Cmd Terminator ^M                                      �

���� Edit  Select  Quit ��������������������������������ͼ

o Cmd Terminator is automatically appended to modem command

strings.

Here we have the modem commands AceComm uses to interact with

your modem.  Please note the bottom of the menu contains the "Cmd

Terminator".  AceComm appends this to all modem commands

automatically.  Notice that "Init" has a ^M followed by ~~~~ and

another command.  This is an example of command stacking.

'^' is an escape character AceComm recognizes and translates byte

following into control characters.  ^M translates into a carriage

return, <CR>, (13d).  A character following '^' is either upper

case or made upper case by AceComm, then subtracted by 64

converting 'A' to 1 etc.

'~' translate into 1/4 second delays.

AceComm modem command lines are limited to 40 characters in

length, including the Cmd Terminator.  This is a standard maximum

command length used by modem manufacturers.

Page 30              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Modem Control

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

����� Serial Port Setup Menu ����������ͻ�

�� Port Number Select      1            ��

�� Line Control Setup      19200L,8N1   ��

�� Modem Connect Strings                ��

�� Modem Response Strings               ��

�� Modem Command Strings                ��

���� Modem Control�������������������ͻ ��

���>DTR      On                       � ��

�Ⱥ RTS      On                       �ͼ�

� GPO1     Off                      �

� GPO2     On                       �

� Loopback Off                      �

��Modem Status���������������������ͺ

� CTS      On                       �

� DSR      On                       �

� RI       Off                      �

� DCD      Off                      �

�� Select  Quit �������������������ͼ

--Modem Control--

o GPO2 must be set "ON" for interrupt to seen by the computer

o Normal operation requires LOOPBACK be left "OFF"

These controls are available for experienced users.  The "Modem

Control" menu will allow setting or re-setting each of the bits

listed.  If you want interrupts to continue to be recognized you

shall make sure and leave GPO2 "ON".  GPO2 "OFF" will disable

interrupts for the port, and AceComm will not function properly.

Same with Loop back.  If in doubt, you can use the above snapshot

as a reference to correct bit setting of the modem control

register.

--Modem Status--

o DSR will normally be SET when a modem is powered  up.   AceComm

will alert the user on DSR not SET when installing on a port.

Page 31              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Default Video Emulation

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

����� Serial Port Setup Menu ����������ͻ�

�� Port Number Select      1            ��

�� Line Control Setup      19200L,8N1   ��

�� Modem Connect Strings                ��

�� Modem Response Strings               ��

�� Modem Command Strings                ��

�� Modem Control Setup                  ��

����Default Video Emulation���������������������������ͻ

�Ⱥ�Video Emulation  <>  Associated KTS file����������ͺ

� TTY                   TTY.KTS                      �

� Ansi                  ANSI.KTS                     �

�>Avatar                AVATAR.KTS                   �

� VT102                 VT102.KTS                    �

�� New_Vid  Asso_KTS  Quit  View_Cur_KTS �����������ͼ

o Video Terminal emulation is built into AceComm

o Associated KTS file is the Keyboard Translation Sequence file

that will be loaded for the given Video Type.

o Keyboard Terminal emulation is defined inside of *.KTS files

AceComm includes a separate utility KTS.EXE which allows the

user to define KTS files for any Keyboard Terminal emulation

desired.

AceComm emulates the selected VIDEO type.  "Associated KTS file"

is the .KTS file you want to use with the VIDEO selection.  Each

time AceComm activates a VIDEO type, the associated KTS file is

loaded and becomes active on the keyboard.  KTS keyboard

definitions override MACRO assignments.

KTS (Key Translation Sequence) files contain assignments for

a sequence of characters to a key press. When a key is pressed

from inside TERMINAL area, AceComm first checks to see if the

key press has been defined in the current KTS data. If so,

AceComm sends the assigned sequence of characters to the modem.

The utility KTS.EXE (included) can be used to create new and or

custom KTS files. See the included .ASC examples for defining

and creating KTS files. Using KTS.EXE, the keyboard can be

deigned to emulate any possible combination of characters

available to the system.

Key press assignments defined in the KTS data area are given

top priority, and thus take presides over MACROS assigned to

Page 32              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Default Video Emulation (continued)

the same key.

Page 33              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Key Translation Sequence file (.KTS)

����������ACE.PRM Data  Edit-Menus������VT102.KTS������������

� General Controls                    � TAB           ^I

����� Serial Port Setup Menu ��������ͺ Del/.

�� Port Number Select      1          � Home/7        ^[[H

�� Line Control Setup      19200L,8N1 � Up/8          ^[[A

�� Modem Connect Strings              � Left/4        ^[[D

�� Modem Response Strings             � Right/6       ^[[C

�� Modem Command Strings              � End/1         ^[[K

�� Modem Control Setup                � Down/2        ^[[B

����Default Video Emulation����������ͺ Keypad-0      ^[Op

�Ⱥ�Video Emulation  <>  Associated KT� Keypad-1      ^[Oq

� TTY                   TTY.KTS     � Keypad-2      ^[Or

� Ansi                  ANSI.KTS    � Keypad-3      ^[Os

� Avatar                AVATAR.KTS  � Keypad-4      ^[Ot

�>VT102                 VT102.KTS   � Keypad-5      ^[Ou

�� New_Vid  Asso_KTS  Quit  View_Cur� Keypad-6      ^[Ov

� Keypad-7      ^[Ow

� Keypad-8      ^[Ox

� Keypad-9      ^[Oy

� Keypad--      ^[Om

�>Keypad-*      ^[Ol

�� Select  Quit �������

KTS files contain a sequence of bytes to be sent to the modem

when a particular key press is made.  AceComm emulates a video

type independently from the keyboard emulation.  KTS files may be

edited to emulate any combination imaginable.  KTS files may be

created and assigned to a video type using the "Associate" option

above.

AceComm comes with ready made KTS files.

Notes: .KTS files are compiled from .ASC files.  .ASC files

may be created, maintained, customized with a TEXT

EDITOR in ASCII TEXT mode.  The utility KTS.EXE is

provided for compiling .ASC files into .BIN files.

Rename the .BIN file with .KTS and copy the .KTS file

into the directory pointed to in PRM-data for ".KTS".

Page 34              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments

��D:\ACE\ACE.PRM��������������ķ

�  General Controls            �

�  Terminal Area Options       �

�  Serial Port Setup           �

�>��KeyBoard MACRO Assignments��ķ

� ��Current Keyboard:ACE_DEF.KBDĺ

� �>      F1 - F12               �

� �   ALT-F1 - F12               �

� �  CTRL-F1 - F12               �

� � SHIFT-F1 - F12               �

� �  ALT+KEY Combo               �

� � CTRL+KEY Combo               �

� �  Extended Keys               �

������ Select  Write  Quit������Ľ

o Each of the above key sets is available for assignment to a

MACRO or MACRO stack.  The MACROS provide a wide range of

services.  All of AceComm's functionality is available with

MACROS.

o The "SHIFT-Fn" key-set is used by TAGGER to store TAGGED ITEMS

������������������������������������Ŀ

� Menu options here allow each key   �

� press be re-assigned. The keyboard �

� can be arranged thru the use of    �

� MACROS to resemble software you    �

� are more accustomed to. AceComm`s  �

� keyboard is totaly MACRO driven.   �

� Keyboard definitions are stored in �

� .KBD files. ACE.CTL can be edited  �

� so future upgrades of AceComm will �

� automatically configure with your  �

� keyboard preference.               �

��������������������������������������

--Write--

Prompts for a filename, and writes the current keyboard to a

new KBD file, or updates an existing KBD file of the same name.

When you choose "Write" the following occurs:

AceComm  prompts  for  "filename"

"filename"  is  forced a ".KBD"  extension

"filename" is written in the same directory as ACE.PRM

resides

AceComm writes the contents of "Key MACRO Assignments" menu

Page 35              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

to the file

AceComm re-reads the original KBD file.

In short:

- All changes, once written to a KBD file with "Write", pertain

to the KBD file written to.

- Making changes and not "Writing" them, will be saved to the

current KBD file at program end.

To further explain, all changes made in this menu area to the

keyboard definitions, apply to the current KBD file.  If you exit

the menu area, without choosing "Write", and say "ACE_DEF.KBD" is

active, the changes will be stored to "ACE_DEF.KBD" when you

leave the PRM-data area.  If you choose "Write" from the above

menu, then the changes are stored to a new or existing KBD file,

and do not affect the current keyboard, as AceComm re-loads the

current KBD file after a "Write".  Unless you "Write" to the

current KBD file, which is un necessary, because AceComm will

update it automatically anyway.

"SHIFT-Fn" is the key-set the TAGGER function assigns TAGGED

ITEMS to.

When you tag a word or line from the screen, the tagged item

becomes associated with the first available SHIFT-Fn key by way

of the <SEND_SendStr> MACRO.

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

���KeyBoard MACRO Assignments��ͻ        �

�� F# Keys ����������������������������������������������������

�  1<SendStr>Michael Phelps^M

�  2<SendStr>[FON_REC_PWD]^M

�  3<SendStr>Phoenix, AZ

�  4<SendStr>[PASSWORD]^M

�  5

�  6

�  7<SendAscii> %P

�  8

�  9

� 10<SendStr>password^M

�>11

� 12<SendFile> message.txt

���� Edit  Zap  Quit ������������������������������������������

- Edit

Page 36              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

Allows editing the MACRO assignment.

- Zap

Clears field (Zaps) the MACRO assignment.

Page 37              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

���KeyBoard MACRO Assignments��ͻ        �

�� ALT F# Keys �������������������������������������������������

�  1<ChgDir>FILES <RunProgram> list <ChgDir>..

�  2<RunProgram> list

�  3<RunProgram> dsz rz -rm

�  4<RunProgram> COMMAND /C dir /w %WK

�> 5

�  6

�  7

�  8

�  9

� 10

� 11

� 12

���� Edit  Zap  Quit �������������������������������������������

"ALT-n keys"  key-set that POP UP REFERENCE displays

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

���KeyBoard MACRO Assignments��ͻ        �

�� CTRL F# Keys ������������������������������������������������

�  1<LocalEcho> ON <DIALNUM> 846-2940 <LocalEcho> OFF

�  2<SetDTR> Off <Delay> 5 <SetDTR> ON

�  3

�  4

�> 5

�  6

�  7

�  8

�  9

� 10

� 11

� 12

���� Edit  Zap  Quit �������������������������������������������

Page 38              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

���KeyBoard MACRO Assignments��ͻ        �

�� SHIFT F# Keys �����������������������������������������������

�  1<SENDSTR>80XXX_92.ARJ

�  2

�  3

�  4

�> 5

�  6

�  7

�  8

�  9

� 10

� 11

� 12

���� Edit  Zap  Quit �������������������������������������������

"SHIFT-Fn" key-set the TAGGER function assigns TAGGED ITEMS to.

When you tag a word or line from the screen, the tagged item

becomes associated with the first available SHIFT-Fn key by way

of the <SEND_SendStr> MACRO.

Page 39              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

��ALT+KEY Combo�������������������������������������������������

�>A   <AutoScrGen>

� B   <Back buffer_>

� C   <CaptureFile>

� D   <FonListDialer_>

� E   <HostMode>

� F   <FileLister_>

� G   <FileRequest_>

� H   <MdmHangUp>

� I

� J   <CommandShell_>

� K

� L

� M

� N

� O

� P   <PrmDataMenus_>

� Q   <EndAceComm>

� R   <RunScrFile_>

� S   <ScrnSnapshot_>

� T   <ScrnTag_>

���� Edit  More  Quit ������������������������������������������

PgDn/PgUp toggles display between above and below

Page 40              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

��ALT+KEY Combo�������������������������������������������������

�>U

� V

� W

� X   <EndAceComm>

� Y

� Z   <ClearWindow>

� 1

� 2

� 3

� 4

� 5

� 6

� 7

� 8

� 9

� 0

� -

� =   <DoorWayMode>

���� Edit  More  Quit ������������������������������������������

PgDn/PgUp toggles display between above and below

Page 41              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

��CTRL+KEY Combo������������������������������������������������

�>A

� B

� C

� D

� E

� F

� G

� H

� I

� J

� K

� L

� M

� N

� O

� P

� Q

� R

� S

� T

� U

���� Edit  More  Quit ������������������������������������������

PgDn/PgUp toggles display between above and below

Page 42              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Keyboard MACRO Assignments (continued)

��CTRL+KEY Combo������������������������������������������������

�>V

� W

� X

� Y

� Z

� CTRL-2/@

� CTRL-6/^

� CTRL--/_

� CTRL-BKSP

� CTRL-[/{

� CTRL-]/}

� CTRL-ENTER

� CTRL-\/|

� CTRL-PRTSC

� CTRL-HOME

� CTRL-PGUP

� CTRL-LEFT

� CTRL-RIGHT

� CTRL-END

� CTRL-PGDN

���� Edit  More  Quit ������������������������������������������

PgDn/PgUp toggles display between above and below

Page 43              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Directories in Use Menu

��D:\ACE\ACE.PRM��������������ķ

�  General Controls            �

�  Terminal Area Options       �

�  Serial Port Setup           �

�  KeyBoarD MACRO Assignments  �

�>��Directories in Use Menu���������������������������������

� �>.FON files: D:\ACE

� � .CAP files: D:\ACE

� � .SCR files: D:\ACE

� � .KTS files: D:\ACE

� �  Downloads: D:\ACE

� �   Nodelist: D:\ACE

� �  .REQ/.MLF: D:\ACE

������ Edit  Quit ������������������������������������������

- .FON

Where AceComm expects .FON files to be.

- .CAP

Where AceComm will create and or re-open CAPTURE files.  A

CAPTURE file is a means for "capturing" all port I/O to DISK.

When you open a capture file, AceComm records everything that

comes into the serial port, exactly as it comes in.  No

filtering, no translation of any kind.(1) AceComm does not

overwrite CAPTURE files.  You have the option of renaming the

file, or AceComm will append new data to the end of the

file(2).

- .SCR

Where  AceComm  will  create  and  or look for .SCR files.  The

MACRO <RunScript_> lists this directory.

- .KTS

Where AceComm expects to find it's KTS files.  It's ok to store

your .ASC files in this directory.

- Downloads

This is the directory that AceComm 's internal protocols will

receive into.  AceComm will also list this directory when

building upload LISTFILES.

- Nodelist

When you select the FileRequest option, AceComm expects to find

either (a) a valid RAW FIDO NODELIST and NLIST.IDX file(3).

Page 44              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Directories in Use Menu (continued)

or (b) a .NOD file(4)

- .REQ/.MLF

Mailer  activities.

.REQ files are used when doing File Requests.  When you ask

AceComm to make a File Request, the filename is created using

the NET/NODE number in ascii hex, and the file extensions is

".REQ".

.MLF files (Mail List File) are used when you want to File

Attach to a NET/NODE.  File Attach may be any file(s), and will

also serve as a means for point mail operators to send their

mail bundle to the HOST.

(1) ANSI.EXE is a provided utility that can filter VIDEO CONTROL

SEQUENCES from DISK files.

(2) If you have edited a CAPTURE file, or by other means used any

utility that added the 26d character to the file, AceComm

will not overwrite the END OF FILE character.  This may

result in some editors not recognizing new data that follows

the 26d character.  There exists utilities that can filter

this and other characters from TEXT files.

(3)  NLIST.IDX is a file created using the AceComm utility

IDX.EXE.  IDX.EXE will create a small index into the RAW

FIDO NODELIST file, for all ZONE:NET pairs.

(4) .NOD files are FBL output for a NET.  FBL.EXE is another

AceComm utility.

Page 45              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Executable Utilities

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

� KeyBoard Text Macros                   �

� Directories in use                     �

��� Executable files Menus ����ͻ        �

��>*.ARJ                        �        �

�� *.ARC                        �        �

�� *.LZH                        �        �

�� *.SDN                        �        �

Ⱥ *.ZIP                        ��������ͼ

� *.ZOO                        �

� *.EXE                        �

� *.COM                        �

� *.DOC                        �

� All others                   �

� External Upload Protocols    �

� External Download Protocols  �

���� Select  Quit ������������ͼ

The extensions are used with the following menus, and are called

when a filename has been selected that contains the .EXT as

listed.  Selections are made while in <FileLister_> menu area.

Executable files are then handed over to AceComm's SPAWN

function.  (<RunProgram> equivalent) SPAWN can find files along

the DOS PATH.  SPAWN can also determine a files extension (.COM,

.EXE) SPAWN will use  XMS,EMS,DISK  swapping  if  the  option  is

selected.   SWAP  for  SPWAN  is  selected  from  inside PRM-data

"General Controls" menu.

Page 46              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Executable Utilities (continued)

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

� KeyBoard Text Macros                   �

� Directories in use                     �

��� Executable files Menus ����ͻ        �

����%Fn = *.ARJ����������������������������������������������

���><RUNPROGRAM> ARJ a -s %FN %WK

��� <RUNPROGRAM> ARJ x %FN %WK

��� <RUNPROGRAM> ARJ v %FN %WK

Ⱥ�

��

��� Select  Edit  Quit �������������������������������������

� *.COM                        �

� *.DOC                        �

� All others                   �

� External Upload Protocols    �

� External Download Protocols  �

���� Select  Quit ������������ͼ

Sample menu when a "Highlighted" file's extension of ".ARJ".

AceComm changes to the directory being listed, so that any WORK

done by the utility is done in the LISTED directory.  AceComm

will use the SWAP functions if allowed.  (see SWAP enable flags

located inside "General Controls" Menu Area.

Page 47              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Executable Utilities (continued)

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

� KeyBoard Text Macros                   �

� Directories in use                     �

��� Executable files Menus ����ͻ        �

�� *.ARJ                        �        �

�� *.ARC                        �        �

�� *.LZH                        �        �

�� *.SDN                        �        �

Ⱥ *.ZIP                        ��������ͼ

� *.ZOO                        �

���%FN = *.EXE����������������������������������������������

��><RUNPROGRAM> %FN %WK

�� <RUNPROGRAM> LIST %FN

��

��

��

��� Select  Edit  Quit �������������������������������������

Sample menu for executable files.  Notice that the <RunProgram>

MACRO will be handed the current "Highlighted" filename with the

%FN MACRO.  In the above example the %WK argument will cause

AceComm to wait for a key press afterward.  This allows the user

to view any finishing screens generated from the executable file

argument to <RunProgram>.

This type of MACRO programming allows the user to fire up that

QWK Mail Reader (and what have you) on the fly without exiting

AceComm.

Page 48              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Auto Engage Sequence Triggers

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

� KeyBoard Text Macros                   �

� Directories in use                     �

� Executable files Menu                  �

��Auto Engage Sequence Triggers���������������������������������

��Sequence Trigger -- Responding MACRO STACK��������������������

�>rz^M**^X           <RecvZmo>

� HS*^B              <RunProgram>hslink -P1 <SendStr>^M

� ^`^H^`^H^`^H^`^H   <RunProgram>bimodem bimodem.pth <SendStr>^M

� MPt^`^V^H          <RunProgram>MPT R <SendStr> ^M

�

�

�

�

� **EMSI_REQA77E      <SendEMSI_CLI>

� **EMSI_IRQ8E08      <SendEMSI_ICI>

��Pre/Post AutoM MACROS�����������������������������������������

�

�

���� Edit  Quit  Help �<>v^�������������������������������������

o The last byte in a Sequence Trigger is the sequence scan

trigger byte

o The last byte in a Sequence Trigger should end in an un

common character (as designed above)

o EMSI_REQ detection can allow swift access to a BBS when

mailer software answers the phone

o EMSI_IRQ is sent by BBS software to tell the caller it can do

Interactive EMSI data handshaking

The above defaults have been working very well for the

protocols associated.

- Sequence Trigger

A sequence trigger is a character string that is used by a

remote HOST, protocol etc. as an attempt to engage some

action from the other end.

- Trigger Character

Page 49              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Auto Engage Sequence Triggers (continued)

AceComm scans back thru it's receive buffer for a trigger

sequence with each occurrence of the "last byte of the

sequence".  This means that AceComm will scan for the string

each time it sees the trigger byte arrive from the modem.  For

this reason a sequence trigger will end in a non-common byte.

While 'rz^M' (^M being <CR>, 13 decimal) would be a valid

sequence trigger, we don't want AceComm to search on every

carriage return character that comes in.  Look at Bimodem's

sequence trigger, we don't want to end the sequence with a <SP>

character (^' is the control code for SPACE CHARACTER 32

decimal).

Characters 32(Space) and below need be escape encoded with

the '^' ^A becomes 1, ^B 2, ^C 3, ^M <CR>, ^` 32 <SP> etc.

When entering sequence text, you may simply type the <SP>

character, AceComm will do the conversion for you.

**EMSI_REQA77E

Most FIDONET mailer software will send an EMSI_REQ string to a

caller.  This string is an attempt to determine the caller

type.  AceComm's <SendEMSI_CLI> MACRO tells the mailer this is

a human caller, bypassing the common "Press ESC to enter BBS"

prompt.  DBridge mailer software is in complete and won't

recognize this advanced EMSI handshaking.  All other mailer

software does.

**EMSI_IRQ8E08

IEMSI capable BBS systems will send an EMSI_IRQ string to a

caller.  Detection of EMSI_IRQ is responded to with IEMSI data

handshaking.  AceComm's <SendEMSI_ICI> MACRO handles IEMSI log

on.  (see PRM-data IEMSI Setup menu)

- Responding MACRO Stack

The MACRO defined to the right of each sequence trigger will be

used when a sequence has been detected.  This allows AceComm to

be configured with auto download detection for up to 10

protocols.  MACRO STACKING is valid with AutoM MACROS.

The sending of a carriage return as the last macro is use full

for moving the BBS display upon return from the external

protocol.  AceComm completely disables the serial port before

calling any external programs.

- PreAutoM MACRO

The "PreAutoM" MACRO, if defined, will be processed prior to

the AutoM MACRO about to be processed.  This allows for some

Page 50              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Auto Engage Sequence Triggers (continued)

extra maneuvering to be done which might otherwise be common to

all AutoM MACROS anyway.  For instance the practice of changing

to your "FILES" directory will insure that downloads are

received into the "FILES" directory, even with protocols that

do not allow file redirection.

Notes: If you use PreAutoM MACRO for changing directories,

remember that protocols such as BIMODEM which you may be

up loading with at the same time, will need full path

names to files they are to send.  Also protocols or what

have you that use PRM, CFG etc.  files need know where

to find these control files.

- PostAutoM MACRO

The "PostAutoM" MACRO, if defined will be processed immediately

following the AutoM MACRO.  Again this is some extra MACRO

processing power for those common to all AutoM MACRO jobs.

- To Disable Auto Engage Sequence Triggers

Located in the "Terminal Area Options" is a over ride flag with

which Auto downloads may be completely disabled.  This flag

affects all Sequence Trigger / AutoM MACROS pairs defined.

Auto Engage                            On    <=-ENABLE

Auto Engage                            Off   <=-DISABLE

Page 51              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Auto Log On Sequence Triggers

��Auto Log On Sequence Triggers���������������������������������

��Sequence Trigger -- Responding MACRO STACK��������������������

� is^`your^`name:     <SendStr>[USER_NAME]^M~Y^M~[FON_REC_PWD]^M

� is^`your^`name:     <SendStr>[USER_NAME]^M^M~[FON_REC_PWD]^M^M

�>Host^`Name:        <SendStr>CIS^M~75120,3306^M~[FON_REC_PWD]^M

� first^`name^`?      <SendStr>[USER_NAME]^MY^M[FON_REC_PWD]^M

�

�

�

�

�

�

���� Select  Edit  Null_Selection  Quit  Help �<>v^�������������

Intelligent Auto Log On (Tm)

o Each Fon.Record FLAGS can be toggled to perform auto log on

by either use of a script file, or character sequence trigger

and associated macro stack.

o AceComm responds to a sequence trigger by processing the macro

stack.

o Log on sequence trigger scanning is available immediately after

connect, and until the sequence has been found, or 2 minutes of

on line time has passed.

o When a log on sequence has been processed, or 2 minutes of on

line time has passed AceComm stops scanning for log on

sequences for the remainder of the on line session.

On Dial/Connect Acecomm will, if the AutoL flag for the

current Fon.Record is SET, begin scanning incoming data for

Auto Log On Sequence Triggers. When any of the 10 possible

sequences is found, the sequence's associated macro stack is

processed.

Once a sequence has been found, scanning stops until the next

Dial/Connect.

If a sequence has not been found after the first 2 minutes of

on line time has passed, Intelligent Auto Log On (Tm) is

disabled until the next Dial/Connect.

- Sequence Trigger

A sequence trigger is a character string that is used by a

remote HOST, protocol etc. as an attempt to engage some

Page 52              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Auto Log On Sequence Triggers (continued)

action from the other end.

- Trigger Character

AceComm scans back thru it's receive buffer for a trigger

sequence with each occurrence of the "last byte of the

sequence".  This means that AceComm will scan for the string

each time it sees the trigger byte arrive from the modem.  For

this reason a sequence trigger will end in a non-common byte.

While 'rz^M' (^M being <CR>, 13 decimal) would be a valid

sequence trigger, we don't want AceComm to search on every

carriage return character that comes in.  Look at Bimodem's

sequence trigger, we don't want to end the sequence with a <SP>

character (^' is the control code for SPACE CHARACTER 32

decimal).

Characters 32(Space) and below need be escape encoded with

the '^' ^A becomes 1, ^B 2, ^C 3, ^M <CR>, ^` 32 <SP> etc.

When entering sequence text, you may simply type the <SP>

character, AceComm will do the conversion for you.

- To Disable Auto Log On Sequence Triggers Located in the

"Fon.Record Edit" FLAGS menu is a over ride flag with which

Auto Log On for the record may be completely disabled.  This

flag only affects the currently selected record.

Page 53              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Mass Menu Attribute Changes

��Menus / Colors������������������������ͻ

�>Status line color  <CR> to change      �

� Curtain color                          �

� Help screen color                      �

� Paint Menus                            �

� Update All Menus                       �

� Video Rows (VGA)   Detect              �

��"Update" uses values below������������ͺ

�       Border Attr  113                 �

�       Border Type  1                   �

�       Title Color  30                  �

�      ITitle Color  116                 �

�    Selector Color  112                 �

�  Left Field Color  112                 �

� Right Field Color  112                 �

�        Burst Mode  Off                 �

�      Menu Chatter  Off                 �

�      Menu Whistle  Off                 �

���� Select  Quit �<>v^�����������������ͼ

----------------------------------------------------------------

o Here you can play with the colors for this menu only, unless

you choose the "Update All Menus" option

- Status Line Color

Toggle the AceComm status line back ground color (1 - 7)

- Curtain Color

Toggle color of back drops for various Areas

- Customize

All  changes are limited to the current active menu "Mass Box".

Just as selecting the "Customize" option from any  other  menu,

changes are reflected and remain permanent.

- Help Screen Colors

AceComm's on line help uses 4 separate colors. Certain error

conditions can trigger a help screen to be displayed. When

this occurs, the error message is displayed at the top of the

help screen, followed by the help screen's Title, help text,

and bottom selector bar.

- Update All Menus

Selecting  this  option  write a clone of the active menu "Mass

Page 54              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Mass Menu Attribute Changes (continued)

Box" to *all* AceComm menus.

The values on the right side of the menu are  in  decimal.   They

may  be  copied  and  used  in  the  .CTL file as defaults.  (See

ACE.CTL(1) "Menu Attributes")

��  Customizer        <ENTER>-Select ���������������������������

���� Color_Border  Type_Border  Left_Field  Right_Field   More

................................................................

��  Customizer        <ENTER>-Select ���������������������������

���� Title  ITitle  Selector  Burst On Chatter Off Whistle On

(1)  ACE.CTL  may  be  renamed.   AceComm  package  contains  the

default  ,CTL file ACE.CTL, but it may be renamed to another

filename.

Page 55              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Interactive EMSI data

��Interactive EMSI data������������������������������ͻ

�  User Name Michael Phelps                           �

�      Alias AceComm                                  �

�   Location Phoenix, AZ  USA                         �

�     Data # (602) 846-2940                           �

�    Voice # (602) 846-4563                           �

�>  Password                                          �

�    CRT def ANSI,23,80,0                             �

�  Protocols Zap,ZMO                                  �

� Capability CHT,TAB,ASCII8                           �

�   Requests NEWS,MAIL,FILE,HOT,CLR,HUSH,MORE,FSED    �

���� Edit  Quit  Help �<>v^��������������������������ͼ

o Interactive Electronic Mail Standard Identification (IEMSI)

o IEMSI is an auto log on interface designed to present caller

information to the HOST quickly and automatically. Few BBS

systems currently support IEMSI. RA supports IEMSI log on.

o Interactive EMSI log on function is called with the

<SendEMSI_ICI> MACRO function

o Data defined here will be used by the <SendEMSI_ICI> MACRO

function

o All IEMSI data need not be defined. The BBS system will

prompt the user for any invalid or missing information.

- User Name and Alias

Enter your name as you want it to appear to a BBS system.

There is also a  PRM-data  MACRO  [USER_NAME]  that  uses  this

field.

There is also a PRM-data MACRO [ALIAS] that uses this field.

- Location

City, State, Country

There is also a PRM-data MACRO [LOCATION] that uses this field.

- Data # and Voice #

If  you have no Data #, leave the field blank.  There is also a

PRM-data MACRO [DATA_PNUM] that uses this field.

There is also a  PRM-data  MACRO  [DATA_PNUM]  that  uses  this

field.

Page 56              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Interactive EMSI data (continued)

- Password

This is a default password, any FON.RECORD.PASSWORD always will

override this IEMSI PASSWORD.

There is also a PRM-data MACRO [PASSWORD] that uses this field.

- Birthdate (currently unused)

- CRT def    (current session only)

��CRT Def����������������ͻ

�>AVATAR/0+ / ANSI        �

� ANSI X3.64              �

� TTY (raw mode)          �

���� Select  Quit �<>v^��ͼ

Consists  of  four sub-fields separated by commas, from left to

right:TERMINAL EMULATION,ROWS,COLUMNS,NULS

AceComm fills in the ROWS,COLUMNS,NULS fields automatically

when building IEMSI data packets.

Example CRTDEF: AVT0,24,80,0

TERMINAL EMULATION

Video type

AVT0        (AVATAR/ANSI)

ANSI

TTY

ROWS

Video rows for current session only

COLUMNS

Video columns for current session only

NULS

00h character required after line feeds.

- Protocols

��Protocols������������������������ͻ

�>ZedZap (Zmodem to 8k)        Yes  �

� Zmodem                       Yes  �

���� Select  Quit �<>v^������������ͼ

AceComm internal protocols support ZMO,ZAP fields.

ZMO is normal zmodem

ZAP is zmodem with up to 8k data frames.

- Capability    (current session only)

��Capabilities���������������������ͻ

Page 57              "AceComm & Utilities"(Tm) v 1.7

Chapter 4  PRM-data MENUS AREA

Interactive EMSI data (continued)

�>Can do full screen chat      Yes  �

� Can handle TAB characters    Yes  �

� Can handle 8-bit characters  Yes  �

���� Select  Quit �<>v^������������ͼ

CHT       Can do full screen on line conversation (CHAT)

TAB       Can handle TAB (ASCII 09h) characters

AceComm supports TAB

ASCII8    Can handle 8-bit IBM PC ASCII characters

- Requests      (current session only)

��Requests�������������������������ͻ

�>Show bulletins, anouncements Yes  �

� Check for new mail           Yes  �

� Check for new files          Yes  �

� Hot-keys                     Yes  �

� Screen clearing              Yes  �

� Do not disturb               Yes  �

� Page pausing, `More`         Yes  �

� Full-screen editor           Yes  �

���� Select  Quit �<>v^������������ͼ

NEWS      Show bulletins, announcements, etc.

MAIL      Check for new mail

FILE      Check for new files

HOT       Hot keys "ON"

CLR       Screen clearing

HUSH      Do not disturb

MORE      Page pausing, often referred to as "More"

FSED      Full screen editor

Page 58              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Receiving Units

��Receiver Protocols������������������������������

��Internal Batch Capable��������������������������

�>Zmodem

� Ymodem 128/1024 adobtive

� YmodemG

��Internal Single File����������������������������

� Xmodem 128/1024 adobtive

� Ascii file          XON/CTS

��External Protocols (user defined)���������������

� BIMODEM %WK

� HSLINK %WK

� MPT %WK

� JMODEM %PF %WK

� DSZ port %P speed %B rz -mrr [DIR_DNLDS]

� Setup Menu (External Protocols)

��Auto Engaging Menu������������������������������

� Setup Menu (Auto Engaging)

��EMSI Protocol�����������������������������������

� Recv Mail / File Request

�� Select  Options  Quit  Help �<>v^��������������

AceComm  contains  internal Xmodem, Xmodem 1k, Ymodem, Ymodem 1k,

Ymodem G, Zmodem, and a Zmodem variant ZedZap (8k).

--Internal Batch Capable--

Batch protocols receive the incoming filenames as part of the

file data sent from the sender.  AceComm will append filenames

received from the remote sender to the directory path contained

in the PRM-data area under "FILES".

--Internal Single File--

Single file protocols require a filename be given locally, by

the user.  The user is prompted for a filename between the menu

selection and calling the internal protocol function.

--External Menu--

"Externals" are defined as menu selectable executable that will

become arguments to the <RunProgram> MACRO.

You may find it simpler to assign external protocols to a "Hot

Key" from inside PRM-data Key Definition menu.  This menu is

provided for those who want a "menu driven" affect.

--Auto Engaging Menu--

Page 59              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Receiving Units (continued)

AutoDnlds are protocols whose sending units begin by sending a

documented sequence of characters.  AceComm allows up to 10

separate sequences be defined, and associates each sequence

with a MACRO STACK.  the MACRO STACK can be setup to call the

external protocol associated with the sequence.  "AutoDnLds" is

documented in more detail above.  Shown below is a snippet

reminder display.

�� AutoDnLd Sequence Triggers ���������������������������������

� rz^M**^X            <RecvZmo>

� HS*^B               <RunProgram>hslink -P1 <SendStr>^M

� ^`^H^`^H^`^H^`^H    <RunProgram>bimodem /C \ace <SendStr>^M

� MPt^`^V^H           <RunProgram>MPT R <SendStr> ^M

� at^`is^`your^`name: <SendStr>Michael Phelps^M~^M~PASS^M

See "AutoDnLd Sequence Trigger" menu for more information.

--Recv Mail / File Request--

"FileRequest" is a point mail function.

Selecting File Request function will bring up the menu below

if the PRM-data directory field for NODELIST points to either

- FIDONET NODELIST and compiled NLIST.IDX file

- FBL.EXE output .NOD file

FBL output .NOD file is used in the example below.

���������������������������������������ͻ

�>           1:114/12                   �

� Format =>  ZONE:NET/NODE              �

���������������������������������������ͼ

This menu prompts the user for a ZONE:NET/NODE number of the

system we want to call. Having entered the above "1:114/12"

the menu below is presented.

��1:114/12�����������������������������ͻ

��D:\ACE\fon\�������������������������ͻ�

�  1_105.NOD    19595   05/12/93 10:09 ��

�> 1_114.NOD    11220   05/12/93 10:09 ��

O�  1_125.NOD    7886    05/12/93 10:09 �

�  1_282.NOD    7623    05/12/93 10:09 �

O�  1_382.NOD    10291   05/12/93 10:09 �

�ͳGrab� Quit �<>v^�������������������ͼ

This menu lists FBL output .NOD files found in the directory

field NODELIST.  FBL produces these files and names them

according to their ZONE_NET number.  We want to select the file

1_114.NOD because 1_114.NOD is an extracted listing of all NET

114 zone 1, from the FIDONET NODELIST.  Selecting the 1_114.NOD

file produces the following menu.

Page 60              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Receiving Units (continued)

��1:114/12�����������������������������ͻ

���1:114/12������������������������������������������ͻ

��    System Name ORAC/2                              �

Ⱥ           City Phoenix_AZ                          �

�  Operator Name Eugene_Glover                       �

�   Phone Number 1-602-277-1334                      �

�      Baud Rate 9600                                �

�          Flags H16,CM,XA                           �

� .CFG  Password mailman                             �

�ͳFile_Request� Human call  Poll_for_Mail  Quit ���ͼ

Here AceComm has found the desired node information from the

file 1_114.NOD.

Notice the "Phone Number" field contains a long distance

number.  The following "Dial Filter" statement" contained in

"Point Mail Setup" menu is used to convert the phone number to a

local number.

1-602-

(A)        (B)

Explanation: Field (A) defines what to look for in phone numbers

Filed (B) defines what to replace field (A) with.

Here we left field (B) blank.  AceComm then

replaces "1-602-" with nothing.

Notice the "Password field. The following "Password"

definition was made in "Point Mail Setup" menu.

1:114/12 mailman

The selector bar at the bottom of the above menu contains

several choices. The first being "File Request". Selecting

"File Request" produces the following screen.

��Input Window���������������������

�����������������������������������

�>FileName:  files

�����������������������������������

AceComm is prompting for file names.  The example uses the file

name "files".  Many systems will recognize "files" as a MAGIC

file name, and send an "all files listing" for the system in

response. Pressing <ENTER> alone when finished entering file

names causes AceComm to begin dialing the target system.

���Record Number 1:114/12����������������������������������

Page 61              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Receiving Units (continued)

�>     System Name   ORAC/2

�     Phone Number   1-602-277-1334

OK   �     Line Control   9600L,8N1

�       Video Type

OK   � Control Flags      AutoMACRO,

�      Dial Prefix

�     Log on Script

�     Capture File

�         Password   mailman

�    Load KBD File

�  Log on triggers

��Last Connect  --   Total Connects �����������������������

� 0

��Cycles     --      Dial String                       Last

�   0                ATEQ0X4DT277-1334

� 00:03    00:03     58

�ͳQuit� <SpaceBar>-ReCycle �<>v^��������������������������

----------------------------------------------------------------

CONNECT 9600/ARQ

Address 1:114/12 Using BinkleyTerm-OS/2 Ver. 2.56

Welcome to ORAC/2 - Home of Log/2 (Log for OS/2)

Press <escape> to enter BBS!

ORAC/2

Eugene Glover, Phoenix, AZ USA1-602-277-1334

BinkleyTerm, 9600, CM,XA,V32B,HST

Outgoing  D:\ACE\mail\0072000C.REQ

File Info  7 06/26/1993 11:55

0        7        0 CPS        00:00        ZEOF

CPS                ZDATA

Incoming  c:\fil\orac.lzh

File Info  90401 06/24/1993 20:00

90401        289        1089 CPS        01:23        ZSendHexHdr

TIMEOUT

NO CARRIER

OK

----------------------------------------------------------------

AceComm uses it's text based display for the Point Mail protocol

displays.

Page 62              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Sending Units

��Sender Protocols��������������������������������

��Internal Batch Capable��������������������������

�>Zmodem

� Ymodem

� Ymodem 1k

� YmodemG

��Internal Single File����������������������������

� Xmodem

� Xmodem 1k

� Ascii file          XON/CTS

��External Protocols (user defined)���������������

� BIMODEM bimodem.pth

� HSLINK -P%P @UPLDS.LST

� MPT S @UPLDS.LST %WK

� JMODEM S %PF %WK

� DSZ port %P speed %B sz @[DIR_START]\UPLDS.LST

� Setup Menu (External Protocols)

��EMSI Protocol�����������������������������������

� Send Mail / File Attach

�� Select  Options  Quit  Help �<>v^��������������

AceComm  contains  internal Xmodem, Xmodem 1k, Ymodem, Ymodem 1k,

Ymodem G, Zmodem, and a Zmodem variant ZedZap (8k).

--Internal Batch Capable--

AceComm's internal "Batch" capable protocols are capable of

sending from a LISTFILE.  A LISTFILE is an ascii text file

containing <CR> <LF> terminated filenames.  One filename per

line.  Filenames may contain a full DRIVE:\PATH specifier.

AceComm looks in the current directory for a filename

"UPLDS.LST".  When selecting any of the "Batch" capable

internal sending units, AceComm will always look for

"UPLDS.LST", if the file exists, AceComm displays the following

screen.

�����������������ķ      AceComm always prompts to send the

� Found UPLDS.LST �   from the LISTFILE "UPLDS.LST", when the

� Send contents?  �   "UPLDS.LST" file exists in the directory.

�>(Y)es/(n)o?:    �   Pressing <CR> defaults to (Y)es, (n)o

�����������������Ľ   brings up the regular file selection

menu.

Page 63              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Sending Units (continued)

�������������������ķ      If the (Y)es selection was selected,

� UPLDS.LST         �   AceComm will, after the transfer,

� Delete this file? �   present the menu at left, prompting to

�>(Y)es/(n)o?:      �   delete the LISTFILE, or leave it exist.

�������������������Ľ

The "Regular file selection" menu (below) allows "on the fly"

LISTFILE creation.  The "on the fly" LISTFILE is not named

"UPLDS.LST", but a temporary unique filename is selected.  "On

the fly" LISTFILE is deleted after transfer without any prompt.

��Make Uploads List���������������ķ

�>Batch Tag                        �

� Manual entry                     �

� Finished  /  ESC-key             �

�� Select  Quit ������������������Ľ

Batch Tag

This selection brings up the "FileLister" menu.  This

menu allows tagging filenames to the LISTFILE.  PRM-data

DIR "DNLDS" is the default directory displayed by

"FileLister" when called by the "Sender Protocols" menu.

Manual entry

The  menu  below  is  presented  with the "Manual entry"

selection.

��Make Uploads List����������������ķ

� Batch Tag                         �

���Input Window�����������������������������������������������

��>FileName:  c:\fil\

Ӻ�Input Window<ENTER> alone when done������������������������

�������������������������������������������������������������

The "Manual entry" filename prompt pre pends the prompt space

with any PATH found in PRM-data DNLDS field.

"Batch Tag/ Manual entry" are recorded to a temporary unique

LISTFILE as explained above.  Temporary unique LISTFILE is

automatically deleted after use.

Page 64              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Sending Units (continued)

- Single File

AceComm's internal protocols that send 1 filename per

invocation use the "Manual entry" window shown below.

��Input Window�����������������������������������������������

�>FileName:  c:\fil\

��Input Window<ENTER> alone when done������������������������

�������������������������������������������������������������

--Externals--

This selection brings up a menu listing up to five

configurations for external protocol setup/selection.  This

menu allows storing configurations for external protocols to be

used.  The menu storage below functions the same as any MACRO

stack assigned to a key press.  Here you have a menu where you

may find it easier to to store specific upload protocol

definitions.

�� External Protocols (UpLd) ����������������������������������

�><RunProgram>BIMODEM %WK

� <RunProgram>HSLINK %PF %WK

� <RunProgram>MPT S @UPLDS.LST %WK

� <RunProgram>JMODEM S %PF %WK

� <RunProgram>DSZ sz %PF

�� Select  Edit  Quit �����������������������������������������

Page 65              "AceComm & Utilities"(Tm) v 1.7

Chapter 5  INTERNAL / EXTERNAL PROTOCOLS

Internal, Defined-External Sending Units (continued)

-- Send Mail / File Attach--

This selection allows the user to send files to EMSI capable

mailer software.  The menu shown below prompts the user for a

FIDONET style ZONE:NET/NODE number.

���������������������������������������ķ

�>                                      �

� Format =>  ZONE:NET/NODE              �

���������������������������������������Ľ

The number is required for naming a LISTFILE.  LISTFILE in the

mode uses "MLF" file extension.  (Mail ListFile) AceComm allows

normal Tag or manual entry when adding filenames to LISTFILE.

Those who want to "File Attach" or are running under a HOST as

a point mail system will use this option.  Point mail operators

may upload their mail bundles with this "Send Mail" selection.

AceComm requires a NODELIST or FBL output .NOD file when using

"Send Mail/Attach".  AceComm uses the ZONE:NET/NODE number

entered above for looking up system information about the HOST

to be called.  AceComm is able to retrieve the phone number and

system name from NODELIST/.NOD files.

Page 66              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

�� Telephone_Directory D:\ACE\1_114.FON, 136 Records������������

��      System Name            Phone Number     Line Control Vid

� 1    AceComm Software Support 1-602-846-2940     9600,8N1    A

� 2    ADTBBS                   1-602-840-5611     9600,8N1    A

� 3    ADTBBS                   1-602-840-5611     9600,8N1    A

� 4    AMUG Preferred           1-602-553-0721     9600,8N1    A

� 5    APPS Software Int'l      1-602-947-7678     9600,8N1    A

� 6    AZ Legislature BBS       1-602-253-0065     2400,8N1    A

� 7    AZ MAC UG                1-602-553-0749     9600,8N1    A

� 8    AZ MAC UG 2              1-602-947-0587     9600,8N1    A

� 9    Am Az! West BBS          1-602-843-6574     9600,8N1    A

� 10   Anime Archive            1-602-863-6599     9600,8N1    A

� 11   Answering Machine        1-602-242-3158     9600,8N1    A

� 12   Answering Machine        1-602-242-3158     9600,8N1    A

� 13   Ariseth Keep             1-602-843-0998     9600,8N1    A

�>14   Artemis Lair bbs         1-602-841-4474     9600,8N1    A

� 15   Astroland                1-602-381-8708     9600,8N1    A

� 16   Bob's Place BBS Node 1   1-602-497-2173     2400,8N1    A

� 17   Bob's Place BBS Node 2   1-602-545-8745     9600,8N1    A

� 18   Broadcaster's BBS        1-602-872-9148     9600,8N1    A

� 19   CADE BBS                 1-602-931-3468     2400,8N1    A

� 20   Cactus Bulletin Board Sy 1-602-945-3493     2400,8N1    A

�  Dial  Edit  <SPACEBAR>-Que  Circular_Redial_Que  Fon_files

�� Add_record  Remove_record  UnMark_All  Import  Paint  Quit �<

The Telephone Directory Area lists binary .FON files in their

full ASCII representation.  The current loaded .FON file is

identified at the top of the menu.  Notice that the number of

records contained in the '1_114.FON' file is presented at the top

right.  .FON files are not fixed to any certain record size, and

can have records added or deleted, from here, in the Directory

list area.  Maximum record count for a FON file is 250 records

per FON file.  AceComm supports unlimited FON files, and provides

easy directory List/Grab function for changing FON files.

- Dial

Dial the "highlighted" record.  The function continues dialing

until:

(a) a connection has been established

(b)  USER ABORT.

- Edit

Another menu contains the "highlighted" record's data is

presented for editing, or just a closer view.  All changes are

automatically save to disk.

Page 67              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

- <SPACEBAR>-Que

Mark the "highlighted" record for 'RedialQue'.

- Circular_Redial_Que

Switch into The 'Que' Menu.  At least 1 record needs be marked

"Queued" for AceComm to present this area.  From the 'Que'

Menu, 'Dial' goes into single dial mode, and each record is

dialed 1 time, followed by the next record until all records in

the 'Que' have been dialed.  The 'Que' then revolves around to

the first record and begins again.  When a connection has been

established, the record is removed from the 'Que'.

�� Circular_Redial_Que D:\ACE\1_114.FON, 6 Records��������������

��      System Name            Phone Number     Line Control Vid

�>14 # Artemis Lair bbs         1-602-841-4474     9600,8N1    A

� 15 # Astroland                1-602-381-8708     9600,8N1    A

� 16 # Bob's Place BBS Node 1   1-602-497-2173     2400,8N1    A

� 17 # Bob's Place BBS Node 2   1-602-545-8745     9600,8N1    A

� 18 # Broadcaster's BBS        1-602-872-9148     9600,8N1    A

� 19 # CADE BBS                 1-602-931-3468     2400,8N1    A

�  Dial  Edit  <SPACEBAR>-Zap  Telephone_Directory

�� Add_record  Remove_record  UnMark_All  Import  Paint  Quit �<

-Fon_files

List PRM-data directory .FON, allow selection of new .FON file

- Add_record

Function appends a new record to the current loaded .FON file,

and immediately brings up the 'Edit' Menu.  The new record is

automatically saved.

- Remove_record

The "highlighted" record is removed  from  the  current  loaded

.FON file.  No hole is left, the .FON file is reduced a 'record

size' in bytes.

- UnMark_All

Clear TAG bit from all records in the current FON file.

- Import

"Import" selection brings up the FON file conversion menu

displayed below. The menu provides the following functions.

() Convert another software's FON type file to AceComm FON

file.

() Merge two AceComm format FON files into one file.

Page 68              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

() Initialize a new FON file of video rows number of

blank records.

�� Telephone_directory D:\ACE\fon\ACE.FON, 9 Records������������

��      System Name              Phone Number     Line Control V

�>1  # ACECOMM Support BBS      1-602-846-2940     19200,8N1   A

� 2    CIS (local phoenix 9600) 468-0285           19200,7E1   A

� 3    CIS (phones)             1-800-346-3247     2400,7E1    A

� 4    Comp USA #2   ��Covert to AceComm FON��ͻ   19200,8N1   A

� 5    Supra Modem BB�                         �   19200,8N1   A

� 6  # Comput Connect�> BOYAN 5                �   19200,8N1   A

� 7  # Comp USE (Dall�  Qmodem 4               �   19200,8N1   A

� 8    Leprechan Cast�  Qmodem 5               �   19200,8N1   A

� 9    Xanathar's Cas�  Procomm 2.43           �   19200,8N1   A

�  Dial  Edit  <SPACE�  PcPlus 1               �ue  Fon_files  S

�� Add_record  Remove�  PcPlus 2               �  Paint  Quit  H

�  Telemate 3.01          �

�  TELIX 3.15 - 3.21      �

�  Combine 2 Ace FON      �

�  Create a blank FON     �

���� Select  Quit �<>v^��ͼ

- Paint

Customize the current menu only.  All changes are save

automatically.

- Quit to Terminal Area. (ESC-key)

.FON files

AceComm uses a binary .FON file for storing it's telephone

directory records.  Each record contains data specific to the

system. FON files must be edited inside AceComm, using The Edit

menu.

Page 69              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

- Fon.Record Edit Menu

�� Telephone_directory c:\fil\PHOENIX.FON, 394 Records����������

��      System Name              Phone Number     Line Control V

� 181  Intimidator BBS, The     492 0560           2400,8N1    A

� 182���Record Number 188���������������������������������������

� 183�      System Name  JCCS Node 3 $L

� 184�     Phone Number  582 4299

� 185�     Line Control  2400L,8N1

� 186�       Video Type  Av/An

� 187�    Control Flags  Macro,DTag,Capture,Text,IEMSI,AutoL

�>188�      Dial Prefix

� 189�>    LogOn Script  ACECOMM_.SCR

� 190�     Capture File  ACECOMM_.CAP

� 191�         Password  jccs

� 192�    Load KBD File

� 193�  Log on triggers

� 194��Last Connect  --   Total Connects �����������������������

� 195� 00  04:26:05pm         4

� 196��Cycles     --      Dial String                       Last

� 197�   0               ATEQ0X4DT582 4299

� 198� 00:00    00:00         60

� 199�� Edit  Paint  Quit  Help �<>v^���������������������������

� 200  Lejentia BBS             493 8633           2400,8N1

�  Dial  Edit  <SPACEBAR>-Que  Circling_dial_queue  Fon_files  S

�� Add_record  Remove_record  UnMark_All  Import  Paint  Quit  H

- System Name            AceComm Software Support

The name defined here, is presented onto the revolving status

line (right side of status line) when "on line".

- Phone Number           1-602-846-2940

This  phone  number, although long distance, can be modified by

AceComm when dialing, with use of "Dial" statements in the

Point Mail Menu.

Example: lets say we are local Phoenix, where our area code is

602, we have an entire phone directory of long

distance numbers.  The following dial statement

defined inside "Point Mail Setup" will cause AceComm

to translate all of the long distance numbers to local

numbers.

Dial 1-602-

AceComm will see this definition and translate it into

Page 70              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

anything following the definition.  In this case, we

did not put anything after the "1-602-", so AceComm

will strip "1-602-" from all phone numbers that

contain "1-602-" as part of the number.  This is valid

for all phone numbers that are processed thru the

AceComm Dialer Menu.  Mailer mode, File Requests which

get their phone numbers from either a FIDO NODELIST,

of FBL output .NOD file do use the Dialer Menu and do

receive number translation from "Dial" statements.

- Line Control           2400L,8N1

Most remote HOSTS will be set up using 8 data bits, No parity,

and 1 stop bit.  Compuserve uses 7 data bits, Even parity, and

1 stop bit.  AceComm will configure the UART to assemble the in

coming bit stream into characters, based on the configuration

defined here.  Baud rate means nothing when LOCKBAUD is set, as

is with the "L" appended to baud in this example.  Other wise,

when LOCKBAUD is not set, AceComm will set the UART up to the

Fon.Record baud rate.

Note: When LOCKBAUD is not set, AceComm is also capable of

resetting the baud rate to a connect string.  If you dial

out at 2400, and connect at 9600, and 9600 is detected in

the the connect string, AceComm will automatically set

the baud to 9600.  If LOCKBAUD is set, AceComm leaves

baud alone.

- Terminal Emulation     Av/An

This option will present the Terminal emulations menu, you may

select the video ,type you want associated with this record.

"Av/An means AVATAR and ANSI video emulation.

- Control Flags  Macro,DTag,Capture,Text,IEMSI,AutoL

��   Control Flags����������������ͻ

� Tag dial attempts    On          �

� Auto log on method   Macro,      �

� Open Capture file on On          �

� Capture mode         Text,       �

�>Allow IEMSI log on   On          �

� Allow auto log on    On          �

���� Edit  Quit �<>v^�������������ͼ

"Script"  when  set,

AceComm will attempt to load and run the filename defined next

to "AutoLogOn Script".  If a script file is not found, AceComm

ignores the error and continues normal operation.

Notes: .SCR files are looked for in the directory defined

Page 71              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

inside PRM-data Area "Directories"

"Macro"  when  set,

AceComm will enable the defined Auto Log on Sequence Triggers

and corresponding MACRO stacks.

"DTag"  when  set,

AceComm will tag un-successful dial attempts (busy, etc.) to

the Que Dialer.  An "*" character appears to the left of a

Fon.Record's display line, when it is either Q tagged or, D

tagged.  When this flag is not set, the only other way to Q tag

a record is by manually selecting the menu option

"<SPACEBAR>-Que" from the fon list display.

"Capture"

When this flag is selected, a capture file is opened upon

Dial/Connect.  Any previously open capture file is first closed.

"IEMSI"

Allow Interactive EMSI log on

"AutoL"

Allow auto log on methods (script/macro)

- Dialing Prefix

The Fon.Record dial prefix overrides The PRM-data default

prefix.  If this field is null (blank), AceComm uses the

PRM-data default dial prefix.

- AutoLogOn Script       ACE_S.SCR

When SCRIPT flag is set, for the current Fon.Record, AceComm

will use this filename, opening it as a SCRIPT file, and run

the contents.  This action takes place upon successful

Dial/Connect.  If the filename does not exist, AceComm simply

goes about it's business, and you will log on normally.

AceComm will look for the SCRIPT filename in PRM-data DIR .SCR

path.

- Capture File           ACE_S.CAP

Default filename presented to user when opening a "capture data

to file" file.

- Password               my/password

See also: Expandable MACRO Parameters <SendStr> [FON_PASSWORD]

would replace "[FON_PASSWORD]" with "my/password" for this

example.  You may define the Fon.Record PASSWORD field to any

character combination, the field maximum length is 19

Page 72              "AceComm & Utilities"(Tm) v 1.7

Chapter 6  TELEPHONE DIRECTORY AREA

characters.

- Custom .KBD File       MY.KBD

KeyBoard File, AceComm will first remember what the current KBD

filename is, load the Fon.Record's custom KBD file, if defined,

and finally restore the original KBD file after session

disconnect.  Say What?

You are about to log onto Compuserve.  You have CIS.KBD file

defined for the Fon.Record.  When dial/connect is detected,

AceComm will load the contents of CIS.KBD, overlaying your

current keyboard MACRO assignments.  When you log off of

Compuserve, and AceComm detects the carrier drop (no DCD),

AceComm will load back the original keyboard file, overlaying

your custom CIS.KBD.

You will need to distinguish KBD files from KTS files.  Where a

KBD file overlays the keyboard MACRO assignments.  The KTS file

defines the ultimate sequence that will be sent to the modem

for a given key-press.  Keys assigned a MACRO, are processed as

MACRO functions.

Keys not assigned a MACRO(s) will be sent to the modem, and are

candidates for KTS.

Keys with extended codes, are sent as they would be seen in

DoorWay mode.  Normal keys (a-z,A-Z,1-0,CTRL A-CTRL Z, etc.)

are sent in their normal lower ascii representation (as you

would expect).  Now, any of these key-presses that have been

defined inside the KTS file, are translated into their defined

sequence.  AceComm & Utilities provides ASCII examples for

compiling KTS files, as well as KTS.EXE the custom Keyboard

Sequence Translation (Tm) compiler.

- Auto Log on Sequence Trigger/MACRO Stack

Each Fon.Record can be assigned to one of the Auto Log on

definitions available from a menu when selecting this option.

AceComm will respond to a sequence trigger by processing the

MACRO stack. Log on sequence trigger scanning is available

immediately after connect, and until the sequence has been

found. When a log on sequence has been processed, AceComm

stops scanning for the log on sequence.

Page 73              "AceComm & Utilities"(Tm) v 1.7

Chapter 7  INTERNAL FILE LISTER

��D:\ACE������������������������������ͻ

�> 1_114.FON    26368   06/27/93 07:25 �

�  ACE.ASM      8269    06/26/93 18:00 �

�  ACE.CTL      13589   06/25/93 08:04 �

�  ACE.EXE      196102  06/27/93 07:24 �

�  ACE.FON      1600    06/27/93 04:13 �

�  ACE.HLP      44572   06/27/93 07:09 �

�  ACE.LOG      3694    06/27/93 07:29 �

...

�  ACTL.ASM     101998  06/25/93 14:22 �

�  Utility  New_Dir  Change_Drive      �

�� Tag  Delete  Quit  Help �<>v^������ͼ

- Utility

This selection brings up a sub-menu of pre-defined executable.

AceComm contains several sub-menus for this purpose.  Sub-menu

selected is determined by the "highlighted" files's extension.

The %FN symbol is documented in MACRO.DOC.  %FN is an

Expandable MACRO Function.  %FN argument to a MACRO from

anywhere inside the "File Lister" menus is replaced with FULL

PATH NAME for the current high lighted filename.

��C:\DOC������������������������������ͻ

�  A.SNA        1242    06/27/93 07:53 �

�  ACE.BAK      117893  06/26/93 12:25 �

�  ACE.CHA      2706    06/27/93 07:46 �

�>��%FN = C:\DOC\ACE.DOC����������������������������������

� � <RunProgram> BB %FN

� �><RunProgram> LIST %FN

� � <RunProgram> QEDIT %FN

� � <RunProgram> GREP %PF %FN %PF

� � <RunProgram> PATCH %FN

� �� Select  Edit  Quit �<>v^�����������������������������

�  COVER        213     06/25/93 16:22 �

�  CREDIT.CRD   2918    06/25/93 15:30 �

...

�  FILE_ID.DIZ  392     06/26/93 13:14 �

�  Utility  New_Dir  Change_Drive      �

�� Tag  Delete  Quit  Help �<>v^������ͼ

- New_Dir

Allows changing directories.  AceComm changes directories right

along with the display.  When you use AceComm's  "Utils"  Menu,

work is done in the currently listed directory unless otherwise

redirected   via   MACRO   arguments   or  overriding  external

Page 74              "AceComm & Utilities"(Tm) v 1.7

Chapter 7  INTERNAL FILE LISTER

executable configurations.

Notes: AceComm always returns to the starting  directory  after

leaving the FileLister Area.

��D:\ACE��������������ͻ

��D:\ACE����������ͺ>>D:\ACE              �

�> 1_114.FON    263� >..                  �

�  ACE.ASM      826� >CAP                 �

�  ACE.CTL      135� >CVTFON              �

�  ACE.EXE      196� >FILES               �

�  ACE.FON      160� >FON                 �

�  ACE.HLP      445� >KTS                 �

�  ACE.LOG      369� >MAIL                �

�  ACE.MAP      920� >SCR                 �

�  ACE.OBJ      271� >SND                 �

�  ACE.PRM      378� >XTRA                �

�  ACEBOX.INC   345� >ZEN                 �

�  ACEM         253� >F                   �

�  ACEORDER.FRM 588�� Select  Quit �<>v^�ͼ

�  ACET.SCR     809     05/17/93 21:33 �

...

�  ACTL.ASM     101998  06/25/93 14:22 �

�  Utility  New_Dir  Change_Drive      �

�� Tag  Delete  Quit   Help �<>v^�����ͼ

- Change Drive

Allows listing another drive.

��D:\ACE���������������������������������ͻ

�> 1_114.FON    263�>A                    �

�  ACE.ASM      826� B                    �

�  ACE.CTL      135� C                    �

�  ACE.EXE      196� D                    �

�  ACE.FON      160� E                    �

�  ACE.HLP      445�� Select  Quit �<>v^�ͼ

�  ACE.LOG      3694    06/27/93 07:29 �

...

�  ACTL.ASM     101998  06/25/93 14:22 �

�  Utility  New_Dir  Change_Drive      �

�� Tag  Delete  Quit  Help �<>v^������ͼ

- Tagged files

Tagged files are stored in the file "UPLDS.LST" when tagged from

inside the <FileLister_> MACRO.  When tagged from any of

AceComm's internal protocol sending units, tagged filenames are

stored into a temporary file.  The temporary file is deleted

after each use.  UPLDS.LST files are not deleted, but instead

Page 75              "AceComm & Utilities"(Tm) v 1.7

Chapter 7  INTERNAL FILE LISTER

stored ready.

AceComm will look for UPLDS.LST when any internal batch capable

protocols are selected.  If found, AceComm will ask the user is

sending the contents of UPLDS.LST is desired, if (Y)es, AceComm

will attempt to send all filenames listed in the UPLDS.LST file,

if (n)o, AceComm will present the the File Input' menu.

��D:\ACE������������������������������ͻ��UPLDS.LST������������

�  1_114.FON    26368   06/27/93 07:25 ��>D:\ACE\1_114.FON

�> ACE.ASM      8269    06/26/93 18:00 ��

�  ACE.CTL      13589   06/25/93 08:04 ��

�  ACE.EXE      196102  06/27/93 07:24 ��

�  ACE.FON      1600    06/27/93 04:13 ��

�  ACE.HLP      44572   06/27/93 07:09 ��

�  ACE_MAIL.CFG 430     06/22/93 14:23 ��

...

�  ACTL         591     06/22/93 14:08 ��

�  ACTL.ASM     101998  06/25/93 14:22 ��

�  Utility  New_Dir  Change_Drive      ������������������������

�ͳTag� Delete  Quit  Help �<>v^������ͼ

- Delete

Delete option presents a full path spec for the high lighted

filename, prompting confirmation before deletion.

��D:\ACE������������������������������ͻ

�  1_114.FON    26368   06/27/93 07:25 �

�>�������������������ͻ 06/27/93 07:31 �

� � D:\ACE\A.SNA      � 06/26/93 18:00 �

� � Delete this file? � 06/27/93 07:31 �

� �>(Y)es/(n)o?:      � 06/25/93 08:04 �

� �������������������ͼ 06/27/93 07:24 �

�  ACE.FON      1600    06/27/93 04:13 �

�  ACE.HLP      44572   06/27/93 07:09 �

�  ACE_DEF.KBD  11120   06/27/93 07:29 �

...

�  ACE_MAIL.CFG 430     06/22/93 14:23 �

�  Utility  New_Dir  Change_Drive      �

�� Tag  Delete  Quit  Help �<>v^������ͼ

Page 76              "AceComm & Utilities"(Tm) v 1.7

Chapter 8  POINT MAIL FUNCTIONS

��Point Mail Setup������������������������������������ͻ

�>    Address 1:114/35                                 �

�    Sys Name AceComm Support                          �

�        City Phoenix, AZ                              �

�    Operator Michael Phelps                           �

�   Sys Phone (602) 846-2940                           �

�        Baud 14.4                                     �

�       Flags v32b                                     �

� Dial filter                                          �

� Dial filter                                          �

� Dial filter                                          �

� Dial filter 1-602-451 1-451                          �

� Dial filter 1-602-                                   �

�    Password 1:114/12 mailman                         �

�    Password 1:114/7 doorman                          �

�    Password 1:114/116 pdnman                         �

�    Password                                          �

�    Password                                          �

�   Nodelist: D:\ACE\fon                               �

�  .REQ/.MLF: D:\ACE\mail                              �

���� Edit  Quit �<>^v���������������������������������ͼ

EMSI handshake data

The following data is presented to the remote mail system

during EMSI handshaking.

Address

Sys Name AceComm Support

City Phoenix, AZ

Operator Michael Phelps

Sys Phone (602) 846-2940

Baud 14.4

Flags v32b

Password  --  Pre arranged between two systems.

The following example will cause AceComm to fill in the EMSI

data field for "password" with 'mailman' when the target

system's address is 1:114/12.

Password 1:114/12 mailman

Dial filter  --  AceComm will use Dial statements to modify

phone numbers. Dial filter statements are used when dialing

from inside of AceComm's phone directory, and or with mail

functions.

A long distance BBS directory list can be used without

editing phone numbers.

Page 77              "AceComm & Utilities"(Tm) v 1.7

Chapter 8  POINT MAIL FUNCTIONS

The following dial statement filter "1-602-" from all phone

numbers the  string is found in.

Dial filter 1-602-

The following dial statement converts "1-602-451" found in

phone numbers to "1-451".

Dial filter 1-602-451 1-451

Dial statements are required when using point mail

functions, because NODELIST phone numbers contain area

codes. AceComm uses dial statements to know how to modify

phone numbers.

Point mail functions look in the PRM-data directory "NODELIST"

for systems directories.  Point mail functions require either

of two types systems directories.

Systems directory type (FIDO  NODELIST)

FIDO NODELIST is a systems directory that most EMSI capable

HOST systems use.  AceComm can use a FIDO NODELIST.  To use

the FIDO NODELIST, you must first compile an INDEX file.

AceComm uses NLIST.IDX.  NLIST.IDX contains a quick look up

table for Zone:Net/Nodes in the FIDO NODELIST.  The AceComm

package provides IDX.EXE as a utility to create the

NLIST.IDX file.  NLIST.IDX and the NODELIST are to be found

inside of the PRM-data directory "NODELIST".

Systems directory type (FBL.EXE output .NOD file)

.NOD file is a NET extraction from the FIDO NODELIST.

FBL.EXE (provided) is capable of creating this file.  .NOD

files contain a listing for a single NET.  .NOD files are

very small and may be preferable for point mail use where

calling 1 or a few pre know NETS is intended.  .NOD files

are to be found inside of the PRM-data directory "NODELIST".

Page 78              "AceComm & Utilities"(Tm) v 1.7

Chapter 9  MOUSE SUPPORT

Support for a pointer device has been implemented into AceComm

using the Microsoft Mouse Driver functions. Your Microsoft

compatible mouse should work with AceComm.

-- Mouse in Terminal Area --

o Pressing the left mouse button sends <CR>

o Pressing right mouse button sends the character the mouse

cursor is highlighting.

From Terminal Area, a one line mouse menu is presented on the top

row of the screen.  This menu presents several of the more

commonly used MACRO functions otherwise available from the

Terminal Area with a key press.

Placing the mouse cursor over a menu choice and pressing the left

button selects the menu item.

The menu can be toggled off from the PRM-data Terminal Options

menu.

The mouse can be used for Character I/O with the HOST.  The left

button sends <CR> character.  The right button sends the current

highlighted character.

-- Mouse in Menu Areas --

o Pressing the left button selects an item.

o Pressing the right button is the same as <ESC> or Quit

Horizontal and vertical sensitivity can be adjusted from

inside the PRM-data general controls menu.

The mouse detection can be toggled "OFF" from inside the

PRM-data General Controls menu.

Turning "OFF" mouse detection completely disables AceComm

from any mouse function related use, including Terminal Area

mouse menu.

Page 79              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

- ACE.PRM

ACE.PRM is a binary file which contains all of AceComm's

memory, so to speak.  When AceComm is started it reads the

ACE.PRM file.  All your serial data, modem data, the keyboard

assignments, menu-usage, customizing and so on is stored in

ACE.PRM.  PRM-data Area edits the ACE.PRM data, and when

AutoSave is "ON", all data is saved each time you leave

PRM-data Area.  When AceComm is terminated, the PRM-data is

saved to ACE.PRM.

- ACE.CTL

ACE.PRM can be rebuilt using the ACTL.EXE utility.  ACTL.EXE

reads and formats a .CTL file into ACE.PRM.  A default .CTL

file comes with the AceComm package.  The default filename is

ACE.CTL.

Page 80              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

KTS files

- .KTS/.ASC/.BIN files

Key Translation Sequence files are used when you want a key

press to be translated into a sequence of characters.  A good

example of KTS is in the ANSI.KTS file.  Here, keys such as Up

Arrow are defined to send an ansi escape sequence that the

HOST'S video can read, and bounce back, decrementing the cursor

position by 1.

A utility KTS.EXE is provided with the AceComm package, which

allows you the user to compile .ASC files into .KTS file.

You can look at the included .ASC files, that come inside

KTS_SFX.EXE, as an example to work from.  The ANSI.ASC file was

compiled by KTS.EXE into ANSI.BIN.  ANSI.BIN is exactly the

same as ANSI.KTS.  The .BIN extensions is used so that a

working .KTS file is not overwritten.  You need to rename .BIN

files to .KTS when ready for use.

Notes: Keys assigned KTS are processed only if not defined in

KBD.  When a key is assigned to a MACRO(s), the MACRO(s)

take priority and are processed.  Do not assign MACRO(s)

to key-presses you want to KTS.

.KTS has it's directory definition optionally defined inside

PRM-data area.

Page 81              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

KBD files

- .KBD

KBD files give the user ability to keep more than one keyboard

layout for AceComm operations.

KBD KeyBoarD files contain the MACRO assignments for the

keyboard.  When AceComm is first installed, the file

"ACE_DEF.KBD" is used as a default keyboard (KBD).  When

AceComm is started, it reads the "current" KBD file.  "Current"

KBD file is the default ("ACE_DEF.KBD" or the last KBD "Load"ed

from inside the PRM-data menu area.

When you "Load" a KBD file from inside PRM-data area, The KBD

file becomes "current".  This means that any changes you make

to the key definitions are saved to the "current" KBD file.

Updating the "current" KBD file is done automatically when

leaving PRM-data area when "AutoSave" is "ON" (default).  When

"AutoSave" is "OFF" you need to manually select the "Save"

option from the main PRM-data menu.  Do not confuse "Save"

with "Write".  "Write" is an option found inside the "KeyBoarD

MACRO Assignments" menu.  "Write does not save changes to the

"current" KBD file, but prompts the user for a filename, and

creates or overwrites a new or existing KBD file.  Then "Write"

re-loads the "current" KBD file.

KBD files are also used with Fon.Records.  The AceComm package

comes with a "CIS.KBD" file example.  The ACE.FON file, also

provided with the AceComm package, contains a record for CIS

(local Phoenix).  This Fon.Record (CIS) has defined as a KBD

file, "CIS.KBD".  When dialing a Fon.Record that contains a KBD

file definition, AceComm will upon successful Dial/Connect,

load the KBD file associated with the Fon.Record.  This custom

KBD file is valid for the time that AceComm detects CARRIER

(DCD), or the user loads another KBD file from inside the

PRM-data area.  When AceComm detects that the Dial/Connect call

is over, (DCD drops),

AceComm will:

Write the Fon.Record's KBD file, saving any

changes that may have been made to it while on

line.

Re-load the KBD file that was in affect before the

connection was established to system that had a

KBD file associated with it's Fon.Record.

Page 82              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

KBD files (continued)

KBD files may be associated to a Fon.Record from inside the

telephone directory area, "Edit" option.

Page 83              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

REQ/MLF,BCH,SCR,FON files

- .REQ/MLF

Filenames created with ".REQ" extensions are used when making

File Requests on EMSI FIDONET style systems.  REQ files are a

list of <CR> <LF> terminated filenames.  This REQ file is sent

to the system the File Request is being made on, the system

will send from the list.

Filenames created with ".MLF" extensions are used for

Attaching (sending) files to EMSI FIDONET style systems.  MLF

files are a list of <CR> <LF> terminated filenames.  AceComm

sends from the list (LISTFILE).

.REQ/MLF has it's directory definition optionally defined

inside PRM-data area.

- .LST

AceComm creates LISTFILES with .BCH extensions.  A .BCH

extended LISTFILE is created from inside the "FileLister"

(<FileLister_> MACRO) area.  Filenames "Tagged" from inside

"FileLister" are stored inside a file "UPLDS.LST".  AceComm's

internal batch protocols will send from "UPLDS.LST" by default,

you do have the option of creating another LISTFILE, or

entering filenames manually.

- .SCR

AceComm forces ".SCR" file extensions onto script files.

Script files may be generated automatically using AceComm's

AutoScriptGen (Tm).  Script files may be loaded from the

terminal area, or used as AceComm's only argument when starting

AceComm.  Script files may be associated with

Fon.Records from inside "Edit" option.  Script files associated

with a Fon.Record are:

Upon successful Dial/Connect AceComm creates an "Event" just

prior to exiting back to terminal Area.  The "Event" is to run

the Script file associated with the Fon.Record.  Typically,

this Script file will contain some auto log on MACROS.  The

Script plays out, end of "Event"

.SCR has it's directory definition optionally defined inside

PRM-data area.

- .FON

Telephone directory files are forced the ".FON" file

Page 84              "AceComm & Utilities"(Tm) v 1.7

Chapter 10  ACECOMM DATA FILES

REQ/MLF,BCH,SCR,FON files (continued)

extensions.  All Fon.Records are stored inside of FON files.

AceComm FON files can hold up to 250 records each.  AceComm

allows unlimited FON files.  Selection of FON files is made

available from inside the Fon.List area using "Fon_Files"

option.

.FON has it's directory definition optionally defined inside

PRM-data area.

Page 85              "AceComm & Utilities"(Tm) v 1.7

Chapter 11  WARRANTY

DISCLAIMER - AGREEMENT

Users of AceComm must accept this disclaimer of warranty:

AceComm is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the

warranties of merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or conse-

quential, which may result from the use of AceComm.

Page 86              "AceComm & Utilities"(Tm) v 1.7

Chapter 12  LICENSE

AceComm is not a Public Domain program and is not free.  AceComm

is copyright (C) 1990-1993 by MGP Programming Services.

Non-registered users of this program are granted a limited 30 day

license to evaluate the programs suitability for their

requirements.  Any usage of AceComm beyond the evaluation time

period requires registration of each copy of the program used.

Use of non- registered copies of AceComm beyond the original

evaluation period is prohibited.

AceComm may NOT be modified in any respect, for any reason,

including but not limited to, de-compiling, disassembling, or

reverse engineering of the program.  The opening title screen,

help screens, and all other proprietary program output must never

be altered, removed, bypassed or modified by any means.

You are free to distribute the PUBLICLY AVAILABLE evaluation

version of AceComm to others subject to the above restrictions

and also the following:

A.   No fee is charged for its use.

B.   No re numeration may be accepted for AceComm.  This does

not apply to computer access charges the system

operators (Sysops) of or organizations owning bulletin

board systems, on line services, etc...  may charge

subscribers.

C.   AceComm must be copied in unaltered form, complete with

files containing license information, the FULL

documentation and all accompanying files.  The

self-extracting archive distributed by MGP Programming

Services must not be altered in ANY respect.

D.   All shareware houses/distribution firms must make

explicitly clear that the diskette purchase containing

any shareware program has NOT registered the software

with the author MGP Programming Services Software/Michael

G Phelps.

Page 87              "AceComm & Utilities"(Tm) v 1.7

Chapter 12  LICENSE

System Operators may make AceComm available for download only if

the above conditions are met.  The archive (Acennnn.EXE)

distributed by AceComm or an equivalent archive may be made

available only in complete form.  Refer to FILELIST.DOC to verify

complete package contents.  If the contents appear altered or not

complete, you may obtain the latest release of AceComm directly

from MGP Programming Services by sending a formatted diskette

labeled with the word "AceComm".  Please include $5 for handling.

The latest version of AceComm is also available on Compuserve.

Commercial distributors of "Public Domain", "Shareware", and/or

User Supported software may distribute AceComm subject to the

above conditions only after obtaining WRITTEN permission from MGP

Programming Services.  This condition statement supersedes all

previous agreements.

Please refer to the section entitled registration/ordering

section for additional information on registration, corporate

site-licensing and related topics.

Page 88              "AceComm & Utilities"(Tm) v 1.7

Chapter 12  LICENSE

The above license statement does not apply to the REGISTERED

version of AceComm.  The registered software of MGP Programming

Services is protected under United States Copyright and Trademark

Laws.  It must be treated just like a book with certain

exceptions as follows:

A.  MGP Programming Services authorizes the making of archival

copies of the registered software for the sole purpose of

backing-up your software and protecting your investment

from possible loss.

B.  The medium on which the registered software is recorded is

transferred to the customer, but not the title to the

software.

C.  The customer may resell or distribute unmodified copies of

the registered software provided the customer has

purchased from MGP Programming Services one copy of the

registered software for each one sold or distributed.

The provisions of this software license shall also be

applicable to third parties receiving copies of the

registered software from the customer.

D.  By saying, "just like a book", MGP Programming Services

means that the registered software may be used by any

number of people and may be freely moved from one computer

location to another so long as there is ABSOLUTELY NO

POSSIBILITY of it being used at one location while being

used at another.  Just like a book that cannot be read by

two different people in two different locations at the

same time.

Page 89              "AceComm & Utilities"(Tm) v 1.7

Chapter 13  ACECOMM User Support

Contacting the author

You can get more help by contacting th

author Michael Phelps at the following

- VOICE 602-846-4563 6 days 12-7pm MST

- BBS   602-846-2940 24 hours

- COMPUSERVE User ID 75120,3306

- FIDONET ADDRESS 1:114/35

- FIDONET ACECOMM ECHO

I distribute the "ACECOMM" echo to

USA NODES, until it makes BACKBONE.

- Write to: AceComm Software Support

3003 North Central Ave

Suite 121-171

Phoenix, AZ  85012  USA

Page 90              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

Registering AceComm gives you access to one of the most

powerful communications programs available.

o  Registering AceComm allows you to legally use the product after

the trial period.

o  You need only Register AceComm 1 time

o  Registration gives you a license to use the AceComm

package for un limited use, and the following advantages.

- Up to date printed bound manual

- Major Version upgrade notices sent

- Commercial AceComm on Disk

Eventually Registered copies of AceComm versions will run 1

year or more ahead of the publicly available "try before you

buy" versions.  AceComm will continue to provide "pace

setting" features, "intelligent software technology" making

the most of your on line time.

- Unlimited software support  (See SUPPORT.DOC)

- 1 Free upgrade to a version of your choice

The FREE upgrade package includes the above and can be

obtained at any time down the road by sending in an AceComm

order form with the FREE upgrade information line filled out.

Most importantly, registered users know they are helping to make

sure that high-quality software like AceComm continues to be

offered at low cost.

To  place  an  order,  please  use  the  order  form contained in

the file "ACEORDER.FRM".

Page 91              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

AceComm & Utilities (Tm)

Version  1.7

ORDER FORM

FROM:

Name: ____________________________________________________

[Company]: ____________________________________________________

[Title]: ____________________________________________________

Address: ____________________________________________________

City: ____________________________________________________

State: ___________________________ Zip Code: ______________

Country: ___________________________ Phone: _________________

TO:

AceComm Software

3003 N. Central Ave. #121-171

Phoenix, AZ    85012   USA

���������Ŀ

���������Ŀ     �              (R)

�ĳ         �o    �������������������

�   ���������Ŀ �  Association of

�   �         ���  Shareware

���ĳ    o    �    Professionals

�����ĳ    �    ���������������������

�����������    MEMBER

Page 92              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

AceComm & Utilities (Tm)

Version  1.7

ORDER FORM

Telephone   (602) 846-4563     AceComm Support Services

AceComm BBS (602) 846-2940     3003 N. Central Ave Suite 121-171

CompuServe  75120,3306         Phoenix, AZ  85012  USA

================================================================

Individual Registration Licensing use of AceComm and Utilities

- commercial version software  & documentation on diskette

- printed bound manual, upgrade notices)

- 1 upgrade sent FREE (you choose the version)

- all future upgrades at very low cost of 20%

1'st FREE & all future upgrades include disk/printed manual

for MS-DOS:   1 Purchase at $45 . . . . . . . . . . . . . ______

================================================================

INTERNAL SITE LICENSE for the use of AceComm & Utilities for DOS

- commercial version software  & documentation on diskette

- 1 upgrade sent FREE (you choose the version)

- all future upgrades at very low cost of 20%

Quantity Registrations

----------------------

10 - 50 ..         at $32 each, nbr computers ___x 32 _____

50+                call for special discount

custom features available

call for quantity manual pricing

================================================================

Overseas add $7.00/order  for shipping and handling        _____

Diskette format (choose one)  5.25" disk (__)  3.5" disk (__)

================================================================

TOTAL PRODUCTS                                             _____

Sales tax for Arizona residents                   6.5%    _____

TOTAL ORDER                                                _____

For payment by non-USA BANK CHECK, add $40.00              _____

Payment method:                            TOTAL ENCLOSED  _____

( ) Check           ( ) Cash

( ) Money order     ( ) Send FREE Upgrade

Page 93              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

( ) Credit card - See instructions in file CREDIT.CRD

================================================================

AUTHORIZED SIGNATURE: __________________________________________

TITLE: ____________________________________  DATE: _____________

Page 94              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

AceComm & Utilities (Tm)

Version  1.7

Credit Card Order Form

As a service to AceComm customers who want to register by  credit

card,  arrangements  have  been  made  with  the  Public Software

Library (PSL) to  handle  all  credit  card  registrations.   PSL

accepts Visa, MasterCard, American Express, and Discover Card.

Non-U.S.   residents  are  encouraged to register by credit card.

In many  cases,  paying  by  credit  card  will  be  easier  than

acquiring U.S.  funds.

PSL will take the order information and forward it to the AceComm

author,   MGP   Programming  Services  &  Michael  G  Phelps  for

processing and shipping.

Page 95              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

AceComm & Utilities (Tm)

Version  1.7

AceComm Software Credit Card Registration Form

PSL Part number 10805

The PSL part number for AceComm is: 10805

Credit card registrations may be made by the following methods:

-- Phone PSL at:

800-242-4775  (Toll free) To register by credit card.

713-524-6394  To  register  by  credit  card  and  for   order

inquiries.

Be sure to have BOTH the credit card form at the end of this file

PLUS the order form in the file ORDERFRM.DOC  available  to  give

order information to PSL.

ACCURACY in phone ordering is important!  Misspellings can result

in delays or an erroneous registration.

-- FAX PSL at 713-524-6398

-- Email PSL at CompuServe user id 71355,470

-- Write PSL at:

The Public (Software) Library

P.O. Box 35705

Houston, TX 77235-5705

USA

PSL Part number 10805

For FAX, email, or mail to PSL, send BOTH the credit card form at

the   end   of  this  file  PLUS  the  order  form  in  the  file

ORDERFRM.DOC.

The Public Software Library also  maintains  a  vast  library  of

shareware  and  public  domain  programs  and  prints  a  monthly

magazine announcing new releases and general software news.

Page 96              "AceComm & Utilities"(Tm) v 1.7

Chapter 14  Registering AceComm

AceComm & Utilities (Tm)

Version  1.7

AceComm Software Credit Card Registration Form

PSL Part number 10805

Please complete this form AND the regular order form in the file

ORDERFRM.DOC and send BOTH *** DIRECTLY *** to:

The Public (Software) Library

P.O. Box 35705

Houston, TX 77235-5705

USA

Date ________________________

Name:  _____________________________________________________

[Company:] _____________________________________________________

Address:  _____________________________________________________

:  _____________________________________________________

:  _____________________________________________________

Payment by:   ( ) MasterCard          ( ) Visa

( ) American Express          ( ) Discover Card

Card #:  ______________________________  Exp. Date:  ___________

Signature of card holder: ______________________________________

PSL Part number 10805

Page 97              "AceComm & Utilities"(Tm) v 1.7