BBcpuclk Rev 1.0

(C) Copyright 1995 Remi Lenoir

BBcpuclk is FREEWARE

What is BBcpuclk ?

------------------

BBcpuclk is a program that allows some CPU upgrades for 386SX machines to

work properly with a Commodore A2386SX BridgeBoard. It has been tested with

the Evergreen RevTo486 (386SX3+) upgrade and a TI486SXLC2-50 chip.

The problem

-----------

The Evergreen SLC3 upgrades uses an enhanced version of the 386SX developed

by IBM. This enhanced version uses, among other things, a clock tripled core

to improve  performance. The problem is when used in a 2386 Bridgeboard

the CPU hangs when the clock tripler circuit is started. In other words, you

can use the CPU at 16, 20 or 25Mhz  (depending on which BridgeBoard you have)

and take advantage of its internal 16KB cache but you can not take advantage

of the clock tripler. Considering the price of the toy, it's pretty annoying.

The reason

----------

I was not able to determine the exact reason of the failure except that the

IBM486SLC3 "doesn't like" the clock it receives from the BridgeBoard.

The solution

------------

What guided me to the solution is that the clock tripler operates properly when

the BridgeBoard BIOS is configured with SYSTEM SPEED = LOW.

The solution consists of giving a "better" clock to the CPU. How is this done ?

This is done by modifying the configuration of the PC chipset used on the

BridgeBoard. One of the chips (VL82C311) contains a unit that generates the CPU

clock and the BUS clock. This unit is quite flexible and looks like this:

TURBO ON/OFF

|

|

|

\|/

|-----|

|---------------------|     |

|                     |     |    \

|   |-----------|     | MUX |----- CLK2

|  \|           |    \|     |    /

TCLK2 ----|---| NON-TURBO |-----|     |

/|  DIVIDER  |    /|------

|           |

|-----------|

/|\

|

|

|

DIVIDE RATIO

(1, 2, 3, 4)

Notes:

- TCLK2 is the output of the oscillator from which the CPU clock is derived.

- CLK2 is the CPU clock. In a standard 386SX it is divided by two before

going to the core giving an operating frequency half of CLK2.

(32Mhz for a 16Mhz system, 40Mhz for a 20Mhz one and 50Mhz for a 25Mhz one).

As you can see, you have two ways of giving the CLK2 clock to the CPU. By going

direct (TURBO option) or by going through the divider and choose a ratio of 1.

When the BridgeBoard is configured as SYSTEM SPEED = HIGH (in BIOS SETUP), the

unit is configured in TURBO mode, when SYSTEM SPEED = LOW, the unit is

configured in NON TURBO with a divide ratio of 2 (half speed).

The solution is to provide CLK2 by going through the divider with a ratio

of 1 rather than using the TURBO mode. In theory it should be equivalent but

the Evergreen upgrade does not operate reliably with the TURBO mode whereas it

does with NON TURBO. I think the clock generated with the NON TURBO mode has

faster rise time/fall time and/or less skew or something like this that keeps

the clock tripler circuit happy.

The program

-----------

You have to execute the BBcpuclk program before you start the clock tripler.

The clock tripler is started when the CPU is configured and this is done by

the Evergreen configuration program launched in the CONFIG.SYS file. The problem

is that I am anything but a PC specialist and I don't know how to write a .SYS

program. Fortunately there is a utility called WRAPPER.SYS that will launch a

.COM or .EXE program from the CONFIG.SYS file. Just insert the following line

in the first line of your CONFIG.SYS file:

DEVICE = WRAPPER.SYS BBcpuclk.com

Then you can insert the call to the Evergreen configuration program.

Notes:

------

I wrote this program because I upgraded my BridgeBoard by changing the CPU: I

removed the original 386SX and replaced it with a TI486SXLC2-50. Then, I

ran into the problem that it would hang when I would start the clock doubler.

So this clock doubler/clock tripler problem also affects the TI chip. Strangely

enough, it doesn't seem to affect the Cyrix SRX2-50 upgrades.

(I will later release an AmigaGuide file giving more information on how to

upgrade a BridgeBoard).

When I finally got my system to work (thanks to Brian for providing me with the

VL82C311 data sheet) I remembered reading posts about the Evergreen upgrade

not working well and thought that people with an Evergreen might have the same

problem. That's around the time that I met Peter on c.s.a.emulations and provided

him with the program. He did BETA testing and later got in touch with Gary who

found the WRAPPER program putting an end to the config.sys problem.

I still don't know the exact reason why the CPU hangs when its clock does not

go through the divider. I might take a closer look at that later but as

everything is working...

Although this program is working for me and at least two other persons, I can

not guarantee that it will work for others, neither I can guarantee that the

information given in this file is valid or accurate. You use this program and

this information at your own risk.

BBcpuclk source code:

---------------------

;

; BBcpuclk.asm

;

; (C) Copyright 1995 Remi Lenoir

;

cseg	segment

assume 	cs:cseg,ds:cseg

org	100h

start:	mov	al,7

out	0ech,al

in	al,0edh		;read CLKTCL register

and	al,9fh		;set NON TURBO divider ratio to 1

out	0edh,al		;write CLKCTL register

mov	al,0

out	0f4h,al		;switch to NON TURBO mode

exit:	mov	al,0

mov	ah,04ch

int	21h

db	'BBcpuclk (C) Copyright 1995 Remi Lenoir'

cseg	ends

end start

This program was assembled using a public domain 80x86 assembler called A86.

A86 can be found on a SIMTEL archive (/msdos/asmutil/a86v372.zip).

If you know how to write a .SYS version of BBcpuclk, I would like to here

from you.

WRAPPER.SYS:

-----------

This program can be found on a SIMTEL archive (/msdos/bootutil/wrapsy10.zip)

or from compuserve.

WRAPPER.SYS is Copyright (1992) by Philip B. Gardner.

Distribution:

-------------

BBcpuclk is FREEWARE. It is not public domain. The author gives permission for

BBcpuclk to be freely distributed provided that copying charges remain in the

same range as Fred Fish and that no modification is made to the original

distribution.

If you want to include BBcpuclk in a commercial product, please contact me.

My current email address is dvs@netcom.com. As I share this mailbox with other

persons, I would rather prefer you  post questions in the c.s.a.emulations

newsgroup that I regularly check.

Credits:

--------

I would like to thank the following persons:

- Brian J. Heyboer for providing me with the VL82C311 data-sheet.

- Peter Ouzts for BETA testing the program on the Evergreen upgrade.

- Gary S. Cooper for BETA testing the program and pointing  out the

WRAPPER.SYS program.

and all other persons who have shared information on the BridgeBoard.

Remi.