R4utils 1.0

Introduction.

In the archive you will find three little utilities:

- rl is a quick and dirty implementation of the Lucy-Richardson restoration

algorithm.

- tf performs a 2D Discrete Fourier Transform (DFT) on a R4 binary image.

- an gives some stats on the distribuition of values in an image and its DFT.

Requirements.

An Amiga computer :)

OS V2+

RAM requirements.

The rl utility is memory hungry: you will need about 2.5MB in order to restore

a 256x256 R4 binary image. All the utilities are a lot faster if they can

allocate FAST RAM.

Installation.

In the archive you will find two versions for each utility:

- #?.881 is for 680X0+68881 systems (with X>2)

- #?.000 is for vanilla 68000 systems

so simply copy the version that fits your system where you want.

Some information about implementation.

Each program uses the Fast Fourier Transform (FFT) C routine I wrote, so

they are quite slow ;) 2D FFT is really a "heavy" computation and should

be computed only on really fast systems: don't expect to use your A500

to compute a rl restoration with 500 iterations. Using a math coprocessor

is zillion times faster if you have one.

Q&A.

What is the R4 image format?

Well, R4 is not actually an image format; in fact it is basically a raw

concatenation of 32 bit floating point values. This type of file is often

used to store real digitised images from some instrument (e.g. the HST

camera, HST being Hubble Space Telescope) and moreover is damn easy to

read and write in every programming languages.

How can I display R4 binary images on the Amiga?

Until now there's only one way I know of: use my sao2pgm conversion

utility as found on Aminet; this program is a quick hack I wrote, it

converts R4 binary images in the PGM P5 format. On X Unix machines you

can use SaoImage to display these files: let's hope someone will port it

under AmigaOS, it would be much nicer :)

Why are these utils shell only? and why is the interface so ugly?

Cause I had to mantain source level compatibility with Unix and so

I could not use Amiga specific features. :( However some standard

Ansi functions were substituted with Exec and Dos ones where possible.

Why are they so buggy?

Well I can't tell you. I've used these utilities for several months now

and on my system all is fine. Contact me if you have problems.

RL.

This utility performs the Lucy-Richardson restoration algorithm on a

degraded image. This method is quite popular for restoration of HST

WF/PC and FOC images. Look at ftp.stsci.edu (the HST internet home) to

know more about this algorithm.

The syntax is:

rl <# iter> <width> <height> <infile> <psffile> <outfile> <adu> <noise> [<startfile>]

where

- <# iter> is the number of iterations

- <width> and <height> the dimensions for the image to restore

- <infile> the degraded image

- <psffile> the image containing the Point Spread Function (PSF)

- <outfile> the restored output image

- <adu> and <noise> are values associated with noise (more on these below)

- <startfile> is the optional image to start with

Currently width and height must be equal and a power of two.

Adu and noise.

Adu is number of electrons per DataNumber(DN)

Noise is the constant noise

These values are used to compute the Square Chi for the pixel distribuition.

A good rule to stop the iteration is to look at this value and stop

when it is near 1 or when it does not change meaningfully.

Startfile specification.

If startfile is specified the rl loads it and starts the iteration

using it; this is useful for subdividing the iteration in steps:

the first time you run rl don't specify startfile and iterate for say 40 times,

then look at the restored outfile...is it looking good? does the Square

Chi seem to have converged? if yes then you have restored it, if no then

run rl another time using the previous outfile as your new startfile and go on

in this way.

Things to look at.

During the iteration the program prints some "markers":

- the Square Chi and the delta from the previous value.

- the Distance from the degraded image and the restored image and its delta.

- the Square Norm for the restored object.

These values are then saved in two separate ASCII files:

- *.log contains the Square Chi,the Distance and the Square Norm

- *.diflog contains the deltas

where * is substituted with the outfile name. You can look at these values

using a text editor and graph them using GnuPlot.

For the mathematician.

The degradation model used is the classical:

g = k*o + w

where g is the degraded image, k is the PSF, o the object, w the noise and

the * denotes convolution.

Let's call f(j) the j restored object and i(j)=k*f(j) its image.

Square Norm for a generic h object is computed as:

||h||**2 = (sum(for all pixels) of h**2)**2

Square Chi for j object is computed as:

sqchi(j) = (sum(for all pixels) of (i(j)-g)**2/(var(i(j))))/(n-1)

where n is the width*height and var(i(j)) is:

var(i(j)) = (noise**2)/adu + i(j)/adu

The Distance for the j image is computed as:

dist(j) =||i(j)-g||**2 / ||g||**2

TF.

This utility simply performs the 2D DFT of a given R4 binary image.

The syntax is:

tf <width> <height> <infile> <outfile>

It simply loads the given <width> x <height> R4 infile, transform it and then

saves the result in the R4 outfile.

AN.

This utility gives you some information about the distribuition of pixels

in a R4 binary image and its DFT.

The syntax is:

an <width> <height> <infile>

It basically load the given <width> x <height> R4 infile,prints values,

transforms it,prints values for real and imaginary parts of the DFT,

decompose each pixel value in modulus and phase (angle) and then prints

the values for them.

The values printed for each set are:

- Min and Max the minimum and maximum values

- Range = Max - Min

- Ratio = Max / Min

- Sum the sum for all pixels

- Mean the mean value

- Sigma the standard deviation from the mean

This utility is almost useless, but sometimes it is handy: for example

the Ratio value printed for the modulus of the DFT of a PSF is the

so called "conditioning number", the bigger it is the more ill posed

is the deconvolution problem.

Thanks.

I would like to thank two persons at STScI for their kindness:

Bob Hanisch

Rick White

Credits, flames, etc.. to

Stefano Agostinelli

via Vesuvio 12/11

16134 Genova - ITALY

IRC: arm

E-mail: agos001@pn.itnet.it