HST Service Script Release 2.0

------------------------------

HST Modem setup under Qmodem 4.0

--------------------------------

Setting up your modem for Qmodem use is quite simple.  Make sure

that the following parameters are specified in the respective

customization options (accessible via Alt-N).  Each option or

set of options is preceded by the Qmodem Alt-N sequence required

to get to the proper configuration menu.

MODEM/COMMUNICATIONS PARAMETERS

-------------------------------

Alt-N-M-C

Default Baud Rate    :  19200 (9600 on slow systems)

Modem Command Delay  :  Default is 0, but if modem does not

recognize commands, start with a value

of 30 and decrease it until you find the

lower threshold for this parameter.

Alt-N-M-M

Initialization String:  ATZ^M

Abort Dial Command   :  DTR (these three letters)

Hangup Command       :  DTR   "      "      "

HOST MODE

---------

Alt-N-H

Answer Command       :  ATA^M

Ring Detect Message  :  RING

RUNTIME OPTIONS

---------------

Alt-N-O-T

XON/XOFF             :  NO

CTS Signal           :  YES

CTS When Writing     :  YES (if fast HD or RAMdisk)

Alt-N-O-D

File Transfers       :  20

That's it.  proper switch settings are:

1,2,6,7,10: UP

All others: DOWN

Dan Moore

1/18/89