��������������������������������������������������������������������������ͻ

�STOP! ARRET! ALTO! HALT! STOP! ARRET! ALTO! HALT! STOP! ARRET! ALTO! HALT!�

�                                                                          �

�                    This program can harm your HARDWARE...!               �

�                        YOU USE IT AT YOUR OWN RISK !!!                   �

�                      ABSOLUTELY NO LIABILITY ACCEPTED !!!                �

�                                                                          �

�The program will not run without the appropriate parameters, so there is  �

�LITTLE risk that you will run it accidentally. But be very careful if you �

�call it from a batch file...!                                             �

�                                                                          �

�STOP! ARRET! ALTO! HALT! STOP! ARRET! ALTO! HALT! STOP! ARRET! ALTO! HALT!�

��������������������������������������������������������������������������ͼ

PORTS.EXE

���������

(Ver 1.3 - 19 Sep 91)

Copyright (c) and developed by

Jos� Campione

2415 SouthVale Cr. U-25, Ottawa, Ont. K1B 4T9 (613) 523-4345.

and

C. Robert Parkinson

5 Ramsgate, Ottawa, Ont. K1V 8M4, (613) 523-7299.

Program suggested by Micheline Johnson

(who also worked out the details for example #4).

*  *  *

NOTICE

������

This program and every file distributed with it are copyright (c)

by the authors, who retain authorship both of the pre-compiled and

compiled codes.  Their use and distribution are unrestricted, as long

as nobody gets any richer in the process.  Although these programs

were developed to the best of the authors abilities, no guarantees

can be given as to their performance.  By using them, the user

accepts all risks and the authors decline all liability.

INTRODUCTION

������������

PORTS.EXE is a small utility to read and/or write a byte or

word value of an input/output (I/O) port.  This ability will enable

you to alter some of the defaults for your system operation.  In

particular, this program was designed to allow you to view and/or

alter the CMOS configuration values in 80286 and higher systems,

although it can be used for other purposes (see the WARNING below).

For those users who are not familiar with the Intel/Microsoft

I/O port address structure, please read the BACKGROUND section below

BEFORE you use this program.

WARNING

�������

This utility can alter the basic operation of your system.

DO NOT use it to write to the ports unless you thoroughly understand

what you are doing.  In particular, DO NOT write to any port which

is concerned with disk operations.  An incorrect value in another

location may crash your system, but an improper value in these

locations may corrupt your disk.  Be very careful to not get

confused between decimal and hexadecimal values.

Merely reading a port may also disrupt your system operation,

although it will not harm any hardware.

You have been warned!

SYNTAX

������

PORTS.EXE uses five possible first parameters.  The "u", "a"

and "w" parameters, while they may be useful in other circumstances,

were specifically created to deal with the CMOS RAM settings in 286

and higher systems.  The parameters are:

i = In (from port).

Input/display a single byte from a port.

o = Out (to port).

Output/send a single byte to a port.

u = Out/In (to port 1/from port 2).

Output/send a single byte to a port, followed

by an input/display from a second port.

The optional parameter "#xxxx" (see below)

permits a delay of xxxx milliseconds to be

inserted between the two port accesses.

a = Out/Out (to port 1/to port 2).

Output/send a single byte to a port, followed

by an output/send of another byte to a second port.

The optional parameter "#xxxx" (see below)

permits a delay of xxxx milliseconds to be

inserted between the two port accesses.

w = Word out (to port).

Output/send a 2-byte word to a port.

Usage:

�����

C:\> PORTS i $INWORD <!>

C:\> PORTS o $OUTWORD <byte value> <!>

C:\> PORTS u $OUTWORD <byte value> $INWORD <!>

C:\> PORTS a $OUTWORD <byte value> $OUTWORD <byte value> <!>

C:\> PORTS w $OUTWORD <word value> <!>

Where:

- INWORD and OUTWORD are port addresses.  These are not in the

normal "segment:offset" form, but rather they are in terms of

an "absolute" port address (e.g., "$061" for the speaker).

The addresses can be entered as hex or decimal numbers,

but hex addresses must be preceded by the "$" sign. (This is

Pascal's way of indicating a hex number).

- <byte value> or <word value> is the decimal or hex value

that you wish written to the port.  Hex values must be

preceded by the "$" sign.

- The optional "#xxxx" parameter will only function with the

first parameters "u" and "a", when making two successive

calls to ports.  In the syntax for this parameter, "xxxx"

represents any decimal number between (and including) 0 to

9999.  A string  representing a number within this range

will cause a delay of "xxxx" milliseconds between the first

and the second port call.  Any string not representing a

number within that range will cause this parameter to be

ignored (i.e., no delay between the port calls).

- The optional "!" parameter causes the display on the screen

of the value of the last input/output byte in decimal, hex

and binary formats.  It will also display the delay value

if "#xxxx" was used.

- Any positive integer below 65536 (0 to 65535 decimal, or

$0 to $FFFF hex) is a valid word.  Similarly, any positive

integer below 256 (0 to 255 decimal, or $00 to $FF hex) is a

valid byte.  When entering bytes or words, $DF is equivalent

to $00DF and both are valid either as bytes or as words.

- Note that parameter "u" causes two successive calls. The

first one is similar to a call using parameter "o" and the

second one is similar to a call using parameter "i".

Similarly, parameter "a" makes two successive calls

equivalent to two separate calls using parameter "o".

Our testing on several systems has determined that these two

successive calls (as, for example, those required to access

the CMOS) could not be carried out by two successive

individual commands from the DOS prompt.  They require a

special "single step" operation with a single call to PORTS.

CMOS CONFIGURATION

������������������

For 80286 and higher systems, configuration information is held

in battery-powered CMOS memory that retains the information even

when the computer is switched off.  That information, originally

entered through the software setup program, can be altered with

PORTS.EXE.  However, to do so you must know the CMOS address and the

desired value.  To display bytes in CMOS, use "PORTS u" (as in the

example below).  To change a CMOS value, using the example below,

simply use "PORTS a" to output the CMOS address (byte) into port 70h

and output the desired byte value into port 71h.

Example:

�������

C:\>PORTS u $70 $14 $71 !

This line causes the following display:

Port byte = 35d, 0023h, 00100011b.

Similarly, the same command line can be entered using decimal

notation as:

C:\>PORTS u 112 20 113 !

which would have caused exactly the same display.

This command outputs byte $14 (a CMOS address) to port $70

and requests the input of the value currently held at that

CMOS address from port $71, which in this case happens to

contain byte $23.  In this example, the displayed byte

is the equipment byte held in the CMOS.  You can interpret

it as follows, remembering that bit 0 is the rightmost bit:

Bits 6,7:      00 - 1 floppy drive

01 - 2 floppy drives

Bits 4,5:      00 - No monitor, or EGA or better.

01 - 40 column, color.

10 - 80 column, color.

11 - monochrome.

Bits 2,3:      (not used)

Bit  1  :      0  - math coprocessor not installed.

1  - math coprocessor installed.

Bit 0   :      0  - No floppies installed.

1  - 1 or more floppies installed.

ERRORLEVELS

�����������

In all cases but one, the byte being addressed is returned as an

ASCII value in the exit errorlevel.  The exception is when using the

"w" parameter (out a 2-byte word to a port), when the returned

errorlevel will be 0.  This exit errorlevel capability makes PORTS

well suited for use in a batch file.  Use "PORTS i" with an

errorlevel test to confirm that this is the proper value to alter,

then use "PORTS u" with another errorlevel test to confirm that it

has indeed been changed to the value that you desire, branching to

an error routine in either case if the correct errorlevel is not

returned.

BACKGROUND

����������

The term "port" has two meanings, so this needs to be cleared

up first.  From a user perspective, "port" is normally thought of

as being a hardware connector.  The user is likely to consider that

he has, for example, two serial ports, one parallel port and,

perhaps, a mouse port and a game port.  However, to a programmer

the term "port" implies an access point to facilitate data

transfer, in other words, an input/output (I/O) address.  To

contrast the two meanings, consider that one serial "port" has, in

fact, eight separate but integral I/O "ports".  This document is

concerned solely with the I/O address meaning of the word "port".

Any Intel 80x8x processor has two separate address modes, using

the same addressing pins.  The first mode addresses memory (i.e.,

normal RAM/ROM addressing).  The second mode, I/O port addressing,

while it uses some of the same numerical addresses as for RAM,

actually sends/receives the information to or from support chips

(e.g., the timer, the keyboard controller, etc.) or peripheral

devices (e.g., the disk controller, the graphics adapter, etc.).

All programming languages have built-in methods of differentiating

between the two modes of addressing.

As we said before, I/O address ports are used by DOS for data

transfer.  The values in these locations govern that part of the

system which is external to the processor itself.  Therefore, the

ports allow direct access to the hardware.  Through ports you can

access components on the mother board, on expansion cards or even

devices external to the system proper, such as modems, printers,

etc.  Each port will have a different meaning for each particular

device and the use of any given port varies from chip to chip or

device to device.  Before using PORTS, check your technical

documentation for the motherboard, the expansion card or the device.

Although we normally speak of the ports as one generic class,

they are actually divided into two types, CPU ports and DOS ports.

CPU ports (also known as "logical" ports) are directly accessible to

the CPU and govern such things as the speaker, keyboard, timer, disk

access, etc.  DOS ports are normally accessible to the processor

only through the use of DOS "interrupts".  They control high-level

data transfer via "serial" ports (e.g., COM1) and "parallel" ports

(e.g., LPT1).  Understanding this difference is essential for those

programmers who write at the hardware level.

There are many differences in the port addressing schemes used

by various Intel processors, although newer processors are always a

superset of older ones.  These differences are necessarily also

supported by the specific ROM BIOS for that system type.  Because of

these differences, compatibility is normally attained by interacting

with the ports only through the processor itself or through DOS/BIOS

interrupts.  But sometimes, in order to achieve a specific purpose,

direct access to the hardware cannot be avoided; PORTS.EXE can do

this for you.

Each port has a specific port number, or address.  In a 16-bit

bus system (8086, 80286 and above) this allows 65,535 ports (0 to

FFFFh). As of 1990, only port addresses 000h to 3FFh had been

assigned.  In an XT system, because of the 8-bit external bus, the

ports are limited to a maximum of 1,024.

Five kinds of operations are usually performed on ports:

1) input (receive from port) a byte;

2) output (send to port) a byte;

3) output (send to a port) a byte and input (receive) a byte

from a second port in one single operation;

4) output (send to a port) a byte and output (send to the same

or another port) another byte in one single operation; and

5) output (send to a port) a word (two consecutive bytes in one

single step).

This last "word" operation is necessary because many hardware

functions are triggered immediately by any operation on the

applicable port.  Any attempt to send two consecutive single-byte

values to this port would necessarily fail.

Some port operations require two successive bytes sent to the

same port, but the hardware may require a small delay between these

bytes.  PORTS can interpose this delay if required.

Many ports are "bit-mapped", in that each bit has a specific

function.  For example, port 14h is the so-called "equipment byte",

since it specifies, in bit values, the elementary hardware

configuration of your system.  Similarily, port 61h uses only the

first two bits to control the speaker, the remaining six being used

for unrelated purposes.  Changing bit-mapped ports with PORTS is a

complex operation, requiring the conversion to and from binary

values.  See Example #3 below.  To assist in this conversion, you

may wish to use the PC Magazine utility CONVERT.COM.

There are also many anomalies in the way in which ports respond.

These are dependent upon the hardware being addressed, the system

being used, the manufacturer's design whims, etc.  As these

anomalies are very much system-specific, trial and error (hopefully

not too much error!) is the only sure test.

This small technical background is NOT designed to make you

familiar with I/O ports and their usage.  Please consult any

reliable programmer's handbook.

EXAMPLES

��������

Example #1 - Controlling the Speaker

������������������������������������

Your computer's speaker is controlled by bits 0 and 1 of the

bit-mapped byte found at port 61h.  If both bits are set to 1 you

will hear a continuous tone in the same frequency as the infamous

DOS beep.  To vary the tone, programs must use ports 42h and 43h to

tell the system programmable timer chip how to vary the output

frequency.  That's beyond what we want to demonstrate here; we're

just going to show you how to output sound.

The remaining bits 2 to 7 of the byte at port 61h are uses for

other purposes, so don't disturb these.  This means that we have to

first find the existing byte at that port.  Do that with this

command:

"PORTS o $061 !"

Now take the binary byte on the screen (e.g., 01001000 or 48h)

and change it so that bits 0 and 1 are set to 1 (e.g., 01001011 or

4Bh).  Make certain that you do not alter the other 6 bits.  The

easiest way to do this conversion is to use the CONVERT.COM utility

from PC Magazine, but you can do it manually using the technique

illustrated in Example #3.

If we just output the changed byte to port 61h, we will get a

continuous tone and we will then have to output the original byte

to turn it off.  But we can use the PORTS "a" parameter to do both,

inserting a variable delay so that we can tell that our ports call

does work.  Using the example byte, this is the command:

"PORTS a $061 $04B $061 $048 #500"

Change the "#500" to vary the duration of the sound.

Example #2 - Initializing the Printer

�������������������������������������

Here is a simple example to reset a printer connected to your

system to the printer's built-in default settings, assuming that it

is a parallel port Centronics-compatible printer.  It also assumes

that your printer is addressed through LPT1.  While this type of

reinitialization can more easily be done using your printer's

built-in "escape" sequences, this example merely illustrates one of

the capabilities of PORTS.  The third of the LPT1 I/O port addresses

is the one that controls the initialization (INIT) line to the

printer.  To initialize the printer we have to drop the normal +5V

signal on that INIT line to 0V, then restore it to +5V again.  To

do this with a software routine, we have to determine that specific

I/O address for our system, then send two sequential values to that

port; one to set bit 2 to zero, then one to set bit 2 to 1.

However, since the INIT line has to be held low (0V) for long

enough for the printer to notice, we must either use two separate

PORTS "o" calls or, preferably, use a PORTS "a" call to the same

I/O address but with a delay ("#xxxx") between the two calls.  Here

is the routine:

- To find the specific I/O address for your system, we will use

DEBUG.  However, you could use any other memory editor.  At the

DEBUG "-" prompt, enter "d 0040:0008".  The values in the first

two bytes are the first LPT1 I/O address.  On our test system,

DEBUG gave "BC 03".  Remembering to reverse the order, we get

an I/O address of "03BCh".  But we want the THIRD I/O address,

so we add 2 (in hex) to that, giving us "03BEh" as the INIT

port address.

- Now, we have to drop bit 2 to 0, then raise it to 1 again.  So

we output binary 11101000 (232) to the port.  Counting from 0

on the right, you can see that bit 2 is indeed a 0.  To raise

bit 2 to 1, we then output binary 11101100 (236) to the same

port.

- We're ready to try our test.  Execute the following PORTS

command, remembering to substitute your own I/O port address if

it differs from 03BEh:

"PORTS a $03BE 232 $03BE 236 #50!"

Experiment with different values for "#xxxx" to find the minimum

delay that your printer requires.  On the test system, for example,

a delay as small as 1 millisecond was acceptable.

Example #3 - Setting Bright Text Background Colours

���������������������������������������������������

Try this test if your system has a CGA card.  Only use it if you

have a CGA card, as it may not work with other adapters.  If your

VGA or SuperVGA adapter is 100% EGA compatible (e.g., the ATI VGA

Wonder) this test will work.  PORTS does not check for the adapter

type.

You can modify the blinking bit that causes blinking characters

on the screen, so that, instead of blinking, it will enable the use

of bright background colors.

Most CGA cards can be accessed through port 03D8h ($03D8). The

configuration word of the CGA card can be obtained from the BIOS (it

is the word at address $0040:$0065).  Usually, this word contains the

value $3029 (if you look into this address using DEBUG, remember that

DOS inverts the bytes in a word), which in binary is:

3    0    2    9 h

0011 0000 0010 1001 b

^

To toggle blinking, bit 5 has to be reset (indicated with the

caret). This can be done by "ANDing" 3029h with $DF:

3    0    2    9 h

0011 0000 0010 1001 b

AND                  D    F h

0000 0000 1101 1111 b

^

���������������������

3    0    0    9 h

0011 0000 0000 1001 b

So enter the following:

C:\>PORTS !

And you'll get the ports message with the blinking string

"WARNING!".

Then enter:

C:\>PORTS w $0D38 $3009 !

If everything went smoothly, you'll see that the blinking of

"WARNING!" has ceased and that, instead, it is being displayed on

a bright background color.

To change your display back to normal, enter:

C:\>PORTS w $0D38 $3029 !

Or replace $3029 with whatever word was originally found at

address $0040:$0065 in your system.

Example #4 - Denying Access to Drives

�������������������������������������

For 286/386/486 systems only.

����������������������������������������������������ͻ

�                   CAUTION                          �

�                   �������                          �

�  This procedure, if improperly done, can result    �

�  in your having great difficulty accessing         �

�  your computer system.                             �

�                                                    �

�  Improper inactivation of a hard drive can be      �

�  rectified by booting from a floppy diskette       �

�  and restoring the CMOS from the setup diskette.   �

�                                                    �

�  Improper inactivation of floppy drives is of      �

�  more concern.  If you have provided for no way    �

�  of restoring the CMOS from the hard drive, then   �

�  you'll probably need to return to your dealer.    �

�                                                    �

�  UNDER NO CIRCUMSTANCES inactivate all floppy      �

�  and hard drives.  Should you do so, even your     �

�  dealer will be very unhappy.                      �

�                                                    �

����������������������������������������������������ͼ

There are several password protection programs that permit the

hiding or disabling of one or more storage devices.  One limitation

of these programs is that they can usually be bypassed by booting

with a system (bootable) diskette from the A: drive.

To overcome this security limitation, it is possible to switch

off the A: and B: drives by appropriate editing of the CMOS RAM.

The retrieval and input of the CMOS configuration setup

information from and to the CMOS EEPROM chip can be done by using

the PC Magazine utilities CMOSGET and CMOSPUT.  In a manual

operation, the byte file generated by CMOSGET can be edited using

any good binary editor and then uploaded to the system using

CMOSPUT.

However, PORTS permits a more elegant and practical approach:

1) Back up the complete hard drive that you wish to hide.

2) Make a copy of the CMOS RAM, using "CMOSGET > CMOS.DAT".

3) Make a bootable system diskette with the following programs (in

addition to the two hidden system files) on it, which will

permit everything to be put back to normal if there are any

problems:

COMMAND.COM

CMOSPUT.COM

CMOS.DAT

PORTS.EXE

4) Note the contents of your CMOS RAM at address 10h (floppy

drives), address 12h (hard drives) and at addresses 2E,2Fh

(checksum).  An example of the contents might be:

- at address 10h, the value 24h (1.2M and 1.4M floppy drives).

- at address 12h, the value F0h (single hard drive type Fh).

- at address 2E,2Fh, the checksum value 04D4h.

5) To switch off the floppy drives, address 10h has to be cleared

(subtract 24h) and to switch off the hard drive, address 12h has

to be cleared (subtract F0h).  Therefore, switching off the

floppy drives requires a checksum of 04B0h.  If you are switching

off the hard drive, a checksum of 03E4h is required.

6) Put the following batch files on the hard drive:

FLOFF.BAT

���������

ports a $70 $10 $71 $00 !

ports a $70 $2E $71 $04 !

ports a $70 $2F $71 $B0 !

and

FLON.BAT

��������

ports a $70 $10 $71 $24 !

ports a $70 $2E $71 $04 !

ports a $70 $2F $71 $D4 !

7) Put the following batch files on the bootable floppy diskette:

HDOFF.BAT

���������

ports a $70 $12 $71 $00 !

ports a $70 $2E $71 $03 !

ports a $70 $2F $71 $E4 !

and

HDON.BAT

��������

ports a $70 $12 $71 $F0 !

ports a $70 $2E $71 $04 !

ports a $70 $2F $71 $D4 !

8) In the above examples, the CMOS RAM port addresses are $70 and

$71.  The parameters following $70 and $71 are the CMOS RAM

internal address and the data to be written there, respectively.

The "$" is the Pascal representation for hex numbers, used by

PORTS.

To Turn Off the Hard Drive

��������������������������

8) Run HDOFF from the bootable diskette in floppy drive A:.  To

test whether you have been successful in hiding the hard drive,

do a boot, without the floppy diskette in drive A:.  If your

efforts have been a success, the computer will now find no hard

drive, and will give the prompt:

"Insert BOOT diskette in drive A:"

After booting from the floppy drive, any request to log on to

the hard drive causes the display of the DOS error message:

"Invalid drive specification".

9) To return everything back to normal, run HDON from the floppy

drive and reboot.

To Turn Off the Floppy Drives

�����������������������������

10) Run FLOFF from the hard drive, and reboot.  The computer will

not even check the floppy drives to see if they contain

diskettes.  Instead, it will boot straight from the hard drive.

A request to log on to one of the floppy drives, regardless of

the presence of a diskette in that drive, causes the display of

the DOS error message:

"Not ready reading drive A

Abort, Retry, Fail ?"

11) To return everything back to normal, run FLON from the hard

drive and reboot.

*  *  *

#