����������������������������������������������������������������������������Ŀ

� Module Player V1.4      (c) 1994,1995 by Lord Excess of Sound Wizards Int. �

������������������������������������������������������������������������������

USING THE SWMP DRIVERS WITH 'TURBO PASCAL 6.0'

����������������������������������������������

First of all you have to unzip the files of the included

archive PAS_SDK.ZIP to a location where the pascal compiler can

find them. This is either the directory where you keep your

source code or even better the Turbo Pascal directory itself.

The files you need in any case are:

MODPLAY.TPU           : main unit

MODPLAY.PAS           : source code

DETECT.TPU            : fully automatic hardware detection

SB_DRV.TPU            : SoundBlaster driver

SBP_DRV.TPU           : SoundBlaster Pro driver

GUS_DRV.TPU           : Gravis UltraSound driver

The rest are example files, which can be important to

understand things better. Now the only step to link all

mod-procedures and functions is to add the following statements

after your program heading:

PROGRAM xxxxx;

{$M $4000,0,0}        : This sets the heap size to zero

: else the drivers cannot alloc memory

USES ModPlay;         : include unit ModPlay

...

..

At program start you have to initialize this unit using the

following procedure:

���INIT DRIVER�����������������������������������������������������������������

�       PROCEDURE Mod_Init(Driver,Port: WORD; IRQ,DMA: BYTE);

�         -> Select and initialize driver for sound output

�

�            Driver = Detection         (0) :  Port = 0

�                                              IRQ  = 0

�                                              DMA  = 0

�

�                   = SoundBlaster      (1) :  Port = SB base address ($220..)

�                                              IRQ  = SB IRQ#

�                                              DMA  = 1 (always)

�

�                   = SoundBlaster_Pro  (2) :  Port = SBP base address

�                                              IRQ  = SBP IRQ#

�                                              DMA  = SBP DMA channel

�

�                   = Gravis_UltraSound (3) :  Port = GUS base address

�                                              IRQ  = GF1 IRQ#

�                                              DMA  = MIDI IRQ# !!

�

A very powerful hardware detection is able to detect any

supported soundcard with all required settings. So in general

the easiest way to init the drivers is:

MOD_Init(Detection,0,0,0);

If all fails you can override detection and select one of the

drivers by yourself, but in this case you have to know (or ask

for) the settings of an installed soundcard.

If you want to know if initialisation was successfull, have a

look at the variable Soundcard (WORD), which will contain the

output device:

0:        no sound available

1:        SoundBlaster            [ 22.2 kHz mono     ]

2:        SoundBlaster Pro        [ 21.7 kHz stereo   ]

3:        Gravis UltraSound       [ 44.1 kHz +stereo+ ]

You do not have to handle this variable. You can also call the

driver's subfunctions even if no soundcard was detected,

nothing will happen in this case.

At program end the unit selfrestores any altered interrupt

vectors etc.

The following functions & procedures are available:

(Note: The first two procedures are enough to playback a mod !!!)

���LOAD MODULE�����������������������������������������������������������������

�       PROCEDURE Mod_Load(FileName: STRING);

�         -> Stops possible sound output, loads module file into memory,

�            but does no start playing. If successful the variable

�            Channels (WORD) will contain the number of channels, else zero.

�

���PLAY MODULE�����������������������������������������������������������������

�       PROCEDURE Mod_Play(Looping: WORD);

�         -> Starts playing previously loaded song. If looping is zero,

�            song is played only once, else continuously.

�

���STOP MODULE�����������������������������������������������������������������

�       PROCEDURE Mod_Stop;

�         -> Stops sound output at once.

�

���MAIN VOLUME�����������������������������������������������������������������

�       PROCEDURE Mod_Volume(Volume: BYTE);

�         -> Changes the driver's main volume setting. Valid values are

�            between 0 (lowest) and 64 (loudest).

�

���STATUS����������������������������������������������������������������������

�       FUNCTION Mod_Status: WORD;

�         -> Returns 1 if sound is being played, else 0.

�

���GET POSITION����������������������������������������������������������������

�       FUNCTION Mod_Position: WORD;

�         -> Returns 256*PatternNumber+LineNumber. Can be useful to

�            syncronize a special graphic effect to the sound.

�

���JUMP BACK�������������������������������������������������������������������

�       PROCEDURE Mod_Rewind;

�         -> Decreases current songposition. No effect in first pattern.

�

���JUMP FORWARD����������������������������������������������������������������

�       PROCEDURE Mod_Forward;

�         -> Increases current songposition. No effect in last pattern.

�

���PEAKS�����������������������������������������������������������������������

�       PROCEDURE Mod_Peak(VAR Peaks);

�         -> Updates the array Peaks by copying the driver's volume

�            setting (0..64) of each channel. The number of channels

�            can be found in the variable Channels (WORD) and is either

�            four, six or eight.

�

In general it remains to be said that you can do as much error

checking as you like, but it is not necessary. In a demo or

intro for example you do not care if there is sound available

to the grafix you produce on the screen (in most cases). But in

a player it is evident that you need to have any sounddevice at

all to output the sound! The best way is to take a look at the

example programs. Have a lot of fun!

Lord Excess in June '95

#