-------------------------------------------------------------------------

The NEGA Programming Language v1.0c

Copyright (C) Jan. 1997 by Tylisha C. Andersen

-------------------------------------------------------------------------

The NEGA language is a block structured substitute for assembly language,

and can be classified as a middle level language.   The code is 99%

compatible with the TERSE language by Jim Neil (for more information, see

the WWW site  http://www.terse.com).  To convert TERSE code to use NEGA,

simply change all data definitions to standard MASM style (var DB ???).

If errors appear when compiling the code, try parenthesizing the offending

statements, etc.

NEGA files have the extension .N; to run the compiler, type  NEGA file[.N]

It will output an assembly language file (file.ASM) that can be compiled

with MASM or TASM.  It can often be useful to examine this file when

debugging.  To completely compile a *.N file:

NEGA file.N

TASM -m file.ASM

TLINK -t file

The compiler is written in 'C'.  The source code compiles under both

Borland and Watcom C, and it probably will also compile under MS Visual C

and Symantec C.

An example of a program written in NEGA (a 5.4K Tetris game) is provided

in the directory TETRIS.  This game was originally written by Tenie Remmel

and I converted it to NEGA so it could be used as an example.

-------------------------------------------------------------

General considerations

-------------------------------------------------------------

Statements must start after the beginning of a line,

following some white-space.  A single line may contain

many statements.  Statements are usually terminated with

a ';', just like in 'C' or Pascal.  In some cases, a '?'

can be used (see 'Compares and Tests').

The comment character is '\'.  All characters between a

'\' character and the end of the line are ignored.

For any operation that is not directly supported, simply

write the standard assembly code.  Example:

lidt  [bx];

If it would conflict with NEGA syntax, you can either

enclose the statement in parentheses, or start it at the

very beginning of a line.  Example:

(O    equ <Offset>);

Operands can also be enclosed in parenteses.  Examples:

mov   ax, (-1);     \ mov ax,-1

mov  (ax, bx);      \ mov ax,bx

If this does not work, an entire line may be output with

no processing at all by starting it with an '!'.

-------------------------------------------------------------

Assignment

-------------------------------------------------------------

This category includes all direct assigment operations:

Assembly        NEGA

mov   A, B      A = B;

movzx A, B      A =+ B;

movsx A, B      A =- B; or A =+- B;

xchg  A, B      A == B;

lea   A, B      A === B; or A =% B;

-------------------------------------------------------------

Chained Assignment

-------------------------------------------------------------

Chained assignment allows several registers or variables

to be set using  a single statement.  For instance, to

copy a segment register, two assembly instructions are

needed, but with chained assignment it can be done using

one statement:

ds = ax = cs;

which produces the following code:

mov   ax, cs

mov   ds, ax

The general syntax of a chained assignment is:

X1 = X2 = ... = XN = R = Y;

where R must be a general register, X1 through XN must

be either registers or variables, and Y can be anything.

This produces the following assembly code:

mov   R, Y

mov   X1, R

mov   X2, R

.

.

.

mov   XN, R

For example, you can do this:

X = Y = Z = AX = 0;

which will set X, Y, Z, and AX to 0.

-------------------------------------------------------------

Arithmetic and Logical

-------------------------------------------------------------

The syntax for arithmetic and boolean logic instructions

is very simple:

Assembly        NEGA

add   A, B      A + B;

adc   A, B      A ++ B;

sub   A, B      A - B;

sbb   A, B      A -- B;

and   A, B      A & B;

or    A, B      A | B;

xor   A, B      A ^ B;

inc   A         A+;

dec   A         A-;

neg   A         -A;

not   A         ^A;

imul  A         *A;

idiv  A         /A;

mul   A         **A;

div   A         //A;

imul  A, B      A * B;

imul  A, B, C   A = B * C;

imul  A, B, C   A, B * C;

Also, a  sub A,A  instruction, which clears a register,

may be coded as  &A.  This has the additional effect of

clearing the carry flag.

The common code sequences  inc A/inc A  and  dec A/dec A

can be coded as  A++;  and  A--;  respectively.  Longer

strings of +'s and/or -'s are not supported.  (In these

cases, an ADD instruction is shorter anyway.)

Note that the  imul A,B  and  imul A,B,C  instructions

work with unsigned values as well as signed values.  The

compiler will optimize an  A=B*C;  which normally would

produce  mov A,B/imul A,C (see below) to  imul A,B,C.

-------------------------------------------------------------

Shifting and Rotation

-------------------------------------------------------------

These instructions are:  shl,shr,sar,rol,ror,rcl,rcr.

Assembly        NEGA

shl   A, B      A > B;

shr   A, B      A < B;

sar   A, B      A +-> B;

rol   A, B      A << B;

ror   A, B      A >> B;

rcl   A, B      A <<< B;

rcr   A, B      A >>> B;

shld  A, B, C   A, B < C;

shrd  A, B, C   B, A > C;

In all of these instructions, the last operand, which is

the shift count, must be either 1, the CL register, or a

constant value.

Note that the order of the first two operands in the

shrd A,B,C  instruction is reversed.  This is because

a  shr B:A,C  is what this instruction really does.

-------------------------------------------------------------

Stack Operations

-------------------------------------------------------------

These include push, pop, pushf, popf, etc.

Assembly        NEGA

push  X         =X;

pop   X         X=;

pushf           =??;

popf            ??=;

pushfd          ????=;

popfd           =????;

A single statement can include multiple pushes and pops.

The syntax for a compound push is:

=X1 =X2 ... =XN;

which produces the following code:

push  X1

push  X2

.

.

.

push  XN

The syntax for a compound pop is:

X1= X2= ... XN=;

which produces the following code:

pop   XN

.

.

.

pop   X2

pop   X1

Note that the values are popped in REVERSE order.  This

is intended to eliminate bugs caused by popping values

in a different order than they were pushed.

Here is an example:

=ax =bx =cx =si;

.

.

.

ax= bx= cx= si=;

This produces the following code:

push  ax

push  bx

push  cx

push  si

.

.

.

pop   si

pop   cx

pop   bx

pop   ax

-------------------------------------------------------------

Compares and Tests

-------------------------------------------------------------

All compares and tests use a '?' terminator instead of a

';' terminator.

A  cmp A,B  can be coded as  A - B?.   It subtracts B

from A, and sets the flags based on the result.

A  test A,B  can be coded as  A & B?.   It ANDs A with B

and sets the flags based on the result.

A  test A,A  can also be coded as  A?.

Any assignment, chained assignment, arithmetic, logic,

shift, rotate, or compound statement can be terminated

with a '?'.  The compiler will generate a test command

on the result.  Note that often this may not be needed

as the flags are also set by many other instructions.

-------------------------------------------------------------

Compound Statements

-------------------------------------------------------------

Often, many instructions are needed to compute a result.

If they all share a common destination register, they

can be combined into one statement.  The syntax for a

compound statement is:

R <op> X1 <op> X2 ... <op> XN

or  R = Y <op> X1 <op> X2 ... <op> XN

where R is a register, X1 through XN are registers or

variables, and Y can be anything.  This generates the

same code as if the following were used:

R = Y;          \ optional

R <op> X1;

R <op> X2;

.

.

.

R <op> XN;

-------------------------------------------------------------

Jumps, Calls and Interrupts

-------------------------------------------------------------

Here is the syntax for unconditional jumps, etc:

Assembly        NEGA

jmp   X (short) .X;

jmp   X (near)  ..X;

call  X         =.X;

int   X         !X;

into            !+-;

ret             .=;

ret   X         .=X;

iret            !.=;

And here is the syntax for conditional jumps:

Assembly        NEGA

ja X            >>X;

jae/jnb/jnc X   >>=X; or =>>X;

jb/jc X         <<X;

jbe/jna X       <<=X; or =<<X;

je/jz X         ==X;

jg X            >X;

jge/jnl X       >=X; or =>X;

jl X            <X;

jle/jng X       <=X; or =<X;

jne/jnz X       <>X;

jno X           -+X;

jns X           ++X;

jo X            +-X;

jpe/jp X        &&X;

jpo/jnp X       &&X;

js X            --X;

These condition codes are used in other commands such as

the SETcc, IF, WHILE, conditional break, etc.

The conditional jumps will only jump up to 128 bytes in

either direction.  For a near jump, precede the command

with a '.' character, like this:

.<> foo;

This is not necessary if you use TASM's JUMPS directive,

which fixes up near jumps automatically.  Use this with

care as it wastes 2-3 bytes if you don't need it.

The syntax for the CX-specific jumps is:

Assembly        NEGA

jcxz  X         ?==X;

jcxnz X         ?<>X;

loop  X         -.X;

loope/loopz X   -==X;

loopne/loopnz X -<>X;

jcxnz  is not actually an instruction; it is emulated,

so it takes 4 bytes instead of the normal 2.

-------------------------------------------------------------

Block Structure

-------------------------------------------------------------

The most important part of the NEGA language is the

block structure.  This is what really makes it a middle

level language instead of low-level, allowing flow

control similar to that used in languages such as 'C'

or Pascal.

The simplest block structure is the DO:

{ ... };

A DO simply encloses a block of statements.  They may be

any valid NEGA statements, including other blocks, or

even nothing at all.  The DO may not seem useful, but it

can be useful if used with a BREAK (see below).

This is the syntax for an IF:

<cond> { ... };

where <cond> can be any of the following:

>   G       <   L       >=  GE      =>  GE

<=  LE      =<  LE      ==  Z       <>  NZ

>>  A       <<  B       >>= AE      =>> AE

<<= BE      =<< BE      --  S       ++  NS

+-  O       -+  NO      &&  P       ||  NP

or  ?<>  which is CXNZ.  The condition may be preceded

by a '.' to span large blocks. The statements inside the

IF are executed only if <cond> is true.  For example:

ax? <>{ ax-; };     \ Decrement AX if it isn't zero.

al - 9? >>          \ Convert AL to hex.  If AL > 9 then

{  al + 7; };       \ add 7 before adding '0'.

al + '0';

The final example produces the following code:

cmp   al, 9

jbe   ?Els0001

?Top0001:

add   al, 7

?Els0001:

?End0001:

add   al, '0'

This is the syntax for an IF-ELSE:

<cond> { ... },{ ... };

<cond> is as above.  The statements in the first (IF)

block are executed if <cond> is true, and if <cond> is

false the statements in the second (ELSE) block are

executed.  If the ELSE block is greater than 127 bytes,

you can use a '.':

<cond> { ... },.{ ... };

Also, an ELSE-IF is possible:

<cond> { ... }, <cond> { ... }, { ... };

The ELSE-IF can be extended as long as you wish.  Here

is an example of the ELSE:

ax - bx? >          \ CX = max(AX, BX).

{ cx = ax; },

{ cx = bx; };

which produces the following code:

cmp   ax, bx

jle   ?Els0001

?Top0001:

mov   cx, ax

jmp   ?End0001

?Els0001:

?Top0002:

mov   cx, bx

?Els0002:

?End0001:

The syntax for a WHILE is:

{ ... } <cond>;

The WHILE repeats the statements in the block as long as

<cond> is true.  <cond> is as above EXCEPT that instead

of the  ?<> (CXNZ)  there are the following five types:

?==     CXZ

-.      Loop

-==     Loopz

-<>     Loopnz

.       Jmp (loop forever)

Examples:

cx = 40;            \ subtract BX from DI 40 times

{ di - bx; }-.;

{                   \ wait for a key, and count iterations

bx+;            \ this could be used to seed a RNG

ah = 1; !16h;

}==;

The final example produces the following code:

?Top0001:

inc  bx

mov  ah, 1

int  16h

jz   ?Top0001

?Els0001:

?End0001:

An IF can be combined with a WHILE, and the loop will

be entered only if the IF condition is true:

<cond> { ... } <cond>;

These can even be combined with ELSE's.  The general

syntax for block structure is:

<cond> { ... } <cond>, <cond> { ... } <cond>, ...

... <cond> { ... } <cond>, { ... } <cond>;

where any of the <cond>'s can be omitted.

To exit a block, a BREAK can be used:

<cond>;

or  <cond> N;           \ N is a digit from 0 to 9

The valid conditions are as above (see IF) EXCEPT that

instead of  ?<> (CXNZ)  there is  ?== (CXZ), and there

are two unconditional jumps:

.;                  \ short jump

..;                 \ near jump

If there is no operand, the current block is exited.  If

an operand is supplied, the current block, plus N more

levels of blocks are exited (where N is a digit from 0

to 9).  Also, the common code sequence  .1; };  can be

shortened to  .};  (this is very useful for selection

statements).  For example:

ax - bx? >          \ sort AX, BX, CX in order

{

ax == bx;

bx - cx? <;     \ sorted already?

bx == cx;

ax - bx? >

{ ax == bx; };

};

{                   \ a CASE statement

bx - 0? ==

{ Var0 + ax; .};

bx - 1? ==

{ Var1 - ax; .};

bx - 2? ==

{ Var2 + ax - di; };

};

The final example produces the following code:

?Top0001:

cmp   bx, 0

jnz   ?Els0002

?Top0002:

add   Var0, ax

jmp   ?End0001

?Els0002:

?End0002:

cmp   bx, 1

jnz   ?Els0003

?Top0003:

sub   Var1, ax

jmp   ?End0001

?Els0003:

?End0003:

cmp   bx, 2

jnz   ?Els0004

?Top0004:

add   Var2, ax

sub   Var2, di

?Els0004:

?End0004:

?Els0001:

?End0001:

-------------------------------------------------------------

Input and Output

-------------------------------------------------------------

These instructions read and write from I/O ports.  They

are represented by the '@' character.  The string I/O

instructions are described in the String Instructions

section.

Assembly        NEGA

in    al,X      =@X;

in    ax,X      =@@X;

in    eax,X     =@@@@X;

out   X,al      @X=;

out   X,ax      @@X=;

out   X,eax     @@@@X=;

in    al,dx     =@;

in    ax,dx     =@@;

in    eax,dx    =@@@@;

out   dx,al     @=;

out   dx,ax     @@=;

out   dx,eax    @@@@=;

-------------------------------------------------------------

String Instructions

-------------------------------------------------------------

String instructions manipulate strings or other arrays

in memory.  If operands are specified, for example:

rep movs byte ptr es:[di], ss:[si]

the actual operands are ignored; they are only used to

figure out the size, and to generate segment overrides

(in the example, the source segment was set to SS).

Here is the NEGA syntax for the simple string

operations:

Assembly        NEGA

lodsb           =*;

lodsw           =**;

lodsd           =****;

stosb           *=;

stosw           **=;

stosd           ****=;

movsb           *=*;

movsw           **=**;

movsd           ****=****;

scasb           *-?

scasw           **-?

scasd           ****-?

cmpsb           *-*?

cmpsw           **-**?

cmpsd           ****-****?

insb            *=@;

insw            **=@@;

insd            ****=@@@@;

outsb           @=*;

outsw           @@=**;

outsd           @@@@=****;

The syntax for the complex string instructions is:

Assembly        NEGA

lods  A         =*A;

stos  A         *A=;

movs  A, B      *A=*B;

scas  A         *A-?

cmps  A, B      *A-*B?

ins   A         *A=@;

outs  A         @=*A;

Any string instruction may be preceded by a REP prefix.

If it is a SCASx or CMPSx, it should be either REPE or

REPNE.  This is useless for a LODSx instruction.

The syntax for a REP prefix or a REPNE prefix is  <>.

The syntax for a REPE prefix is  ==.  For example:

<> *=*;             \ rep movsb

== ****-****?       \ repe cmpsd

<> **-?             \ repne scasw

Note:  SCASW and SCASD can act as a compact way to add

2 or 4 to DI.  They save 1 byte relative to the normal

method.

-------------------------------------------------------------

Pointer Bumping

-------------------------------------------------------------

The NEGA language provides a shorthand syntax for the

common operation of incrementing or decrementing a

pointer before or after using it.  For instance, the

code sequence

mov   al, [di]

inc   di

can be coded as

al = [di+];

The operator acts only on the nearest operand inside the

brackets.  The operator must appear either just after

the opening bracket or just before the closing bracket.

If the operator appears before the operand, the compiler

will place the INC or DEC instruction(s) before the

command, and vice versa.  For example:

al = [-di];         \ dec di/mov al,[di]

ch = [+si-];        \ inc si/mov ch,[si]/dec si

bx = es:[bp+si-];   \ mov bx,es:[bp+si]/dec si

cx = [var+];        \ mov cx,[var]/inc var

bl = [ebx+edi*4+];  \ Not allowed!  This generates:

\ mov bl,[ebx+edi*4]/inc 4

\ I don't know of a good way to fix this.

-------------------------------------------------------------

Miscellaneous

-------------------------------------------------------------

The syntax for a Lx A,B (LDS, LES, etc.) is:

x,A = B;

The syntax for a SETcc A is:

A = <cond>;

where <cond> can be any of the following:

>   G       <   L       >=  GE      =>  GE

<=  LE      =<  LE      ==  Z       <>  NZ

>>  A       <<  B       >>= AE      =>> AE

<<= BE      =<< BE      --  S       ++  NS

+-  O       -+  NO      &&  P       ||  NP

Other miscellaneous instructions:

Assembly        NEGA

aaa             "+;

aad             "/;

aad   A         "/A;        (unofficial instruction)

aam             "*;

aam   A         "*A;        (unofficial instruction)

aas             "-;

cbw             *;

cdq             ****;

clc             &;

cld             +;

cli             !-;

cmc             ^;

cwd             **;

daa             '+;

das             '-;

hlt             ....;

lahf            =?;

sahf            ?=;

setalc          =;          (unofficial instruction)

stc             |;

std             -;

sti             !+;

wait            ...;

xlatb           ><;

xlat  A         ><A;

Examples:

al - 0Ah? al -- 69h; '-;    \ convert number in AL to hex!

!-; !;                      \ hang the computer

al = <>; *;                 \ AX = 1 if NZ

cl - 30? =?;                \ test now, but jump later

.

.

.

?=; >>label;

--------------------------------------------------------------------------

--------------------------------------------------------------------------