BACKDOWN.EXE (Version 1.0)

------------------------------------------------------------------------

Bob Flanders and Michael Holmes                            May 12, 1992 (Utilities)

------------------------------------------------------------------------

BACKDOWN: A TSR (terminate-and-stay-resident) utility, BACKDOWN lets you

start and manage communications sessions and file downloads in the

background while you continue working in your foreground application.

You can run BACKDOWN interactively, or you can use its script facilities

to automate all or part of its operations.

Specifically, BACKDOWN provides five major functions:

a "dumb terminal" interface, script execution, a script compiler,

protocol-based download capability, and background execution. The program

requires DOS 3.1 or later and occupies about 16K when resident in memory.

A complete set of source code files are available in case you wish

to study or customize them. They are included in BD.ZIP or listed as

BDSRC.ZIP. The source can be assembled with Microsoft MASM 5.1 or 6.0 or

Borland's Turbo Assembler 2.5 or 3.0. (Other assemblers may work, but

we have tested it only using these.) Using the Microsoft assembler, you

create BD.EXE from its .ASM source with the two commands

MASM BD;

LINK BD;

The statements needed to assemble the BACKDOWN downloading protocol

files are as follows:

MASM BXMODM;

LINK BXMODM;

EXE2BIN BXMODM BXMODM.BDP

MASM BBPLUS;

LINK BBPLUS;

EXE2BIN BBPLUS BBPLUS.BDP

MASM BASCII;

LINK BASCII;

EXE2BIN BASCII BASCII.BDP

Note that if you know assembly language, BACKDOWN provides a fairly

simple interface for adding further protocol modules without requiring

you to recompile the main program.

BRINGING UP BACKDOWN

The full syntax for BACKDOWN is

BD [/Pn|/Hxxx,i] [/Sz] [/C script] [/T] [/U] [script [operands]]

Note that all the command line parameters shown are optional. If you

simply enter BD alone, BACKDOWN will be loaded into memory with the

following defaults: COM1, 2400 bps, no parity, 8 data bits, 1 stop bit,

7-bit screen display, automatic recognition of the CompuServe B+ protocol

enabled, and a script file buffer size of 1K. The command line parameters

specifically related to operating BACKDOWN from a script rather than

interactively are explained in the sidebar "Creating and Using a BACKDOWN

Script." (See Below)

The /Pn command line switch lets you specify any of the standard

PC COM ports, as follows:

/P1 = Port 3F8 Interrupt 4 (COM1)

/P2 = Port 2F8 Interrupt 3 (COM2)

/P3 = Port 3E8 Interrupt 4 (COM3)

/P4 = Port 2E8 Interrupt 3 (COM4)

If you want to use a port that the /Pn switch does not support,

the /Hxxx,i switch lets you specify a particular I/O port and hardware

interrupt. For example, if you have a COM port at address 230h and you

want it to use interrupt 5, you would enter the command BD /H230,5.

Since BACKDOWN uses COM1 as the default, the following three commands

to load the program into memory are functionally identical:

BD

BD /P1

BD /H3F8,4

When you have finished using BACKDOWN, you can remove it from memory

by typing the command BD /U. Note that if you try to uninstall BACKDOWN

as it is  processing a download or script, it will ask you if you are

sure you want to uninstall it. If you answer yes and the BD/U command

is successful you will get a message indicating that BACKDOWN has been

uninstalled. The only time the /U command will not work is if you loaded

another program after BACKDOWN. In this case simply uninstall that program first.

You can test whether BACKDOWN is actively engaged by using the

/T command line switch. This switch returns an ERRORLEVEL and a screen

message that indicates whether BACKDOWN is installed, and if so, whether

or not it is busy. Specifically,

ERRORLEVEL 2 indicates that BACKDOWN is not installed.

ERRORLEVEL 1 indicates that BACKDOWN is installed, but it is either

running a script or a receiving a download.

ERRORLEVEL 0 indicates that BACKDOWN is installed but is in its "dumb

terminal" mode and so may be uninstalled.

As an example of the /T and /U switches in action, the following

batch file will repeatedly test for activity until BACKDOWN is through

processing and then it will unload the program.

@echo off

:testbd

BD /T >NUL

IF ERRORLEVEL 2 GOTO NOTHERE

IF ERRORLEVEL 1 GOTO TESTBD

BD /U >NUL

Echo Backdown now uninstalled!

GOTO END

:NOTHERE

Echo BACKDOWN not in memory!

:END

When you load BACKDOWN into memory, it looks for any download protocol

modules you may have on your system. These modules each have a .BDP

extension (for BackDown Protocol). BACKDOWN looks for these modules

in either the directory specified by the environment variable BDP or in

the current directory.

You can create the BDP environment variable with the DOS SET

command. For example, if you have stored the protocol modules in your

C:\PCMAG\BACKDOWN directory, you can relay this information to BACKDOWN

by using the command

SET BDP=C:\PCMAG\BACKDOWN

either from the DOS prompt or, more conveniently, as a line in your

AUTOEXEC.BAT file.

When loading, if BACKDOWN determines that it is already loaded or

that the COM port specified is not in your system, it displays the

appropriate error message and returns to the DOS prompt. Otherwise,

BACKDOWN displays the names of the available protocol modules and

finishes with a message indicating that it has loaded. Until you issue

the BD /U command to unload it, BACKDOWN will remain memory-resident.

Once BACKDOWN is resident in memory, you pop it up by pressing

the key combination Ctrl-Alt-B. The current screen is saved and

replaced with the initial "dumb terminal" screen. This screen doubles

as a noncontextual help screen that can be accessed by pressing F1.

The use of the various other function keys shown in the figure will be

explained shortly. To return to DOS or to your application (keeping

BACKDOWN active if it is downloading a file), the hotkey is Alt-X.

When you are using the ``dumb terminal'' interface, any information

you enter will be sent out over the specified COM port. You can verify

this by typing a few random letters at the keyboard and noting that the

SD (Send Data) light on your modem flickers. Similarly, any characters

received will be displayed. If you're using a Hayes-compatible modem

(as we'll assume for all examples in this article), entering the command

ATV1E1 will set the modem for Verbose Responses (V1) (such as OK or NO

CARRIER) and Echo On (E1), so you can see what you type. After typing

the command, hit the Enter key and the modem should respond "OK".

At this point you can proceed along one of several paths. You can

change BACKDOWN's settings by pressing F2. As we have just illustrated,

you can type specific modem commands to connect to or use a communications

service simply by typing them on the dumb terminal screen and hitting

Enter. By hitting F9, you can initiate a download. You can execute a

script that will perform one or more predetermined functions by hitting

F10. Or you can pop down the TSR and return to your application or to

the DOS prompt by pressing Alt-X.

SELECTING SETTINGS

When you press the Settings key (F2), BACKDOWN displays a four-column

list of the various communications parameters you can set with BACKDOWN.

The first column shows the available baud rates, and the second indicates

the available data bits, parity, and stop bit combinations. The third

column lets you choose how many bits will be used when characters are

displayed on your screen. If you select 7BIT mode (the default), the

high-order bit of incoming data will be stripped off before being shown.

The fourth column selection determines whether automatic CompuServe B+

protocol invocation is enabled or disabled; this will be explained below.

To make a selection from the options shown, simply press the

character that corresponds to it. BACKDOWN will display the setting you

selected and will prompt you for another entry. Hitting Enter returns

you to the dumb terminal mode. After setting BACKDOWN's parameters, you

can enter the necessary modem commands. For example, if your local

ZiffNet access number were 555-1235, you would type

ATDT555-1235

and then hit the Enter key. The modem would dial ZiffNet and respond

with some version of the Connect message.

DOWNLOADING FILES

Any time you want to download a file or capture information, you must

use a protocol. A protocol consists of a set of rules that allows two

computers to talk to each other; both computers must abide by the agreed

upon protocol during the transfer of files. Protocol agreement is

established by a process known as handshaking, which takes place when

the two computers first connect.

Four protocols come with the initial distribution of BACKDOWN:

Xmodem, Xmodem/CRC, CompuServe B+, and ASCII. Among these, Xmodem,

Xmodem/CRC, and CompuServe B+ provide correction facilities to ensure

error-free file transmission. The ASCII protocol allows any information

displayed during the session to be captured to a disk file, but because

it lacks error-correction facilities it provides no assurance that the

data is transferred reliably.

Which protocol should you use? If you are just trying to capture

information that is being displayed on the dumb terminal screen, use

the ASCII protocol. It is the only protocol that performs this function.

For file transfer via ZiffNet or CompuServe, CompuServe B+ is the

protocol of choice. In the absence of CompuServe B+, Xmodem/CRC is the

next best, followed by Xmodem. Xmodem and Xmodem/CRC are implemented with

"relaxed" Xmodem timing (where the timing requirements are not as

stringent), so that they work properly with ZiffNet or CompuServe.

Both Xmodem and Xmodem/CRC use the same basic BXMODM.BDP module.

Systems vary in the exact sequence of events that leads up to a

file download, but the usual scenario runs something like this: Select

the file to download, tell the remote system which protocol you want to

use, wait for the remote system to tell you to start it at your end,

and execute the protocol on your machine. From there, the two computers

enjoy a friendly conversation while transferring the file you requested.

On completion of the transfer, you are usually returned to some type of

command prompt.

To see how BACKDOWN implements this procedure, let's assume that you

want to retrieve the latest copy of CDX.COM from ZifNnet.

First, from the BACKDOWN dumb terminal screen you connect to ZiffNet

by giving your modem the AT commands to dial your local ZiffNet access

number. When you see the Connect message, you press Ctrl-C. The system

then prompts for your user Id and password. After you enter these,

several initial messages will be displayed.

Eventually, you'll get to a prompt (usually an exclamation point),

at which you type

GO ZNT:UTILFOR

and hit Enter. This will take you to the Utilforum/Tips Forum, where

you will enter the command LIB 2. (All of the PC Magazine utilities are

kept in library 2. PCMAG UTILS)

For this example, we will download CDX.COM using the Xmodem/CRC

protocol. Type the command

DOW CDX.COM /PROTO:XMODEM

and press the Enter key. ZiffNet will respond by giving you some

information about the file and will then ask you to start the protocol.

This means that ZiffNet is ready to send the file.

Now you finally get to press the BACKDOWN Download key (F9), which

will bring up the screen to start the download process. From that screen

you select Xmodem/CRC by typing a C, followed by Enter. You will then be

prompted for a filename, which you must enter and which (optionally) may

include a full path. In this case you type CDX.COM and press Enter.

BACKDOWN tells ZiffNet that it is ready to receive the file, and

the transfer begins. After a few minutes (depending on the speed of your

modem), the file CDX.COM will be on your hard disk and BACKDOWN will tell

you that the transfer has been successful. ZiffNet requires you to press

the Enter key to get back to the LIB prompt. Entering the command BYE

will then disconnect you from ZiffNet.

When using the Xmodem and ASCII protocols, you may enter the filename

to be received when you specify the protocol. In the above example, when

asked for a protocol, you could have entered

C CDX.COM

and BACKDOWN would not have prompted you again for the filename.

If you are using the CompuServe B+ protocol, the above procedure

must be altered slightly. First, when ZiffNet displays the LIB prompt,

you respond

DOW CDX.COM /PROTO:B

Next, instead of prompting you to start the protocol on your machine,

ZiffNet requests that you enter a filename for your computer. The

filename you enter is used to store the information when it is placed

on your local disk. After typing the filename, ZiffNet sends a message

to BACKDOWN requesting that it start a B+ transfer.

This is where your setup choice of B+ Enabled or B+ Disabled comes

in. If you do not have B+ enabled, the request will be ignored and a

"club" character (ASCII 5) will be displayed on your screen. If B+ is

enabled, however, BACKDOWN will automatically load the protocol module

and tell ZiffNet that it is ready to receive. From here on, the transfer

is similar to the Xmodem transfer.

Why would you ever want to disable automatic B+ detection? You would

do so if you are connecting to a service that does not use B+ and that

needs to send you a "club" character (known as an ENQ). In such a case,

disabling CompuServe B+ recognition will prevent BACKDOWN from sending

a response that is not used by the service.

The example above describes a perfect file transfer, but life is

not always perfect, nor are file transfers. The error messages that

BACKDOWN may display during a download are included in the sidebar

"BACKDOWN Messages."

The ASCII protocol, unlike the others, captures text just as it is

displayed on the dumb terminal screen. It also lets you enter keystrokes

as if no protocol were running. One noticeable difference is that none

of the function keys work while the ASCII protocol is active.

To cancel any protocol before completion of a file download, you

just press the End key. In response, the ASCII protocol simply closes

the receive file, whereas the Xmodem and CompuServe B+ protocols attempt

to abort the transfer gracefully before returning to the dumb terminal

or script.

Remember, while using BACKDOWN to download a file, you may return

to your foreground program at any time except when you are answering

a question that BACKDOWN has displayed for you to answer. You return

to the foreground program by pressing Alt-X.

One final note on downloads: If you initiated the download from

the dumb terminal (as opposed to a script) and you have exited BACKDOWN

and returned to the foreground, BACKDOWN will automatically pop up when

the download is complete.

ADDITIONAL FUNCTIONS

There are three other function keys that perform important tasks. The

first is the F4 key, which causes BACKDOWN to send a single 0.5-second

break signal. The string <<BREAK>> on-screen tells you that the signal

has been sent. The need for and use of the break function varies from

service to service, but wherever you need it, you've got it.

When you want to disconnect from a service, pressing the F3 key

momentarily drops the voltage of the Data Terminal Ready (DTR) signal

to the modem and displays the string <<HANGUP>>. If your modem is

configured to honor DTR, it should disconnect immediately.

If your modem is not configured that way, you may have to type

something like +++ATH and hit Enter. The +++ usually causes a Hayes

modem to return to the command state; the ATH command tells the modem

to hang up.

Generally, you should log out of a remote service before

disconnecting with F3. Logging out is the best way to be sure that

the service knows you have completed your session. This can avoid

anything from a minor annoyance to additional charges for connect

time. In many cases, the remote service breaks the connection

automatically as the result of a logout command.

Finally, the F10 key allows you to enter the name of a BACKDOWN

script to be executed. Scripts free you from regularly retyping commands

that perform everyday tasks, such as dialing a number, logging you into

a service, or retrieving files. A sample script for logging onto ZiffNet

is shown in Figure 4 (See Below), but it must be compiled before BACKDOWN

can run it. After pressing F10 you can enter not only the script name but

also up to nine blank delimited operands after that name.

Figure 5 (See Below) provides an example of how such parameters are used,

but for a full discussion of BACKDOWN's script language and procedures

you should turn to the sidebar "Creating and Using a BACKDOWN Script."

There are several things that BACKDOWN does not do. For instance,

it has no upload capability. Nor does it provide any terminal emulation

beyond a basic teletype-style interface. Further, its script language is

more limited than we would like it to be.

Even in the absence of these features, however, we hope you are among

the many PC Magazine readers who will not only use BACKDOWN but will

wonder how they ever got along without it.

Bob Flanders is president and senior systems analyst and Michael Holmes

is vice president and senior systems analyst at NCI Systems in Annandale,

Virgina.

-----------------------------------------------------------------------------

SIDEBAR 1: CREATING AND USING A BACKDOWN SCRIPT

by Michael Holmes

Typing in the same commands every time you use a communications service

is tedious and time-consuming. By using BACKDOWN's script language you

can automate such repetitive procedures. With a short script you could

log on to a BBS or set up to download a file; a longer script might

handle the whole session, from when you log-on through when you hang-up.

The three optional command line parameters that relate to the use

of BACKDOWN scripts were not explained in the main article. These are

BD [Sn] [/C script] [script [operands]]

When loading initially, BACKDOWN normally reserves a 1KB buffer to hold

a compiled script file and store script variables. Should you write an

exceptionally long script (or one with many script variables), you may

need to use the /S switch to increase the size of the script buffer

to n bytes. For example, if you have a script that requires 1,500 bytes

and you are starting BACKDOWN on COM port 2, you would enter

BD /P2 /S1500

You can write script source files with any text or word processor that

produces an ASCII file. To save memory space and execution time, this

file must be compiled by BACKDOWN before it can be used. To compile a

script file, you enter the following command at the DOS prompt:

BD /C script

Although it is not mandatory, your ASCII script source file should

normally be given the file extension .BDS. (If no extension is specified,

BACKDOWN assumes this extension.) The compiled script will have the same

filename but will automatically be given a .BDC extension. On completion

of the compile, BACKDOWN displays the size of the .BDC file. The number

shown is the minimum number of bytes needed for the script buffer;

depending on the number and length of parameters used in the script,

more buffer space may be required.

Once written and compiled, a BACKDOWN script file can be executed

either from the command line or by pressing F10 from the interactive dumb

terminal screen. For example, to execute the TESTPARM.BDS script shown

in Figure 5, passing its space-delimited parameters to it, you could

enter the following command:

BD TESTPARM Alpha Beta Gamma

Note that if this command were entered when BACKDOWN was not already

resident in memory, the utility would automatically be loaded and the

script would be started in the background. The same script and parameters

could be specified after installing BACKDOWN by pressing F10.

THE BACKDOWN SCRIPT LANGUAGE

In preparing an ASCII script source file for compilation, the BACKDOWN

script language recognizes 14 commands, together with their operands.

These are listed, explained, and exemplified below.

Command

O [300|1200|2400|9600]

[N81|E71|O71]

[7BIT|8BIT]

[B+|B-]

[CASE|NOCASE]

The operands for the O (Options) command generally correspond to the

settings that may be selected by pressing F2 after BACKDOWN is

memory-resident. These options include communications parameters for

BACKDOWN's COM port, its display mode, and automatic recognition of

CompuServe's B+ protocol. In addition, the O command can control case

recognition of responses from the communications service.

In addition to the baud rates listed, BACKDOWN supports data

formats of no parity, 8 data and 1 stop bit (N81); even parity, 7 data

and 1 stop bit (E71); and odd parity, 7 data and 1 stop bit (O71). The

7BIT option strips the high-order bit from characters before they are

displayed. With this option specified, data will display correctly if

your communications parameters are none,8,1 while those of the service

are set at even or odd,7,1. The 8BIT operand causes the display of all

characters as full 8-bit values, with no masking of the most significant

bit.

The B+ and B- operands respectively enable and disable the

automatic recognition of the CompuServe B+ protocol. The CASE operand

enables case recognition of responses to the R command (explained below);

NOCASE disables case recognition. For example, if you specified NOCASE

and used the command

R FORUM

any string containing the word forum would satisfy the wait, regardless

of case.

Multiple operands, separated by a space, may appear on the same

line as an O statement, and the O statement can occur more than once in

the script.

Example:

O 2400 N81 7BIT B+ NOCASE

Command

S text

The S (Send String) command sends a text string to the remote computer.

This command may contain embedded control characters, such as ^M (carriage

return) or ^G (bell). It may also contain references to variables that

will be replaced with specified character strings at execution time.

(See the P command below for more about variables.)

For example:

S DOW PCSPOO.ZIP /PROTO:B^M

Command

N text

The N (Notify Operator) command displays a text string on BACKDOWN's

screen when the TSR is popped up. The text string for this command, like

that for the S command, can contain both control codes and replacement

variables.

For example:

N ^G -- Download complete -- ^G^M

Command

R text

The R (Wait for Remote) command waits for the data stream from a remote

computer to contain a specified character string (text). A time-out occurs

if the remote does not send the proper characters in the time specified

by the T command (explained next).

The text string for the R command may contain control characters,

but replaceable variables are not supported. The CASE and NOCASE option

of the O command determines whether the R command is case-sensitive.

For example:

R Userid:

Command

T nn [label]

The T (Set Time-out) command sets a time-out value of nn seconds, to

be used by the R command. The optional label designates a specified

statement to be executed should a time-out occur. If no label is given,

control passes to the next statement in the script.

For example:

T 60 NotSeen

S GO ZNT:UTILFOR^M

R Forum !

.

.

:NotSeen

n Timeout waiting for Forum prompt.^M

s bye^M

Command

H

The H (Hangup) command causes the DTR line on the serial port to drop

momentarily, disconnecting the machine from a remote service.

For example:

H

Command

D p [filename]

The D (Download) command invokes a specified download protocol module,

p.  The protocol letter is the same as that used in interactive mode.

The additional download filename is optional, but the Xmodem and ASCII protocols will prompt for it if one is not specified.

When the ASCII protocol is invoked, even from a script, control

is returned to the keyboard. After opening the received file, ASCII

sends a carriage return to the remote service. Thereafter, characters

entered at the keyboard are sent out the active COM port; received

characters are both displayed and placed in the download file. Pressing

the End key stops the download and returns control to the script. If 30

seconds elapse with no data received, the ASCII protocol closes the

receive file and returns to the script.

For example:

R LIB

S TYPE CDX.DOC

D A CDX.DOC

In the example above, note that the S command does not end with a

carriage return (^M). This is because the ASCII protocol will send a

carriage return when it starts running.

If Xmodem is used, downloading begins as soon as the receive file

is opened and progresses until the download is complete. To cancel

receipt of a file prematurely, press the End key. The following example

shows script statements to start an Xmodem download from ZiffNet.

R LIB

S DOW CHKFRG.ZIP /PROTO:XMODEM^M

R complete.

D X C:\DL\CHKFRG.ZIP

S ^M

CompuServe's B+ protocol should not be invoked with the D statement.

The B+ invocation occurs automatically when the system is ready to send

a file.  Use the R or the W command to wait for the invocation of the

CompuServe B+ protocol.

For example:

R LIB

S DOW SLICE.ZIP /PROTO:B

W 60

Command

P letter prompt_string

The P (Prompt and Reply) command displays the specified prompt string

and loads the response into a variable designated by a single letter in

the range from A to Z. Variables are referenced by placing a percent

sign in front of the letter of each variable.

For example:

P A Enter your name:

N ^M Your name is: %A^M

Command

= letter [text]

The = (Equate) command assigns a text string to a script variable letter.

When the text operand is omitted, BACKDOWN assigns a null string to the

variable.

For example:

= F ZCOPY.COM

Command

W nn

The W (Wait) command temporarily halts execution of the script for

nn seconds. During this period of time, any arriving characters are

displayed on BACKDOWN's pop-up screen.

For example:

W 25

Command

E n

The E (Echo Control) command sets the display characteristics to

n during script execution. These are the values of n:

0 stops display of all data.

1 displays only the data received by BACKDOWN. The host may

echo any data sent from BACKDOWN.

2 displays only data sent by BACKDOWN. Received data is not displayed.

3 displays data sent from and received by BACKDOWN. This may

result in characters appearing twice on the screen.

If no E command is specified, the value defaults to 1.

For example:

E 3

Command:

G label

The G (Goto) command transfers control to the point in the script at

which label is found. The transfer is an unconditional forward or

backward jump.

For example:

G GetLaserlst

Command

:label

The :Label command marks a particular point in a script as the target

for a G or a T command.

For example:

:GetLaserlst

S DOW LASERL.ZIP /PROTO:B

W 15

Command:

; comment

You can include documentation in your scripts by using comment statements.

A comment statement contains a semicolon as the first character on the

line.  Any text appearing after the semicolon is ignored by the compiler.

-----------------------------------------------------------------------------

SIDEBAR 2: BACKDOWN MESSAGES

by Bob Flanders

When two computers have to communicate across telephone lines, occasional

errors are bound to occur. When this happens, BACKDOWN will notify you,

either on the screen directly (if you're working interactively) or by

popping up over your application.  Many of the messages the program sends

depend on the specific protocol you have selected, so they are listed by

protocol and followed by brief explanations.

XMODEM: Timeout, protocol ended

B+: Timeout, protocol ended

ASCII:  No Characters seen for 30 seconds, protocol ended

The download is canceled when the message above is displayed.

XMODEM: Too many errors, download cancelled

B+: Too many errors

As a transfer proceeds, BACKDOWN is constantly checking for errors.

If too many errors occur in a row, the download is canceled.

B+: Receiving filename

The message showing the name of the file being received is displayed

when a transfer begins with the CompuServe B+ protocol.

XMODEM: User cancelled download

B+: Download cancelled

If you press the End key during a transfer, BACKDOWN will display this

message and attempt to end the transfer gracefully.

XMODEM: Sender cancelled download

If the service you are using stops the download, BACKDOWN tells you

about it with this message.

XMODEM: Write error. Protocol aborted

B+: Aborted: write error

This error occurs if BACKDOWN is unable to write data to the file you

have specified, as may happen, for example, if the disk fills up.

XMODEM: Packet sequence error

This is an unusual error in Xmodem where a piece of the file being

transferred has been irrecoverably lost, so the transfer is canceled.

XMODEM: Download completed OK

B+: Download completed OK

This message appears after a successful download.

----------------------------------------------------------------------------

ZIFFNET.BDS     COMPLETE LISTING

Figure 4: After you compile it under BACKDOWN, this sample script will log you onto ZiffNet. Note that the user ID and password shown must be replaced with your user Id and password.

; Dial up Ziffnet and log in.

; This script is designed to work with a modem using an AT-command set.

O 2400 8N1 7BIT B+ NOCASE

; Init the modem

T 5 ReInit

:Init

S ATZ^M

W 3

S ATE0V1^M

R OK

G Dial

; Only reaches here if first init fails

:ReInit

T 5 FailInit

G Init

; Die gracefully if modem does not respond

:FailInit

N Modem did not respond^M

G End

; Dial ZiffNet

:Dial

N ^MDialing ..

T 60 NoAnswer

W 1

; Replace "xxx-xxxx" with your local Compuserve telephone number

S ATDTxxx-xxxx^M

R CONNECT

W 1

N ^MConnected^m

W 2

; Send userid

S ^C

R User ID:

; Replace "yyyyy,yyy" with your account number

S yyyyy,yyy^M

; Send Password

R Password:

; Replace "Your:password:Here" with your password

S Your:password:Here^M

N ^M^G --- Login Complete --- ^G^M

G End

: NoAnswer - Hangup and tell operator

H

N ^G -- Line busy or your modem isn't working -- ^G^M

: End

; Recompile this script with the command BD /C ZIFFNET

----------------------------------------------------------------------------

TESTPARM.BDS     COMPLETE LISTING

Figure 5: This short script demonstrates how command line parameters are specified. Much like a batch file's parameters, they are referenced as %1, %2, and so on.

If you have the following script called TESTPARM:

N Parm 1 is %1^M

N Parm 2 is %2^M

N Parm 3 is %3^M

Here is a sample execution:

Script name: TESTPARM Alpha Beta Gamma

Parm 1 is Alpha

Parm 2 is Beta

Parm 3 is Gamma

----------------------------------------------------------------------------