MR. MACHINIST (tm) COPYRIGHT (c) 1996 BY F1 COMPUTING

���������������������������������������������������������ͻ

�                                                         �

�               Mr. Machinist OnLine Manual               �

�               ===========================               �

�                   by Allan J. Bremer                    �

�                   ( fone@flash.net )                    �

�                                                         �

�        Copyright (c) 1992 - 1996 by F1 Computing        �

�                                                         �

�                 Manual Date: 08-15-96                   �

�                                                         �

���������������������������������������������������������ͼ

Customer Service

Customer Service Phone Number: 1-800-949-4724

1-817-221-4724 (Metro)

Mr. Machinist is always available on the Internet from:

www.flash.net/~fone

This manual is continually  being up-dated and may or may not

contain the most recent information.  The Help File is always

up-dated at the same time the program is so  if you don't see

information  on  a  certain  function  in this manual try the

Help File by striking the 'F1' key at any time.

A) PREFACE

Welcome to Mr. Machinist.

Mr. Machinist is a program for the machinist, the engineer, and any

one else who is involved in the process of design and engineering.

It was developed for the machinist who is new at computers and

especially revolves around the CNC milling and turning machinist

and operator. The program is a useful tool and time saver for the

CAD/CAM programmer also. In fact, Mr. Machinist can give you an

answer to many problems faster than the shops CAM software can

start up!

This easy to use menu driven program is designed to solve the every

day geometry and trigonometry problems that machinist and

industrial engineers encounter.

Get the chord of a circular sector, convert decimal degrees to

hours, minutes, and seconds, find the proper speed and feed for a

machining center, view a decimal chart, use the scientific

calculator for formulas, edit multiple files that were created

manually or by your CAM software, insert or delete N sequence

numbers from CNC text files, 'split' a large CNC text file into

their appropriate program numbers instead of cutting and pasting.

You also have the option to strike F1 for instant help on hundreds

of subjects. These are just some of the many possibilities of Mr.

Machinist.

One major advantage of Mr. Machinist is its ability to continually

keep an answer file of all the functions you choose.  This Answer

file is constantly being updated as you require answers to

functions.  You may view the answer file or print it at will.  This

answer file, called MRMSOLVE.TXT, is created or appended to every

time you run the program. You can start a new answer file any time

by using 'Alt-Minus Key'. There are other files that are created if

certain functions are preformed and they are the Drill Speed and

Feed function, the Bolt Circle function, the Formula function, and

the Circle Divide function.  The reason behind this is that these

functions, at times, require a large file or separate piece of

information.  You may read about these functions in more detail in

their appropriate sections of this manual.

The Mr. Machinist program is intended for any manufacturing system

that wants to save time and money. Some operators of machine tools

can spend a lot of time trying to figure out problems like

trigonometry or getting the chord of a circle. Some of these

functions require more than a few entries in a calculator and can

cause a machinist to sit down and scratch there head for an hour

causing all kinds of lost time.

Ultimately, the best way to use the Mr. Machinist program is to put

a computer in the center of the shop and let everyone who needs it

use it. Buy an old used computer with a mouse and run Mr. Machinist

from it. The program can run on any DOS based computer, as long as

it is a 286 or greater processor with at least 2 megs of RAM

memory.  An 'XT' version will be out shortly  that will run on any

DOS based computer.  (Give us a call!)

B) LEGAL NOTICE

Mr. Machinist is distributed as is. The author disclaims all

warranties, expressed or implied. The author will assume no

liability for damages either from the direct use of this product or

as a consequence of the use of this product.

1 INTRODUCTION

1.1 WHAT IS THE MR. MACHINIST PROGRAM?

Mr. Machinist is a program that was developed by a CNC machinist

for other machinist, programmers and engineers.  It is a menu

driven, text based environment with the novice computer user in

mind.

Mr. Machinist's main purpose is to save time and money and

increase production in a shop environment by solving math

calculations with the least amount of data input.

The Mr. Machinist program is not a CAD/CAM program nor is it a

drawing program. It is meant to be used along with CAM software to

further the programmers capabilities and speed up production time.

One example is finding the chord of a circle. If you wanted to find

the chord of a circle in a major CAM software package, nine times

out of ten you will have to draw the circle and check the distance.

It takes about ten seconds to find a chord in Mr. Machinist!

Mr. Machinist can also find the solutions to many other shop

math orientated calculations like trigonometry, geometry,

conversions, areas and volumes, and the program can also help the

machinist in calculating speeds and feeds, chip load and surface

feet per minute.  It also has the capability to create G-Code

programs for Bolt Circles and other shapes.

1.2 WHY USE THE MR. MACHINIST PROGRAM?

Use Mr. Machinist to increase productivity, decrease down time, and

increase machining knowledge.  Mr. Machinist is also a great

program to introduce to machinist who want to get into using a

computer for the first time.  90% of the people in the average

machine shop have never had the opportunity to acquire knowledge of

computers. Here is a program that will do that and at the same time

make there job a lot easier!

1.3 HISTORY OF THE MR. MACHINIST PROGRAM

1985 - A passing thought...

1990 - A trig problem on a Vic 20...

1991 - Mr. Machinist Beta Version in Pascal...

1992 - Started developing program in October of 1992 and began

to release the Beta versions to major bulletin boards

in January of 1993.

1993 - Version 19.0 Beta, the last Beta version. ( 10-01-93 )

1993 - Version 1.00 - Released 10-31-93

1.4 HOW TO USE THIS MANUAL

We suggest you read the first four chapters to acquaint yourself

with the basics of this program, the requirements and options of

the program, and the installation procedure.  Then browse the

remainder to get an idea of its content.  Once the program is

installed and running, refer to the references for the specific

area that you may have a question about.

A HINT:  If this file is plain ASCII text, you could save the

original and make a copy and then insert any specific key for a

text search to any topic you use frequently.  You could also delete

portions and modify the table of contents.  Then call up the file

in a second ( or third ) window in the editor and you would have an

instant reference.  This process would give a searchable,

scrollable supplement to the on-line help from the F1 key.

*< NOTE >*  The editor in this program has a capacity of about 64K

which is smaller than this file.  You will not be able to view this

manual using the Editor in Mr. Machinist.  To read this manual just

choose 'Read Manual' from the 'File' menu.

To get the most out of the Mr. Machinist program, you must read

this manual carefully.  It will answer any questions you might have

about Mr. Machinist's operation.  You are able to search through

this document by line numbers if you are viewing it from a computer

program.  There are many programs on the market that can do this

but I believe the LIST program by Vernon D. Buerg to be the best.

It is assumed that you have access to a DOS manual and you will try

your best to get acquainted with the different commands that DOS

has to offer. Even though Mr. Machinist is an easy to use menu

driven program it is still a good idea to get an understanding of

the basics of DOS.

Instead of using the phrase, 'Mr. Machinist' in this document you

will see the letters 'MRM' which mean the same thing.  This will

save a lot of time and space.

1.5 UN-REGISTERED VERSION INFORMATION

The free distribution of the Mr. Machinist program gives users a

chance to try it before buying it.  If you try this program and

continue using it, you are expected to register.

Copyright laws apply to the Mr. Machinist program and the copyright

holder retains all rights, with a few specific exceptions as stated

below.

The author specifically grants the right to copy and distribute the

software, either to all and sundry or to a specific group.

The Mr. Machinist program fits your needs easier, because you can

try it before you buy it.

The goal of F1 Computing is to continually evolve Mr. Machinist

into a program that will meet the needs of the manufacturing

community at large while at the same time teach it's users the

calculations and methods used in the industry today.

1.6 DISCLAIMER - Agreement

Users of Mr. Machinist must accept this disclaimer of  warranty:

"Mr. Machinist is supplied as is.  The author disclaims all

warranties, expressed or implied, including, without limitation,

the warranties of merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or

consequential, which may result from the use of Mr. Machinist."

Mr. Machinist is provided at no charge to the user for evaluation.

Feel free to give it to your friends, but please do not give it

away altered or as part of another system.  The essence of the Mr.

Machinist program is to provide personal computer users with

quality manufacturing software without high prices, and yet to

provide incentive for the author to continue to develop new

functions and procedures related to manufacturing.  If you find

this program useful and find that you are using Mr. Machinist and

continue to use Mr. Machinist after a reasonable trial period, you

must make a registration payment of $98.00 to F1 COMPUTING.  The

$98.00 registration fee will license one copy for use on any one

computer at any one time.

You must treat this software just like a book.  An example is that

this software may be used by any number of people and may be freely

moved from one computer location to another, so long as there is no

possibility of it being used at one location while it's being used

at another.  Just as a book cannot be read by two different persons

at the same time.

Commercial users of Mr. Machinist must register and pay for their

copies of Mr. Machinist within 30 days of first use or their

license is withdrawn.  Site-License arrangements may be made by

contacting F1 Computing.  There are many great advantages of

obtaining a Site-License.  Up-grades are 25% cheaper!  If you plan

on using Mr. Machinist on more than one computer then you will need

a Site-License.  The MRMORDER.DOC file is provided for

registration.

Anyone distributing Mr. Machinist for any kind of remuneration must

first contact F1 Computing at the address below for authorization.

You are encouraged to pass a copy of Mr. Machinist along to your

friends for evaluation.  Please encourage them to register their

copy if they find that they can use it.  All registered users will

receive a copy of the latest version of the Mr. Machinist system.

2. GETTING STARTED

2.1 BACKING UP YOUR DISK AND DATA

The first thing you must do is to make a working copy of your

original disk. Because disks are susceptible to damage, it is

unwise to use the original program disk for regular use. If it is

ever lost or damaged you would have to purchase a new disk or spend

more time on the phone lines downloading the program.

If you should lose or damage the working copy, you can always use

the original program disk to make another working copy.

In order to make a working copy of your original disk or file you

will need to use one of the DOS commands that can transfer or copy

the contents of your original disk onto another disk.

Here is one example that will copy ALL of the contents of one disk

onto another:

Let us assume that you have the Mr. Machinist program on a high

density 3.5 disk and you want to make a backup copy on another 3.5

high density disk. You could use the command called DISKCOPY. One

thing to keep in mind about the DISKCOPY command is that if you use

it on a 3.5 high density disk then you have to have a 3.5 high

density disk to copy it to.  Both disk must be the same format!

Insert the original disk into your drive, get onto that drive by

typing its letter and a colon, ( A: ).

Now type the command, ( DISKCOPY ).  At this point your system will

ask you to insert the SOURCE disk.  The source disk is your

original disk.  After it reads a sufficient amount of data into

your RAM memory it will ask you to insert your TARGET disk, that's

your backup or working copy disk.  Be careful not to get your

source and target disks mixed up.  If your using high density disk

and there is a lot of data on it then it can take a few times of

inserting the target and source.

The only real way to get efficient at using DOS commands is to use

them and read a book that explains DOS in a way that you can

understand.  Some people can learn it well from the DOS manual and

others may want to go out and purchase a beginners guide to DOS.

Either way, you will do yourself a favor by getting good at DOS!

2.2 HARDWARE REQUIREMENTS

Any IBM or IBM compatible 286 + computer with at least 2megs of RAM

can operate the Mr. Machinist program.  It does not matter what type

of video adapter you have either, it will run in color, in mono, in

Hercules, CGA, EGA, VGA, SVGA, it's all automatic.  The program

needs at least 2 megs of RAM  memory and it also uses the standard

RAM memory.  It also has no use for a math co-processor but having one

would be a good idea.  A mouse is an excellent way to run the

Mr. Machinist program but it is not required.  The Mr. Machinist program

must be executed from a hard drive.  It is possible for it to run on a

floppy disk but it is very slow in execution and very non-productive.

DOS version 3.3 or above is required to run the program. It is very

advisable to run your system under DOS version 5.0 or above.  This

version is a much better version than the older versions and it

also has a little help file that you can use to get help on almost

any DOS command.  Many people never read their DOS manual and

therefore they are lacking in the knowledge of operating system

commands.  If you have DOS version 5 or above a good command to

remember is the help command.  Let's say that you need help on the

COPY command.  You would type, 'COPY /?' and then on-line help will

pop up.

Mr. Machinist can utilize a printer also and one is highly

recommended.  There may be times when a CNC programmer may want to

take a copy of the program to there machine for a reference.

One thing that is not needed to run Mr. Machinist is an expensive

computer.

2.3 FILES

There are three files that are needed to run Mr. Machinist: MRM.EXE

(MRM.EXE), DPMI16B.OVL, and RTM.EXE.  There are other files that

come with the program disk but are not required to execute the

program.  The fourth most important file is MRMHELP.HLP, the help

file.

Here is a list of the files that are included in the Mr. Machinist

System:

FILES.TXT    =  File List

RTMRES.EXE   =  Run Time Manager Set Up (Optional)

DPMIUSER.DOC =  How to use DPMI, Dos Protected Mode Interface

SFM.TXT      =  The Surface Per Minute file

INSTALL.EXE  =  Mr. Machinist's installation program.

MRMORDER.DOC =  The order form to register the program.

MRM.EXE      =  The main executable file.

DPMIINST.EXE =  Sets up DPMI for Problem Systems

DPMI16B.OVL  =  The Dos Protected Mode Interface Overlay File.

MRMANUAL.DOC =  The Mr. Machinist Manual. (Your reading it!)

MRMHELP.HLP  =  The help file.

README.TXT   =  First time information about Mr. Machinist.

READMRM.EXE  =  Executable Manual Reading Program

RTM.EXE      =  The Run Time Manager

2.4 FILES CREATED ON YOUR DRIVE

Mr. Machinist Text Files

========================

The Mr. Machinist program can create, edit, and delete five

different files on your hard drive upon each execution.

- MRMSOLVE.TXT -

This is the answer file that holds all of the answers to each

function except Drill Speeds/Feeds, Bolt Circle Calculations,

Circle Divide Functions, and Formula Writes.  'Ctrl-F1' views it

and 'Ctrl-P' Prints it.

- MRMDRILL.TXT -

This is a file that is created only when you choose the Drill Speed

and Feed Function.  'Ctrl-F4' views it and 'Ctrl-F6' prints the

file.

- MRMBOLT.TXT -

This is a file that is created only when you choose the bolt circle

calculation function.  'Ctrl-F8' views it and 'Ctrl-F9' prints the

file.

- MRMCIRCD.TXT -

This is a file that is created only when you choose the Circle

Divide Function.

- FORMULA.TXT -

This is a file that is created when you choose the Formula

function. 'Shift-F9' views it and 'Shift-F10' prints it.

All of these files are created the very first time you execute

there appropriate function.  After that they are continually

appended (added) to.  If any of these files exist then they will

continue to grow and grow with each execution.  It is advisable to

do a 'Alt-Minus' function now and then in order to delete these

unwanted files or save them.  By starting the program with the

'/clean' parameter, you will do the same thing as an 'Alt-Minus'.

2.5 INSTALLATION   (Read README.TXT file for install updates!)

The installation program makes it easy for a user to transfer the

proper files from a disk to their hard drive.  The installation

program is only meant to be executed from a disk drive and not the

hard drive.  Go to your floppy drive by typing 'A:' and then type

the word 'install'.

All of the files listed on page 14 are in a compressed file called

MRMMAIN.EXE.  After the installation program is complete you are

taken to the directory where the files were copied to and you are

asked to execute MRMMAIN.EXE.  This will un-compressed all files.

The reason for this compression is to allow the distribution of all

files on one disk.  When these files are un-compressed you may then

delete the file called MRMMAIN.EXE.

2.6 QUICK START

If you have been following this manual then you should have a

working copy of the Mr. Machinist program and your original disk is

put away some where.  If your not running the program from a disk,

which is true 99% of the time, then you should have two working

copies of the Mr. Machinist program and also your original disk.

To start-up Mr. Machinist simply enter, 'MRM'.  To preform a

function, hold down the 'Alt' key and strike the highlighted letter

of your choice from the upper menu.  If you wanted to edit a file

you would hold down the 'Alt' key and then strike 'F', this will

bring down the file menu.  From there you could choose from a new

file to one that already exists.  At this time you can see all the

other menus by simply pressing your right or left arrow keys.

Press the 'Enter' key to view the choices under the menu item.

Always remember that help is always near by simply striking the

'F1' key.

MOUSE Notes - The Mr. Machinist program is meant to be used with a

mouse.  By using a mouse you can resize windows, cut  and paste

text faster, hold your coffee in one hand and work with the other,

etc.  Some computers are set up to use Windows(tm) only and

therefore do not have a mouse driver installed.  To install a mouse

driver, check your mouse package or get your computer person to

hook you up.

HELP FILE Notes - Please remember, the most powerful type of help

is not this manual but the help file within the program.  F1

Computing is continually up-grading this software, sometimes on a

daily basis.  Because of this, the manual is reformatted and up-

graded only about once a month.  When a new function is put into

the program, a help file command has to be assigned to it and that

is why the emphasis is on the help file and not the manual.

2.7 ADDITIONAL MATERIALS

It is very advisable to own your own Machinery's Handbook.  The

latest one at this writing is the 24th Edition.  This book is

considered to be the Machinist's 2nd Bible, the 1st Bible should be

the real one! (KJV, NIV, NAS, Phillips, Amp, etc.)

There are many books about formulas and feed rates and engineering

calculations available at your public library.  There is one

excellent publisher that is considered to be the main supplier of

industrial books and that is the Industrial Press, Inc.  Write or

call them for a free catalog of all their books.

INDUSTRIAL PRESS, INC.

200 MADISON AVENUE

NEW YORK, NY 10016

PHONE (212)889-6330

2.8 REGISTRATION  (Print an Invoice from the 'File' menu!)

MR. MACHINIST (TM) for MS-DOS

INDIVIDUAL USE

* Diskette with programs and documentation ... $98.00

SITE LICENSE

============

The following information describes the Mr. Machinist

registration fees for use on more than one computer:

Site licenses are available for Mr. Machinist.  A site

license is needed if Mr. Machinist will be used on more than

one computer.  The rates of these licenses are as follows:

2  to     9 computers ..... at $78 each

10  to    24 computers ..... at $60 each

Write or call for pricing on quantities over 24.

FUTURE ENHANCEMENTS

Job log database

Points/Tangent math

Larger Edit Buffer

CNC Communications

News letters (Registered users)

F1 Computing

PO Box 1093

Newark, TX 76071-1093

email: fone@flash.net

www.flash.net/~fone

1-817-221-4724

1-800-949-4724

See the file called MRMORDER.DOC for more registration

information.

2.9 CUSTOMIZATION

If there are certain formulas or functions and/or procedures that

your company or you would like to see in MRM let us know on the

registration order form.  You or your place of employment may use

a standard formula that requires a lot of input and time to get the

result.  Well, with Mr. Machinist you only need input the known

values and all other formulas are done instantly!  Still figuring

out formula upon formula using your little calculator?  Get it into

Mr. Machinist and get productive!

Customization can be done for registered users.  The cost depends

upon the time and size of the procedure but most formulas can be

implemented at the standard cost of 50% of the registration fee.

For further information on customization, contact the author at the

address given.

3. SUPPORT POLICIES

3.1 TECHNICAL SUPPORT BBS

There is not a support BBS for the Mr. Machinist program at this

time.  One of the best ways to check out the latest version is

through the Internet!  The Mr. Machinist home page is located at:

http://www.flash.net/~fone or you may call these BBS's:

SME BBS:           313-271-3424                28800 - 24hrs

Exec PC:           414-789-4390                28800 - 24hrs

If you have trouble getting a copy of the latest version please

leave email to: f1@sunlink.net.  You may also request a copy

from the author and you will be sent the latest version on a disk

size that you specify.

3.2 VOICE CALLS

There is a customer service number for registered users to call.

If unregistered users wish to speak to the author to confirm his

existence or to give comments and suggestions then they are free to

do so.  We cannot, however, accept collect calls. Customer service

phone number:(817)221-4724 (Voice)

3.3 MAIL ( Snail Mail or e-mail )

If you have comments or suggestions or bug reports then you can

certainly send them in the mail.  When you register you will print

and fill out the included file called MRMORDER.DOC and send it to:

F1 COMPUTING

PO BOX 1093

NEWARK, TX 76071

4. REFERENCE

4.1 INTRODUCTION

The reference section of this manual is designed to give you the

information you need to understand the functions of the Mr.

Machinist program and at the same time to explain the reasoning

behind some of the methods.  It takes you through all of the

functions of the program and gives you many hints on how to operate

the program in such a way as to give you peak performance and

productivity.

Remember, if your viewing this file from your computer you may use

a program like LIST by Vernon D. Buerg (c) and use the line number

functions to search the document.  The line number functions for

the list program are the '+', '-', and the '#' keys.  If you hold

Shift and strike the '#' key you will be prompted for the line

number to go to.

If you strike the '+' key you will be prompted for the line number

to go to forward from the point you are at.  If you strike the '-'

key you will be prompted for the line number to go to backward from

the point you are at.  This method makes it very simple to get to

where you want to be in the document at any given time.

4.2 BRINGING UP MENUS

In this type of standard interface you will find there are many

different ways to do the same thing.  You can strike the 'F10' key

and the cursor will enter into the top menu words.  From there you

can move your arrow keys to the left or right to land on the menu

of your choice.  Once your on the menu title you can then strike

'Enter' and the menu will be 'pulled down' for your viewing.  Using

your arrow keys once again you can go up or down in the menu to

highlight your choice.  When your menu choice is highlighted you

may strike 'Enter' to 'pop up' your menu.

The fastest method of menuing in Mr. Machinist is by using your

mouse to point and shoot.  To bring up a menu using this method you

would click on the menu title to pull down the menu and then click

on your menu choice to execute the function.  For right handed

people the left mouse button is used to click.  If your left handed

you can use the mouse option in Mr. Machinist to switch buttons so

that the right mouse button is the execute switch.  Keep in mind

that there is a time limit to your double clicking. If you double

click too slow your choice will not be executed.  Mr. Machinist

does have an option that you can choose in order to slow down or

speed up your mouse double click.

4.3 MOVING AROUND

In no time at all you will begin to feel at home with this type of

interface or 'look'.  As you get familiar with the program you will

learn to 'drag' the windows around, edit files, print, and do math

functions without ever getting rid of a window.  This type of

interface is used very widely in the computer environment.  The

things you will learn while using this program will be used in many

other programs that you come across.

4.4 ENTERING DATA

In order to get answers you must give questions and the way to give

questions in MRM is to input data in input lines.  The three most

important keys that have to do with an input line is the 'Enter'

key, the 'Tab' key and the 'Shift' key.

These are the keys that cycle you through the dialog boxes.  In

other words you can move to the different choices in a dialog box

by using these keys.  Using a mouse you would have to click once in

the input line and you will be ready for input.  You may also use

your arrow keys to cycle through input lines.

Some input lines will only take certain types of data.  In most

input lines you will be expected to insert numerical data and not

letters.  If you happen to input the wrong type of data into an

input line, you will be displayed an error message telling you what

you did wrong and then you will be taken back to the dialog box so

you can start over again.

4.5 THE TITLE BAR

Every window has a title bar.  The title bar is simply the name of

the file that is associated with this particular window.  The title

bar of a dialog box us usually an indicator as to the function of

that dialog.

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

4.6 SYSTEMS DATE

If your computer system has the proper date set then MRM will

display today's date at the top right of your screen.  Today's date

will also be inserted automatically into the answer file called

MRMSOLVE.TXT which is created on your hard drive each time the

program is executed or run.  You will also observe the date that

the program and help file was compiled or created.  From the main

program you can do an 'Alt-F1' and the 'about' information box will

be displayed.  An 'Alt-F1' is done by holding down the 'Alt' key

and then tapping the F1 key.  After you tap 'F1' you then let up on

the 'Alt' key.

4.7 EXITING MR. MACHINIST

You exit MRM by 'Alt-X', clicking on the bottom portion of the

screen over the letters 'Alt-X', or pulling down the file menu and

then selecting 'Alt-X'.  You will find that through out MRM there

will be many ways to do the same thing.  This gives you the chance

to do what you like to do best.

In the un-registered version of the program you will be asked if

you would like to print a registration invoice so you can be a

registered user.  If you choose 'Yes' you will print a copy of the

file MRMORDER.DOC on your printer.  If your printer is not working

properly for some reason then you will be displayed an Error box

telling you that there is a problem.  If you choose 'No' from the

print invoice message you will exit with a information message just

before DOS takes over.

These 'nag' screens will not appear in the registered version.  An

unregistered version of MRM is a little slow due to these start and

finish nag screens.  To be productive with the program you must

become a registered user.

4.8 MANIPULATING WINDOWS

Many windows can be open at one time while your using MRM.  At

times there may be windows that are blocking the view of other

windows, this is when a user needs to know how to manipulate

windows quickly.

Windows are manipulated by the mouse, the keyboard, or both mouse

and keyboard.  Manipulation is done by resizing and/or moving.

4.8.1 Using a mouse to move windows

You can move a window using the mouse by aiming for the

'header' of the window and then 'dragging' the mouse.  The

header is the top double line of a window and it usually has a

file name at the top or a caption.  To 'drag' the mouse means

to click the mouse and hold the button down while moving the

mouse around.  Once you get to where you want to be you release

the button.  You can resize the window using the mouse by

'dragging' the lower right hand corner of the window.  The

resize corner looks different than any other corner of the

window.

4.8.1 Using a mouse to move windows (Con't)

To 'zoom' a window using the mouse you must double click

anywhere on the top line of a window.  When this is done the

window will take up all of the screen except the program

headers and footers.  If you double click the top of the window

again it will go back to it's original size.  You can also

cycle through window sizes the same way by striking the 'F5'

key.

You can activate a window by clicking anywhere on it or in it.

The active window is shown by a double outlined box and all the

other windows are single lined.  Always remember, some windows

may be hidden by other windows and therefore you can't find

them to click on them!

Well, no problem, you could either move your windows around to

see if there are other windows behind them or you can strike

the 'F6' key which is the 'next' key.  Using the 'next'

function you will be cycled through the windows in the order

that they were brought up.

To close a window using the mouse you would go to the upper

left of any window and there you will see a small box that is

enclosed in brackets, that is the 'close box'.  Clicking on the

close box will close the window for good.

4.8.2 Using the keyboard to move windows

To fastest way to access the windows menu is to use 'Alt-W'.

This will bring up the windows menu.  You will notice six

different choices:

Resize/move Ctrl-F5

Zoom             F5

Next             F6

Close        Alt-F3

Tile

Cascade

To resize a window using the keyboard you would highlight the

Resize/move menu choice and then strike 'Enter'.  Of course for

this function to work you have to have a window open in the

first place.  After you strike 'Enter' you will notice that the

window border goes from a double line to a single line.  This

indicates that the computer is in the process of manipulating

that window.  You will not be able to strike other keys or do

other functions until your move is complete.  When the window

has a single line border it is ready and waiting to be moved.

You now must use the arrow keys to move the window in the size

that it is now at.  If you want to make the window smaller or

larger you hold down the 'Shift' key while you use the arrow

keys.  Using these two methods you can place a window anywhere

on the screen and make it any size you want.

After you have completed the move or resize you must do one

last important thing, strike the 'Enter' key to complete the

process!  If you forget to do this you might think that you

locked up your computer when all you really need to do is to

strike 'Enter'.

To 'Zoom' a window to full size and then back to its previous

state you would toggle the 'F5' key or highlight the menu

choice and strike 'Enter'.  To toggle means to go off and on,

back and forth, etc.

The 'F6' key is the 'Next' key which cycles you through your

open windows.  This function is activated by actually striking

the 'F6' key or highlighting the 'Next' key using your arrow

keys and then striking 'Enter'.

To 'Close' a window use 'Alt-F3' or highlight the 'Close' menu

choice using your arrow keys and then strike 'Enter'.

To 'Tile' a window means to place all windows that are

currently open, next to each other.  The meaning of the word

lies behind floor tile.  When you look at a floor that has tile

on it you will notice that all the squares are next to each

other.  This is just how the 'Tile' function works.  To

activate this function you would highlight the 'Tile' menu

choice and then strike 'Enter'.

To 'Cascade' a window means to place one window on top of

another while letting the previous windows header be exposed.

To activate this function you would highlight the 'Cascade'

function and then strike 'Enter'.

4.9 THE HELP FILE  ( F5 will Resize it )

The help file is one of the major functions of MRM.  It is

basically a huge book with many cross references.  When you see

words that are highlighted in yellow you can 'click' on them for

there cross reference.  If you have a monochrome monitor you will

notice the difference of these words by a brighter color.  If you

do not have a mouse you can use the 'Tab' key to access the cross

references.

You will also notice that you can go to an index by striking

'Enter' at any time while your in the help file.  The index is

actually a dictionary of manufacturing terms geared toward a

Machinist or Industrial Engineer.

The help file is actually the manual on-line!  There is a help

function for everything in MRM!  If at any time you have a question

about what you are doing or your stuck at comprehending a certain

function then you should go to the help file.  This help file is

growing rapidly as new versions of MRM come out.

The help file is becoming larger all the time and is a very useful

extension to reference books in the manufacturing field.

There is much more information in the help file than one may

expect.  Decimal charts, formulas, G and M code definitions,

manufacturing history, machining hints, and much more.

4.10 DATE, TIME, AND SCREEN SAVER

The date that your system has accepted will be shown in the upper

right hand corner at the top of the screen.  This date should be

today's date.  If it is not then you should re-set your systems

date by using the DATE command through DOS.

If you have the wrong date on your system then all the files that

your system creates will have the wrong creation date and time

stamped in them.

The time is shown in the lower right hand corner of the screen.

This is the system time and it should be set the same way as the

date.  If it is wrong then you should use the TIME command through

DOS.

There is a screen saver in the Mr. Machinist program that helps

computer monitors from burn-in.  This especially helps monochrome

monitors because they are easily done in by burn-in.  There is no

option for a user to set the time of the screen saver activation at

this time.  Possibly in future versions this option will be

installed.  For the time being, the screen saver is set to begin

after five minutes of nothing happening.  If you touch a key or

move the mouse then the count down will begin all over again.

The message that the screen saver will display can be customized by

registered users when they specify on the registration form.  The

un-registered version shows:

'Move your mouse or strike 'Esc' key to return to Mr. Machinist'

Take advantage of your screen saver message.  It could state your

company's mission statement, a humorous message or even a serious

'Get back to work!' message.

You can blank the screen at any time by executing a 'Shift-F5'.

4.11 FILE MENU

The file menu brings down a menu full of choices that have to do

with editing and printing files.  This menu also has the change

directory function which helps you search certain directories

without typing in full file names.

There is also a Dos Shell function within this menu that 'shells'

you out to DOS.  To 'shell' out means to leave your program

temporarily and go into DOS so you can execute other programs.

Keep in mind that when you do this, Mr. Machinist still takes up a

little memory.  If you are running the Extended Memory version of

Mr. Machinist then it will shell out to the extended memory so that

you will have plenty of memory available to run other programs.  If

you are running the 'XT' version of Mr. Machinist then you will

only have about 200K of RAM memory available.  To get back into

your program after shelling out type 'EXIT' at the DOS command

line.  One thing you can also do to make DOS shell easier is to

write a small batch file so that when you strike 'E' a batch file

called E.BAT executes and gives the command 'EXIT'.

The last command in the file menu is the 'Alt-X' or 'Esc' command.

This command will exit Mr. Machinist.  Strike it any time to exit

the program.  If your editing a file and you have not saved it, the

program will ask you to save it before exiting.

4.11.1 File Menu - Open   [F3]

To access the 'Open' menu choice using the keyboard you must

use 'Alt-F' to first bring down the file menu and then strike

'Enter'.  You do not need to use the arrow keys to highlight

the menu choice if it is your first time into the 'File Menu'

because the 'Open' menu choice will already be highlighted by

default seeing that it is the first choice on the list.

Using the mouse you would click on the 'File' menu and then

click on the 'Open' line.

The ultimate way to get to the 'Open  a File' dialog box is to

strike the 'F3' key!

This will open a file selection dialog box that will display

all the files in your current directory.

Using the 'XT' version of Mr. Machinist, there is a limit on

the number of files in a directory that this file dialog box

will seek.  Let's say you have 400+ files in one directory,

well when you strike 'F3' to bring up the file dialog box it

may say, 'To many files'.  This means that you have to many

files in the current directory for this function to handle.

At the top is an input line under the 'Name' designation.  You

may type in the file name of your choice and then strike

'Enter'.  You do have the option of using wild card characters.

If you do not know the name of the file you want to edit or the

file is in another directory then you have two choices in the

method of retrieval.

1) From the file menu select, 'Change Dir'.  A directory

selection box will appear and you can use the keyboard or the

mouse to highlight your choice, then strike 'Enter' or double

click with the mouse to get to that directory.  Then the

'Change Dir' box will display the files and sub-directories of

that particular choice.  At that point you can highlight the

file name of your choice and strike 'Enter' or double click.

2) If you do not know the name of the file you want to edit

then you will have to search for it.  If you remember the first

few characters of the file name, you could use wild cards to

cause the selection box to display only those files that meet

your criteria.

Example: RA??.* will show you all files that start with RA and

have one or two characters after it and any extension or no

extension.

RAT.TXT

RA.DOC

RA

RA55

If you type a file name in the input line that does not exist

and then strike 'Enter', the editor will create and open an

empty file for you by that name.

You will notice a down arrow symbol to the right of the input

line, this is a history list, a list of names you have already

used.  In order to view certain files that you have already

viewed using this dialog then you simply strike the down arrow

and a list of previous viewed files will appear.

You have the option to choose file names from it by

highlighting the file and striking enter or double clicking it

with your mouse.

Choosing the 'Open' box will load the file you have chosen into

a new edit window.

The file list box is the largest box of the file selection

dialog box.  This box will show you every file that matches up

to your specifications in the input line above it.  It will

also display the child and parent directories of the current

directory.

(You can further your understanding of the principles of

directories and sub-directories by referring to your DOS

manual.)

There are some helpful functions you can use while your in the

file list box.  One way to get into the file list box is to

strike 'Enter' after you have chosen 'Open' from the 'File'

menu.  Another way is to click on any file name in the box with

your mouse.  Your 'Tab' key on the key board will cycle you

through the many choices of your file selection box in one

direction, while holding down the 'Shift' key and then striking

'Tab' will cycle you through in the other direction.  The Page

Up and Down keys will let you view more files if there are more

than can fit in one windows view port.  The arrow keys will

cycle you through individually.

When your in the file list box you can also type a lower case

letter to search for a file name that starts with that letter.

If you type a second letter, it will search for the file name

with the first letter you typed and the second letter you

typed.  You can continue this for all file letters or number

characters.  If you type a capital letter you will search for

a directory name in the same manner.

At the bottom of the file dialog box is an information panel.

This will tell you all of the information of the file that is

currently highlighted.

4.11.2 File Menu - New

The 'New' command will bring up an edit window with no title.

At that time you can start editing a new file.  If you want to

save it you can bring up the file menu and choose 'Save as' or

'Save'.

Either way you will be asked to give the file a name because it

does not have a name and it needs one.  If you try to 'Exit'

the program without saving it you will be asked if you want to

save it or not.  It's always a good idea to save the file your

working on about every 5 minutes or so.

4.11.3 File Menu - Save   [F2]

The 'Save' menu will save a named file to your disk.  You will

be asked to give the file a name if you try to save an un-named

file.  You must be editing a file before this can be active.

If your editing a file that is already named, the 'Save'

command will save the file without showing you a thing.  You'll

see your hard drive light come on if your saving it to the hard

drive or you will see your floppy light up.

The fastest way to save a named file is to strike the F2 key!

4.11.4 File Menu - Save As

Save text in the current window to disk under a new file name.

You are asked to which file you would like to save the text.

The file may be an existing disk file, or a new file.

Once your text is saved, you can continue using the editor on

the current edit window.  Note that the window title will

change to reflect your new file name.

This command is disabled until you open an edit window.

4.11.5 File Menu - Save Done   [Ctrl]-[K]-[D]

Saves text in the current window to disk and closes the edit

window.  If your window is titled UNTITLED,  you will be asked

to supply a file name.  Otherwise, the text is saved to disk

using the current window title.

This command is disabled until you open an edit window.

4.11.6 File Menu - Print

You will be able to search through a dialog box and choose the

file that you would like to print.  If you are editing a file

or files and you would like to print one of them, then strike

'F8'.  If you have any trouble printing then it may be because

you have your printer hooked up to LPT2 port instead of the

usual LPT1.  See the command line options in the programs help

file or type 'mrm/?'.

You may print the current file that is being edited by striking

the 'F8' key.  A window will pop up asking you if you really

want to print the current file.  Using the mouse you may click

on yes, strike 'spacebar' or 'Enter' and your file will print.

Strike the 'Esc' key to exit and say no, or you may click on

the 'no' button.

This will open a file selection dialog box that will display

all the files in your current directory.  At the top is an

input line under the 'Name' designation.  You may type in the

file name of your choice and then strike 'Enter'.  You do have

the option of using wild card characters.  If you do not know

the name of the file you want to print or the file is in

another directory then you have two choices in the method of

retrieval.

1) From the file menu select, 'Change Dir'.  A directory

selection box will appear and you can use the key board or

the mouse to highlight your choice, then strike enter or

double click with the mouse to get to that directory.

Then the 'Change Dir' box will display the files and

sub-directories of that particular choice.  At that point

you can highlight the file name of your choice and strike

'Enter' or double click to print that particular file.

2) If you do not know the name of the file you want to

print then you will have to search for it.  If you

remember the first few characters of the file name, you

could use wild cards to cause the selection box to display

only those files that meet your criteria.

Example: RA??.* will show you all files that start with RA

and have one or two characters after it and any extension

or no extension.

RAT.TXT, RA.DOC, RA, RA55

If you type a file name in the input line that does not

exist and then strike 'Enter', an error will occur and you

will be told that there is no such file in that path, by

that name.  You will notice a down arrow symbol to the

right of the input line, this is a history list, a list of

names you have already used.  Strike the down arrow while

your in the input line to activate it.

You have the option to choose file names from it by

highlighting the file and striking enter or double

clicking it with your mouse.

Choosing the 'Open' box will print the file you have

chosen.

The file list box is the largest box of the file selection

dialog box.  This box will show you every file that

matches up to your specifications in the input line above

it.  It will also display the child and parent directories

of the current directory.  (You can further your

understanding of the principles of directories and

sub-directories by referring to your DOS manual.)

There are some helpful functions you can use while your in

the file list box.  One way to get into the file list box

is to strike 'Enter' after you have chosen 'Open' from the

'File' menu.  Another way is to click on any file name in

the box with your mouse.  Your 'Tab' key on the key board

will cycle you through the many choices of your file

selection box in one direction, while holding down the

'Shift' key and then striking 'Tab' will cycle you through

in the other direction.  The Page Up and Down keys will

let you view more files if there are more than can fit in

one windows view port.  The arrow keys will cycle you

through individually.

When your in the file list box you can also type a lower

case letter to search for a file name that starts with

that letter.  If you type a second letter, it will search

for the file name with the first letter you typed and the

second letter you typed.  You can continue this for all

file letters or number characters.  If you type a capital

letter you will search for a directory name in the same

manner.

At the bottom of the file dialog box is an information

panel.  This will tell you all of the information of the

file that is currently highlighted.

4.11.7 File Menu - Print Answer File   [Ctrl]-[P]

There is a file that is created or appended to on your drive

each and every time you execute MRM called MRMSOLVE.TXT.  This

file holds the answers to every mathematical function that is

executed in the program.  It gives the date and time of file

creation and neatly formats your answers so that you may choose

to print them out at any time by holding down the 'Ctrl' key

and then tapping the 'P' key.

MRMSOLVE.TXT is not something that is in memory, it is a file

that gets written to every time you do a math function in the

program.  If you choose to keep the file for later reference

you must rename the file.

4.11.8 File Menu - Show Answer File   [Ctrl] [F1]

'Ctrl-F1' will bring up the answer file called MRMSOLVE.TXT in

the editor for you to view or edit.  This is the file that

contains all of your answers to the current session.

4.11.9 File Menu - Change Directory

From the 'File' menu select, 'Change Dir'.  The 'Change

Directory' box will appear and you can use the key board or

double click the mouse to highlight your choice, then strike

'Enter' or double click to enter that directory into the input

line.  At that point you can click the mouse on the 'OK' button

or strike 'K' to make the choice valid and make this directory

the default one.  If you choose the 'Chdir' button you will

make the input lines directory your choice also but it will not

continue to be the default.

Your 'Tab' key on the key board will cycle you through the many

choices of the 'Change Directory' box in one direction, while

holding down the 'Shift' key and then striking 'Tab' will cycle

you through in the other direction.  The Page Up and Down keys

will let you view more directories in the directory tree list

box if there are more than can fit in one windows view port.

The arrow keys will cycle you through individually.

If you type a directory name in the input line that does not

exist and then strike 'Enter', or click on 'OK' you will get an

error message that says, "Invalid drive or directory".

You will notice a down arrow symbol to the right of the input

line, this is a history list, a list of directories that you

have already used.  You have the option to choose from it by

highlighting the directory and striking 'Enter' or double

clicking it.

Choosing the 'Revert' button makes your previous choice the

current directory.

4.11.10 File Menu - DOS Shell

To 'shell' out means to leave your program temporarily and go

into DOS so you can execute other programs.  Keep in mind that

when you do this, Mr. Machinist still takes up a little memory.

The Extended Memory version of Mr. Machinist 'Shells Out' to

the extended memory so that just about any program can run

using the RAM or more extended memory.  The 'XT' version of the

program only leaves you around 200K or so in RAM once you

'Shell Out'.  To get back into your program after shelling out

type 'EXIT' at the DOS command line.

One thing you can also do to make DOS shell easier is to write

a small batch file so that when you strike 'E' a batch file

called E.BAT executes and gives the command EXIT.  If you need

more information on creating BATCH files you can look into a

DOS manual.

Remember that the library has rows of computer books too!

4.11.11 File Menu - Print Registration Invoice   [F9]

This option will check to see if your printer is online and

then it will print an invoice so that you can register this

software.  If you have registered the Mr. Machinist program

than this menu choice is here for you to give order forms to

other companies and friends who may want the program.

For a $98.00 registration fee you will become a registered user

of the Mr. Machinist program with a license for use on one

computer only.

For SITE LICENSE REGISTRATION and other information see the

enclosed file called MRMORDER.DOC.

To view the order form before you print it, tap your 'tab' key.

That will cause the cursor to shoot down to the first help word

which in this case is MRMORDER.DOC.  Once the phrase is

highlighted you can strike the 'Enter' key to view the

document.

4.11.12 File Menu - Sequence a CNC File   [Ctrl]-[F3]

A sequence number is usually the first word that appears in a

block or sentence of a text file for CNC machines.  There are

two main reasons behind a sequence number.

1) Lets the operator know which line the machine is currently

processing.

2) Provides the operator a way to search or find a particular

line.

These sequence numbers usually go to four digits and begin with

an N.

The very first line of a CNC program or 'text file' always

begins with an O on the CNC machine but not always in a file on

a personal computer.

Here is an example of a G and M code file with sequence

numbers:

:0203 (05-07-93)

N0005 (ARM-ROCKER)

N0010 (123456 JOB#1234)

N0015 G00 G17 G40 G G80

N0020 G30 G91 Z0

N0025 T01M6

N0030 (.625 EMILL)

N0035 G55 G90 G00 X-.65 Y0.0

N0040 S1800 M03

N0045 G43 Z1. H12

N0050 M08

N0055 Z.1

N0060 G01 Z-.031 F20.0

N0065 X.65 F5.0

The Mr. Machinist program has a function in it that can insert

N sequence numbers into an un-sequenced G and M code file on

the personal computer.

There is also the capability to insert or remove spaces between

letters and numbers in a 'G' coded file for the sequence and

un-sequence function.  Some shops like to get rid of the spaces

for more disk space.  The CNC controller can handle and except

a file whether it has spaces or not.

To Insert Sequence Numbers into a File

======================================

A Sequence dialog box will first display and it will ask you

for the 'source' file, the file that needs to be sequenced.  At

this input line you insert the file that you would like to

sequence.

The next input line is the destination file.  The Mr. Machinist

program will not over write the old file but it will create a

new file so that you will always have your original to go back

to.

The next input line needs to know the 'N' start number.  All

'N' numbers will have leading zeros and four digits but you do

not need to type in the leading zeros.  If you want your first

number to be 'N0010' you would type '10' in this input line,

you do not have to insert the 'N', input only the number.

Next you will be asked for the increment number. This number

can not be greater than 10 because if you incremented at a very

large number your file could go over the CNC control's allotted

amount of 4 digits.

Lastly you will be asked to input 'y/n/a'.  This input line has

to do with spaces.  Here is the definition of what 'y/n/a'

means:

y = (y)es, add one space between every letter and number

in the file, except for the first letter 'N'.  If

there is already a space between each letter and

number in the file then after this function there will

be two spaces between them, you probably do not want

that.  Some 'CAM' programs generate 'G' coded files

that have no spaces between letters and numbers.

Spaces create what is called, 'white space'.  White

space can make a file much more readable!

n = (n)o spaces.  If you want a single space removed

between each letter and number then type 'n'.  If the

file does not have a space between a letter and number

then nothing will happen.  If the file has the spaces

between a letter and a number then it will remove one

space and leave the other after this function goes to

work.  This 'remove space' function can save disk

space on a computer storing large G-Code files.

a = (a)lone or you could say, 'leave it alone'.  This

choice will sequence a file but will leave it the way

the original was, except for the sequenced 'N' numbers

of course.

4.11.13 File Menu - Un-Sequence a CNC File [Ctrl]-[U]

The Mr. Machinist program has a function in it that can remove

'N' sequence numbers from a CNC text file on your computer.  In

the first input line you will be asked for the source file that

you want the 'N' numbers removed from.  In the next input line

you will insert the name of the new destination file.  If the

new file exists on your drive you will be shown an error box

and then you will start the process over.

The removal function will un-sequence every line except lines

that do not have 'N' sequence numbers in them.

Lastly you will be asked to input 'y/n/a'.  This input line has

to do with spaces.  Here is the definition of what 'y/n/a'

means:

y = (y)es, add one space between every letter and number

in the file, except for the first letter 'N'.  If

there is already a space between each letter and

number in the file then after this function there will

be two spaces between them, you probably do not want

that.  Some 'CAM' programs generate 'G' coded files

that have no spaces between letters and numbers.

Spaces create what is called, 'white space'.  White

space can make a file much more readable!

n = (n)o spaces.  If you want a single space removed

between each letter and number then type 'n'.  If the

file does not have a space between a letter and number

then nothing will happen.  If the file has spaces

between a letter and a number then it will remove one

space and leave the other after this function goes to

work.  This 'remove space' function can save disk

space on a computer with large G-Code files.

a = (a)lone or you could say, 'leave it alone'.  This

choice will sequence a file but will leave it the way

the original was, except for the sequenced 'N' numbers

of course.

4.11.14 File Menu - Split a CNC File   [Ctrl] [F10]

Before we get into 'Split a CNC File' let's begin with clearing

a CNC's storage capacity:

Clear Your CNC's Storage Capacity

=================================

Some machine shops have to download there CNC files to a

personal computer in an office some where.  Many shops now are

supplying there workers with computers right on there bench

near there machine.  As a CNC's control gets filled with

programs it can become full and cause the operator to have to

download one or more programs to make room enough for the

current program to load.

Many CNC controls offer a function that lets them download ALL

files in the CNC's storage to ONE file on the personal

computer.  This is the fastest method of clearing the CNC

storage.  There is just one big problem that people face when

they have this huge file on there computer with many programs

all in one file, they have to manually cut and paste to get

even one program back to the CNC.

Mr. Machinist has a one of a kind function in it that can

'split' a large file that has been downloaded from a CNC.  The

purpose of this function is to rewrite ALL of the individual

programs to separate files on the personal computer and they

are named by there four digit program number just as they were

on the CNC with an additional extension of .TXT.

The main purpose of the 'Split' function is to split a large

text file that was downloaded to a personal computer by a CNC

machine into all of the original separate programs.  You would

have to manually cut and paste using an editor to do this if

you did not have this function to work with and that would take

a lot of time and money!

Mr. Machinist will ask you for the file name of the large text

file and then it will look for the first occurrence of a line

that starts with a ':'.  Once it finds the colon Mr. Machinist

will get the next four digits and add an extension of .TXT to

it and give the new file the proper program number.  It will

then begin to copy every line after the colon to the new file

UNTIL it reaches a blank line.

Once a blank line is encountered the program will look for the

next occurrence of a ':' which should be the next line.  At

that time the same process occurs until every file is written

to the specified directory.

The split function will show the MRMSOLVE.TXT answer file after

the process is complete.  If a file was found to already exist

on your drive and the split function could not create the file

then you will be shown an error message stating that the file

already exist.  These error messages will go to the answer file

also in order that you can view the results at the end of the

function.  If a file exists you can either rename it or delete

it out of your system and then run the split function again.

The files that were already created will not be created again.

After this process is complete, the original large file will

still remain for you to decide what you want to do with it.

One little note: A good CNC programmer should know a lot about

DOS.  Read manuals carefully and study to show thyself

approved!

4.11.15 File Menu - Exit Mr. Machinist   [Alt]-[X]

If you hold down the 'Alt' key with one finger and strike the

'X' key with another finger you will exit the program.  Before

you exit you will be given an information box asking if you

really do want to exit.  Strike 'Y' for yes or just strike

'Enter'.  The only reason that you may not be able to exit is

that you may have some active windows on your desk top that you

need to close or your editing a file that has been modified and

not saved.

4.12 EDIT MENU

This menu allows you to cut, copy, and paste text to and from the

clipboard.  You can also undelete your last text operation, clear

selected text from your document without putting it into the

clipboard, or view current clipboard contents.

The Edit menu gives you six choices that have to do with editing a

file.

1. Undo  - Undeletes the last line deleted or changed.

2. Cut   - Removes selected text from file to clipboard.

3. Copy  - Copies selected text to clipboard.

4. Paste - Copies text from clipboard to file.

5. Show  - Shows context of clipboard.

6. Clear - Removes selected text without going to

clipboard.

4.12.1 Edit Menu - Undo  [Ctrl]-[Q]-[L]

Undo your last text operation.  Whether you entered text that

you don't really want, or accidentally deleted some text you

would like to restore, UNDO will restore the text to its

original context.

However, you must select UNDO before you use any cursor key.

The UNDO buffer is based on cursor movement.  As long as you

are entering or deleting text you can UNDO it.  This also

applies to marking blocks of text and deleting them.  The

deletion can be undone, as long as you don't move the cursor

with a cursor key (i.e., [PgUp], [Right-Arrow], [Home], etc.).

Once the cursor is moved, the UNDO buffer flushes itself and

the original text context is forever gone.

This does not apply to using any of the text reformatting keys.

Wordwrap can not be undone.  Nor can paragraph or document

reformatting.

This command is disabled until you enter or delete text in an

edit window.

4.12.2 Edit Menu - Cut    [Shift]-[Del] or [Ctrl]-[K]-[Y]

Cut selected text out of your document to the clipboard.  Using

CUT indicates that you want to "cut" your selected text,

removing the text from your document.

This command is disabled until you place text on the clipboard.

4.12.3 Edit Menu - Copy  [Ctrl]-[Ins] or [Ctrl]-[K]-[K]

Copy selected text to the clipboard.  Using COPY indicates that

you want to take a "copy" of your selected text, leaving the

text in your document intact.

This command is disabled until you place text on the clipboard.

4.12.3 Edit Menu - Paste  [Shift]-[Ins] or [Ctrl]-[K]-[C]

Paste text into your document from the clipboard. In order to

use this option, text must have been placed on the clipboard

previously using COPY or CUT commands.

This command is disabled until you place text on the clipboard.

4.12.4 Edit Menu - Show Clipboard

Show Clipboard Contents      Alt-E-S

View the contents of the clipboard.  The clipboard will be

blank unless you previously put something on it using the COPY

or CUT commands.

To exit the clipboard, press [Alt]-[F3] and you will return to

your current edit window.

4.12.5 Edit Menu - Clear

Clear - Removes selected text without going to clipboard

Selected text is that text which you highlight in your

document.  Once text has been selected, you may COPY, CUT, or

CLEAR the text.  Selected text has a different color than

regular text.

Clear selected text from your document.  This is much like the

CUT option, but the selected text is not put on the clipboard.

Use this command only if you want to delete selected text for

good.

This command is disabled until you select text.

4.13 WINDOWS MENU

This menu allows you to navigate amongst the different windows open

on the desktop, resize or move a window, or arrange your windows in

a tile or cascade pattern on the desktop.

The Windows menu gives you six window manipulation choices.

1. Resize/move - Use arrow keys or shift-arrow keys to move or

resize.

2. Zoom - F5 will toggle off and on the 'zoom' window function

3. Next - F6 will toggle between active windows.

4. Close - Alt-F3 will close the current window.

5. Tile - View all edit windows.

6. Cascade - Stacks all edit windows.

4.13.1 Windows Menu - Resize/Move

There are two windows that can be resized, the edit window and

the help window.  All windows can be moved.  Learning to resize

and move windows at a good pace can really speed up the job of

multiple editing and/or viewing multiple formula answers.

Using the mouse is the quickest way to resize and/or move a

window.  To resize a window using the mouse, 'drag' the mouse

on the lower right corner of the window.  To 'drag' the mouse

means to hold down the left mouse button while you have the

mouse cursor positioned on a certain item.  As you hold down

the left mouse button on the lower right corner, begin to move

the mouse around.  You will now see that the window resizes

from the lower right position.  The window will stay at its

current position as soon as you take your finger off of the

mouse.

To resize a window using the keyboard, you hold down the 'Alt'

key and then strike 'W'. This will pop down the 'Window menu'.

Strike the 'R' key to resize a window.  After striking the 'R'

key you can use the arrow keys to resize the window.  When you

have the window where you want it, strike the 'Enter' key. This

will take you out of the resize mode.  Don't forget to strike

the 'Enter' key when your through because it will appear as

though the system is 'locked up' until you do.

To move a window using the mouse you drag the mouse on the top

heading of the window.  The top heading of a window is the

wording that is centered on the top line of the current window.

To move a window using the keyboard you should hold down the

'Alt' key and then strike the 'W' key and the 'Window menu'

will pop down.  Then strike the 'M' key and you will be able to

move the window using the arrow keys.

4.13.2 Windows Menu - Zoom

The zoom icon is located in the upper right corner of most

windows.

It means that this window can be zoomed in on to fill the whole

desktop or can be zoomed out of to resume its normal size.  Not

all windows can be zoomed.  Dialog boxes are usually not

"zoomable."

Zoom Icon [#]

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

4.13.3 Windows Menu - Next  [F6]

Go to the next window on the desktop, if you have multiple

windows on the desktop.

This command is disabled unless there is at least one window on

the desktop.

4.13.4 Windows Menu - Close   [Alt]-[F3] or [Esc]

The close icon appears in the upper left corner of almost every

window.  By clicking on it with your mouse, you can quickly

close a window.  If you are not using a mouse, press [Alt]-[F3]

or [Esc].

Close Icon:[�]

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

4.13.4 Windows Menu - Tile

Arrange all windows on the desktop in a tile pattern.  An

example of four windows tiled on the desktop would look like

this:

������������������������������������Ŀ

� �[�]�������1�[ ]��������������2�Ŀ �

� �               ��               � �

� �               ��               � �

� �               ��               � �

� ���������������ͼ����������������� �

� �������������3�Ŀ�������������4�Ŀ �

� �               ��               � �

� �               ��               � �

� �               ��               � �

� ���������������������������������� �

��������������������������������������

Tiled Windows

This command is disabled unless there is at least one window on

the desktop.

4.13.5 Windows Menu - Cascade

Arrange all windows on the desktop in a cascade pattern.  An

example of four windows cascaded on the desktop would look like

this:

���������������������������1Ŀ

���������������������������2Ŀ

���������������������������3Ŀ

����[�]�����������������4�[ ]�

����                         �

����                         �

����                         �

����                         �

����                         �

����������������������������ͼ

Cascaded Windows

This command is disabled unless there is at least one window on

the desktop.

4.14 SEARCH MENU

The Search menu is used to find certain text in a file and this

function can also search and replace chosen text with new text.

This menu has three functions to choose from.

1. Find - Finds the text of your choice.

2. Replace - Finds text and then replaces it with new text.

3. Search again - Do another search using the same choice.

4.14.1 Search Menu - Find  [Ctrl]-[Q]-[F]

Type in the text that you wish to find.  You can also press the

down arrow key to bring up a history list of the last text you

looked for.  Clicking the mouse on the history list icon to the

right of the input line will do the same thing.

You have two options available when trying to find text.

You can do a case sensitive search, which means text must match

the case of the text you entered before a match is found.

You may search for whole words only, which means that no match

will be found if the text you input is part of a word (i.e,

TIME -- "sometime" is NOT a match).

You use the mouse or up/down arrow keys to highlight the option

you want, and click on it or press the spacebar to toggle it

on/off.

Find a particular bit of text in your document.  A dialog box

will appear asking you what text to find, and gives you the

option of finding text that is case sensitive or whole words

only.

[Ctrl-L] will let you do a find action or a search and replace

action again.

This command is disabled until you open an edit window.

4.14.1 Search Menu - Replace

Type in the replacement text you want.  You can also press the

down arrow key to bring up a history list of the last

replacement text.  Clicking the mouse on the history list icon

to the right of the input line will do the same thing.

The first two options are for finding text.

You can do a case sensitive search, which means text must match

the case of the text you entered before a match is found.

You may search for whole words only, which means that no match

will be found if the text you input is part of a word (i.e,

TIME -- "sometime" is NOT a match).

You use the mouse or up/down arrow keys to highlight the option

you want, and click on it or press the spacebar to toggle it

on/off.

4.14.2 Search Menu - Search Again [Ctrl-L]

This function will let you continue a search or a search and

replace function without starting all over again.

4.15 CIRCLE MENU

The Circle menu is filled with math functions that have everything

to do with circles.  There are seven main functions.

1. Circ. Segment -Find the angle, chord, height, or Radius.

2. Circle Area - Find the surface area of a circle.

3. Circ. Ring - Find the surface area of a circular ring.

4. Circ. Ring Sector Area - Same as 3 but only a section.

5. Circ. Sector Area - Find a circular sector area. Only two inputs

needed.

6. Circumference - Find circle circumference.

7. Three Points - Find X and Y center and radius of a circle.

8. Pin in a Diamond - Find the pin size/circle diameter inscribed

within a four sided symmetrical shape.

4.15.1 Circle Menu - Circle Segment

This function allows a user to find the Chord, Height, Radius,

Total Segment Angle, and Arc Length from two knowns.

There are two instances where there can be no solution, knowing

just the Chord and Arc Length or the Height and Arc Length.

The first step is to input the two known pieces of information.

Certain types of input are not feasible.  Take for example a

Height input that is greater than or equal to the Radius.  If

the Height is greater than the Radius, subtract the Radius from

the Height.  Another example of improper input is a Chord that

is greater than the Diameter of the Circle.

Remember that your answers not only go to a dialog box but also

to a file that exists on your hard drive called MRMSOLVE.TXT.

This file can be viewed or printed at any time.

4.15.2 Circle Menu - Circle Area

This choice simply finds the area of a given circle diameter.

Input the Diameter of the Circle you would like to find the

Area for.

Area = Pie x Radius Squared

4.15.3 Circle Menu - Circular Ring Area

This function will find the Area of a Circular Ring. Input the

Outside Diameter and then the Inside Diameter.

Area = Pie x (OD� - ID�)

4.15.4 Circle Menu - Circular Ring Sector Area

This function is able to give you the Area of the Sector of a

Circular Ring.  You need to know three things: the Angle of the

Sector, the OD and the ID.

Area of a Circular Ring Sector Formula

======================================

( OR = Outer Radius, IR = Inner Radius )

((Sector Angle x Pie) / 360)) x (OR� - IR�)

4.15.5 Circle Menu - Circular Sector Area

This function gives you a choice of entering the two knowns and

then calculates the answer.  You may either enter the Radius

and Angle, Radius and Arc Length, Radius and Area, Angle and

Arc Length, Angle and Area, or Area and Arc Length...any

combination is possible!

4.15.6 Circle Menu - Circumference

This function will find the Circumference of a Circle.

The circumference of a circle is the total length of a circle,

as if you were to 'cut' the circular line and lay it flat and

then measure it.

Circumference = Pie x Diameter

4.15.7 Circle Menu - Three Points On A Circle

A geometrical law says that if three point are in a plane, in

this case X and Y, and not in a straight line or under one

another, then it's possible to have a circle pass through those

three points.  You will find X and Y coordinates and radiuses

of large amounts if you input three points that are almost a

straight line.

Formula

=============================================

(X3�-X1�)+(Y3�-Y1�)   (X2�-X1�)+(Y2�-Y1�)

------------------- - -------------------

2(X3-X1)              2(X2-X1)

Y =  ----------------------------------------

Y3-Y1                Y2-Y1

-----        -       -----

X3-X1                X2-X1

(X3�-X1�) + (Y3-Y1)((Y3+Y1)-2Y)

X = -------------------------------

2(X3-X1)

R = Sqrt( (X3-X)� + (Y3-Y)� )

4.15.8 Circle Menu - Pin in a Diamond

This function will calculate a pin or circle diameter inscribed

within a four sided symmetrical shape.  You input the length of

two sides to calculate the answer. One side is the 'X' axis

corner to corner and the other side is the 'Y' axis corner to

corner.  It does not matter which side you decide to call 'X'

or 'Y'.

One important note to remember about this function is that the

four sided figure must have all square corners, that is to say

that the corners do not have a radius in them.

4.16 MATH MENU

The Math menu has nine functions to do with math.

1. Trig. - Right angle trig. (Give the two knowns)

2. Conversions: a) Millimeters to Inches

b) Inches to Millimeters

c) Degrees to Degrees, Minutes, Seconds

d) Degrees, Minutes, Seconds to Degrees

e) Celsius to Fahrenheit

f) Fahrenheit to Celsius

3. Polar Coordinates - Gives an X and Y from a radius and a

distance.

4. Dist. Across Square Corners - Input height for answer.

5. Dist. Across Hex Corners - Input height for answer.

6. Dist. Between Two Points - Give two X and Y's.

7. Middle of Two Points - Give two X and Y's.

8. Three Points Shift/Rotate - Give three X and Y's and new X and

Y origins and the degree of rotation, (if any).

9. Calculator - Do formulas and calculations.

4.16.1 Math Menu - Trig.

Choosing this function will give you a screen that will ask you

for the two knowns of a right triangle. The function will then

give you all the angles and side dimensions of that right

triangle and also the perimeter, inner circle diameter, outer

circle diameter, and perpendicular line that has to do with

that particular right triangle.

A triangle with a 90 degree angle in it.  With two known pieces

of information, the solution to all other sides and angles can

be obtained as long as one of those pieces of information is a

side of the triangle.  The three angles added up must equal 180

degrees.

When you hold down the 'Alt' key and strike the 'T', the Trig.

screen will appear.  The best way to cycle through the input

lines is to strike 'Enter'.  If you happen to put the maximum

amount of digits (12) into the input line then you will be

automatically cycled into the next input line.  When you have

your two pieces of information entered you may then hold down

the 'Alt' key and strike 'O' for OK or you may strike 'Enter'

until you have reached the OK button and then strike 'Enter'.

You may also use the up and down arrows to move up and down the

dialog box.

The 'Tab' key cycles you through the dialog box in one

direction.  If you hold down the 'Shift' key and then strike

'Tab' you will cycle through the dialog box in the other

direction.

The Perpendicular Line calculation that is given in the answer

box is the distance of the line that extends from the

intersection of the side opposite and the side adjacent

perpendicular to the hypotenuse.  This distance is sometimes

needed when a milling machinist must put a part up on an angle

in a vise in order to machine a chamfer on it.

Theorem of Pythagoras:

a� + b� = c�

Where c = Length of hypotenuse,

a and b = the other sides.

Formula

�

� �

�   �

a �     �  c

�       �

�         �

�       ->A �    (A = The acute angle)

� � � � � � � �

b

a   Side Opposite

Sin A = - = -------------

c   Hypotenuse

b   Side Adjacent

Cos A = - =   -------------

c   Hypotenuse

a   Side Opposite

Tan A = - = -------------

b   Side Adjacent

b   Side Adjacent

Cot A = - = -------------

a   Side Opposite

c   Hypotenuse

Sec A = - = -------------

b   Side Adjacent

c   Hypotenuse

Csc A = - = -------------

a   Side Opposite

4.16.2 Math Menu - Conversions

1) Millimeters to Inches

2) Inches to Millimeters

3) Degrees in Decimal to D.M.S.

4) Degrees in D.M.S. to Decimal

5) Celsius to Fahrenheit

6) Fahrenheit to Celsius

Use your up and down arrow keys to choose the conversion you

would like, then strike the 'Tab' key to move into the input

line.  Enter the number that you would like to convert and then

strike 'Enter'.

Degrees, Minutes and Seconds input

**********************************

Input the Degrees to the left of the decimal place and minutes

two places to the right of the decimal place, and the seconds

after the minutes, (two places also).

INPUT EXAMPLE:   45.5434 = 45 Degrees

54 Minutes

34 Seconds

4.16.3 Math Menu - Polar Coordinates

This function will ask you for a radius and an angle and then

it will give you the X and Y coordinate of the point where that

angle line meets that radius circle from center X/Y zero.

4.16.4 Math Menu - Distance Across Square Corners

This function will ask you for the height of the square and

then it will give you the distance from corner to corner.

4.16.5 Math Menu - Distance Across Hex Corners

This function will ask you for the height of the hex from top

to bottom, (From flat to flat) and then give you the distance

across hex corners.

4.16.6 Math Menu - Distance Between Two Points

This function will ask you for two X and Y coordinates and then

give you the distance between these two points.

4.16.7 Math Menu - Middle Of Two Points

This function will ask you for the two X and Y coordinate

locations.  You will then be shown the X and Y coordinate of

the point that lies exactly in the middle.

4.16.8 Math Menu - Three Points, Shifted And Rotated

This function will ask you to input three X and Y coordinates,

a shift location, and a rotation angle. The X and Y Datum

position is what is being shifted here or in other words the

reference or datum point.

The rotation begins at the 3 O'clock position and swings around

in a counter clock wise direction.

4.16.9 Math Menu - Bolt Circle

This function will ask you for the diameter of the Bolt Circle

and then the number of holes and then the angle of the first

hole.  Just as in most CAM software, the angle starts at the

right side of X zero on the horizontal plane and swings its way

counter clock wise until it goes the full 360 degrees.  You may

insert a negative angle for the first hole if you like.

When the Bolt Circle function is executed, it writes its

answers to a file on your drive called MRMBOLT.TXT.  After Mr.

Machinist writes the answers to the file it will display it.

You can print the MRMBOLT.TXT file at any time by striking,

'Ctrl-F9'. You can view the MRMBOLT.TXT file at any time by

striking, 'Ctrl-F8'.

Keep in mind that every time you strike 'Ctrl-F8' and the

answer file 'pops' up, it shows you the current state of

MRMBOLT.TXT.  When your done viewing the file you should close

the window by clicking on the close icon or striking 'Alt-F3'

or striking 'Enter'.

4.16.10 Math Menu - Calculator

You can use the keyboard or the mouse to operate the

calculator.  'F4' brings up the calculator at any time and

'Alt-F3' or 'Esc' will close the calculator window.

Left arrow erases the last character typed.

The � key toggles the number from + to -.

( � on keyboard is the 'Underscore' key )

The C key clears the input line.

Once the calculator window is on screen you will be ready to

enter your calculation or formula.

You can strike the 'Tab' key to move around the calculator

dialog box.  Striking the 'Tab' key will take you to the 'Ok'

button, once more will take you to the 'Cancel' button and once

more will take you back to the input line.  If you hold down

the 'Shift' key and then strike 'Tab' you will move around in

the opposite direction.  You can also use the mouse to click on

one of these three items and activate them.

Once your in the input line you can start to enter your data.

When numerical entry is complete you may strike 'Enter' or

click on 'Ok' with your mouse or strike 'O' for 'Ok'.  An

example of this would be: 4+4*87/456*tan(56) then 'Enter'.

The order of precedence is at work here.  The calculator

evaluates the parts of an expression inside the most deeply

nested parentheses before anything else.  In the example, the

tangent of 56 is calculated and then 87/456 is calculated, then

the two are multiplied and 4 is added.

Upper and lower case input of functions are treated the same.

You may also append an O for octal or an H for hex after a

number.  If you input 55o, your answer will be 45 because 55 in

octal is 45.  If you input 640H, your answer will be 1600

because 640 in hex is 1600.  When you type a hex constant you

must be sure to type a number before it.  If you type FF for a

hex number then you will get an error, but if you type 0ff then

you will get 255.

You may enter assignment statements such as, a:=3, and b:=3,

and also c:=sqrt(a*b).  Always use a single character as your

variable, A to Z.  If you use assignment statements within

expressions always be sure they are predeclared.  Here is an

example using the formula for RPM (R) of a spindle:

12 x F (Feet per Minute)

R = --------------------------

Pi x D (Diameter of Endmill)

You would enter f:=200 and then strike 'Enter',  d:=0.5 and

'Enter', and for the end result,  r:=(12*f)/(pi*d).  If you

were to leave out the  parentheses your result would be in

error due to the  fact that the calculation would divide first

and  then do its multiplication.

Standard Calculator Keys:

Tab       = Activate next item

Shift-Tab = Activate in opposite direction.

F4        = Bring up calculator window

Alt-F3    = Close window.  (Any time)

Esc       = Close window.  (Any time)

Enter     = Calculate Answer.  (Valid Input)

Calculator Functions

====================

Be sure to inclose functions with more than one  character

within parentheses.

Example: 5*tan(45)*sqrt(7)

+         = Addition

-         = Subtraction

/         = Division

*         = Multiplicational

^         = Exponentiation

O         = Octal (Precede Number)

H         = Hex (Precede Number)

Pi        = 3.1416.........

Frac      = Fractional Value

Factor    = Factorial (1 x 2 x 3 x 4....)

Exp       = Natural Exponent

Int       = Integer Value

Abs       = Absolute Value

Log       = Log Base 10

Sqrt      = Square Root

Ln        = Natural Log

ArcCos    = ArcCosine

ArcCsc    = ArcCosecant

ArcSin    = ArcSine

Cos       = Cosine

Csc       = CoSecant

Sin       = Sine

ArcCot    = ArcCotangent

ArcSec    = ArcSecant

ArcTan    = ArcTangent

Cot       = CoTangent

Sec       = Secant

Tan       = Tangent

4.17 AREAS AND VOLUMES MENU

4.17.1  Areas/Volume Menu - Acute Angle

An acute angle is an angle that is less than 90 degrees.  An

acute angled triangle is one with all the angles less than 90

degrees.

a and b and c are the sides of the acute triangle.

Formula

=======

S = 1/2 x (a + b + c)

Area = Sqrt(S x (S - a)) x (S - b) x (S - c)

4.17.2  Areas/Volume Menu - Barrel

Barrel - A cylindrical object where the middle diameter is

greater than the end diameter.  The sides of a barrel shape can

either be a circular arc or a parabola arc. In the parabola arc

the radius from the center of the arc shape is constantly

changing until it's peak at the top of center.

The formula used here in Mr.  Machinist is for the circular

arc.

Formula

=======

V = Approximate Volume

D = Middle Diameter

d = End Diameter

h = Height of Barrel Shape

Arc of a Circle (Barrel)

V = 0.262 x h x (2 x Sqr(D) + Sqr(d))

Arc of a Parabola (Barrel)

V = 0.209 x h x (2xSqr(D+Dxd+3/4xSqr(d))

4.17.3  Areas/Volume Menu - Cone

This function will ask you for a radius and a height of a cone

and then it will give you the area and the volume of that cone

in inches or millimeters.

4.17.4  Areas/Volume Menu - Cube

This function will ask you for a side dimension of a cube and

then will give you the volume of the cube in inches or

millimeters.

4.17.5  Areas/Volume Menu - Cycloid

This function will ask you for the radius of the generating

circle of a cyclode and then give you the area.

Cycloid

=======

The path of a point on the circumference of a circle as it

rolls along on a straight line.

4.17.6  Areas/Volume Menu - Cylinder

This function will ask you for the radius and height of a

cylinder and then give you the area, volume, and the total area

of cylindrical surface and end surfaces.

4.17.7  Areas/Volume Menu - Ellipse

This function will ask you for the major axis dimension and the

minor axis dimension and then give you the area and

circumference of the ellipse.

4.17.8  Areas/Volume Menu - Ellipsoid

This function will ask you for the major axis dimension and the

minor axis dimension and then give you the volume of the

ellipsoid.

4.17.9  Areas/Volume Menu - Frustum of a Cone

This function will ask you for the base diameter and the top

diameter and the height of the cone and then give you the

volume.

4.17.10 Areas/Volume Menu - Frustum of a Pyramid

This function will ask you for the top area and the bottom area

and the height of the pyramid and then give you the volume.

4.17.11 Areas/Volume Menu - Hexagon

This function will ask you for the outside radius, the inside

radius, and the length of a side but you do not have to answer

all of the input lines!  You only need to answer one of the

questions and all other answers will be given!

By inputting just the length of a side you get the area, outer

radius, inner radius and distance across flats.

By inputting just the outer radius you get the area, inner

radius and distance across flats.

By inputting just the inner radius you get the area, outer

radius and distance across flats.

4.17.12 Areas/Volume Menu - Hollow Cylinder

This function will ask you for the outside diameter, the inside

diameter and the height of the cylinder and will give you the

volume.

4.17.13 Areas/Volume Menu - Hollow Sphere

This function will ask you for the outside diameter and the

inside diameter of the hollow sphere and then will give you the

volume.

4.17.14 Areas/Volume Menu - Obtuse Angle

This function will ask you for the length of 3 sides and then

will give you the area of an obtuse angled triangle.

(An angle that is between 90 degrees and 180 degrees.)

4.17.15 Areas/Volume Menu - Octagon

Octagon - A polygon with eight angles and eight sides.

This function will ask you for the length of a side, the

outside radius, and the inside radius.

You only need to input one piece of information in order to get

your answers.  The answers that are given are the side length,

the outside radius, the inside radius, and the area of the

octagon.

4.17.16 Areas/Volume Menu - Parallelogram

This function will ask you to input the length of side A and

the length of side B.  You will then be displayed an answer box

giving you the 'area' of the parallelogram.

4.17.17 Areas/Volume Menu - Polygon

This function will ask you to input four pieces of data.  You

need not input all four.

You may input the number of sides, either ID or OD, and the

length of a side.  You will then be displayed an answer box

giving you the 'area' of the Polygon.

4.17.18 Areas/Volume Menu - Portion of a Cylinder

A cylinder that is cut at the top or bottom on an angle.  If

the cylinder was level at both ends then it would be a

cylinder, not a portion of a cylinder.  A portion of a cylinder

has 2 heights, the upper most portion of the angle cut and the

lower most portion.  Insert these heights as Height1 and

Height2.

4.17.19 Areas/Volume Menu - Prism

This function will ask you to input two pieces of data, the

area of the prism and the height.  You would first have to find

the area of the 'end surface' of the prism which could be of

different shapes.  Once you know the area of the end surface

you can then input the height which is measured perpendicular

to the end surface.  You will then be displayed an answer box

giving you the 'Volume' or the prism.

4.17.20 Areas/Volume Menu - Pyramid

This function will ask you to input two pieces of data, the

area of the pyramid's base and the height of it.  The base of

the pyramid could be different shapes so you would have to

first find the area of the base.  After you have input the

proper data you will be displayed an answer box giving you the

'Volume' of the pyramid.

4.17.21 Areas/Volume Menu - Rectangle

This function will ask you for two pieces of data, the length

of one side and the length of the other. You will then be

displayed an answer box giving you the 'Area' of the rectangle.

4.17.22 Areas/Volume Menu - Right Triangle

This function will ask you for two pieces of data, the length

of the side adjacent and the length of the side opposite.  You

will then be displayed an answer box giving you the 'Area' of

the right triangle.

4.17.23 Areas/Volume Menu - Spandrel/Fillet

This function will have two input lines requiring you to input

data into one of them.  Either input the radius of the spandrel

or fillet or input the chord. You will then be displayed an

answer box giving you the 'Area' of the spandrel or fillet.

4.17.24 Areas/Volume Menu - Square

This function will ask you for the length of a side only.  With

that information you will be given the diagonal length and the

'Area' of the square.  The diagonal length is the distance from

one corner to the opposite corner.

4.17.25 Areas/Volume Menu - Square Prism

This function will ask you for the length, width, and height of

the square prism or 'square box'.  You will then be shown an

answer box giving you the volume of the 'box'.

4.17.26 Areas/Volume Menu - Sphere

This function will ask you to input the radius of the sphere.

You will then be shown an answer box giving you the 'Area' and

the 'Volume' of the sphere.

4.17.27 Areas/Volume Menu - Spherical Sector

Chord, Volume, and Area of a Spherical Sector

=============================================

In the input box you must enter the Radius and Height while in

the spherical sector menu.  While in the spherical 'segment'

menu you must enter the height and one other input, either the

radius or the chord.  The height can not be greater than 2 x

Radius!  You will then be displayed an answer box giving you

the chord, volume, and area of the spherical sector.

A sphere is a closed surface where all points are the same from

a given distance.  Another simple name for a sphere is a ball.

A spherical sector is a section of a 'ball' or sphere that is

cut out from the whole part.  It consist of a full revolution

and not just a segment.  A spherical segment is a cut section

from the 'ball' or sphere but it is cut at two parallel lines.

Formula   (Spherical Sector)

============================

V = Volume, A = Area, C = Chord:

V = (2 x pie x Sqr(radius) x height) / 3

A = pie x radius x (2 x height + .5 x chord)

C = 2 x Sqrt(height(2 x radius - height))

4.17.28 Areas/Volume Menu - Spherical Segment

See 4.17.27 (Spherical Sector)

Formula   (Spherical Segment)

=============================

V= pie x Sqr(height) x (radius - height / 3)

A= 2 x pie x radius x height

C= 2 x Sqrt(height(2 x radius - height))

R= (Sqr(chord + 4 x Sqr(height))/(8 x height)

4.17.29 Areas/Volume Menu - Spherical Wedge

This function will ask you for the angle of the wedge and the

radius.  You will then be shown an answer box giving you the

'Area' and the 'Volume' of the Spherical Wedge.

4.17.30 Areas/Volume Menu - Spherical Zone

This function will ask you for various input.  You must input

the height of the spherical zone and in addition you must

either input the radius or both chord 1 and chord 2.  If you

input the radius and the height you will be given the 'Area' of

the zone.  If you input the height and chord 1 and chord 2 you

will be given the radius and volume.

4.17.31 Areas/Volume Menu - Torus

A shape that looks like an anchor ring or a doughnut. If you

were to take a ball and orbit it around a center distance, the

shape that it would leave is a Torus.  There are two radius

involved, the radius of the ball that is orbited and the orbit

radius.

Formula:

========

Radius1 = The orbiting ball.

Radius2 = The orbit radius.

Area   = 4 x sqr(pi) x Radius2 x Radius1

Volume = 2 x sqr(pi) x Radius2 x Radius1

4.17.32 Areas/Volume Menu - Trapezium

A four sided figure in which no two sides are parallel.

Help on Trapezium input

=======================

Let's assume that the longest side of the four sided

figure is down.

�

�   �

�       � �

�           �       ( A Trapezium )

�              H1   �

��               �

� H2              �     �

�  �               �

� � � � � � � � � � � � � � �

�L2������� L1 ��������� L3 Ĵ

H1 = Tall height of the figure.  (Right)

H2 = Short height of the figure.  (Left)

L1 = Distance between heights.

L2 = Dist. from left edge to left height.

L3 = Dist. from rt. edge to rt. height.

(H1 + H2) x L1 + (L2 x H2) + (L3 x H1)

Area = --------------------------------------

2

4.17.33 Areas/Volume Menu - Trapezoid

A four sided figure in which two sides are parallel.  Let's

assume that the longest side of the four sided figure is down.

Side 1 = One parallel line.  Side 2 = The other parallel line.

Height = Height of the figure.

(Side1 + Side2) x Height

Area = ------------------------

2

4.17.34 Areas/Volume Menu - Wedge

A three dimensional shape with a base that has length and

width.  The length of the base travels up to a sharp point

which is the height of the wedge.  The top and the bottom of

the wedge are parallel lines.

Side 1 = The base length.

Side 2 = The base width.

Side 3 = Top of the wedge, parallel to Side1.

Height = Height of base to top of wedge.

Formula

=======

(2 x Side1 + Side3) x Height

Volume = ----------------------------

6

4.18 RATES MENU

This menu choice provides you with speed and feed information.  You

input the required data and you end up with a general speed and

feed to begin to work with.

4.18.1 Rates Menu - Speed and Feed For Drilling

This choice will cause a menu to pop up that asks you what

material and drill diameter you are using.  Strike the F1 help

key for a list of the type of materials that you can input into

this function.

Materials are listed under AISI and SAE steels.

Insert the type of material that you are drilling and the size

of the drill in decimal form.  It will then display a dialog

box that will give you the RPM and the minimum and maximum feed

rate in inches per minute for each material condition.

This information will also go to the MRMDRILL.TXT file on your

drive so you may print it out at any time and/or view it.  To

view the drill file with complete information on all Brinell

hardness numbers click on the 'Details' button or hold down the

'Ctrl' key while striking the 'F4' key.

To print the drill file with complete drill information hold

down the 'Ctrl' key while striking the 'F6' key.

Remember that the drill file called MRMDRILL.TXT gets created

or appended to each time you start a new session with MRM.  To

delete an old drill file do an 'Alt-Minus' function.

Material List

=============

This is a list of materials that can be input into the Mr.

Machinist Drill Speed and Feed input line.  The total list

can be found in the help file while this function is in

effect.

(See the Machinery's Handbook for comparison.)

4.18.2 Rates Menu - Speed and Feed For Milling

This function will ask you for the surface feet per minute, the

work piece or cutter diameter, the chip load, and the number of

teeth on the cutter.  A general chart is positioned on the left

of the screen for handy reference of surface feet and chip

loads.

Determine the Feet Per Minute from the chart on the left and

input the data into the cutting speed input line.  Insert the

chip load data by referring to the bottom of the chart on the

left.  Keep in mind that these are very general speeds and

feeds.

Start slow and work your way up when using feeds/speeds

The three main factors that determine the rate of metal removal

are cutting speed, feedrate and depth of cut.  First, take into

consideration, the depth of cut, then feed rate and then the

cutting speed.

Formula

=======

IPT = Inches Per Tooth

T   = Number of Teeth

FPR = Feed Per Revolution

RPM x IPT x T

Feed Rate = -------------

FPR

RPM x IPT x T

FPR = -------------

Feed rate

Feet Per Minute Chart

=====================

Material            HSS     Carb.

********            ***     *****

Stainless Steel      50       150

Cast Steel           50       150

Cast Iron            75       225

High-Carbon Steel    50       150

Medium-Carbon Steel  75       200

Low_Carbon Steel     125      375

Brass                300      800

Magnesium            600     1000

Aluminum             600     1000

Copper               300     1000

Inches Per Tooth

Cutter                        IPT

******                        ***

Metal Cutting Saws           .001 +

Form Cutters                 .002

End Mills ( less than 1/2" ) .001 +

End Mills ( more than 1/2" ) .003

Plain Cutters                .005

Side Cutters                 .007

Face Mills                   .010

====================================

Use these calculations as a starting

point only.

====================================

4.18.3 Rates Menu - Speed and Feed For Tapping

This menu function in Mr. Machinist will give you the speed and

feed of a tap used in a CNC machine that utilizes ridged

tapping.  High speed tapping is used widely in the machining

industry and it is used with high accuracy.  A hole can be

bottom tapped within 0.0010 using ridged tapping.

Input the tap size and you will be given the speeds and feeds

for the three main categories of materials.

Acceptable tap sizes

====================

4-40, 5-40, 6-32, 8-32, 10-24, 10-32, 12-24

1/4-20, 1/4-28, 5/16-18, 5/16-24, 3/8-16

3/8-24, 7/16-14, 7/16-20, 1/2-13, 1/2-20

These are some general speeds and feeds for tapping.  This

information is general and you can differ from these given

speeds and feeds.  If you are tapping with a machine that can

ridged tap you can increase the speeds and feeds.  Rigid

tapping is an excellent method of tapping that can be very

accurate.  You may also want to multiply your feed by .9 for

spring loaded taps to achieve some 'give' in the

synchronization of the spindle speed and the machine feed.

S = Speed     F = Feed     T = Threads Per Inch.

================================================

T = S / F    S = TPI x F    = S / TPI

================================================

3.82

R.P.M. = S.F.P.M. x -----

O.D.

================================================

Material   Feet Per Minute  Lubrication

========   ===============  ===========

Alum.         90 - 100      Kerosene & Lard Oil

Mach.         40 - 60       Compound, Sulfur-base

or Kerosene & Paraffin

Tool          25 - 35       Sulfur-base or Kerosene

and Lard.

4.19 OPTIONS MENU

This is the user options menu.  Using the functions of this menu,

the user can change the way things look and react.

4.19.1 Options Menu - Mouse

Using this function can adjust the speed of a double click on

your mouse.  It can also change the active button from left to

right.  (For left handed people)

4.19.2 Options Menu - Save Desktop

The 'DeskTop' is the way your screen looks at any given time.

You may have two or three files up on your screen that you are

editing and you may have a few answer boxes and calculations up

also.  You can 'save' the things that your screen is showing

you to a file called, 'MRM.DSK'.  The DSK extension stands for

Desktop.  Then at a later time you may 'retrieve' or 'restore'

your desktop by choosing the 'Retrieve DeskTop' menu choice.

When you exit the program the Desktop is automatically saved

but you can also save they way your Desktop looks during a

session so you can go back to a view before exiting.

By choosing 'Save DeskTop' you will create a file called,

'MRM.DSK' in your current directory.  This file will hold all

of your current views so that you can 'Retrieve' them at a

later time.

Be aware that when you 'Change Directory' in the file menu you

will then 'save' the desktop in the directory that you changed

to.  So if you start the program at a later time and expect to

find your last desktop but you can not, then you may have saved

your desktop in another directory.  You could use this feature

to have multiple views in multiple directories.

4.19.3 Options Menu - Retrieve Desktop

The 'DeskTop' is the way your screen looks at any given time.

You may have two or three files up on your screen that you are

editing and you may have a few answer boxes and calculations up

also.  You can 'save' the things that your screen is showing

you to a file called, 'MRM.DSK'.  The DSK extension stands for

Desktop.  Then at a later time you may 'retrieve' or 'restore'

your desktop by choosing the 'Retrieve DeskTop' menu choice.

This option will search your current directory for the file

called 'MRM.DSK'.  You can also retrieve Desktops from multiple

directories.  You could use this feature to retrieve multiple

views in multiple directories.

Be aware that when you 'Change Directory' in the file menu you

will then 'save' the desktop in the directory that you changed

to.

So if you start the program at a later time and expect to find

your last desktop but you can not, then you may have saved your

desktop in another directory.

4.19.4 Options Menu - Use .BAK File

Using this option you can choose to create or not to create

backup files. Backup files are created with the file extension

.BAK by default.

When you edit a file that exists on your drive, that file is

actually in two places: On your disk or drive, and in the RAM

of your computer.  When you first create a file, it only exists

in RAM until you save it to disk.  When you save a file to disk

or drive for the first time it exists with the name that you

chose to give it.  Mr. Machinist will create a backup file by

default when you save a file that already exists on your disk

or drive.  You can choose to shut this feature off therefore

saving on disk space but loosing the chance to have a file that

has not been over written.

5.0 EDITOR HELP

Movement                     Command

========                     =======

Character Left               Arrow

Character Right              Arrow

Word Left                    Ctrl-Left Arrow

Word Right                   Ctrl-Right Arrow

Line Up                      Arrow

Line Down                    Arrow

Page Up                      PgUp

Page Down                    PgDn

Beginning of a Line          Home Key

End of a Line                End Key

Beginning of a File          Ctrl-PgUp

End of a File                Ctrl-PgDn

Delete Line                  Ctrl-Y

Delete Word Right            Ctrl-T

Start Marking a Block        Shift-(arrow keys)

Copy Block to Clipboard      Ctrl-Insert

Paste Block from Clipboard   Shift-Insert

Delete Block or Clipboard    Ctrl-Del

Hide Block Marker            Ctrl-K-H

Cut into Clipboard           Shift-Del

Show Clipboard Contents      Alt-E-S

Go to Menu Bar               F10

Return to Editor from Menus  Esc

Save current Workspace       F2

Save File as Another Name    Alt-F-A

Search                       Ctrl-Q-F or Alt-S-F

Search Again                 Alt-S-S

Search/Replace               Ctrl-Q-A or Alt-S-R

Replace Last Modification    Alt-E-U

Move Edit Window             Ctrl-F5

(Arrow Keys and Shift-Arrow)

Zoom Resized Edit Window     F5 or Alt-W-Z

Next Window                  F6 or Alt-W-N

Tile (One on top of another) Alt-W-T

Cascade (Headings vertical)  Alt-W-C

Close Current Window         Alt-F3

Start a New File             Alt-F-N

Open an Existing File        F3 or Alt-F-O

Quit Mr. Machinist           Alt-X

You may open and manipulate many edit windows at the same time, cut

and paste text to and from any window.  If you like, you can open

the same file more than once for reference purposes.  If you want

to reference some code from another CNC program you can bring the

file up into an edit window, resize it and cut and paste from it to

your current CNC program.  In doing this you could create a type of

'database' for your machine files.

Let's say you have a machine file, (CNC program) called K0154.TXT.

You could create a file called K0154N.TXT for the 'notes' to this

file.  In K0154N.TXT you could put the job number, cycle time,

tools used, set up notes and what ever you like!  Then when it

comes time to look at that CNC program you could call up these two

files, resize them to a smaller size and look at both of them!

When it comes time to save your changes, strike F2 and it's done.

You'll know that you have made a change to the edit window when you

see a little star '*' just to the right of the lower left edit

window.  If you strike Alt-X to quit when the star is there (you

have not saved your changes) you will be asked if you want to save

your changes.  At this time you can strike 'Y' for yes or 'N' for

no or 'C' for cancel.  Whenever you save a file, Mr. Machinist will

keep your old one and put a BAK extension to it unless you use the

.BAK function in the 'Options' menu.

Let's say your editing a file called README.TXT and you make a few

changes to it and then you save it.  You will now have 2 files on

your system by that file name but with different extensions,

README.BAK and README.TXT.

Do remember this though, if you make a save and a README.BAK exist,

the old README.BAK file will be replaced by the newer README.BAK

file.

Note: README.BAK is used here just as an example!

Using a Mouse

=============

It's faster for a user to use the keyboard instead of the mouse,

but when it comes to resizing and moving windows, the mouse is the

preferred method.  The left mouse button is the one you'll always

use.  Just point and shoot! Pick a menu, click the left button and

down comes a menu.  Choose from the menu and click for results.  To

move around edit windows, 'drag' the mouse on the edit window

heading.  Dragging the mouse means to hold down the left mouse

button while moving the mouse.  When you want to resize the current

window, drag the mouse on the lower right corner of the edit

window.  To highlight a block of data while editing drag the mouse

over the text you desire to mark and that text will become

highlighted.

The editor incorporated into Mr. Machinist can manipulate a file of

65 K or under.  That is the limit at this time.  If you want to

edit a very large file you will have to use a more powerful editor

or 'split' your file and then 'paste' it back together.

The editor in Mr. Machinist has the capabilities to edit almost all

CNC programs.  If the CNC program is larger than this limit then

maybe you can create a smaller program by using sub-programs.  A

small CNC program that does a lot is the sign of a good program!!!

This editor is a standard editor that uses commands that are

universal in most programs.  A small book can be created telling

you how to be proficient in using an editor but here are some

basics.

Features

========

Full mouse support

Move, resize, overlap, tile, or cascade

Open and view several files at once

Search and replace text

Cut, copy and paste using the clipboard

Paste from one file into another

Use the editor while you use the math functions

View the clipboard contents

Restore deleted text

Do block operations, large deletes or inserts

Jump to Line    [Ctrl]-[J]-[L]

==============================

Quickly jump to any line in your document.  A dialog box will

appear that requests the line number you wish to jump to.  Valid

entries are 1 through 9999.

Your cursor will remain on the current line if you enter an invalid

line number.

This command is disabled until you open an edit window.

Cancel   |   No

===============

Clicking on these buttons, or pressing [Esc], says "No, go back to

what I was doing -- don't change or do anything!".

Drag Mode

=========

The current window is being moved or resized.  Press [Enter] to

keep the current position, or [Esc] to restore the former position

and/or size.

Line Number

===========

Enter a line number from 1 to 9999.  You may press the down arrow

to select from a history list of previous entries, or use the mouse

to click on the history icon to the right of the input line.

Reformat Options

================

Select one of the buttons to tell the editor where you want to

start searching.  You may click on the appropriate button with your

mouse, or use the up/down arrow keys to highlight the proper

button.

Replace Text

============

Type in the replacement text you want.  You can also press the down

arrow key to bring up a history list of the last replacement text.

Clicking the mouse on the history list icon to the right of the

input line will do the same thing.

Right Margin

============

Enter a right margin value from 10 to 255.  Press the down arrow or

click on the history icon to the right of the input line to bring

up a history list of previous entries.

Tab Stops

=========

Use your right/left cursor keys to move the cursor to the tab stop

you want to delete/set.  Press the spacebar to delete tabs, or any

other character key (like [x]) to set a tab stop.

IMPORTANT:  If you press spacebar immediately after you bring this

dialog up, all tab stops will be erased!

You can push the down arrow or click on the history icon between

the OK and CANCEL buttons to bring up a history list of previous

tab stop settings.

History Icon

============

The history icon is a little box with the down arrow in it.  It is

usually situated near an input line within a dialog box.

The history icon is selected by pressing the down arrow, or

clicking on it with the mouse.  It brings up a new window that

contains a list of all previous commands that may have been input.

The list is usually 10 items deep.

You can select from this list by using the mouse to scroll through

it, or the up/down arrow keys.  Pressing [Enter] accepts the input

and places it on the input line.  This feature allows you to

quickly pick previous entries.

Input Line

==========

An input line is always found within a dialog box.  It allows you

to enter data to allow some further action to take place.  Input

lines will usually have a history icon associated with them,

thereby allowing you to quickly select from amongst a list of

previous entries.

Resize Corner

=============

The "resize corner" of a window is the bottom right corner that

appears a bit different from the rest of the window border.  By

clicking on it with a mouse, you can resize a window to the

dimensions you want.  Not all windows have this corner, for not all

windows can be resized.

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

Window Border

=============

All windows have a border around them, be it an edit window or a

dialog box.  The border tells you several things about a window.

If the border is highlighted, it means that window is the active

window, or the one you are using.  If the border is not

highlighted, it means the window is not active.  If the border

changes to a color that is not normal for active or inactive

windows, it means you are resizing and/or moving it.

Del Char Right  [BackSpace]

==============  [Ctrl]-[H]

Move the cursor to the left one space, deleting the character

before it.

Char Left       [Left Arrow]

=========       [Ctrl]-[S]

Move the cursor to the left one space.  No characters are deleted.

Char Right      [Right Arrow]

==========      [Ctrl]-[D]

Move the cursor to the right one space.  No characters are deleted.

Del Char Left   [Del]

=============   [Ctrl]-[G]

Delete the character to the right of the cursor.  All other

characters on the line are moved to the left to fill the vacant

space.

Del To End Line [Ctrl]-[Q]-[Y]

===============

Delete all characters from the current cursor position to the end

of the current line.

Delete Line     [Ctrl]-[Y]

===========

Delete the current line.  All lines below it are moved up by one

line.

Del To Start Line [Ctrl]-[Q]-[H]

================= [Ctrl]-[Backspace]

Delete all characters from the current cursor position to the start

of the line.

Delete Word     [Ctrl]-[T]

===========

Delete characters from the current cursor position to the last

character in the word, if the cursor is sitting in a word.  If the

cursor is sitting in spaces, all spaces to the beginning of the

next word will be deleted.

Page Bottom     [Ctrl]-[End]

===========

Move the cursor to the bottom of the current page.  A page is

defined as all text that is within the current window frame.

Hide Selection  [Ctrl]-[K]-[H]

==============

Unhighlight any text you may have selected.  You press this key

sequence if you decide you don't need to perform any action on the

selected text.

Pressing the down arrow without the [Shift] key will do the same

thing.

Page Top        [Ctrl]-[Home]

========

Move the cursor to the top of the current page.  A page is defined

as all text that is within the current window frame.

Auto-Indent     [Ctrl]-[Q]-[I]

===========

Toggle the editor auto-indent feature on/off.

Auto-indent will automatically indent the cursor to the same

position on the next line as the first character on the current

line when [Enter] is pressed.

If Auto-Indent is on, an "I" will appear on the edit window

indicator line.

This command is disabled until you open an edit window.

Insert Line     [Ctrl]-[N]

===========

Insert a carriage return and line feed pair at the current cursor

position.

All characters at the cursor position and to the end of the line

will be moved down to the next line.

Unlike the [Enter] key, the cursor stays at its current position

and does NOT move down to the next line.

Insert Mode     [Ins]

===========     [Ctrl]-[V]

Toggle the cursor between insert and overstrike mode.  In insert

mode, characters are inserted into the text at the current cursor

position and all characters after the cursor are moved to the left.

In overstrike mode, characters at the cursor position are replaced

with the new character you type.

Line Down       [Down Arrow]

=========       [Ctrl-[X]

Move the cursor down to the next line.

Line End        [End]

========        [Ctrl]-[Q]-[D]

Move the cursor from its current position to the end of the line.

Line Start      [Home]

==========      [Ctrl]-[Q]-[S]

Move the cursor from its current position to the start of the line.

Line Up         [Up Arrow]

=======         [Ctrl]-[E]

Move the cursor up one line.

New Line        [Enter]

========        [Ctrl]-[M]

Insert a carriage return at the current cursor position.  The

cursor will move down to the next line.  If Auto-Indent is on, the

cursor will indent to the same position as the first character of

the line the cursor was on when [Enter] was pressed.  Otherwise the

cursor goes to the first position of the new line.

Page Down       [PgDn]

=========      [Ctrl]-[C]

Move the cursor down a page at a time through your text. Note that

a page is defined as all text that is within the current window

frame.

This is not a "true" page down feature, in that the first time you

press the key the cursor simply goes to the bottom of the current

page.  You'll probably have to press it twice to get to the next

page.  Thereafter, it works as a true page down feature.

Page Up         [PgUp]

=======         [Ctrl]-[R]

Move the cursor up a page at a time through your text.  Note that

a page is defined as all text that is within the current window

frame.

This is not a "true" page up feature, in that the first time you

press the key the cursor simply goes to the top of the current

page.  You'll probably have to press it twice to get to the

previous page.  Thereafter, it works as a true page up feature.

Document Reform [Ctrl]-[Q]-[U]

===============

Quickly reformat your document text.  A dialog box will appear that

allows you to select whether to begin reformatting at the current

cursor position or from the beginning of the document.

Reformatting is based on the position of the first character in the

first line of a paragraph and the current right margin setting.

You must be VERY careful when using this feature for you can easily

ruin your document's appearance using the reformatting feature.

You can NOT undo the reformatting process.  It is permanent.

Therefore, you are advised to save your document BEFORE you use

this feature.

This command is disabled until you open an edit window.

Paragraph Reform  [Ctrl]-[B]

================

Quickly reformat a paragraph.

Reformatting is based on the position of the first character of the

current line the cursor is on and the current right margin setting.

Reformatting takes place from the current line to the end of the

paragraph.

You can NOT undo the reformatting process.  It is permanent.

This command is disabled until you open an edit window.

Right Margin    [Ctrl]-[O]-[R]  {EDITOR Cmds:Editor_Commands}

============

Set a right margin value for the editor.  A dialog box will appear

that asks you to enter a right margin value.  Right margin values

may range from 10 to 255.  The default right margin is 76.

The right margin is used by the editor wordwrap, reformatting, and

centering features to determine where to wrap a line of text.

This command is disabled until you open an edit window.

Scroll Down     [Ctrl]-[Z]

===========

Scrolls the screen up while maintaining the cursor position.  If

the original cursor position scrolls off the screen, the cursor

scrolls down a line.  This forces the cursor to stay in the upper

left corner of the screen should the original cursor position

scroll away.

Scroll Up       [Ctrl]-[W]

=========

Scrolls the screen down while maintaining the cursor position.  If

the original cursor position scrolls off the screen, the cursor

scrolls up a line.  This forces the cursor to stay in the lower

left corner of the screen should the original cursor position

scroll away.

Select Word     [Ctrl]-[K]-[T]

===========

Highlight the current word the cursor is sitting on.  The word is

marked from the current cursor position to the next space or end of

line, whichever comes first.  The marked word is put into the

clipboard without further effort on your part.  Marking works in

the normal manner, i.e., it disappears the moment you move the

cursor.

Note that NOTHING will happen if you try to use this command when

the cursor is sitting on a space or the end of a line.

Tab Stops       [Ctrl]-[O]-[I]

=========

Enter custom tab stop preferences in a dialog box.  These tab stops

are used to allow you to quickly insert spaces, or jump over text

to a designated tab stop.

This command is disabled until you open an edit window.

Start Select [Ctrl]-[K]-[B]

============ [Shift]-[Down Arrow]

Start selecting text.  Selected text is that text which is not the

same color as normal text.  You select text when you want to

perform a particular action on that text and no other text, such as

COPY, CUT, CLEAR, etc.

Tab Key         [Tab]

=======         [Ctrl]-[I]

Move the cursor to the next tab stop.  Tab stops default to every

fifth character on a line.  You may reset the tab stops by using

[Ctrl]-[O]-[I].

When in INSERT mode, the cursor will attempt to go to the next tab

stop, happily INSERTING SPACES as it looks for that next stopping

point.  If it is at the last tab stop, it stops inserting spaces

and takes a big jump to the first character of the next line.  Try

to keep tab use to a minimum when the cursor is in insert mode.

You eat up valuable buffer space real quick!

When in OVERSTRIKE mode, the cursor will attempt to go to the next

tab stop, happily skipping over characters as it looks for that

next stopping point.  If it is at the last tab stop, OR the line is

too short to allow it to reach the next tab stop, it takes a big

jump to the first character of the next line.

Document End    [Ctrl]-[PgDn]

============    [Ctrl]-[Q]-[C]

Move the cursor from its current position to the end of the

document.

Document Start  [Ctrl]-[PgUp]

==============  [Ctrl]-[Q]-[R]

Move the cursor from its current position to the start of the

document.

Word Left  [Ctrl]-[F]

=========  [Ctrl]-[Right-Arrow]

Move the cursor to the beginning of the previous word.  Use this

feature when you want to quickly jump from word to word.

Word Right  [Ctrl]-[A]

==========  [Ctrl]-[Left Arrow]

Move the cursor to the beginning of the next word.  Use this

feature when you want to quickly jump from word to word.

Wordwrap        [Ctrl]-[O]-[W]

========

Toggle the editor wordwrap feature on/off.

Wordwrap allows you to type in text and automatically wrap to the

next line when the cursor reaches the current right margin setting.

If wordwrap is on, a "W" will appear on the edit window indicator

line.

This command is disabled until you open an edit window.

Goto Marker     [Ctrl]-[Q]-[#]

===========

Using the WordStar [Ctrl]-[Q]-[#] key sequence, where "#" is a

number 1,2,3,4,5,6,7,8,9, or 0, will allow you to access up to 10

place markers you have previously set in your text.  Your cursor

will go nowhere if you try to jump to a non-existent marker.

Hold down the 'Ctrl' key and then strike 'q', let up on both keys

and then strike a previously set number key and if this number key

has been set you will go to the marked line #.

This command is disabled until you open an edit window.

Set Marker      [Ctrl]-[K]-[#]

==========

Using the WordStar [Ctrl]-[K]-[#] key sequence, where "#" is a

number 1,2,3,4,5,6,7,8,9, or 0, will allow you to set up to 10

place markers in your text.

While holding down the 'Ctrl' key strike 'k', then let up on both

keys and strike a number key.  Nothing will appear to have happened

but in actuality you have set a place marker to the number key you

have struck.

This command is disabled until you open an edit window.

6.0 QUICK-KEY CHART

F1  = Help File

F2  = Save Current File Being Edited

F3  = Open a File to Edit

F4  = Calculator

F5  = Zoom any Window

F6  = Change Window Views

F7  = Rates (Speeds and Feeds)

F8  = Print Current File Being Edited

F9  = Print Unregistered Version Invoice

F10 = Activate Menu Choices

Ctrl-F1  = View  MRMSOLVE.TXT (Answers)

Ctrl-F2  = Circular Segment Menu

Ctrl-F3  = Sequence a CNC File

Ctrl-F4  = View  MRMDRILL.TXT (Drill Speeds/Feeds)

Ctrl-F5  = Move Edit Window

Ctrl-F6  = Print MRMDRILL.TXT (Drill Speeds/Feeds)

Ctrl-F7  = Drill Speeds/Feeds Dialog

Ctrl-F8  = View  MRMBOLT.TXT (Bolt Circle Answers)

Ctrl-F9  = Print MRMBOLT.TXT (Bolt Circle Answers)

Ctrl-F10 = Split a CNC File

Ctrl-A   = Word Left

Ctrl-B   = Paragraph Reformat

Ctrl-C   = PageDown

Ctrl-D   = Character Right

Ctrl-E   = Line Up

Ctrl-F   = Word Right

Ctrl-G   = Delete Character Right

Ctrl-H   = Delete Character Left

Ctrl-I   = Tab Key

Ctrl-J-L = Jump to Line Number

Ctrl-#   = Set Marker

Ctrl-K-B = Start Select Text

Ctrl-K-C = Paste

Ctrl-K-D = Save/Done

Ctrl-K-F = Save As

Ctrl-K-H = Hide Block Marker

Ctrl-K-K = Copy Selected Text

Ctrl-K-T = Select Word

Ctrl-K-Y = Cut Selected Text

Ctrl-L   = Search Again

Ctrl-M   = New Line

Ctrl-N   = Insert Line

Ctrl-O-I = Tab Stops Set

Ctrl-O-R = Right Margin

Ctrl-O-W = Word Wrap

Ctrl-P   = Print MRMSOLVE.TXT (Answer File)

Ctrl-Q-# = Go to Marker

Ctrl-Q-A = Search/Replace

Ctrl-Q-C = Bottom of File

Ctrl-Q-D = Line End

Ctrl-Q-F = Search

Ctrl-Q-H = Delete to Start of Line

Ctrl-Q-I = Auto Indent

Ctrl-Q-L = Undo

Ctrl-Q-R = Document Start

Ctrl-Q-S = Line Start

Ctrl-Q-U = Reformat Document

Ctrl-Q-Y = Delete to End of Line

Ctrl-R   = Page Up

Ctrl-S   = Character Left

Ctrl-T   = Delete Word Right

Ctrl-U   = Un-Sequence a CNC File

Ctrl-V   = Insert Mode

Ctrl-W   = Scroll Up

Ctrl-X   = Down a Line

Ctrl-Y   = Delete Line

Ctrl-Z   = Scroll Down

Ctrl-Left Arrow  = Word Left

Ctrl-Right Arrow = Word Right

Ctrl-PgUp        = Beg. of a File

Ctrl-PgDn        = End of a File

Ctrl-Insert      = Copy Block to Clipboard

Ctrl-Delete      = Delete Block or Clipboard

Ctrl-BackSpace   = Delete to Start of Line

Ctrl-End         = Page Bottom

Ctrl-Home        = Page Top

Alt-A = Area Menu

Alt-C = Circle Menu

Alt-E = Edit Menu

Alt-F = File Menu

Alt-M = Math Menu

Alt-N = Replace Text

Alt-O = Options Menu

Alt-R = Rates Menu

Alt-S = Search/Replace Menu

Alt-T = Trig. Menu

Alt-V = Conversions Menu

Alt-W = Window Menu

Alt-X = Exit Mr. Machinist

Alt-F1 = About Mr. Machinist (Version Date)