Date: 10 Jul 1985 19:19:24 PDT

Subject: Swapping "Esc" and "~" on AT -- SWPKEYAT

From: Craig Milo Rogers  <ROGERS@USC-ISIB.ARPA>

I've recently gotten an IBM-PC/AT.  Boy, am I burned by the

ill-positioned "Esc" key.  So, I decided to "fix" things.

I started by prying up the key tops of the "Esc" and "~" (tilde)

keys, and swapping them.  Of course, that didn't really fix anything,

but it did show me what my goal looked like.

Next, I decided to write a low-level BIOS extension which intercepts

and swaps key codes to implement the key cap swap I described above.  The

result was SWPKEYAT.ASM, which has been submitted to the INFO-IBMPC library.

SWPKEYAT swaps by intercepting the keyboard hardware interrupt, since

I thought that would fool as much software as possible.  The program is

rather ugly, though:  after swapping key scan codes it jumps into the

AT's ROM BIOS keyboard hardware interrupt routine.

So far, so good.  DOS is perfectly happy, and so are any programs

which do DOS or BIOS keyboard I/O.  In particular, both VDTE and KERMIT

run OK.

But, what about programs that do their own sub-BIOS keyboard

scan conversions?  Luckily for me, I don't run many programs like that.

However, I absolutely had to fix my copy of Lugaru Epsilon.  So, I used

DEBUG to disassemble and trace Epsilon until I found the (single) location

where it was reading the keyboard port.

It takes several instructions to swap the keyboard scan codes.

I couldn't find an easy way to insert the entire sequence into Epsilon.

So, I added an additional interrupt handler to SWPKEYAT.  The new interrupt

does an IN AL,60 (reading the keyboard port) and swaps the appropriate

pair of scan codes.  I selected INT 3FH from the end of the "Reserved for

DOS" range for this function.

The appropriate location to patch in Epsilon 2.02 is offset

BDBE.  I copied EPSILON.EXE to EPSILON.XXX and ran DEBUG on EPSILON.XXX.

The .EXE-file offset became C4BE.  I patched that location, wrote the

file back out, and renamed the result to XXX.EXE.  Once I had confirmed

that my patched version worked, I renamed XXX.EXE to EPSILON.EXE.

So much for this hack.  If you use SWPKEYAT and run into a program

which doesn't see the swap (CodeSmith ((tm) Visual Age) may be a candidate),

I'll be interested in hearing about the problem (but probably won't fix it).

Craig Milo Rogers

------------------------------

Date: 11 Jul 85 15:26:00 EDT

From: TD9H@CMU-CC-TE.ARPA <TD9H@CMU-CC-TE>

To: rogers%usc-isib@USC-MAILNET

cc: brackenridge%usc-isib@USC-MAILNET

Subject: Epsilon Patch for SWPKEYAT

Epsilon 3.0 allows you to remap keys at the level of the keyboard

interrupt (far below the command/key binding level).  It sounds like

your patch will do till then.

Todd Doucet

Lugaru Software Ltd.

[Todd informs me version 3.0 of Epsilon will be released "real soon now". -wab]

------------------------------

To: kpetersen@wsmr-simtel20.army.mil

From: jrv@mitre-bedford.arpa

Subject: SWPKEYAT

-------

Keith:

I am submitting SWPKEYAT.ARC, which includes updated versions of two

files listed in "nodescr.idx":

"PD1:","<MSDOS.KEYBOARD>","SWPKEYAT.ASM",1,7140,7,850710,""

"PD1:","<MSDOS.KEYBOARD>","SWPKEYAT.DOC",1,2966,7,850715,""

SWPKEYAT swaps the escape and tilde (~) keys on the AT keyboard.  The

version in the library conflicts with QuickBasic.  Therein lies a tale...

In SWPKEYAT, Craig Milo Rogers at USC/ISI implemented a new interrupt

which acts like an IN from the keyboard data port with the appropriate

scan codes swapped.  The new interrupt is for use in patching programs

like EPSILON that have their own keyboard interrupt handlers.  For

this, he picked interrupt 3FH, and states:

"INT 3FH is the last 'Reserved for DOS' interrupt.  DOS 3.10

doesn't use it, but that's no guarantee about future availability."

From the phrase 'Reserved for DOS' I assumed that I needed to worry

about compatibility only when upgrading to a new version of DOS.

However, I eventually discovered conflicts with QuickBasic and QuickC.

It turns out that they use interrupts 3DH, 3EH, and 3FH.  Apparently

the warning should have been 'reserved for Microsoft'!

I rewrote SWPKEYAT to implement 3CH.  I also rewrote it so that

SWPKEYAT itself never invokes that interrupt.  Now it would fail only

if:

1) some program changes 3CH,

2) that program doesn't restore 3CH

3) you run some third program which was patched to invoke 3CH.

SWPKEYAT runs on early ATs (BIOS ROM date 01/04/84), and finishes scan

processing by jumping into the ROM at a magic address.  This is clearly

not portable to other ROM BIOSs.  It was also the only reasonable way

to accomplish the function.

Newer ATs implements a "keyboard intercept" feature (function 4FH of

INT 15H) which make scan code swapping much cleaner.  To switch the

escape and tilde keys on such a machine, see the program SWAPKEY in the

same ARC file.

Here's the ARC file contents:

Filename        Length      Date

--------        ------      ----

SWAPKEY.ASM       2269    01-18-89 swap escape and tilde on new ATs

SWAPKEY.COM         70    01-13-89  with keyboard intercept feature

SWPKEYAT.ASM      7783    01-18-89 swap escape and tilde

SWPKEYAT.COM       131    07-07-88  on old ATs (BIOS

SWPKEYAT.DOC      2894    02-26-89   dated 1/4/84)

----            ------

0005             13147

I suggest the following entry for SIMIBM.IDX:

"PD1:","<MSDOS.KEYBOARD>","SWPKEYAT.ARC",1,,,,"swap ESC and tilde on AT kbd"

- Jim Van Zandt