This feature will be included in a future version

of the software. I include the info here for use

in planning how we will use these satellites.

P feature letter and PF command.

The PF command can be used without arguments to request a forwarding

list, or with arguments to send a forwarding list.

The main use for this command is to allow a satellite bbs to obtain

forwarding information from a ground station.

Call the ground station G and the satellite S.

Case 1: G connects to S.

S sends SID and prompt.

G sends SID and prompt.

At this point, G knows it is connected to S and has seen the SID with

the P feature letter. S knows it is connected to G and has also seen

the P feature letter.

G sends messages to S, driven by the usual forward file entries.

Time to reverse forward.

G sends PF d1 d2 d3 ... dn to S.

S responds with a prompt.

G continues with the normal reverse forwarding sequence ("F>" prompt).

S sends all messages with designators d1 d2 d3 ... dn

Case 2: S connects to G.

G sends SID and prompt.

S sends SID and prompt.

At this point, G knows it is connected to S and has seen the SID with

the P feature letter. S knows it is connected to G and has also seen

the P feature letter.

S sends the PF command with no arguments to G.

G sends PF d1 d2 d3 ... dn to S.

S sends messages to G, driven by the list of designators it received from G.

Time to reverse forward.

S continues with the normal reverse forwarding sequence ("F>" prompt).

S sends  messages to G, driven by the usual forward file entries.