+----------------------------------------------------+

|     WDCG -- Wake Digital Communications Group      |

|                   presents                         |

|    PTP -- Packet Terminal Program Version 3.0      |

|       with XPACKET binary transfer protocol.       |

|                                                    |

|          Includes NET MASTER feature and FAX       |

|             support for the AEA PK-232             |

|      BY N4PY .... FOR AMATEUR RADIO USE ONLY       |

|     Split-screen routine complements of AA4L       |

|             Documentation by AB4S                  |

|                                                    |

+----------------------------------------------------+

PTP is a terminal program written for the application layer of a packet

radio station.  This program was developed specifically for use with

the IBM PCjr and the TAPR TNC, but it works with other versions of the

IBM PC and other TNCs as well.  PTP requires IBM DOS 2.1 or later, and

will work with the monochrome adapter or the CGA adapter.  256K of RAM

minimum is required to run this program.  Very little testing has been

done on any other combinations.  It was developed originally for

private use by Carl, N4PY, but at the urging of Ed, AB4S, Carl agreed

to allow this one out of his basement.  For use with the IBM PCjr, see

the note in the Appendix.

Three modes of file transfer are supported:

ASCII file transfer using hardware flow control

XMODEM protocol (with application and AX.25 error correction)

XPACKET protocol (with AX.25 error correction)

There will be no discussion of the first two, as they are both well

documented elsewhere.  XPACKET, however, is the invention of N4PY, and

therefore not too widely known, so far.  XPACKET is a binary transfer

method using speeds that are the same as ASCII dumb transfers.

XPACKET, simply put, is nothing more than a binary transfer in

transparent mode so that all eight bits of each byte can be sent, and

the TNC will not mistake certain characters as commands to itself.

What is the reason for XPACKET if that's all there is to it?  Well, the

justification is to allow the orderly start and end of a file transfer.

Without XPACKET, each operator has to enter TNC transparent mode and

coordinate the start of the transfer.  This can be awkward, as you may

know.  Also, without XPACKET, there is no way for the receiver to

cancel the operation when he realizes that he didn't get something

right.  XPACKET tries to account for the coordination problem, and

allow the "reverse channel interrupt" function to occur.

When the transfer is begun, the receiving station will automatically

send a NAK character every ten seconds until it has received some data

from the sender.  At this time, the only character the receiver can

send is a CAN (cancel) which will cause the sending station to abort

the transfer.  The CAN may be sent one of two ways:

Automatically -- if a block is received which does not agree with the

byte count which was sent at the beginning.

Manually -- if the receiving operator presses the Ctrl-N key.

The sending station operator can also cause the transfer to abort by

pressing Ctrl-N while the transfer is taking place.  You should rarely

encounter the automatic abort, but if you do, you should try starting

the transfer over again.  It is likely that one of the two stations

involved in the transfer has his TNC set up wrong or a PCjr is being

used at the receiving station and is attempting to write his file

directly to a physical diskette.  See the "Recommended" list and the

PCjr note in the Appendix.

USER INSTRUCTIONS

First of all, the only thing you should have to remember is to press

the Home key when you can't remember anything else about PTP options.

This will bring up a help screen which you can customize to help with

your own selections.  (See PTP.HLP in the FILES section of this

document.)

Use a text editor (such as IBM Personal Editor II, or Professional

Editor) to create a file named PTP.DEF.  This file will consist of the

numbers 11 through 40 and the information you want to be sent to the

TNC when the appropriate F-Key is pressed.  It will also contain

information about how to route traffic listed on the WDCG PBBS or

W0RLI-type BBS and whether or not to add line feed characters after

each CR during ASCII uploads.  All information in PTP.DEF is in pairs

of lines with the first of each pair describing an option, and the

next defining your choice for that option.

PTP.DEF might look like:

11

c wa5szl-1+cr  +----------------------------------------------+

| This file is optional.  The usage            |

12             | of the F-keys is: 11-20 = Shift-F1 - F10     |

c k4iww+cr     |                   21-30 = Ctrl-F1 - F10      |

|                   31-40 = Alt-F1 - F10       |

.              +----------------------------------------------+

39

My name is Carl and my QTH is Franklinton, NC.+cr

40

73 and good luck on packet radio.+cr

CONNECT TEXT   Default text for Alt-M connect message.  This can be up

text_string    to 255 characters of text.  Automatically selected if

MSGDEF is ON.

|     STARTUP        Commands you wish sent to the TNC upon starting PTP.

|     cmd_string     You can have up to 20 of these, each prefaced with

|                    "STARTUP."  Prior to sending the commands you place here,

|                    PTP will send XON (Ctrl-Q), ECHO ON or OFF (depending on

|                    split or single screen selection), MYCALL and CONNECT.

|                    Also, if AUTOMSG YES is set, CMSG OFF will be send and if

|                    AUTOTIME YES is set, the TNC clock will be set from the

|                    DOS clock.  All of these will be sent before the STARTUP

|                    commands will be sent.

|     EXITING        Commands you wish sent to the TNC upon ending PTP.  You

|     cmd_string     can have up to 20 of these, each prefaced with

|                    "EXITING."  Prior to sending the EXITING commands, PTP

|                    will send CMSG ON if AUTOMSG YES was set in this file when

|                    PTP was started.

AUTOTIME       TNC-2 or compatible only.  Sets TNC clock time from the

YES            system clock.  Default NO.

AUTOMSG        TNC-2 or compatible only.  If YES, sets CMSG OFF at

YES            program startup and ON at exit.  Default NO.

HOLDBUF        This describes the size of the retrieve buffer lines.

128            (10 - 256) default is 128

STREAM CHAR    This describes the ASCII value of the character selected

124            for the STREAM character in a TNC-2 or compatible.

MYCALL         This describes the identifier to be used for the control

N4PY           station when using Net Master mode.

CONNECT BELL   Connect alarm bell.  If YES, alarm will sound when

YES            connection is detected.

FOREGROUND     This describes the color choice for text on the screen.

15             (0 to 15 -- See your BASIC manual for color numbers)

DEFBELL        This will determine the initial state of the BELL option

OFF            when PTP is started.

AUTOSTART      If this is set to YES, PTP will go to communicate

YES            (terminal) mode automatically after the main menu is

displayed.

DEFQSVE        The initial state of QSAVE will be active if this is

ON             defined as ON.  The default is OFF.

MSGDEF         The initial state of your connect message (from PTP, not

ON             the one from the TNC) will be the default message in

CONNECT TEXT.

BACKGROUND     This describes the background color for text screen.

0

STATE          This should be the 2-letter Postal Code for the state in

NC             which the PBBS you are using is located.

NEAR      _____________

VA                     |

|

NEAR                   |____ These are the nearby states that might be

SC                     |     expected to access the PBBS upon which you

|     are placing the NTS traffic generated with

NEAR      _____________|     PTP.

GA

SENDLF         This will determine whether or not you want PTP to add

NO             LF characters after CR's during an ASCII upload.

DCD            This will determine whether or not you want PTP to use

YES            the DCD line to determine when a connection occurs.  If

NO, receipt of *** CONNECTED will be used to detect

connection.

USERS          This is used only with the TNC-2 or compatibles.  When

3              you are involved in a file transfer, the USERS command

for the TNC will be set to one.  After the transfer is

completed, USERS will be set to the value specified

here.  If this option is not specified USERS will not be

changed.

CONNECT FILE   The full filename to which you want connected text to be

B:MYMSGS.TXT   saved when Qsave is enabled.  DCD YES is required.

NOTE:  All "keywords" used in this file (STATE, NEAR, etc.) must be in

upper case.  There must not be any blank lines in this file.

NOTE:  If you do not specify FOREGROUND and BACKGROUND, they will be

set to 7 and 0 respectively.  Also, if FOREGROUND and BACKGROUND are

set to the same value, the defaults of 7 and 0 will be used.

NOTE:  There is no order dependence to the keyword pairs used in

PTP.DEF.  You may put those in any order you like as long as you keep

the pairs together.

You do not have to define every possible F-key, but be sure that the

number for the key you want to use precedes the text for that key.

Also, if you do not desire to set the other parameters, you do not need

to include them in your PTP.DEF file.

If you assign a string of characters to an Fkey, whether it is Alt-

Fkey, Ctrl-Fkey, Shift-Fkey, or just Fkey 7 through 10, you have

complete control of the string.  If you desire a CR character to follow

your string you simply type "+cr" at the end of it.  If you desire an

imbedded CR character, use "+cr+" at the place where you desire the CR.

To start a string with the CR character, you must put "cr+" before the

string to be sent.  Also, if you want to have the Ctrl-C character as a

part of the Fkey definition, you can do that by using "cmd+", "+cmd+" or

"+cmd" at the point where the Ctrl-C is to appear.

Also, you may want to create a file to be used as setup information for

your TNC.  This file is optional, and may have any name you like.  It

is a command list for the TNC and will be sent to the TNC when you

press Alt-L from the terminal communications screen.  An example of

this file would look like:

MYCALL N4PY   +----------------------------------------------+

SCREENL 0     | This file is optional.  The commands are     |

.             | the ones listed in the owner's manual for    |

.             | your TNC.  Be sure they are set the way you  |

.             | want your TNC to be configured.              |

PACTIME 1     +----------------------------------------------+

A suggested method of building this file is to enable capture (via

Qsave or ASCII rcv mode) and enter DISP.  Your TNC should give you a

list of all the commands and the present settings for these commands.

In a similar fashion, you can make other files like the PTP.DEF file

which will alter the characteristics of your system.  All the same

rules apply for the additional DEF files.  At any time, you can refresh

PTP's "personality"

by using the Alt-E and answering the prompt with the path and filename

of the new DEF file you wish installed.

NOTE: Any time you install a new DEF file, any parameters not set in

the new file will remain the same as they were last set.  That

is, PTP defaults will not be used, the ones from the previous DEF

file will be.

PTP will be updated from time to time.  Check the source from which you

received this copy for updated versions.  The author does not plan to

release the source code, but comments and suggestions about future

releases are appreciated.

When you first RUN the program, you will be given a menu screen that

will allow you to set up several operating parameters.  The numbered

parameters are:

1  Line bit rate  [1200] (300..9600)

2  Data bits  [8] (7 or 8)

3  Parity type  [n] (e,o, or n)

4  Split Screen  [y] (y or n)

5  Comm Port  [1] (1 or 2)

6  QSave File Name  [B:CAPTURE.TXT]

7  User Defined  []

8  User Defined  []

9  User Defined  []

10  User Defined  []

11  Save Setup  [PTP]

12  Load Setup  [PTP]

These parameters can all be set by the user.  The procedure for each

follows:

Line bit rate --- Type 1,9600<CR> if the desired baud rate is not

shown.  The 1200 (or any other ABAUD speed you

select) will become the operating speed for the

terminal program.  Must match TNC.

Data bits ------- type 2,8<CR> if the desired word length is not shown.

Must match TNC.

Parity type ----- type 3,n<CR> if desired parity is not shown.

Even parity, Odd parity, or No parity.  Must match

TNC.

Split Screen ---- type 4,y<CR> if you desire a split screen.  If you do

not select the split screen, a full screen mode will

be used with no keyboard buffer, and the TNC will

provide echoing.

Comm Port ------- type 5,2<CR> if you need to use com2 port for TNC

connection.  Use the comm port (1 or 2) to which you

connected your TNC.

QSave File Name - type 6,c:message.sav<CR> if you want to have the

Quick Save buffer saved under a different name when

you use it.  See the explanation of CAPTURE.TXT under

FILES USED BY PTP TERMINAL PROGRAM.

User Defined ---- type 7,XXX<CR> if you want to use the F keys for

frequently sent commands, or sequences.  Note that

items 8, 9 and 10 are also used in the same way.  In

addition to entering the sequence to be sent, you can

type 7,,XXXX<CR> where XXXX will be displayed on the

terminal screen as a reminder of what you have

attributed to that key.  This is true for keys 8, 9

and 10 also.  Try it; you'll like it.

Save Setup ------ type 11,c:whatever<CR> to select the file name for

this parameter file.  If you type only "11," your

parameter file will be saved as "PTP.MOD"  All

parameter files are .MOD so don't try to assign an

extender to them -- the pgm will do that for you.

The file PTP.MOD will be automatically loaded for you

when the program is started.

Load Setup ------ type 12,c:whatever<CR> to select the file name of the

parameter file you wish to use.  The rules are the

same as for 11 (Save Setup).

After the menu is written to the screen, you will see a list of all

.MOD files present on your default disk.  You may make as many of these

files (with different names) as you like.  Each can have a different

personality.

You will notice that when you type nothing and simply press the Enter

key, you will be placed in terminal mode and the previously selected

commands will be issued.  Take a few minutes to set up your parameters

the way you like to operate, then save the .MOD file before going into

terminal mode.  The only prompt shown on the main menu screen is

F2=Exit.  When F2 is pressed, you must also press the Enter key from

this screen.

NOTE: If AUTOSTART is set to YES in the PTP.DEF file, PTP will go to

terminal communications mode automatically upon initial entry to the

menu screen.

If you have selected the split-screen mode for terminal communications,

you will have an inverse video bar across the screen near the bottom.

This bar contains some reference information about the status of the

connect message and status of the bell action, etc.  If you use F5 to

return to DOS, depending on what you do while in DOS, this line might

be gone when you return via EXIT.  If this is the case, simply press

Alt-C (clear screen) to restore the screen divider line.  The Alt-key

definitions are:

Alt-A   Abort the sending of NTS traffic.  This key will also halt the

loading of TNC parameters via Alt-L.

Alt-B   Toggles the BEL character action.  This affects only whether or

not the BEL will cause the alarm to sound when received.  This

has nothing to do with the Connect alarm in PTP.DEF.

Alt-C   Clears all text from the communications screen.

Alt-D   Display the definitions for the extra 30 F-keys.  This only

works if you have the PTP.DEF file on your disk.

|     Alt-E   Sets up new .DEF file.  You can have as many .DEF files as you

|             like with different characteristics.  The names of these files

|             can be anything you like and it can be wherever you like.  You

|             will be prompted for a complete path/filename.ext.  When you

|             install a new DEF file, any parameters not changed by it will

|             remain the same as they were before; that is they will not go

|             back to the PTP default if they were set by PTP.DEF when the

|             program was started.

Alt-F   Exit the Net Master mode.  See the Appendix for an explanation

of Net Master.

|     Alt-G   Toggles the system printer (LPT1) on and off.  When on,

|             everything received to the terminal will be also printed as

|             displayed on your screen.

|

|     Alt-H   Display the holding buffer.  When invoked, this function will

|             display the last 5000 bytes received in the order they were

|             received.  As it scrolls, you may press the S key to start/stop

|             the scrolling.

Alt-L   Load the TNC parameters.  You must make a file (using a text

editor or by capturing the results of DISP) which contains the

setup you desire for your TNC.  This option allows you to send

the entire file of commands to the TNC.

Alt-M   Connect message screen.  You are able to edit or remove the

connect message you have selected or set up via the PTP.DEF

file.

Alt-N   Enable Net Master mode.  See Appendix for an explanation of Net

Master.

Alt-P   Purge the communications buffer.  This key will get rid of all

the data which has accumulated in the communications buffer.

Alt-R   Send traffic files(s) to a W0RLI type BBS or MAILBOX system.

(See Alt-W.)  When logged onto a W0RLI-type system, you may at

any time begin sending a previously-entered piece of NTS

traffic by pressing Alt-R.  You will be shown a list of the

prepared traffic available on your default disk(ette) drive.

You will be asked to enter the number (only) of the message you

wish to send.  The rest is automatic.

Alt-S   Send traffic file(s).  (See Alt-W.)  When logged onto a WDCG

PBBS system, and when you are in the message section (or at the

main menu) you may send your NTS traffic files automatically by

pressing this key.  When the message is entirely sent, PTP will

cause the message to be saved on the PBBS and will rename your

message file to MSGnnn.LOG.

Alt-T   Escape from transparent mode.  When you are in the split screen

mode, you cannot go back to command mode because the characters

are buffered until Enter is pressed.  This key will cause a

one-second delay, send 3 ^C characters, and delay for one more

second.

Alt-W   Write traffic.  This key will take you to a full-screen editor

which has a template for message traffic.  The last message

which was written will be displayed again, and you can

overwrite the message with a new one.  If you want to enter a

field that is completely different from the one shown, you may

press Alt-E to erase the field before typing.  See NTS TRAFFIC

GENERATION.

In addition to the Alt-keys, from either communications screen you may

enter the following:

F1   [Menu] This key will immediately take you to the main menu.

F2   [Exit] This key will cause an end to the program.  When you press

F2, you will be asked if you are sure you want to exit.

F3   [Rcvf] This key will allow you to receive a file from the PBBS or

from another packet station.  You are asked for the

transfer protocol desired, and the drive to store the file

on.  If you wish to have the received file stored on a

subdirectory other than the current (default) subdirectory,

you must specify this at the time you specify the drive

(i.e. b:\subdir\).  Next you are asked to provide a name

for the file, unless it is an XPACKET file.  You may

(optionally) provide a name for the XPACKET file to be

received.  When you are receiving an ASCII file, you will

use F3 again to close the file unless you and the other

station are both using PTP to do the transfer in which case

the file will be automatically closed when the file

transfer is completed.  Note that the definition changes on

the bottom of the screen when you are receiving an ASCII

file or using Qsave.

F4   [Xmtf] This key will allow you to upload (send) a file from any

disk drive on your system.  When you are asked for the

drive be sure to include the colon (i.e. A:).  The protocol

choices are the same as for F3.  If you are using

subdirectories on your disk(ette), and you wish to send a

file from other than the current subdirectory, you must

specify the subdirectory when you give the file name (i.e.

\subdir\filename.ext).  The subdirectory portion of the

file name will be stripped from the file name before it is

sent in Xpacket mode.

F5   [DOS ] This key causes an immediate exit from the terminal program

and you will get the DOS prompt.  An XOFF is sent to the

TNC when you go to DOS via F5, and an XON is sent when you

return.  You may do any DOS functions that your amount of

memory will support.  Remember that the terminal program

stays in memory, so the usable memory available to you is

reduced by the amount required by the program.  When you

are ready to return to the terminal program, Enter "exit"

at the DOS prompt.

NOTE: If you leave the program altogether while you are in DOS, you

will have to send a CTRL-Q to the TNC to restore normal

operation.  If you return to PTP via "exit", it will be done for

you.  Each time PTP is started it issues an XON to your TNC on

the selected comm port.

NOTE: You must be running IBM PC DOS version 2.1 or higher to use F5.

Additionally, you must have COMMAND.COM somewhere that DOS can

find it or the program will abort.

F6   [Qsve] This key will enable the "Quick Save" option.  On the main

menu you selected a file name for Quick Save.  When F6 is

pressed, the file will be appended with all that is

received until you press F6 again, or F3 to close the file.

The F6=[Qsve] will flash to let you know that the file is

open.  You can, if you desire, capture any messages or

monitored packets which are being held in your TNC due to

either XOFF or because of the DTR line not being raised

until the comm port is opened by pressing F6 immediately

after you do the null enter to go to the communications

screen.

F7 - F0     These keys are user defined.  See the main menu to set up

the definition and the 4-character description for these

keys.  Don't forget the rules for insertion of the CR

character.  (See information under PTP.DEF for rules

concerning CR character and Ctrl-C character.)

Home        This key will cause the PTP.HLP file to be displayed, even

if you no longer have the PTP diskette installed.  When the

program is loaded, PTP.HLP is read from the diskette and

kept in storage for use when Home is pressed.

Cursor Up   These keys are used to recall previously entered text.  The

and      PTP.DEF  file has an entry with the keyword HOLDBUF.  This

Cursor Dn   determines the length of the recall line.  You should set

this to the same length of your PACL in the TNC.  When you

press Cursor Up, you will step through the ten-step buffer

in one direction, and pressing Cursor Dn will step the

other direction.  Experiment with this and set the length

so it is proper for your operation.  This is most useful

for correcting typographical errors in connect attempts, or

for CQ messages when in CONVERSE mode.  This function is

not available in single screen mode.

NTS TRAFFIC GENERATION

While in communications mode, if you desire to write a piece (or more)

of traffic, you can simply swap screens (by pressing Alt-W) and enter

the traffic on a full screen template.  The fields are defined and you

can type the required information in the highlighted fields.  There are

several keys which are used for NTS traffic entry.  They are:

Enter    Moves the cursor to the beginning of the next line, if multi-

line field.  If you are on the last line of a multi-line field

or if the field is a single line field, the cursor will move

to the beginning of the next field.

Tab      Moves cursor to the next field.

Shft-Tab Moves cursor to beginning of the previous field.

Alt-E    Erases to the end of the current field.

Alt-C    Returns to communications mode.

Esc      Saves the message under the name MSGnnn.MSG.

The nnn in the message name is the number field from the

message itself.  When this message has been uploaded to

the WDCG PBBS or W0RLI-type Mailbox, the message file will

be renamed to MSGnnn.LOG.

Note that the check is not required in the message outline.  It will be

automatically generated when the message is saved.

The TIME field is special.  It is not usual that this field be used and

it will normally be left blank.  If you desire to use the time field,

there are two options:

* Put an "A" in the first position of the time field.  This will

cause PTP to enter the current time from the system clock.  There

will be a "Z" appended to the time.  Be sure your system clock is

set to UTC if you use this option.

* You may enter the time directly in the time field.  This time will

not be changed by PTP.

|     The DATE field can be automatically filled in for you from the system

|     clock if left blank, or you can enter a date if different from the

|     current date or if your DOS system clock is not set properly.

After you have finished with the signature line on the message form,

the cursor will move to a field called "@BBS" and you will be able to

specify the call sign (only) for the store and forward feature of the

W0RLI-type MailBox systems.  Do not include the "@" since PTP will add

that automatically for you.

Later, when logged on either a WDCG PBBS system, or a W0RLI-type

MailBox system, you can send your accrued traffic by:

PBBS:  Go to the <M>essage menu and at the prompt, press Alt-S.

W0RLI-type MailBox:  At any system prompt, press Alt-R.

You will be shown a list of all unsent messages from your default

disk(ette) and you will be asked for the number (only) of the message

to be sent.

The rest is automatic (including the <S>ave function on PBBS system

after the message is complete).  You may now send another message by

pressing Alt-S or Alt-R again if desired.  Note that the sending of NTS

message traffic can be halted (aborted) at any time by pressing Alt-A.

When each MSGnnn.MSG file is sent, it will be renamed to MSGnnn.LOG on

your disk.

FILES USED BY PTP TERMINAL PROGRAM

PTP.EXE       The terminal emulator program.  This file can be loaded

with a .bat file, or directly from the DOS prompt by

typing "PTP".

PTP.MOD       This file is used to store your main menu data.  It

contains the personality you assigned when you

configured your PTP program.  You may have as many .MOD

files as your diskette space allows, and each can be a

new personality.

PTP.DEF       This file is the only one which requires a separate

editor to create.  There is a skeleton PTP.DEF file

provided with PTP which you may use to make your own

PTP.DEF.  30 additional F-KEYs are defined by this file

(Alt-Fn, Ctrl-Fn, and Shift-Fn).  This is an optional

file.  If you do not create a PTP.DEF, your F-keys will

be usable, but the extra 30 definitions will not exist.

In addition, there is information included in the PTP.DEF

file which defines screen color and routing information

for NTS traffic to be uploaded to WDCG PBBS or W0RLI-type

BBS.  See PTP.DEF under USER INSTRUCTIONS for details on

this file.

MSGnnn.MSG    These files are NTS messages which have been written and

and        are ready to be sent (.MSG) and logged messages which

MSGnnn.LOG    have already been sent (.LOG).  These files are generated

as a part of the NTS message process.

PTP.CKP       This file is automatically generated and contains

information about the last NTS message sent.  It is used

as the default text for beginning the next message.  Do

not alter or erase this file if you expect NTS message

writing to work properly.

CAPTURE.TXT   This file is the default name which is used for QSAVE and

may be renamed at a later time, or may be changed in the

main menu.  From either full-screen terminal mode or

split-screen terminal mode, you can press F6 and this

file (or other specified file) will be opened for

capture.  The file will continue to be updated with

everything received until F3 (Close file) or F6 is again

pressed.  NOTE: This file is appended each time it is

opened and the previous contents will remain.

PTP.SET       (Or other file name as desired.)  This file is used to

prime your TNC whenever you desire.  When used via Alt-L

from the communications screen, the contents of this file

will be sent to the TNC in cmd: mode.  It is useful for

setting, or restoring any set of parameters you desire.

A suggested use is to capture (via Qsave) the results of

"disp" and use an editor to change this file so that it

will restore your TNC in the event of a loss of data due

to any unforeseen event.

PTP.HLP       This file is required if you desire to use the Home key

for online help.  You may, at your option, use a text

editor to add your own key definitions to the lower half

of the .HLP screen.  NOTE: This file must remain the same

size -- 23 lines.

PTP.SCN       This is the first screen that you see when PTP is

started.

It contains information that must be presented to the

user of this program.  This file MUST be present and MUST

be unaltered in order for PTP to run.  It must be on the

default disk(ette) drive.

PTP.DOC       The documentation file (this file).  Print this file on

your system printer if you desire to have hard copy.

This file MUST be copied along with PTP if you desire to

share this program with another Amateur Radio operator.

PTP.WRK       This is a temporary file which is generated by PTP when

downloading.  If you break, or end the program, before

the download is completed, you might see this file on the

selected disk drive.  You may erase it, or rename it if

you desire.

MYMSGS.TXT    Or other name to your liking.  This is the file which you

cause to be created when you have CONNECT FILE in the

PTP.DEF list.  This file will contain only messages that

were received by you while you were connected and F6

(Qsave) was active.  All monitored traffic will be saved

in CAPTURE.TXT.

|     xxxxxxxx.xxx  Files of TNC commands you have created to be sent to the

|                   TNC using the Alt-L function.

|

|     xxxxxxxx.xxx  Files you have created to replace or add to the PTP.DEF

|                   file which gets installed at the beginning of program

|                   execution.

|

|     xxxxxxxx.xxx  Files which have been created with Qsave function (F-6)

|                   and have a name assigned by you on PTP's main menu

|                   screen.

When you are receiving an ASCII file via F3, the file will be

automatically closed when a BEL character is received.  The WDCG PBBS

always ends downloads with the BEL character.  Also, when PTP uploads a

file, it will put a BEL character as the last byte of the file.  This

can be very handy when you are transferring a file to a friend.  If he

is not using PTP, have him send you a Ctrl-G when he has finished

sending the file, and it will close your disk file, thus ending the

transfer.

SPLIT SCREEN CONSIDERATIONS

For the most part, split screen operation is as simple as typing on the

bottom of the screen and seeing what is received on the upper part of

the screen.

There are, however, a few things to be considered.  Because the split

screen code is an assembly code routine, the BIOS calls are not handled

through DOS and are not the same as the single screen mode.  The

characters from the keyboard are buffered in PTP until you press the

Enter key, reach the size of your buffer (which you established in .DEF

file).  Since you sometimes want to send a character to the TNC without

the CR, a function is provided to allow this.  If you press the Tab

key, the buffer will be sent to the TNC without a trailing CR.  This is

useful for entering command mode by pressing Ctrl-C and following this

with a Tab.  You will be unable to send the HT character in split

screen mode but this will be a very rare inconvenience.  If you desire

to send the Tab character it will be necessary to use the single screen

mode.

Also, since characters will not normally be sent to the TNC until Enter

is pressed, a special key is defined to allow you to exit transparent

mode from the split screen.  See the description of Alt-T for this

procedure.  Split-screen operation will seem a little unusual to you

until you get used to the benefits.  For instance, you can be typing to

someone and he might send a packet to you before you are finished.  You

should see this information appear on the upper part of your screen

even as you type.  Additionally, it is very nice to have all of your

text on the lower part of the screen and the received information above

the divider line.  It will not scroll off your screen nearly as fast

this way.

APPENDIX

XPACKET FLOW DIAGRAM

Example of an UNsuccessful transfer.

SENDER                                      RECEIVER

Ready to send file.

<------------------------------------ NAK (repeated every

10 seconds until

data transfer

begins)

filename ---------------------------------->

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

<------------------------------------ CAN

< Both stations abort transfer and return to terminal mode. >

Example of a successful transfer:

SENDER                                      RECEIVER

Ready to send file.

<------------------------------------ NAK (repeated every

10 seconds until

data transfer

begins)

filename ---------------------------------->

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

128 bytes data ---------------------------->

(listens)

109 bytes data ---------------------------->

(listens)

End Of File ------------------------------->

< -- Both stations go to terminal mode. -- >

NOTE: If nothing is received by the receiving station after 10 seconds,

another NAK is sent.  This is repeated every 10 seconds until the file

transfer is started.

NOTE: For details about the XPACKET protocol, see XPACKET.PRO, a brief

description of the protocol.

What's all this about "filename"?  Well, the XPACKET protocol sends the

file name in its first burst of data.  The receiving station can do

with this what he will.  The PTP program shows how this can be handled

by allowing the file to be saved under the name received, or under the

name specified by the operator.  This program gives the receiver a

chance to select a name under which he wants the received file to be

stored, or if you choose to let the sender provide the name, you simply

press Enter at the prompt for a file name.

The PTP program uses "hardware flow control" which means that it does

not honor XON or XOFF from the TNC.  Instead, it relies on the

asynchronous port hardware to control the sending of data to the TNC.

When the TNC buffer becomes full, the TNC will drop CTS (Clear To Send)

and the terminal will halt in its tracks until CTS is again raised.

This is how the TNC keeps from being overrun with data when the flow is

otherwise continuous.

When the receiver XPACKET station is waiting to receive the first

packet of data it will send NAK every 10 seconds until the first packet

is received.  The last block of data will not necessarily be 128 bytes,

but it will be followed by an EOF character to signify the end of the

transfer.  The receiving station will then save the file according to

the filespec received, or the one entered by the operator.

TNC SETUP

There are two categories for the TNC command setup.  They are:

|     o  Required    (for PTP to work as intended)

|     o  Recommended (Local area determined)

|

|     Required list

|     (TNC-1 and compatibles)

|     +---------------------------------------------------------------------+

|     | AWLEN 8          DELETE OFF          TXFLOW OFF         CR ON       |

|     | AX25 ON          FLOW ON             SENDPAC $0D        COMMAND $03 |

|     | BKONDEL ON       PACTIME EVERY 1     XFLOW ON                       |

|     | CMDTIME 1        PARITY 4            XMITOK ON                      |

|     | CONMODE CONVERS  START $11           STOP $13                       |

|     +---------------------------------------------------------------------+

|

|     Required list

|     (TNC-2 and compatibles)

|     +---------------------------------------------------------------------+

|     | AWLEN 8          DELETE OFF          PARITY 0           COMMAND $03 |

|     | AX25 ON          FLOW ON             SENDPAC $0D        8BITCONV ON |

|     | BKONDEL ON       PACTIME EVERY 1     XFLOW ON           TRFLOW OFF  |

|     | CMDTIME 1        CR ON               XMITOK ON          TXFLOW OFF  |

|     | CONMODE CONVERS  START $11           STOP $13                       |

|     +---------------------------------------------------------------------+

|

|     Required list

|     (PK-232)

|     +---------------------------------------------------------------------+

|     | AWLEN 8          DELETE OFF          PARITY 0           CR ON       |

|     | AX25 ON          FLOW ON             SENDPAC $0D        COMMAND $03 |

|     | BKONDEL ON       PACTIME EVERY 1     XFLOW ON           8BITCONV ON |

|     | CMDTIME 1        PARITY 4            XMITOK ON          TRFLOW OFF  |

|     | CONMODE CONVERS  START $11           STOP $13           TXFLOW OFF  |

|     +---------------------------------------------------------------------+

|

|     Recommended list

|     (TNC-1 and compatibles)

|     +---------------------------------------------------------------------+

|     | AUTOLF ON        BEACON EVERY 0      CONOK ON     LCOK ON           |

|     | CWID OFF         HBAUD 1200          LFADD OFF    TXDELAY 6         |

|     | MCON OFF         RETRY 10            SCREENL 0                      |

|     +---------------------------------------------------------------------+

|

|     Recommended list

|     (TNC-2 and compatibles)

|     +---------------------------------------------------------------------+

|     | AUTOLF ON        BEACON EVERY 0      CONOK ON     LCOK ON           |

|     | CWID OFF         TXDELAY 24          LFADD OFF    NEWMODE ON        |

|     | MCON OFF         RETRY 10            SCREENL 0                      |

|     +---------------------------------------------------------------------+

|

|     Recommended list

|     (PK-232)

|     +---------------------------------------------------------------------+

|     | AUTOLF ON        BEACON EVERY 0      CONOK ON     LCOK ON           |

|     | CWID OFF         HBAUD 1200          LFADD OFF    NEWMODE ON        |

|     | MCON OFF         RETRY 10            ACRDISP 0    TXDELAY 24        |

|     +---------------------------------------------------------------------+

NET MASTER MODE (TNC-2 and compatibles)

This feature will allow a multiconnect QSO to be conducted with one

station acting as the Net Master and relaying all packets received to

each of the connected stations.

The Net Master station will have two parameters included in the PTP.DEF

file which will specify his identifier (i.e. call sign), and the ASCII

value of his selection for the stream character in his TNC-2 or

compatible.  If these parameters are not specified, the defaults will

be NULL for MYCALL, and the vertical bar (|) for the STREAM CHAR.

This mode is active only in converse mode.  If your TNC is in command

mode when NM mode is enabled, it will be placed in converse mode as a

result.  You will not be able to enter command mode while Net Master is

enabled.  More on that later.

The Net Master operator will connect to the other stations which are to

be a part of the "net," or roundtable QSO.  After all stations are

connected to the Net Master, and asked to stand by, the operator of the

NM station will press Alt-N.  After about ten seconds, the letters NM

will begin to flash on the right side of the stripe which divides the

upper and lower parts of the split screen.  (Net Master works only in

split screen mode.)

Hereafter, all received packets will be re-transmitted to each of the

other stations in the "net" preceded with an identifier which tells

the others from whom this packet was sent (i.e.  ":N4PY:Good afternoon,

fellows.  K") Also, everything which originates from the NM operator

will also be sent to each of the stations in the net.

This mode will be somewhat slower than just an UNPROTO roundtable since

the Net Master will have to repeat the received packet to each station

involved, but it should be more accurate since collision detection will

be accomplished as when only two stations are connected.

Please note that while NM mode is in effect, the NM operator cannot

enter command mode on his TNC.  When the Ctrl-C is attempted, it will

be replaced with an asterisk (*) on his screen.  If a change must be

made to the TNC operating parameters, the NM operator must first

disable NM mode, then enter command mode on the TNC, and re-enable NM

mode.

Anytime an additional connect is made, the NM operator will have to re-

enable Net Master mode to cause the new station to be included in the

net.

This mode will take a little practice to get used to it, but it has

worked very well during our tests in Wake County, NC.

The net "member" station operators do not have to use the TNC-2.  They

should see nothing different from a normal connection, except for the

:CALL SIGN:  preceding each packet of text which appears on their

screens.

If you are using a TNC-2 or compatible, be sure you don't have newmode

set up to cause cmd: mode when someone disconnects.  This will leave

you in a strange condition.

Net Master is terminated by the NM operator by pressing Alt-F to

disable the mode.

FINAL COMMENTS FROM WDCG

The PCjr cannot write directly to a physical disk file during file

reception.  If you have enough RAM installed to define a ramdisk, you

can use this as the target drive and should have no problems.

If you are using an IBM PCjr without the internal modem, you will need

to run COMJRNIM.COM program which is included on this diskette.  The

PCjr's RS232 port is normally referred to as COM1 when the internal

modem is not installed, and as COM2 when the internal modem is there.

The hardware addresses are not changed.  Be sure to select COM2 from

the main menu screen if you use COMJRNIM.COM.

We hope you find the Packet Terminal Program as useful and as much fun

as we have.  Please feel free to pass this program along to other

Amateur Radio operators and Packeteers, but please DO NOT attempt to

sell this program, or to use it in any commercial way whatsoever.  This

program is intended for the private use of Radio Amateurs and your

cooperation is greatly appreciated in keeping it just as the author

intended.  If you give a copy of PTP to someone please be sure to

include this document file also.

The author of PTP assumes no responsibility for the use of this program

and no guarantees are implied or intended.

Comments and suggestions are welcome, especially about the binary

transfer protocols.  Send comments to:

+----------------------------+        +----------------------------+

| Carl Moreschi, N4PY        |        | Ed Stephenson, AB4S        |

| Rte 3, Box 260             |        | 700 Madison Ave.           |

| Franklinton, NC  27525     |  <or>  | Cary, NC  27511            |

| 919-494-2363               |        | 919-467-6832               |

| N4PY @ WA4LPD              |        | AB4S @ WA4LPD              |

+----------------------------+        +----------------------------+

Additional hints from PTP users

Producing hard copy:

It has been suggested that some users like to have the printer echo

everything which is received to serve as a "log."  There are three ways

to accomplish this.

1. If you are using single-screen mode (not split screen), you can us

the DOS function Ctrl-PrtSc to cause everything going to the screen

to also go to the printer.  This method will work only in the

single-screen mode.

2. If you are using split-screen mode, you can cause everything

received to go to the printer by setting the QSave File Name (main

menu item 6) to LPT1: or LPT2: (for line printer 1 or 2).

When you are in communicate mode, turn QSave (F6) on.  When you no

longer wish to print received data, press F6 again to disable QSave.

|     3. If you are using a PK-232 and you enjoy printing FAX pictures, there

|        is no need to disconnect your printer from the computer and

|        reconnect it to the cable supplied with the PK-232 for printing.

|        Use PRCON OFF, PROUT OFF, and PRFAX ON.  Use the new feature, Alt-G,

|        to direct everything received out to your printer on the computer

|        port.  You should be able to, optionally, save the results of the

|        FAX receive by using Qsave feature with the main menu selection of a

|        file name for Qsave.  Be sure you are using 8-bit mode with no

|        parity from the TNC to the terminal (computer).  To stop the

|        printing, press Alt-G again; it will toggle from ON to OFF state.

*** End of document ***