HERC.BGI - Supplemental Documentation

HERC.BGI is a "unified" driver that support both the Hercules

monochrome graphics cards (in 2 colors) and Hercules InColor Card

(in 16 colors).  This B.G.I. driver is appropriate for use with

Turbo C 2.0 and Turbo Pascal 5.0.

Since the present BGI kernal does not recognize a InColor card

when it tries to autodetect the attached video card in the system,

this HERC.BGI has built the DETECT function built into the driver

itself.  If an InColor card is present in the system, the driver will

detect it during its installation and act accorrdingly - that is,

operate in 16 colors.

The InColor card will share the same GraphDriver and GraphMode

values with the monochrome graphics cards.  That is,

GraphDriver = HERCMONO, and GraphMode = HERCMONOHI.

To autodetect an InColor card, the following sequence of code

could be used: (in C syntax)

graphdriver = DETECT;

initgraph(&graphdriver, &graphmode, "");

/* ...

graphics error processing

*/

if (graphdriver == HERCMONO) {

if (getmaxcolor() == 15) {

... /* InColor card */

}

else {

... /* monochrome graphics cards */

}

}

....

NOTE - The Autodetect function of Borland's BGI will always want to load the

driver of the device that it identifies as the highest performance

adapter in the system.  This means that in some dual monitor

configurations the DETECT function may force the BGI driver to

what it identifies as the higher performance graphics mode.  For

example, in systems that include both a Hercules InColor Card and a

Hercules VGA Card, the BGI Autodetect function will load the EGAVGA.BGI

driver even though you may want to execute the program on the InColor

Card.  In such a case, you must explicitly name the HERCMONO as the

graphdriver rather than use the DETECT function.