PRELIMINARY

PROGRAM SPECIFICATION

STORAGE MANAGEMENT EVALUATOR

(EVAL)

SPECIFICATION NO. XX-XXXXXX

FEBRUARY 4,1988

1.0 SCOPE

This   document   describes  the  functional  characteristics  of  the	Storage

Management  Evaluator  (EVAL).	EVAL  is  a diagnostic program designed for use

with  the  IBM	AT   to  test  and  evaluate hard and floppy storage management

subsystems.  It  is  especially  suited  for  error  rate testing. The Features

section  describes  the operation and features of the program, including how to

invoke	EVAL,  diagnostic screen definition command structure error methodology

and  miscellaneous  features.  The  Commands section describes the command set,

giving	a  detailed explanation of each command. Appendix A and B describe each

status code, task file, and associated error reporting.

2.0 FEATURES

2.1 Invoking EVAL

EVAL  is  invoked  at  the PC-DOS command line,  or batch file command line, as

follows:

c>EVAL [cmd-file]

The  optional  cmd-file  parameter  specifies  a file from which EVAL will read

commands  until  it encounters an "end" command. All commands described in this

specification  may  be	used  in   command  files  except "edit", "run", "def",

"duw", and "dur".

2.2 Floppy Diagnostic Screen

Figure	1 shows the main floppy diagnostic screen. The screen is divided into 4

windows,  a  command  line,  and  a  message  line.  The  command  line will be

described  in  3.0,  and  the  message	line will be referred to throughout the

specification. The following is a brief description of the 4 windows.

2.2.1 Parameter Window

The  upper  left  window  of the screen is the parameter window. The parameters

shown  will  be  used  for  the  next disk operation. The parameters are: drive

type,  drive  number,  cylinder  number,  head number, sector number, and block

size.  The  parameters are updated dynamically during command execution, if the

operation changes a parameter (see individual commands in section 3).

2.2.2 Status Window

The  status  window  (lower  left)  shows  the	latest	status	returned  after

execution  of  a  command which accesses the controller. The status information

includes  the  status  code,  status  registers  and cylinder, head, and sector

numbers  returned by the controller. Appendix B gives a detailed explanation of

status codes and associated errors.

2.2.3 Error Count Window

The  error  count  window  (upper  right)  shows total error counts, as well as

individual  error  counts  for 5 groups of errors. The counts are kept for both

hard  and soft errors. See 2.5.1 for an explanation of hard and soft errors and

appendix B for a detailed explanation of error reporting.

2.2.4  Error Report Window

The  error  report  window  (lower  right)  shows  the last 5 errors which have

occurred  since  the  diagnostic  began,  or since the last "zero" command was.

issued	Appendix B explains each error in full detail.

2.3 Hard  Diagnostic Screen

Figure	2  shows  the main hard diagnostic screen. The screen is divided into 4

windows,  a  command  line,  and  a  message  line.  The  command  line will be

described  in  3.0,  and  the  message	line will be referred to throughout the

specification. The following is a brief description of the 4 windows.

2.3.1 Parameter Window

The  upper  left  window  of the screen is the parameter window. The parameters

shown  will  be  used  for  the  next disk operation. The parameters are: drive

number,  cylinder  number,  head  number,  sector  number,  and block size. The

parameters  are  updated dynamically during command execution, if the operation

changes a parameter (see individual commands in section 3).

2.3.2 Disk Type Window

The  disk  type  window  (lower left) shows the disk drive boundary parameters.

The  type  information	includes  the maximum cylinders, maximum heads, maximum

sectors,    precomp  cylinder  number,	retry  flag,  step rate, and interleave

value.

2.3.3 Task File Window

The  task  file  window  (upper  right)  shows	the task file returned from the

controller  after  the	command  was  completed.  Appendix  A  gives a detailed

explanation of the task file and associated errors.

2.3.4  Error Report Window

The  error  report  window  (lower  right)  shows  the last 5 errors which have

occurred  since  the  diagnostic  began,  or  since the last "zero" command was

issued. Appendix A explains each error in full detail.

2.4 Command Structure

In  the  interactive  mode (commands being input from the keyboard), the cursor

will  reside  on  the command line, which begins with the CMD> prompt. Commands

are  typed  in lower or upper case. A carriage return  <CR>  ends command entry

and  initiates	command execution. Multiple commands may be entered on the same

line,  separated  by   commas. Commands are separated from parameters by one or

more of the following:	space  tab  =  /  :  (	)

2.4.1 Command Types

From a command entry standpoint, there are 6 types of commands:

type 1 	  cmd

type 2 	  cmd|<choice>

type 3 	  cmd|<param>

type 4 	  cmd|<string>

type 5 	  cmd|<test>

type 6 	  cmd|<special>

The  six  types  of  commands may be freely mixed on the same line. See section

3.0 for complete listing.

2.4.2 Command Line Repeat

A  command  line  may  be  repeated  up to 32,762 times, simply by entering the

desired  command  (or  a  string  of  commands)  followed by a #, followed by a

number.   After  the <CR> is entered, the commands on the line will be executed

repeatedly,  just  as  if  the	line were entered the number of times specified

behind	the  #.  In  addition, a repeat count is shown on the message line, and

decrements down with each command line execution.

If  the  Escape  key  is  hit  during  a  line	repeat,  the  repeat  count  is

automatically zeroed.

Example:

CMD>r,incs #30

will perform a read, increment the sector count operation 30 times.

2.5 Error Methodology

2.5.1 Hard and Soft Errors

Errors	are separated into two categories - hard errors and soft errors. A hard

error  is defined as an error which is not recoverable after a specified number

of  retries  (see "rc"). A soft error is defined as an error which is recovered

during	retries.  In  other  words, a hard error persists through the specified

number of retries; a soft error does not.

A  hard  error	is  distinguished  in the error report window, and in the error

file by a 'H' prefix and a soft error is prefixed by a 'S'.

2.5.2 Error logging

In  addition  to  showing  the	last 5 errors in the error reporting window, an

error  log  file  can  be  kept  on disk for later review. Upon invocation, all

errors	are  logged  to  a specified file (see "errlog"). The file may later be

dumped to the screen interactively (see "def").

2.6  Buffer Screen

The  buffer  screen  can  be reached from the Floppy or Hard screen by giving a

dur  (dump  read buffer) or duw (dump write buffer). The buffer screen displays

one  sector  of  data  at a time that can be edited. The arrow keys are used to

select the byte to edit and the page keys are used to choose the sector.

2.7 Help Screen

The  help screen can be reached from the floppy or hard screen by giving a help

command.  The  help  screen  displays all of the commands in alphabetical order

with a short description after it.

3.0 COMMANDS

The  command  set in EVAL closely resembles that of VHD. Bellow one will find a

list  of  all  the  commands  by  type	and alphabetic. Before getting into the

commands we should look at the formats of the commands.

type 1 	  cmd

type 2 	  cmd|<choice>

type 3 	  cmd|<param>

type 4 	  cmd|<string>

type 5 	  cmd|<test>

type 6 	  cmd|<special>

cmd	     this is the command name

|	     one or more of the following delimiters:

space = : / ( ) tab

some examples :

h 3

h=3

h:3

h = 3

h/3	are all valid commands to set h to 3

h3	is not valid

<choice>    one of the following

true	    t

false     f

yes	    y

no	    n

<param>     a parameter value of one of the following forms

var1	    the value of var1 is used

.	    .

.	    .

.	    .

var8	    the value of var8 is used

0  ... 32762

0h ... 7FFFh

rstat     the value in the status register

rerr	    the value in the error register

rsecc     the value in the sector count register

rsecn     the value in the sector number register

rcyll     the value in the cylinder low register

rcylh     the value in the cylinder high register

rsdh	    the value in the SDH register

rdata     the value in the data register

rastat    the value in the alternate status register

rdin	    the value in the digital input register

<string>    a string of text

<test>      an expression to test that might look like this:

(var1 <= 4)

NOTE: there must be a space between the operands and

the operator i.e.  (var1 <=4) is not valid

operators can be of the form:

==	    equal

!=	    not equal

<=	    less than or equal

>=	    greater than or equal

<	    less than

>	    greater than

<special>   can be any thing, check individual command description

command type 	  command example

1		    r

2		    upd yes

3		    cyl 614

3		    s=var1

3		    c = 1DFh

4		    print(hello out there)

5		    while(var3 != var7)   this type can only be

used in a batch file

6		    etc 1e10

Commands  are  entered at the EVAL command prompt "CMD>". More than one command

can  be  entered  on  a line but each command must be separated by a coma. EVAL

reads and executes each command starting from the left most  command i.e.

CMD> h 0,s 3,c 26,sk

EVAL  allows  the  user  to  repeat  a string of commands a specified number of

times. In the example bellow 100 sectors will be read.

CMD> r,incs #100

ALPHABETICAL LISTING OF COMMANDS

alt

alt|<choice>

Enable alternate sector during format

hard drive only

b

b|<param>

Set block size. Enter the number of sectors to read/write/verify

hard/floppy

bct

bct|<special>

Set bit terminate count

<special> is scientific notation    bct=3e9

stop execution after 300,000,000 bits have been read/write/verify

floppy drive only

c

c|<param>

set cylinder value for next operation

hard/floppy

cmd

cmd|<param>

write to command register of task file

hard drive only

cmp

cmp

compare read buffer to write buffer and flag all errors

hard/floppy

copy

copy

move read buffer to write buffer

hard/floppy

cyl

cyl|<param>

set total number of cylinders on drive

hard drive only

d

d|<param>

set drive number for next operations

hard drive	  0 <= param <= 1

floppy drive	  0 <= param <= 3

def

def

dump error file. change screen to display error log

hard/floppy

diag

diag

diagnostic command. send diagnostic command to controller

hard drive only

dt

dt|<param>

set drive type

0	  1.2 Meg floppy drive in a 1.2 Meg drive

1	  360 K   floppy drive in a 1.2 Meg drive

2	  360 K   floppy drive in a 360 K   drive

floppy drive only

dur

dur

dump read buffer. display contents of read buffer

hard/floppy

duw

duw

dump write buffer. display contents of write buffer

hard/floppy

ecmd

ecmd|<param>

write to 1005 esdi task file command register

1005 esdi hard drive controller only

ecc

ecc|<param>

set number of ECC bytes

hard drive only

ecct	ecct

test ECC and return the number of bits corrected

hard drive only

ecmd

ecmd|<param>

send <param> to esdi command reg

1005 only

edit

edit

enter a batch file from eval.

can not be used in a batch file.

hard/floppy

eelse

eelse

marks end of else statement in batch file

used in batch file only

eif

eif

marks end of if statement in batch file

used only in batch file

else

else

marks start of else statement in batch file. This block will

be executed only if the previous if command was false

used only in batch file

end

end

marks end of a batch file also used to exit the edit screen

used only in a batch file

er

er|<choice>

enable retries. will enable/disable retries on a hard drive

controller based on <choice>

true   enable retries

false  disable retries

errlog

errlog|<string>

enter name of error file to use

file will be opened in the append mode and a header is placed at

the top of the file depending if opened from floppy or hard window

esdi

esdi|<choice>

opens/closes a window for the 1005 esdi controller  esdi task file

etc

etc|<special>

error terminate count

<special> is scientific notation    ect=3e9

execution will stop after 10,000,000,000 errors have been found

ewhile

ewhile

marks end of while statement in batch file

used only in batch file

exfr

exfr|<param>

write to 1005 esdi transfer register

used only wilt 1005 esdi controller

fbs

fbs|<param>

format <sector number> as bad

hard drive only

fbt

fbt

format all of track as bad

hard drive only

fd

fd

format all of drive

floppy

floppy

goto floppy drive mode

hard drive only

ft

ft

format track

h

h|<param>

set head number

hard/floppy

hard

hard

goto hard drive mode

floppy drive only

heads

heads|<param>

set total number of heads

hard drive only

help

help

goto help screen

i

i|<param>

set interleave for next format command

hard/floppy

idc

idc

send idc command to controller with parameters of drive

hard drive only

if

if|<test>

marks start of if block. The if block will be executed

only when the condition of <test> is true.

used only in batch files

incc

incc

increment cylinder count

hard/floppy

inch

inch

increment head count

hard/floppy

incs

incs

increment sector count

hard/floppy

int

int|<choice>

enable / disable interrupts from controller

hard drive only

lrun

lrun|<string>

load and run a batch file <file name>

hard/floppy

ltt

ltt

write/read/compare all of disk

hard/floppy

multi

multi|<param>

set multi block transfer mode with <block size>

only on hard controllers that support this command

nop

nop

no operation does nothing

o

o|<param>

set the offset into the write buffer used with v command

pat

pat|<special>

fill write buffer with pattern  pat=incr,pat 6db

<special>

incr 	  fill write buffer with incriminating data

rand 	  fill write buffer with random numbers

this command will take about 3 mins to complete

ilt		  fill write with interleave pattern

X...X	  fill write buffer with hex pattern X...X

i.e. pat 6db will make a pattern of 6db6db6db...

pause

pause

wait for key press. Will prompt user to press any key

an escape key will stop execution of batch file or repeat line

hard/floppy

precomp

precomp|<param>

set precomp cylinder number

hard drive only

print

print|<string>

print message to lower left corner of screen	   print(HI MOM)

hard/floppy

quit

quit

exit EVAL

hard/floppy

r

r

read command. read number of sectors described by block size

hard/floppy

ra

ra

read all of drive

hard/floppy

rb

rb

read buffer. read buffer of controller

hard drive only

rc

rc|<param>

retry count. set number of retries before calling an error hard

floppy drive only

rcmd

rcmd|<param>

write <param> to command register

hard drive only

rcylh

rcylh|<param>

write <param> to cylinder high register

hard drive only

rcyll

rcyll|<param>

write <param> to cylinder low register

hard drive only

rdata

rdata|<param>

write <param> to data register

hard drive only

recal

recal

recalibrate drive. move heads to track zero

hard drive only

reset

reset

reset drive. does  a soft reset to drive.

in hard drive mode must be followed by 2 recals

hard/floppy

rfixd

rfixd|<param>

write <param> to fix disk register

hard drive only

rl

rl

read long command. Read all of block including ECC bytes

hard drive only

rm

rm

read mulit block

only on hard controller that support multi block

rsdh

rsdh|<param>

write <param> to SDH register

hard drive only

rsecc

rsecc|<param>

write <param> to sector count register

hard drive only

rsecn

rsecc|<param>

write <param> to sector number register

hard drive only

rtf

rtf

read task file. read task file from controller

hard drive only

run

run

run batch file. start execution of batch file

hard/floppy

rv

rv

read verify command. read block of sectors but do not transfer data

hard/floppy

rva

rva

read verify all. do a read verify on all of disk

hard/floppy

rwpre

rwpre|<param>

write <param> to write precomp register

hard drive only

s

s|<param>

set sector number for next command

hard/floppy

sectors

sectors|<param>

set total number of sectors per track

hard drive only

sk

sk

seek command. seek to track specified

hard drive only

skew

skew|<param>

set skew for next format

hard drive only

skt

skt

do a crescendo seek test

hard/floppy

show

show|<param>

display value to screen. will display value in lower right

corner of screen

hard/floppy

st	 st

start timer

this will zero the real time stop watch

ts	 time stamp

this will print the real time stop watch

to the screen and to the error file

upd

upd|<choice>

update command. enable/disable update of screen after every command

hard/floppy

v

v|<param>

set a value in write buffer at offset defined by o

v will also increment the offset value

var1...var8

var1...var8|<param>

set value of var1...var8

var1 = 2	    set var1 to 2

var1 += 3	    var1 = var1+3

var1 -= 1	    var1 = var1 - 1

var1 &= 4	    var1 AND 4

var1 |= 4	    var1 OR 4

note : when using += ... |= there must be spaces seperating the operator

hard/floppy

w

w

write command. write number of sectors described by block size

hard/floppy

wa

wa

write all. send write buffer to all of drive

hard/floppy

wb

wb

write buffer. send write buffer to controller

hard drive only

while

while|<test>

mark start of while block The while block will be executed

as long as the condition of <test> is true.

used only in batch files

wid

wid

write the cylinder/head/sector number to the data sector on drive

hard drive only

wl

wl

write long command. write the write buffer to the drive

including ECC bytes

hard drive only

wm

wm

write multi block

only on hard controller that support multi block

wtf

wtf

write task file

send task file to controller

hard drive only

zero

zero

clear error counters

hard/floppy

4.0 Batch Files

Commands  can  be  executed  from  a  batch  file  in one of two ways. First by

entering  the  name  of the batch file after EVAL at the DOS command line or by

using  the  edit  command.  In	either case the format of the batch file is the

same.  In  a  batch file all the command from the HARD or FLOPPY windows can be

used  except  : "edit", "run", "def", "duw", and "dur". While in the batch file

the  "if",  "else",  and  "while"  commands  can be used to control the flow of

commands.  A  batch  file  must  end  with  the "end" command or it will not be

loaded.  To execute a file one must type the "run" command. Below is an example

of a batch file:

Appendix    (A)

HARD DRIVE TASK FILE

Here  is a brief out line of the Task File for more information check the speck

sheet for the controller you are using.

D7					  D0

�����������������������������������������������Ŀ

Error register	     � BB  � ECC �  0  � ID  �	0  � AC  � TK  � DM  �

�����������������������������������������������Ĵ

Sector count	     �	     number of sectors	to transfer	     �

�����������������������������������������������Ĵ

Sector number	     �		       sector number		     �

�����������������������������������������������Ĵ

cylinder low	     �		 cylinder number (low byte)	     �

�����������������������������������������������Ĵ

cylinder high	     �	0  �  0  �  0  �  0  �	0  �  0  � cyl. MSB  �

�����������������������������������������������Ĵ

SDH		     �	1  � sec. size � DS  � HS3 � HS2 � HS1 � HS0 �

�����������������������������������������������Ĵ

Status reg.	     � BSY � RDY � WF  � SC  � DRQ � CRD � IDX � ERR �

�������������������������������������������������

Error reg.

BB	     Bad Block

ECC	     Bad ECC bytes

ID	     ID not found

AC	     Aborted Command

TK	     Track zero error

DM	     Data Address Mark not found

Status reg.

BSY	     Busy flag

RDY	     Drive ready

WF	     Write Fault

SC	     Seek Complete

DRQ	     Data Transfer Request

CRD	     Corrected Data Flag

IDX	     Index pulse

ERR	     Error Flag

Sector count

The number of sectors to transfer

Sector number

The sector to start transfer at

Appendix    (A)

Cylinder low/high

The cylinder to start transfer at

SDH

sec.size    sector size

00    256 bytes

01    512 bytes

10   1024 bytes

11    128 bytes

DS	     drive select

HS3..HS1    head select

HARD DRIVE ERRORS

The format of the error messages from the hard drive looks like this

Err#  CMD# SR ER   C  H  S  CMD  MSG

Where

Err#	     is the error count

Cmd#	     is the command count

SR	     the value of the status register

ER	     the value of the error register

C	     the cylinder that the error occurred on

S	     the sector that the error occurred on

H	     the head that the error occurred on

CMD	     the command that was being executed

MSG	     a description of the error

Example:

Err#  CMD# SR ER   C  S  H  CMD  MSG

23	 45 01 80   7 1A  3   r    Bad Block mark

Appendix     (B)

Floppy Status Codes:

Status Code		Condition

0	Successful operation

2	Bad address mark.   Either a data AM or IDAM  was  bad.

3	Diskette is write protected.

4	Record not found.

6	The media has been changed since the last operation.

8	DMA  overrun.	 An  illegal  DMA  condition  has

occurred.     The  diagnostic  is  in  an  unknown

condition.  The system should be re-booted.

9	DMA boundary exceeded.	 The system should be  re-

booted.

10	Bad CRC.  Either ID or data CRC did not match its

associated field.

20	Bad UPD 765 chip.

40	Bad seek.

80	Time-out error.  Response from the floppy control-

ler was not received before the system timed out.

Can occur if the drive door is open.

Appendix	  (B)

FLOPPY ERROR MESSAGES

Messages not related to media records:

"Write protected" -

Results from a status code "3".

"Media change indicated" -

Results from a status code "6".

"DMA overrun!!!" -

Results from a status code "8".

"DMA boundary exceeded!!!" -

Results from a status code "9".

"Bad UPD 765 chip" -

Results from a status code "20".

Messages related to media records:

Message is composed of three parts -

1) Type of operation - Read, Write, Reset, Format, Verify.

2) Location -  "c:n  h:n  s:n"

3) Type of error (in parentheses) -

"End of track"

"DAM not found"

"IDAM not found"

"Data CRC error"

"ID CRC error"

"Undefined"

Appendix    (B)

Examples:

"Read error  c:0 h:1 s:10 (DAM not found)"

"Write error c:10 h:0 s:1 (ID CRC error)"

COMMAND LIST BY FUNCTION

Setup Commands

alt	    |<choice>	       enable alternate sectors T/F

b	    |<param>	       set block size

bct	    |<special>	       set bit terminate count	i.e.  btc=1e30

cyl	    |<param>	       set total number of cylinders

dt			       drive type { floppy }

ecc	    |<param>	       set number of ecc bytes

er	    |<param>	       retries enable T/F

errlog	    |<string>	       enter name of error file to use

esdi	    |<choice>	       1005 esdi mode

etc	    |<special>	       error terminate count  i.e.   etc=1e30

floppy	    |<choice>	       goto floppy drive mode

hard	    |<choice>	       goto hard drive mode

heads	    |<param>	       set total number of heads

i	    |<param>	       set interleave

idc			       send idc command

multi	    |<param>	       set multi block size

reset			       reset drive parameters

rc	    |<param>	       retry count { floppy }

sectors     |<param>	       set total number of sectors per track

skew	    |<param>	       set skew size

precomp     |<param>	       set precomp cylinder number

zero			       clear error counters

Transfer Commands

cmd	    |<param>	       write to command register

copy			       move read buffer to write buffer

dur			       dump read buffer

duw			       dump write buffer

ecmd	    |<param>	       write to 1005 esdi command register

exfer	    |<param>	       write to 1005 esdi transfer register

fbs	    |<param>	       format bad sector  <sector number>

fbt			       format bad track

fd			       format drive

ft			       format track

ltt			       write/read/compare all of disk

pat	    |<special>	       fill write buffer with pattern pat=incr,pat=6db

o	    |<param>	       offset into write buffer (1 sector = 520)

r			       read command

ra			       read all

rb			       read buffer

rl			       read long command

rm			       read multi block

rv			       read verify command

rva			       all

v	    |<param>	       value to put into write buffer at offset

w			       write command

wa			       write all

wb			       write buffer

wid			       write cylinder/head/sector number to each setor

wl			       write long command

wm			       write multi block

wtf			       write task file

Batch File Commands

edit			       enter batch file from eval

eelese			       end of else statement { batch file }

eif			       end of if statement { batch file }

else			       else statement { batch file }

end			       end of batch file

ewhile			       end of while statement { batch file }

if	    |<test>	       if statement { batch file }

lrun	    |<string>	       load and run <file name>

nop			       no operation

run			       run batch file

while	    |<test>	       while statement { batch file }

Head Movement Commends

c	    |<param>	       set cylinder count

d	    |<param>	       set drive number

h	    |<param>	       set head count

incc			       increment cylinder count

inch			       increment head count

incs			       increment sector count

recal			       recalibrate drive

s	    |<param>	       set sector count

sk			       seek command

skt			       seek test

Hard Drive Register commands

rcmd	    |<param>	       write <param> to command registor

rcylh	    |<param>	       write <param> to cylinder high registor

rcyll	    |<param>	       write <param> to cylinder low registor

rdata	    |<param>	       write <param> to data registor

rfixd	    |<param>	       write <param> to fix disk registor

rsdh	    |<param>	       write <param> to SDH registor

rsecc	    |<param>	       write <param> to sector count registor

rsecn	    |<param>	       write <param> to sector number registor

rwpre	    |<param>	       write <param> to write precomp registor

Other

cmp			       compare buffers

def			       dump error file

diag			       diagnostic command

ecct			       ecc test

help			       goto help screen

pause			       wait for key press

print	    |<string>	       print message to screen

quit			       exit eval

rtf			       read task file { hard }

show	    |<param>	       display value to screen

st			       start timer

ts			       time stamp

ta			       test all

upd	    |<choice>	       update command T/F

var1...var8 |<param>	       set value of var1...var8

ALPHABETICAL LISTING OF COMMANDS

alt	    |<choice>	       enable alternate sectors T/F

b	    |<param>	       set block size

bct	    |<special>	       set bit terminate count	   bct=3e9

c	    |<param>	       set cylinder count

cmd	    |<param>	       write to command register

copy			       move read buffer to write buffer

cyl	    |<param>	       set total number of cylinders

d	    |<param>	       set drive number

def			       dump error file

diag			       diagnostic command

dt	    |<param>	       drive type { floppy }

dur			       dump read buffer

duw			       dump write buffer

ecmd	    |<param>	       write to 1005 esdi command register

edit			       enter batch file from eval

eelese			       end of else statement { batch file }

eif			       end of if statement { batch file }

else			       else statement { batch file }

end			       end of batch file

er	    |<choice>	       retries enable T/F

errlog	    |<string>	       enter name of error file to use

esdi	    |<choice>	       1005 esdi mode

etc	    |<special>	       error terminate count	etc=1e10

ewhile			       end of while statement { batch file }

exfer	    |<param>	       write to 1005 esdi transfer register

fd			       format drive

fds	    |<param>	       format drive with skew

floppy	    |<choice>	       goto floppy drive mode

ft			       format track

fts	    |<param>	       format track with skew

h	    |<param>	       set head count

hard	    |<choice>	       goto hard drive mode

heads	    |<param>	       set total number of heads

help			       goto help screen

i	    |<param>	       set interleave

idc			       send idc command

if	    |<test>	       if statement { batch file }

incc			       increment cylinder count

inch			       increment head count

incs			       increment sector count

o	    |<param>	       offset into write buffer (1 sector = 520)

pat	    |<special>	       fill write buffer with pattern  pat=incr,pat 6db

pause			       wait for key press

precomp     |<param>	       set precomp cylinder number

print	    |<string>	       print message to screen

r			       read command

ra			       read all

rb			       read buffer

rc	    |<param>	       retry count { floppy }

recal			       recalibrate drive

reset			       reset drive parameters

rl			       read long command

rtf			       read task file { hard }

run				run batch file

rv			       read verify command

rva			       all

s	    |<param>	       set sector count

sectors     |<param>	       set total number of sectors per track

sk			       seek command

show	    |<param>	       display value to screen

st			       start timer

ts			       time stamp

upd	    |<choice>	       update command T/F

v	    |<param>	       value to put into write buffer at offset

var1...var8 |<param>	       set value of var1...var8

w			       write command

wa			       write all

wb			       write buffer

while	    |<test>	       while statement { batch file }

wl			       write long command

wtf			       write task file

zero			       clear error counters