---------------------------------------------------------------------------

KBRATE -- Set PC-AT Keyboard Auto-repeat Rate and Initial Delay

---------------------------------------------------------------------------

Version 1.01, 31 January l985

KBRATE.COM is a program which adjusts the auto-repeat rate and initial

delay for the keyboard of an IBM PC-AT.  It is invoked by the following

command:

C>KBRATE  xy

"x" is one of the letters A through Z.  It gives the repetition rate in

characters per second, according to the following table:

A  30.0                 N   9.2

B  26.7                 O   8.6

C  24.0                 P   8.0

D  21.8                 Q   7.5

E  20.0                 R   6.7

F  18.5                 S   6.0

G  17.1                 T   5.5

H  16.0                 U   5.0

I  15.0                 V   4.6

J  13.3                 W   4.3

K  12.0                 X   4.0

L  10.9                 Y   3.7

M  10.0                 Z   3.3

"y" is one of the numerals 1 through 4.  It gives the time delay between a

key's first depression and the start of its repetition, in units of a

quarter of a second.

There may multiple spaces preceding "xy" but there must be no space between

"x" and "y".

This program runs ONLY on an IBM PC-AT (although there is no internal test

for this).  It sends commands to the keyboard's internal microprocessor to

set the rate and delay.  The default values which are established after a

system reset are a rate of 10 characters per second and a delay of 500

milliseconds, which corresponds to an "xy" value of "M2".  The program does

not remain resident nor does it alter any data in the BIOS working RAM

storage, so it should be entirely transparent to programs which are

intimate with the keyboard or which replace the normal BIOS keyboard

interrupt handler.  It can be invoked from the AUTOEXEC.BAT file to set the

desired rate whenever DOS is started or at any time thereafter.

KBRATE.COM is copyright (c) 1985 by Robert F. Morse.  Permission is hereby

given for private non-commercial use only.  Kudos and brickbats should be

directed to:

Robert F. Morse

17 Bowdoin Street

Cambridge, MA  02138

(617) 864-4250

DTN 223-6720

MLO5-2/B6

LEWS2::MORSE