#O#P#A

#2#A

LET'S BUILD A COMPILER!

By

Jack W. Crenshaw, Ph.D.

16 April 1989

Part IX: A TOP VIEW

#P#A

#2#A

*****************************************************************

*                                                               *

*                        COPYRIGHT NOTICE                       *

*                                                               *

*   Copyright (C) 1989 Jack W. Crenshaw. All rights reserved.   *

*                                                               *

*****************************************************************

INTRODUCTION

In  the  previous  installments,  we  have  learned  many of  the

techniques required to  build  a full-blown compiler.  We've done

both  assignment   statements   (with   Boolean   and  arithmetic

expressions),  relational operators, and control constructs.   We

still haven't  addressed procedure or function calls, but even so

we  could  conceivably construct a  mini-language  without  them.

I've  always  thought  it would be fun to see just  how  small  a

language  one  could  build  that  would still be useful.   We're

ALMOST in a position to do that now.  The  problem  is: though we

know  how  to  parse and translate the constructs, we still don't

know quite how to put them all together into a language.

In those earlier installments, the  development  of  our programs

had  a decidedly bottom-up flavor.  In  the  case  of  expression

parsing,  for  example,  we  began  with  the  very lowest  level

constructs, the individual constants  and  variables,  and worked

our way up to more complex expressions.

Most people regard  the  top-down design approach as being better

than  the  bottom-up  one.  I do too,  but  the  way  we  did  it

certainly seemed natural enough for the kinds of  things  we were

parsing.

You mustn't get  the  idea, though, that the incremental approach

that  we've  been  using  in  all these tutorials  is  inherently

bottom-up.  In  this  installment  I'd  like to show you that the

approach can work just as well when applied from the top down ...

maybe better.  We'll consider languages such as C and Pascal, and

see how complete compilers can be built starting from the top.

In the next installment, we'll  apply the same technique to build

a  complete  translator  for a subset of the KISS language, which

I'll be  calling  TINY.    But one of my goals for this series is

that you will  not only be able to see how a compiler for TINY or

KISS  works,  but  that you will also be able to design and build

compilers for your own languages.  The C and Pascal examples will

help.    One  thing I'd like you  to  see  is  that  the  natural

structure of the compiler depends very much on the language being

translated, so the simplicity and  ease  of  construction  of the

compiler  depends  very  much  on  letting the language  set  the

program structure.#AB#2#AB

- 2 -#A*#2#A*

#P#A

#2#A

It's  a bit much to produce a full C or Pascal compiler here, and

we won't try.   But we can flesh out the top levels far enough so

that you can see how it goes.

Let's get started.

THE TOP LEVEL

One of the biggest  mistakes  people make in a top-down design is

failing  to start at the true top.  They think they know what the

overall structure of the  design  should be, so they go ahead and

write it down.

Whenever  I  start a new design, I always like to do  it  at  the

absolute beginning.   In  program design language (PDL), this top

level looks something like:

begin

solve the problem

end

OK, I grant  you that this doesn't give much of a hint as to what

the next level is, but I  like  to  write it down anyway, just to

give me that warm feeling that I am indeed starting at the top.

For our problem, the overall function of a compiler is to compile

a complete program.  Any definition of the  language,  written in

BNF,  begins here.  What does the top level BNF look like?  Well,

that depends quite a bit on the language to be translated.  Let's

take a look at Pascal.

THE STRUCTURE OF PASCAL

Most  texts  for  Pascal  include  a   BNF   or  "railroad-track"

definition of the language.  Here are the first few lines of one:

<program> ::= <program-header> <block> '.'

<program-header> ::= PROGRAM <ident>

<block> ::= <declarations> <statements>

We can write recognizers  to  deal  with  each of these elements,

just as we've done before.  For each one, we'll use  our familiar

single-character tokens to represent the input, then flesh things

out a little at a time.    Let's begin with the first recognizer:

the program itself.#A6#2#A6

- 3 -#A*#2#A*

#P#A

#2#A

To translate this, we'll  start  with a fresh copy of the Cradle.

Since we're back to single-character  names, we'll just use a 'p'

to stand for 'PROGRAM.'

To a fresh copy of the cradle, add the following code, and insert

a call to it from the main program:

{--------------------------------------------------------------}

{ Parse and Translate A Program }

procedure Prog;

var  Name: char;

begin

Match('p');            { Handles program header part }

Name := GetName;

Prolog(Name);

Match('.');

Epilog(Name);

end;

{--------------------------------------------------------------}

The procedures  Prolog and Epilog perform whatever is required to

let the program interface with the operating system,  so  that it

can execute as a program.  Needless to  say,  this  part  will be

VERY OS-dependent.  Remember, I've been emitting code for a 68000

running under the OS I use, which is SK*DOS.   I  realize most of

you are using PC's  and  would rather see something else, but I'm

in this thing too deep to change now!

Anyhow, SK*DOS is a  particularly  easy OS to interface to.  Here

is the code for Prolog and Epilog:

{--------------------------------------------------------------}

{ Write the Prolog }

procedure Prolog;

begin

EmitLn('WARMST EQU $A01E');

end;

{--------------------------------------------------------------}

{ Write the Epilog }

procedure Epilog(Name: char);

begin

EmitLn('DC WARMST');

EmitLn('END ' + Name);

end;

{--------------------------------------------------------------}#A6#2#A6

- 4 -#A*#2#A*

#P#A

#2#A

As usual, add  this  code  and  try  out the "compiler."  At this

point, there is only one legal input:

px.   (where x is any single letter, the program name)

Well,  as  usual  our first effort is rather unimpressive, but by

now  I'm sure you know that things  will  get  more  interesting.

There is one important thing to  note:   THE OUTPUT IS A WORKING,

COMPLETE, AND EXECUTABLE PROGRAM (at least after it's assembled).

This  is  very  important.  The  nice  feature  of  the  top-down

approach is that at any stage you can  compile  a  subset  of the

complete language and get  a  program that will run on the target

machine.    From here on, then, we  need  only  add  features  by

fleshing out the language constructs.  It's all  very  similar to

what we've been doing all along, except that we're approaching it

from the other end.

FLESHING IT OUT

To flesh out  the  compiler,  we  only have to deal with language

features  one by one.  I like to start with a stub procedure that

does  nothing, then add detail in  incremental  fashion.    Let's

begin  by  processing  a block, in accordance with its PDL above.

We can do this in two stages.  First, add the null procedure:

{--------------------------------------------------------------}

{ Parse and Translate a Pascal Block }

procedure DoBlock(Name: char);

begin

end;

{--------------------------------------------------------------}

and modify Prog to read:

{--------------------------------------------------------------}

{ Parse and Translate A Program }

procedure Prog;

var  Name: char;

begin

Match('p');

Name := GetName;

Prolog;

DoBlock(Name);

Match('.');

Epilog(Name);#A*#2#A*

- 5 -

#P#A

#2#A

end;

{--------------------------------------------------------------}

That certainly  shouldn't change the behavior of the program, and

it doesn't.  But now the  definition  of Prog is complete, and we

can proceed to flesh out DoBlock.  That's done right from its BNF

definition:

{--------------------------------------------------------------}

{ Parse and Translate a Pascal Block }

procedure DoBlock(Name: char);

begin

Declarations;

PostLabel(Name);

Statements;

end;

{--------------------------------------------------------------}

The  procedure  PostLabel  was  defined  in  the  installment  on

branches.  Copy it into your cradle.

I probably need to  explain  the  reason  for inserting the label

where I have.  It has to do with the operation of SK*DOS.  Unlike

some OS's,  SK*DOS allows the entry point to the main  program to

be  anywhere in the program.  All you have to do is to give  that

point a name.  The call  to  PostLabel puts that name just before

the first executable statement  in  the  main  program.  How does

SK*DOS know which of the many labels is the entry point, you ask?

It's the one that matches the END statement  at  the  end  of the

program.

OK,  now  we  need  stubs  for  the  procedures Declarations  and

Statements.  Make them null procedures as we did before.

Does the program  still run the same?  Then we can move on to the

next stage.

DECLARATIONS

The BNF for Pascal declarations is:

<declarations> ::= ( <label list>    |

<constant list> |

<type list>     |

<variable list> |

<procedure>     |

<function>         )*#A6#2#A6

- 6 -#A*#2#A*

#P#A

#2#A

(Note  that  I'm  using the more liberal definition used by Turbo

Pascal.  In the standard Pascal definition, each  of  these parts

must be in a specific order relative to the rest.)

As  usual,  let's  let a single character represent each of these

declaration types.  The new form of Declarations is:

{--------------------------------------------------------------}

{ Parse and Translate the Declaration Part }

procedure Declarations;

begin

while Look in ['l', 'c', 't', 'v', 'p', 'f'] do

case Look of

'l': Labels;

'c': Constants;

't': Types;

'v': Variables;

'p': DoProcedure;

'f': DoFunction;

end;

end;

{--------------------------------------------------------------}

Of course, we need stub  procedures for each of these declaration

types.  This time,  they  can't  quite  be null procedures, since

otherwise we'll end up with an infinite While loop.  At  the very

least, each recognizer must  eat  the  character that invokes it.

Insert the following procedures:

{--------------------------------------------------------------}

{ Process Label Statement }

procedure Labels;

begin

Match('l');

end;

{--------------------------------------------------------------}

{ Process Const Statement }

procedure Constants;

begin

Match('c');

end;

{--------------------------------------------------------------}

{ Process Type Statement }#A6#2#A6

- 7 -#A*#2#A*

#P#A

#2#A

procedure Types;

begin

Match('t');

end;

{--------------------------------------------------------------}

{ Process Var Statement }

procedure Variables;

begin

Match('v');

end;

{--------------------------------------------------------------}

{ Process Procedure Definition }

procedure DoProcedure;

begin

Match('p');

end;

{--------------------------------------------------------------}

{ Process Function Definition }

procedure DoFunction;

begin

Match('f');

end;

{--------------------------------------------------------------}

Now try out the  compiler  with a few representative inputs.  You

can  mix  the  declarations any way you like, as long as the last

character  in  the  program is'.' to  indicate  the  end  of  the

program.  Of course,  none  of  the declarations actually declare

anything, so you don't need  (and can't use) any characters other

than those standing for the keywords.

We can flesh out the statement  part  in  a similar way.  The BNF

for it is:

<statements> ::= <compound statement>

<compound statement> ::= BEGIN <statement>

(';' <statement>) END

Note that statements can  begin  with  any identifier except END.

So the first stub form of procedure Statements is:#A6#2#A6

- 8 -#A*#2#A*

#P#A

#2#A

{--------------------------------------------------------------}

{ Parse and Translate the Statement Part }

procedure Statements;

begin

Match('b');

while Look <> 'e' do

GetChar;

Match('e');

end;

{--------------------------------------------------------------}

At  this  point  the  compiler   will   accept   any   number  of

declarations, followed by the  BEGIN  block  of the main program.

This  block  itself  can contain any characters at all (except an

END), but it must be present.

The simplest form of input is now

'pxbe.'

Try  it.    Also  try  some  combinations  of  this.   Make  some

deliberate errors and see what happens.

At this point you should be beginning to see the drill.  We begin

with a stub translator to process a program, then  we  flesh  out

each procedure in turn,  based  upon its BNF definition.  Just as

the lower-level BNF definitions add detail and elaborate upon the

higher-level ones, the lower-level  recognizers  will  parse more

detail  of  the  input  program.    When  the  last stub has been

expanded,  the  compiler  will  be  complete.    That's  top-down

design/implementation in its purest form.

You might note that even though we've been adding procedures, the

output of the program hasn't changed.  That's as  it  should  be.

At these  top  levels  there  is  no  emitted code required.  The

recognizers are  functioning as just that: recognizers.  They are

accepting input sentences, catching bad ones, and channeling good

input to the right places, so  they  are  doing their job.  If we

were to pursue this a bit longer, code would start to appear.

The  next  step  in our expansion should  probably  be  procedure

Statements.  The Pascal definition is:

<statement> ::= <simple statement> | <structured statement>

<simple statement> ::= <assignment> | <procedure call> | null

<structured statement> ::= <compound statement> |

<if statement>       |

<case statement>     |

<while statement>    |#A*#2#A*

- 9 -

#P#A

#2#A

<repeat statement>   |

<for statement>      |

<with statement>

These  are  starting  to look familiar.  As a matter of fact, you

have already gone  through  the process of parsing and generating

code for both assignment statements and control structures.  This

is where the top level meets our bottom-up  approach  of previous

sessions.  The constructs will be a little  different  from those

we've  been  using  for KISS, but the differences are nothing you

can't handle.

I  think  you can get the picture now as to the  procedure.    We

begin with a complete BNF  description of the language.  Starting

at  the  top  level, we code  up  the  recognizer  for  that  BNF

statement, using stubs  for  the next-level recognizers.  Then we

flesh those lower-level statements out one by one.

As it happens, the definition of Pascal is  very  compatible with

the  use of BNF, and BNF descriptions  of  the  language  abound.

Armed  with  such   a   description,  you  will  find  it  fairly

straightforward to continue the process we've begun.

You  might  have  a go at fleshing a few of these constructs out,

just  to get a feel for it.  I don't expect you  to  be  able  to

complete a Pascal compiler  here  ...  there  are too many things

such  as  procedures  and types that we haven't addressed yet ...

but  it  might  be helpful to try some of the more familiar ones.

It will do  you  good  to  see executable programs coming out the

other end.

If I'm going to address those issues that we haven't covered yet,

I'd rather  do  it  in  the context of KISS.  We're not trying to

build a complete Pascal  compiler  just yet, so I'm going to stop

the expansion of Pascal here.    Let's  take  a  look  at  a very

different language.

THE STRUCTURE OF C

The C language is quite another matter, as you'll see.   Texts on

C  rarely  include  a BNF definition of  the  language.  Probably

that's because the language is quite hard to write BNF for.

One reason I'm showing you these structures now is so that  I can

impress upon you these two facts:

(1) The definition of  the  language drives the structure of the

compiler.  What works for one language may be a disaster for

another.    It's  a very bad idea to try to  force  a  given

structure upon the compiler.  Rather, you should let the BNF

drive the structure, as we have done here.#A6#2#A6

- 10 -#A*#2#A*

#P#A

#2#A

(2) A language that is hard to write BNF for  will  probably  be

hard  to  write  a compiler for, as well.  C  is  a  popular

language,  and  it  has  a  reputation  for  letting you  do

virtually  anything that is possible to  do.    Despite  the

success of Small C, C is _NOT_ an easy language to parse.

A C program has  less  structure than its Pascal counterpart.  At

the top level, everything in C is a static declaration, either of

data or of a function.  We can capture this thought like this:

<program> ::= ( <global declaration> )*

<global declaration> ::= <data declaration>  |

<function>

In Small C, functions  can  only have the default type int, which

is not declared.  This makes  the  input easy to parse: the first

token is either "int," "char," or the name  of  a  function.   In

Small  C, the preprocessor commands are  also  processed  by  the

compiler proper, so the syntax becomes:

<global declaration> ::= '#' <preprocessor command>  |

'int' <data list>           |

'char' <data list>          |

<ident> <function body>     |

Although we're really more interested in full C  here,  I'll show

you the  code corresponding to this top-level structure for Small

C.

{--------------------------------------------------------------}

{ Parse and Translate A Program }

procedure Prog;

begin

while Look <> ^Z do begin

case Look of

'#': PreProc;

'i': IntDecl;

'c': CharDecl;

else DoFunction(Int);

end;

end;

end;

{--------------------------------------------------------------}

Note that I've had to use a ^Z to indicate the end of the source.

C has no keyword such as END or the '.' to otherwise indicate the

end.#A*#2#A*

- 11 -

#P#A

#2#A

With full C,  things  aren't  even  this easy.  The problem comes

about because in full C, functions can also have types.   So when

the compiler sees a  keyword  like  "int,"  it still doesn't know

whether to expect a  data  declaration  or a function definition.

Things get more  complicated  since  the  next token may not be a

name  ... it may start with an '*' or '(', or combinations of the

two.

More specifically, the BNF for full C begins with:

<program> ::= ( <top-level decl> )*

<top-level decl> ::= <function def> | <data decl>

<data decl> ::= [<class>] <type> <decl-list>

<function def> ::= [<class>] [<type>] <function decl>

You  can  now  see the problem:   The  first  two  parts  of  the

declarations for data and functions can be the same.   Because of

the  ambiguity  in  the grammar as  written  above,  it's  not  a

suitable  grammar  for  a  recursive-descent  parser.     Can  we

transform it into one that is suitable?  Yes, with a little work.

Suppose we write it this way:

<top-level decl> ::= [<class>] <decl>

<decl> ::= <type> <typed decl> | <function decl>

<typed decl> ::= <data list> | <function decl>

We  can  build  a  parsing  routine  for  the   class   and  type

definitions, and have them store away their findings  and  go on,

without their ever having to "know" whether a function or  a data

declaration is being processed.

To begin, key in the following version of the main program:

{--------------------------------------------------------------}

{ Main Program }

begin

Init;

while Look <> ^Z do begin

GetClass;

GetType;

TopDecl;

end;

end.#A*#2#A*

- 12 -

#P#A

#2#A

{--------------------------------------------------------------}

For the first round, just make the three procedures stubs that do

nothing _BUT_ call GetChar.

Does this program work?  Well, it would be hard put NOT to, since

we're not really asking it to do anything.  It's been said that a

C compiler will accept virtually any input without choking.  It's

certainly true of THIS  compiler,  since in effect all it does is

to eat input characters until it finds a ^Z.

Next, let's make  GetClass  do something worthwhile.  Declare the

global variable

var Class: char;

and change GetClass to do the following:

{--------------------------------------------------------------}

{  Get a Storage Class Specifier }

Procedure GetClass;

begin

if Look in ['a', 'x', 's'] then begin

Class := Look;

GetChar;

end

else Class := 'a';

end;

{--------------------------------------------------------------}

Here, I've used three  single  characters  to represent the three

storage classes "auto," "extern,"  and  "static."   These are not

the only three possible classes ... there are also "register" and

"typedef," but this should  give  you the picture.  Note that the

default class is "auto."

We  can  do  a  similar  thing  for  types.   Enter the following

procedure next:

{--------------------------------------------------------------}

{  Get a Type Specifier }

procedure GetType;

begin

Typ := ' ';

if Look = 'u' then begin

Sign := 'u';#A*#2#A*

- 13 -

#P#A

#2#A

Typ := 'i';

GetChar;

end

else Sign := 's';

if Look in ['i', 'l', 'c'] then begin

Typ := Look;

GetChar;

end;

end;

{--------------------------------------------------------------}

Note that you must add two more global variables, Sign and Typ.

With these two procedures in place, the compiler will process the

class and type definitions and store away their findings.  We can

now process the rest of the declaration.

We  are by no means out of the woods yet, because there are still

many complexities just in the definition of the  type,  before we

even get to the actual data or function names.  Let's pretend for

the moment that we have passed all those gates, and that the next

thing in the  input stream is a name.  If the name is followed by

a left paren, we have a function declaration.  If not, we have at

least one data item,  and  possibly a list, each element of which

can have an initializer.

Insert the following version of TopDecl:

{--------------------------------------------------------------}

{ Process a Top-Level Declaration }

procedure TopDecl;

var Name: char;

begin

Name := Getname;

if Look = '(' then

DoFunc(Name)

else

DoData(Name);

end;

{--------------------------------------------------------------}

(Note that, since we have already read the name, we must  pass it

along to the appropriate routine.)

Finally, add the two procedures DoFunc and DoData:

{--------------------------------------------------------------}

{ Process a Function Definition }

procedure DoFunc(n: char);#A*#2#A*

- 14 -

#P#A

#2#A

begin

Match('(');

Match(')');

Match('{');

Match('}');

if Typ = ' ' then Typ := 'i';

Writeln(Class, Sign, Typ, ' function ', n);

end;

{--------------------------------------------------------------}

{ Process a Data Declaration }

procedure DoData(n: char);

begin

if Typ = ' ' then Expected('Type declaration');

Writeln(Class, Sign, Typ, ' data ', n);

while Look = ',' do begin

Match(',');

n := GetName;

WriteLn(Class, Sign, Typ, ' data ', n);

end;

Match(';');

end;

{--------------------------------------------------------------}

Since  we're  still  a long way from producing executable code, I

decided to just have these two routines tell us what they found.

OK, give this program a try.    For data declarations, it's OK to

give a list separated by commas.  We  can't  process initializers

as yet.  We also can't process argument lists for  the functions,

but the "(){}" characters should be there.

We're still a _VERY_ long way from having a C compiler,  but what

we have is starting to process the right kinds of inputs,  and is

recognizing both good  and  bad  inputs.    In  the  process, the

natural structure of the compiler is starting to take form.

Can we continue this until we have something that acts  more like

a compiler. Of course we can.  Should we?  That's another matter.

I don't know about you, but I'm beginning to get dizzy, and we've

still  got  a  long  way  to  go  to  even  get  past   the  data

declarations.

At  this  point,  I think you can see how the  structure  of  the

compiler evolves from the language  definition.    The structures

we've seen for our  two  examples, Pascal and C, are as different

as night and day.  Pascal was designed at least partly to be easy

to parse, and that's  reflected  in the compiler.  In general, in

Pascal there is more structure and we have a better idea  of what

kinds of constructs to expect at any point.  In  C,  on the other

hand,  the  program  is  essentially  a  list   of  declarations,

terminated only by the end of file.#A*#2#A*

- 15 -

#P#A

#2#A

We  could  pursue  both  of  these structures much  farther,  but

remember that our purpose here is  not  to  build a Pascal or a C

compiler, but rather to study compilers in general.  For those of

you  who DO want to deal with Pascal or C, I hope I've given  you

enough of a start so that you can  take  it  from  here (although

you'll soon need some of the stuff we still haven't  covered yet,

such as typing and procedure calls).    For the rest of you, stay

with me through the next installment.  There, I'll be leading you

through the development of a complete compiler for TINY, a subset

of KISS.

See you then.

*****************************************************************

*                                                               *

*                        COPYRIGHT NOTICE                       *

*                                                               *

*   Copyright (C) 1989 Jack W. Crenshaw. All rights reserved.   *

*                                                               *

*****************************************************************#AI#2#AI

- 16 -#A*#2#A*

#@