TIF2GIF is a program which converts TIFF-G uncompressed (4 or 8

bits/pixel) monochrome image files to GIF image files.  It is not robust

in the TIF formats it will accept; I wrote it for the two cases I

needed.  I am open to suggestions on additional TIFF variations to support.

You have the option if stretching the image in the X direction, making

the resulting file more suitable for the benighted non-square pixels of

a CGA, EGA or Hercules display board.

If the same number of bits is used for the GIF file, no dithering is

used.  Otherwise, you have a choice of Floyd-Steinberg or Stucki

dithering.  FS is quick, and gives reasonable results.  Stucki will give

better detailing, at the expense of some time.

For multi-bit output, the dithering uses gray-levels, and a "color"

table is written to the file for use by display programs.  Try using 3

bits on a VGA, or 2 bits for an EGA.  ( Be wary of some VGA programs

which do not set color tables correctly! For example, CSHOW5 only allows

4 shades per primary, just like an EGA.) Most EGA/VGA monitors/cards I have

seen have a disproportionally bright "full white", which gives poor

results, though you could try adjusting the contrast & brightness.  The

IBM 8514, for those of you with $, looks wonderful with 4 bits.  I

have added an option (for Stucki dithering only) to disuse the brightest

level, which helps EGAs with the above problem considerably.

Please direct comments/complaints to me, CompuServe 7377,1761.

.. Steve