_____________________________________________________________________________

Parallel.Exe                                  Parallel Technologies, Inc.

Parallel Port Information Utility                                PO Box 7

(C) Copyright 1993 Jay Lowe                        Redmond, WA 98073-0007

This program may be used                                Lab  206-869-1136

free of charge subject to the                           Fax  206-869-9767

Restricted License Agreement                     Compuserve     76640,203

as stated below                         Internet 76640.203@COMPUSERVE.COM

_____________________________________________________________________________

Welcome to the Parallel Port Information Utility

-----------------------------------------------------

Parallel is a simple utility which displays the configuration of the

parallel ports (LPT ports) in your system. Parallel is useful for:

1.  Configuring parallel port software such as Windows printer

connections, parallel port device drivers, and print spoolers.

2.  Resolving technical support problems with parallel port devices,

including printers and external peripherals such as network

adapters, portable disks, tape drives, CD-ROMs, floppies, etc.

3.  Configuring your parallel ports for best performance.

4.  Resolving pesky IO address and IRQ contention problems.

5.  Discovering the performance capabilities of your parallel ports.

6.  Learning how your particular system's CMOS configuration

relates to the actual configuration of your parallel ports.

7.  Making informed decisions when buying parallel ports and devices.

8.  Getting the best parallel port performance for your dollar.

9.  Experimenting with your parallel port configuration.

10. Solving problems associated with print spoolers and network print

capture programs which redirect printer output by changing the

port mappings kept in the LPT Bios data area.

Now is a good time for you to learn something about parallel ports,

since parallel ports are becoming increasingly useful due to the recent

explosion in the popularity of portable and external peripherals, and

because new parallel port standards (EPP, ECP, IEEE 1284) are breaking

performance barriers and allowing full bus speed operation. There are

major performance differences between different types of ports and

these differences are becoming of increasing significance to consumers.

These applications of Parallel are dicussed in more detail below. But

first let's take a quick look at how you run the program itself and

interpret the various information it displays.

PARALLEL.DOC                                                              Page 1

Parallel Port Information Utility  ������������������������������  User's Manual

------------------------------

Quick Overview of Parallel

------------------------------

Parallel operates by testing each of the standard addresses in your

system where parallel ports might be found and gathering a broad

assortment of useful information about the specific port behavior.

Parallel tests the ports and then produces a compact display which

is convenient and easy to read. You may need to refer to these

instructions or the built-in reference screen a few times until you

learn what the various columns of information mean.

How to use Parallel:

Step 1 => Disconnect parallel port peripherals (printers, disks, etc)

Step 2 => Disable parallel port software (device drivers, spoolers, etc)

Step 3 => Attach the optional loopback connector(s) for IRQ detection

Step 4 => Run PARALLEL

Step 5 => Read the display...

Parallel will display five columns of information containing one or

more lines about each port. The columns are:

Port Type        The parallel port's IO capabilities. Types are:

No Port   --   No port is exists at the address

4 bit     --   A slow "unidirectional" port

8 bit     --   A faster "bidirectional" port

EPP       --   A very fast Enhanced Parallel Port

Address          IO Base Address of the port (3BC, 378, 278)

IRQ              Interrupt level associated with the port (5 or 7)

BIOS Name        Logical device name of the port (LPT1, LPT2, LPT3)

Notes            A wide assortment of comments, warnings, etc.

Port Strangeness Factor. On a single line, after the columnar display,

Parallel reports a magic number called the "Port Strangeness Factor".

This is simply the total number of strange or unusual properties that

Parallel has found when examining your configuration. If your ports are

considered strange by Parallel, you might want to take some action

to correct the situtation or at least be aware of the peculiarity.

The Port Strangeness is not reported if your configuration is nominal.

PARALLEL.DOC                                                              Page 2

Parallel Port Information Utility  ������������������������������  User's Manual

------------------------------

Detailed Program Operation

------------------------------

Before running Parallel, temporarily remove all parallel port devices

and software to establish a baseline configuration. Temporarily

disconnect printers and other parallel port devices such as disks,

network adapters, tape drives, etc, and disable any special parallel

port software you may have running, like print spoolers or parallel

port device drivers. The interaction of Parallel with external devices

is discussed in some detail later on, and you may find that disconnecting

these devices is simply not necessary in your configuration.

Then, type PARALLEL at the command line.  Parallel will display the

parallel port type, IO address, IRQ level, BIOS name equivalence, and

an assortment of informative notes about your parallel ports.

If you have a relatively slow PC, you may notice that Parallel produces

a certain amount of temporary "debugging" output as it attempts to

determine your port configuration and then overlays this output with

the final display. This is normal and has been included in the first

release of the program for possible support purposes.

Type PARALLEL ? for a reminder screen on interpreting the display.

If, perchance, the display scrolls off the screen, which might happen

if Parallel is reporting extensive notes about interesting conditions

on your ports, then use the MORE command or redirect Parallel's output

to a file. Redirection to a file can also be handy if you need to fax

Parallel's output to someone.

PARALLEL | MORE          ; pauses the output every screen

PARALLEL > filename      ; redirects output to a file

Parallel works best when you disconnect any parallel port devices, such

as printers, external peripherals, or "laplink" cables. Even our 4bit,

8bit, or Univeral High-Speed Turbo cables for our handy parallel port

network drivers (Pipx and Pndis) should be temporarily disconnected

when you run Parallel, at least until you understand how Parallel can

be affected by interference from devices attached to the port(s).

Parallel may also affect active devices attached to parallel ports

(printers, network adapters, external parallel port disks, etc),

although we've made every attempt to minimize these effects. Parallel's

interactions with devices are discussed in some detail below.

PARALLEL.DOC                                                              Page 3

Parallel Port Information Utility  ������������������������������  User's Manual

-------------------

Sample Displays

-------------------

�����������������������������������������������������������������������������Ŀ

� Parallel Port Information Utility, v1.00 (930308). Use PARALLEL ? for help. �

� (C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.   We thank you.    �

�����������������������������������������������������������������������������Ĵ

�  Port Type  � Address � IRQ � BIOS Name � Notes                             �

�����������������������������������������������������������������������������Ĵ

� 8 bit port  �   3BC   �  5  �   LPT1:   � IRQ determined by loopback        �

� 4 bit port  �   378   �  7  �   LPT2:   � IRQ determined by loopback        �

�   No port   �   278   �  -  �     -     � No port found at this address     �

�����������������������������������������������������������������������������Ĵ

� Experts in parallel port technology and devices since 1987.  CIS# 76640,203 �

�������������������������������������������������������������������������������

The system above (a 486/33 clone with two LPT cards) shows:

o  an 8BIT true bidirectional port at 3BC using IRQ 5

o  a 4BIT unidirectional port at 378 using IRQ 7

o  loopback connectors are installed in both ports

o  the IRQ configuration is healthy, but reversed from "standard"

o  the ports are well behaved (there are no special notes)

�����������������������������������������������������������������������������Ŀ

� Parallel Port Information Utility, v1.00 (930308). Use PARALLEL ? for help. �

� (C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.   We thank you.    �

�����������������������������������������������������������������������������Ĵ

�  Port Type  � Address � IRQ � BIOS Name � Notes                             �

�����������������������������������������������������������������������������Ĵ

� 4 bit port  �   3BC   �  7  �   LPT1:   � IRQ determined by loopback        �

�             �         �     �           �  NOTE: Possible IRQ conflict!     �

� 4 bit port  �   378   �  7  �   LPT2:   � IRQ determined by loopback        �

�             �         �     �           �  NOTE: IRQ sense is INVERTED      �

�   No port   �   278   �  -  �     -     � No port found at this address     �

�����������������������������������������������������������������������������Ĵ

� Your Port Strangeness Factor = 2.   We suggest that you read Parallel.Doc.  �

�����������������������������������������������������������������������������Ĵ

� Experts in parallel port technology and devices since 1987.  CIS# 76640,203 �

�������������������������������������������������������������������������������

The system above (an original IBM-PC) shows:

o  a 4BIT unidirectional port at 3BC using IRQ 7

o  a 4BIT unidirectional port at 378 using IRQ 7

o  an IRQ Conflict - both ports are using the same IRQ

o  the interrupt sense of the second port is inverted

Notes:

o  we can't use IRQ5 in this case because the IBM/PC disk uses IRQ5

o  these old expansion cards do not have IRQ5 capability anyway

o  when a new LPT card in placed at IRQ5, the interrupt test fails

because the disk controller's interrupt line interferes with it

PARALLEL.DOC                                                              Page 4

Parallel Port Information Utility  ������������������������������  User's Manual

-------------------------------

Sample Displays (continued)

-------------------------------

�����������������������������������������������������������������������������Ŀ

� Parallel Port Information Utility, v1.00 (930308). Use PARALLEL ? for help. �

� (C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.   We thank you.    �

�����������������������������������������������������������������������������Ĵ

�  Port Type  � Address � IRQ � BIOS Name � Notes                             �

�����������������������������������������������������������������������������Ĵ

�   No port   �   3BC   �  -  �     -     � No port found at this address     �

� 4 bit port  �   378   �  ?  �   LPT1:   � Need loopback to determine IRQ    �

�             �         �     �           �  See instructions in PARALLEL.DOC �

�             �         �     �           �  Otherwise assume IRQ 7 or IRQ 5  �

�             �         �     �           �  Port has latent EPP tendencies   �

�             �         �     �           �  EPP BIOS is not installed        �

�   No port   �   278   �  -  �     -     � No port found at this address     �

�����������������������������������������������������������������������������Ĵ

� Your Port Strangeness Factor = 1.   We suggest that you read Parallel.Doc.  �

�����������������������������������������������������������������������������Ĵ

� Experts in parallel port technology and devices since 1987.  CIS# 76640,203 �

�������������������������������������������������������������������������������

The system above (a DELL NL25 laptop) shows:

o  a 4BIT unidirectional port at 378 with unknown IRQ

o  IRQ is unknown because the loopback connector is not installed

o  The port has EPP capability, but it is not currently enabled

o  There is no EPP BIOS installed in the system

�����������������������������������������������������������������������������Ŀ

� Parallel Port Information Utility, v1.00 (930308). Use PARALLEL ? for help. �

� (C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.   We thank you.    �

�����������������������������������������������������������������������������Ĵ

�  Port Type  � Address � IRQ � BIOS Name � Notes                             �

�����������������������������������������������������������������������������Ĵ

�   No port   �   3BC   �  -  �     -     � No port found at this address     �

�  EPP port   �   378   �  7  �   LPT1:   � IRQ determined by EPP hardware    �

�             �         �     �           �  EPP BIOS is installed            �

�   No port   �   278   �  -  �     -     � No port found at this address     �

�����������������������������������������������������������������������������Ĵ

� Experts in parallel port technology and devices since 1987.  CIS# 76640,203 �

�������������������������������������������������������������������������������

The system above (the same DELL NL25) shows:

o  the port has been switched into EPP mode with our own EPP BIOS

Notes:

o  Enabling EPP on most EPP laptops requires a special EPP BIOS

o  These are usually provided by the laptop or peripheral vendor

PARALLEL.DOC                                                              Page 5

Parallel Port Information Utility  ������������������������������  User's Manual

----------------------------

Interpreting the Display

----------------------------

Understanding the Port Type

---------------------------

This can be 4bit, 8bit, EPP or No Port. We will add recognition for

other ports such as Microsoft/HP ECP ports, and/or other IEEE 1284

ports, as they begin to appear in end-user systems.

No Port     No port was found at the given IO Address.

Parallel tests all the base addresses and displays at

least one line of information about each base address,

even if there is no port installed at that base address.

Parallel uses a rather more robust technique for

detecting a port than is used in any of the BIOSes we

have seen. This means that Parallel can detect ports

which fail the BIOS boot-time tests. Parallel uses a

rotating bit test and assumes the port exists if any of

the bits at the port location respond correctly.

False Positives.  Parallel can be fooled into thinking

that a port is present when it is not (false positive)

only if there is some other device which behaves like a

parallel port located at the address being tested.  We

have only seen this on one system, an off-shore 286

laptop which had several other design problems as well.

False Negatives.  There is one way in which Parallel

can be fooled into thinking that a port does not exist

(false negative). If an active device is connected to

the data lines and is providing enough power to

"overdrive" the port, then the port existance test will

fail because the incoming data from the device will

foil the readback test. We have only seen Parallel

produce false negatives when an external device is

actively driving the port's inputs or when the Loopback

is incorrectly installed. By the way, in this case, the

PC's BIOS will also fail to detect the port. Again,

Parallel works best when you disconnect any parallel

port devices.

4bit ports  The port can do 8bit byte output and 4bit nibble input.

These ports are often called "unidirectional", and are

most commonly found on desktop bus cards (also called

IO expansion cards, serial/parallel cards, or even 2S+P

cards). This is still the most common type of port,

especially on desktop systems.

4bit ports are capable of effective transfer rates of

about 40 Kbytes/sec (KBPS) in typical devices and can

be pushed upwards of 80 KBPS with certain design

tricks. The port speed is also affected by electrical

PARALLEL.DOC                                                              Page 6

Parallel Port Information Utility  ������������������������������  User's Manual

characteristics of the noise suppression circuitry.

Some ports have completely ridiculous arrangements of

noise suppression circuits which slow them down quite

dramatically and can prevent their functioning with

parallel port peripherals.

NOTE:  The sample transfer rates given here reflect the actual

performance of parallel port network adapters, disks,

etc. The raw transfer rate of the port itself is higher

but since most parallel devices use software strobing to

control data access, the effective transfer rate is

dependent on the quality of the driver software supplied

with the device. Due to the recent explosion in the

number of parallel port peripherals entering the market,

there is an extreme degree of variability in the quality

and performance of these devices and their drivers.

4bit ports are found on about 50% of all systems and

about 75% or more of desktop systems.

We do not recommend 4bit ports; 8bit ports are better.

NOTE:  There is a class of "semi-bidirectional" parallel ports

which appear to the PC to be 4bit ports, but actually

have 8bit capability when they are connected to

external devices with strong drivers. These include

the original PC/AT parallel port. Parallel will report

these ports as 4bit ports but they may actually function

as 8bit ports with a few parallel port peripherals.

A good example of a peripheral which handles these ports

well are IQ Technologies' parallel port network adapters.

It is actually possible to identify this type of port

using a side-effect of Parallel's loopback test. See

the special section below on Semi-Bidirectional ports.

8bit ports  These ports can do both 8bit input and output.

These ports are sometimes called "bidirectional ports",

but that term is often misused by some vendors to refer

to 4bit ports as well. Most newer laptops have 8bit

capability although it may need to be enabled with the

laptop's vendor-specific CMOS setup function. This is

discussed below. A relatively smaller percentage of LPT

bus cards have 8bit capability which sometimes must be

enabled with a hardware jumper on the board itself.

True 8bit ports are preferable to 4bit ports because

they are considerably faster when used with external

devices that take advantage of 8BIT capability.

8bit ports are capable of speeds ranging from 80-150

KBPS, again depending on the speed of the attached

device, the quality of the driver software, and on the

port's electrical characteristics.  8bit ports are

PARALLEL.DOC                                                              Page 7

Parallel Port Information Utility  ������������������������������  User's Manual

found on 25% of desktops, 50% of laptops, and about

90% of newer laptops built since 1991.

CAVEAT:  A number of vendors claim "bidirectional" capability

EMPTOR   for their parallel ports when they are actually just

4bit ports. When buying a parallel port expansion card,

or an entire system, you might want to use Parallel to

find out the REAL capabilities of the port. Be aware

that a vendor's reputation for quality in other areas

may not translate into good parallel ports.  There are

examples of well-known vendors making false claims for

their parallel ports... sometimes, it appears, simply

because they don't understand them... claims or ports.

Of the 8bit add-in LPT cards we have tested, we like the

Everex MagicIO the best, but it can be hard to find.

We are not going to pan anyone's "bidirectional" add-in

cards specifically, but be warned that the use of terms

like "bidirectional" or "PS/2 compatible" may have

nothing to do with the port's actual IO capability.

You can use Parallel to test add-in cards to determine

the parallel port's IO actual behavior.

EPP ports   Can do both 8bit input and output at bus speeds.

These ports are as fast as 8bit bus cards and can

achieve transfer rates of 1 Mbyte/sec and faster. For

example, our EPP-capable external SCSI disk is faster

on a very popular SL laptop than it's internal drive.

EPP ports are supposed to conform to a well defined

electrical standard and many of them actually do so.

However, if you are buying a laptop and want to be sure

of using fast EPP-capable external peripherals, then

definitely arrange to try before you buy.

Xircom has been active in creating and defining the EPP

standard, and since their EPP-capable network adapters

are fairly common, it's worth mentioning that these

adapters can be used as a practical benchmark for the

goodness of a particular EPP port. Crude but effective.

EPP ports are most often found on laptops using the

Intel 386SL chipset with the 360SL IO support chip. You

can usually identify these laptops by the 'SL' suffix

in their name, HOWEVER this is not a hard and fast

rule, since there are non-'SL' systems with EPP ports

as well as 'SL' systems which (in our tests) have non-

functioning or sub-standard EPP ports.  Other vendors

offer add-in EPP cards for the ISA bus. Parallel will

not currently recognize non-Intel EPP ports; we will be

adding this feature in the near future. We know of one

vendor now offering EPP-capable add-in cards:

FarPoint Communications       805-726-4420

The FarPoint card is quite nice because it is a "soft"

PARALLEL.DOC                                                              Page 8

Parallel Port Information Utility  ������������������������������  User's Manual

design with downloadable personalities. It comes with

support for fast Windows printing and a fast Netware

spooling NLM. We like it.

SMC's Component Products Division is due to release

EPP and ECP chips in the very near future, so users

can expect to see EPP and ECP capabilities become

available on a wider range of systems.

HOW TO:  Run Parallel on any system you intend to buy to see

what kind of parallel port(s) are provided.  If you are

buying a desktop with a built-in LPT card, try to get

real 8bit capability instead of the more typical 4bit.

Some LPT cards have hardware jumpers that are required

to enable the bidirectional behavior. So make sure you

get a manual for the LPT card when you buy a system.

If you are buying a laptop and have any desire to use

external peripherals like network adapters, portable

hard disks, CD-ROMS, tape drives, etc, then seriously

consider getting a laptop with an EPP port. They are a

LOT faster running EPP-capable external peripherals. In

any case, try before you buy. Few vendors (and users)

are well-informed about parallel port technology and

inappropriate claims, however inadvertant, are common.

COMING:    Microsoft, Hewlett-Packard, and others are espousing a

SOON --    new standard for parallel ports, the ECP port, which is

ECP PORTS  may become part of the proposed IEEE 1284 specification,

which also covers EPP. Some vendors are reported to

have silicon for this port, but we haven't seen it in

any end-user systems yet. The ECP port is pretty much

the same as EPP ports in terms of performance but uses

a quite different hardware design. The major difference

is that ECP has Microsoft's blessing and they will

probably support it (but perhaps only in Windows) if it

actually materializes. Quite typically, there is no

guarantee that MS will ever support EPP.

When using Parallel to make buying decisions, please

make sure you've got the most recent release. Parallel

port technology will be changing rapidly over the next

year and we will be updating Parallel frequently.

PARALLEL.DOC                                                              Page 9

Parallel Port Information Utility  ������������������������������  User's Manual

Understanding the IO Address

----------------------------

This is the base address of the port. It will be 3BC, 378, or 278 hex.

The IO address specifies the port's location in the PC address space.

Normally the first port is configured to 3BC, the second to 378, etc,

however the ports can be assigned to any of these three addresses.

On bus cards, the IO address is usually set by a arrangement of jumpers

unique to the particular board. You will need a manual or the ability

to read PC board traces (gacck!). These jumpers are usually arranged to

allow you to select one of three addresses: 3BC, 378, 278.

Sometimes these addresses are called by the "logical device names" of

LPT1, LPT2, and LPT3, although this practice is quite misleading (see

BIOS discussion below). Some boards offer a more limited selection of

configurations (usually 378 and 278) and a few are actually hardwired

to a specific address. Vendors do this to save a little money and also

to eliminate the chance of creating an address collision with the

"standard" address of 3BC. Because of the increasing number of clones

which are assembled from various components, we often see systems with

one LPT card, configured to address 378. In fact, a fair number of

systems are shipped with this 378 configuration.

A slightly interesting result we've seen in several desktops spins off

when someone adds a second LPT card to a system which already has a LPT

at 378, IRQ 7. Sometimes they naively reason that since 378 is taken,

they can use 3BC and slap in the card without checking the IRQ level.

This creates a configuration of 3BC and 378 both using IRQ 7, a

situation which can lead to intermittent crashes when using interrupt

driven parallel port software.

By the way, a few systems have parallel ports and com ports built onto

the motherboard itself and, in this case, the configuration jumpers

will be found on the motherboard.

Notice that there are three commonly used IO Addresses (3BC, 378, 278)

and two available interrupt levels (IRQ 7 and IRQ 5). This means that

if you have three LPT cards in a system, two of them will probably be

sharing a hardware interrupt level. This is hardwarily permissible, and

generally doesn't create any problems since most DOS and Windows

software doesn't use interrupts for printing anyway. However, if you

are using LPT software like an interrupt-driven print spooler or

interrupt-driven LPT devices like external hard disks, external network

adapters, and so forth, then you have to be sure that either (1) the

ports are not using the same interrupt level, or (2) the software is

capable of "interrupt sharing".

When adding or configuring LPT cards, we suggest using 3BC, IRQ 7 for

the first card; 378, IRQ 5 for the second card; and 278, no IRQ for the

third card. This gives you two interrupt-capable LPT ports, and one

port (at 278) which you should use only for non-interrupt driven usage,

like the status-driven spooling provided by DOS.

In any case, armed with this understanding and the Parallel Utility,

you are equipped to select a configuration which works for you.

PARALLEL.DOC                                                             Page 10

Parallel Port Information Utility  ������������������������������  User's Manual

HOW TO:  The IO ADDRESS is a standard parameter in network

configuration files where it takes on different names

and syntaxes to confuse innocent users.

Some simple-minded parallel port peripherals (ones

without auto-configuring capability) require that you

specify the IO address, IRQ level, and sometimes the

Port Type in a vendor-specific way. Parallel provides

you with this information. See Configuring Networks and

Devices below.

HOW TO:  Resolving IO Address conflicts is usually fairly simple,

since it is rare to find a device which uses the

standard parallel port addresses.

A good procedure for eliminating potential IO address

conflicts when installing parallel port add-in boards

is to remove all the boards, install one, use Parallel

to see what addresses it finds, install the next, and

so forth.  If Parallel finds each board at a separate

addresses as you install them, then you can be sure of

your configuration.

PARALLEL.DOC                                                             Page 11

Parallel Port Information Utility  ������������������������������  User's Manual

Understanding the IRQ Level

---------------------------

This is the hardware interrupt level assigned to the port. It will

always be either 7 or 5. Parallel displays "?" when it cannot detect

the IRQ level of the port and "-" when the port doesn't exist. Parallel

needs an external loopback connector to automatically detect the IRQ.

With appropriate precautions (described in horrible and excruciating

detail below) you can use a simple paperclip as a loopback connector.

Normally IRQ 7 is assigned to the first port and IRQ 5 is assigned to

the second port. However, due to the explosion of clones assembled from

disparate parts, it is not uncommon to see systems with one LPT port

located at 378 with an IRQ of 7 or 5. This is because clones are often

assembled using offshore add-in LPT cards which are shipped configured

to operate as a second LPT card.

This is a good time to point out that there are few hard and fast rules

when it comes to parallel port configuration. Ports can be located at any

one of the three standard base address and can use either of the IRQ

levels. If you are developing parallel port software, you should auto-

detect the actual configuration rather than assume any standard usage.

We offer libraries and source code for this purpose. Here at Parallel

Technologies, we test our software using many strange configurations and

delight in adding idiosyncratic ports to our extensive collection.

The most common idiosyncrasy of port IRQ is IRQ inversion.  The PC

standard calls for interrupts to be generated on 0->1 transitions of

the interrupt line.  A small percentage of parallel ports generate

their interrupts on 1->0 transitions.  Ports with inverted IRQ can

fail to operate with some interrupt software, particularly certain

external network adapters, disks, and so forth. Parallel identifies

these ports with a note "IRQ style is INVERTED".

Some parallel ports have non-functioning IRQ lines. They are simply not

capable of generating interrupts. Only a very few pieces of software

are capable of dealing with these ports by using a polled mode of

operation. It is sometimes possible to repair such systems by jumpering

the interrupt enable line to the parallel port chip.

On ISA cards (bus cards in ISA slots) the interrupt level is usually

set with jumpers on the board - these jumpers are different for every

card so you need to hunt around for the manual in order to change them.

If you're lucky there will be readable labels on the card itself.

On laptops, the interrupt level can usually be set using the vendor's

specific CMOS configuration (setup) program. Sometimes these CMOS setup

programs are built into the ROM; sometimes they exist as separate

programs or as part of the vendor's diagnostic program(s). Refer to

your manual to find out how to enter the CMOS setup mode. BE CAREFUL

when using the CMOS setup, since it gives you access to parameters that

can disable your system if wrongly changed. KNOW WHAT YOU ARE DOING.

PARALLEL.DOC                                                             Page 12

Parallel Port Information Utility  ������������������������������  User's Manual

******************

* IMPORTANT NOTE *

******************

Misuse of your system's CMOS setup program can prevent

your system from operating correctly, or in some cases,

from operating at all! Be sure you know what you are

doing; read your user's manual or get qualified help if

you have any doubts.

Another strange feature of these software configurable ports is that

they often have very peculiar ideas about nomenclature. Often these

CMOS setup programs use vendor specific "magic code words" to select

various parallel port configurations, and these magic words can have

non-obvious effects. Sometimes the vendor's manuals don't even provide

a secret decoder ring to explain what their magic words really mean in

terms of IO Address, IRQ, and Port Type. Parallel will be useful for

determining the actual effect of the CMOS setup choices on your system.

For example, on my Dell NL25 laptop, 4bit mode is called "Normal" and

8bit is called "Enhanced" and there is no choice at all for the real

Enhanced Parallel Port (the SL's EPP port). So someone might obviously

think they have enabled EPP by choosing "Enhanced". Futhermore, the IRQ

level cannot be set directly, but rather is set as a by-product of

selecting combinations of the IO address and the Port Type. Most of the

choices on my Dell yield IRQ 7, but one lonely choice - LPT1, 278,

Normal mode results in IRQ 5. It makes a strange sort of sense when you

are experienced with parallel ports, but is likely to be confusing to

the average user (if we ever find him, or her). The Dell is not alone

in it's strange naming conventions, most laptops have confusing,

misleading, or incomplete labels and controls in their CMOS setup

programs. This sort of indirect control in very common in CMOS setups

and creates a lot of tech support calls from frustrated users.

In addition, most SL laptops don't have the capability to enable their

EPP ports from CMOS. Usually you need to run a separate vendor-supplied

BIOS extension to enable EPP capability. These take the form of a tiny

TSR which adds INT 17h BIOS extensions for controlling the EPP port,

and handles command line arguments for enabling and disabling EPP. An

interesting sidelight of switching into EPP mode can be that your port

IO address may change on the fly. The Intel EPP hardware allows EPP

ports to be configured at IO Addresses 378 and 278. So if you have set

your port address to 3BC (with CMOS for example) and then switch into

EPP mode, your port will appear to have moved.

Parallel will come in handy in trying to make sense of what your

laptops' CMOS settings are actually doing to your parallel ports.

PARALLEL.DOC                                                             Page 13

Parallel Port Information Utility  ������������������������������  User's Manual

Understanding the BIOS Name

---------------------------

This is the "human-readable" (hah!) name which you use to refer to the

port when using DOS commands, Windows picklists, and so forth. It will

be one of the following: LPT1, LPT2, LPT3.  LPT4 is extremely rare and

Parallel currently ignores it.

The port address is NOT the same as the BIOS name for the port. For

example, LPT1 can refer to a port at 3BC, 378, or 278.

So, how does the BIOS associate the LPT names with the actual ports?

Whenever your system boots, the BIOS goes out and tries to write and

readback data from locations 3BC, 378, and 278. If it can read back

what it writes, then the BIOS decides there is a port there and adds

the IO address to a LPT table in the "Bios Data Area". This table is

four words located at 40:8. A command like "copy foo.txt lpt2:" means

"copy the file foo.txt to the second printer that BIOS knows" and DOS

uses a software interrupt, INT 17h, to do just that. INT 17h looks in

the Bios Data area to find out the IO address of the "second printer",

which can be any of the IO Addresses - 3BC, 378, or 278.

The result is that LPT1 refers to the first port BIOS finds at boot

time, wherever it may be. So "LPT1" means "the first printer", not

necessarily 3BC, 378, or 278. Some BIOSes have unique methods for

finding their LPT ports.

When you see LPT1, LPT2, or LPT3 used in hardware manuals to refer to

port locations, be alert for possible misleading information. Some

well-known vendor's manuals use LPT1 to refer to 378 and LPT2 to refer

to 278, an entirely spurious practice which has confused many users.

It is possible for some parallel port software to set the port into an

IO mode that causes it to fail the BIOS test and seem to "disappear"

from your system even after a soft boot (CTRL-ALT-DEL). Parallel has a

better test, and finds such ports. A hard reset (a power-off reset)

will clear the port. Parallel ports are not completely trivial devices;

they have a number of states that can affect their operation. So if you

are having a parallel port problem, one of the first things to do a

power-off reset.

Another factor here is that various programs can fiddle with the

LPT table. The utility SWAPPRT, for example, can be used to swap

LPTx with LPTy which it does by moving the entries in the LPT table.

There are variations of this redirection theme.

If Parallel does not display a BIOS name for a port, that means the

IO Address for that LPT position has been replaced with a value that

does not match a valid hardware port.  Some parallel port device

drivers do this as a way to grab possesion of the port and prevent

other software from writing to the port while the driver is installed.

PARALLEL.DOC                                                             Page 14

Parallel Port Information Utility  ������������������������������  User's Manual

Understanding the Notes and Comments

------------------------------------

The "Notes" field of the display will contain various comments,

depending on what Parallel has detected. Each note may be followed by

one or more comments giving further information or directions. These

secondary comments are indented one space under their note.

"No port found at this address"

No parallel port is installed at this IO Address.

However, it is also possible that an active external

device is overdriving the port's inputs, which can

prevent the software readback test from working.

Be sure that you have disconnected all external devices.

"Need loopback to determine IRQ"

The port exists, but does not respond to attempts to

generate IRQs. This is because a loopback connector

is required for IRQ detection in most cases.

"Port IRQs are not functioning"

This is pretty bad. It means that the loopback connector

is present and working correctly, but Parallel still

can't generate any interrupts. Technically it means

that transitions in the ACK line don't affect the PIC.

Practically it means that interrupt devices and software

probably won't work, unless they have polling support.

Possible reasons for failing the IRQ test include:

1. The port's interrupt capability is disabled

Check your CMOS configuration and/or the

hardware jumpers on the parallel port board(s)

to be sure that the interrupt is enabled.

The function of CMOS parameters and/or hardware

jumpers varies from system to system, so check

your manual or contact your vendor if you have

problems enabling the port's interrupt line.

Note that most CMOS and jumper arrangements

combine enabling the IRQ with selecting the

level, ie, "enable IRQ5" or "enable IRQ7".

Removing the jumper generally disables IRQ.

2. The port does not have IRQ capability

Most ports DO have interrupt capability,

but a small percentage (at a guess %1 to %5)

do not. More laptops lack interrupts than

add-in cards. Add-in cards without IRQ are rare.

Most reports of non-functional parallel port

PARALLEL.DOC                                                             Page 15

Parallel Port Information Utility  ������������������������������  User's Manual

IRQs in desktop systems can be tracked to

configuration problems which create interrupt

conflicts (see #3) which effectively disable IRQ.

3. There is an IRQ conflict with another device

It is possible that another device in your

system is preventing the parallel port IRQs

from functioning.  For example, in IBM-PCs

and XTs, the disk controller which uses IRQ 5

can prevent parallel ports configured to IRQ5

from creating interrupts.

To isolate this problem, remove all boards from the

system that are not required to perform the test.

In other words, strip down to just the parallel port,

floppy controller and video, and repeat the test.

If possible, configure the parallel port to IRQ7.

If the port can create IRQ7 interrupts, but not IRQ5

interrupts, then an interrupt conflict is likely.

"IRQ determined by loopback"

The loopback was detected, is operating correctly,

and the port's IRQs are functioning correctly. The

IRQ level found is given in the IRQ column.

"IRQ estimated by port float"

No loopback connector was detected, but the port is

generating IRQs anyway. This probably means that the

port is connected to an inactive (unpowered) external

device. Treat IRQs determined in this way with some

skepticism; better to disconnect the device and use a

loopback connector.

"IRQ determined by EPP hardware"

If the Port Type is determined to be EPP, then Parallel

does not use the IRQ test, but rather reports the IRQ

which is standard for an EPP port at that IO Address.

"IRQ noise is present"

Parallel has detected extraneous interrupts occuring

on IRQ level 5 or 7 during the IRQ test.

This condition is reported when the total number of

interrupts occurring on both IRQ5 and IRQ7 exceeds

the number of interrupts Parallel tests for.

This can mean several things:

1. PIC errors. The PC's interrupt controller (PIC)

is generating errors as described below.

PARALLEL.DOC                                                             Page 16

Parallel Port Information Utility  ������������������������������  User's Manual

2. An active device other than the parallel port

is generating interrupts on IRQ 5 or 7.

3. The port's interrupt line is electrically noisy.

This has been seen with boards which have two

parallel ports where transitions in one port's

IRQ line causes noise on the other port's IRQ.

This result may be ignored if you are not having any

problems, but be informed that this condition is quite

likely to confuse parallel port software which tries

to autodetect the port's IRQ level.

TECHNICAL NOTE: PIC errors are usually found with parallel port chips

whose "open" condition on the ACK line may allow the

IRQ signal to float randomly. This effect creates

8259 interrupt controller errors which are directed to

IRQ7 where they can be mistaken for "real" interrupts.

This effect may confuse some parallel port software.

An 8259 error condition is defined to occur when the

8259 attempts to service the requesting device but the

device's interrupt line has gone inactive.  These

are also known as "lost interrupts", which are not

really lost but rather redirected to IRQ7. A noisy IRQ

line can create the appearance of lost interrupts.

This effect of lost interrupts explains why some

parallel port driver software works well on IRQ7 but

not IRQ5, or vice versa. Parallel port interrupt

handlers have to account for conditions that are

generally not seen by other device drivers. So if you

are having problems with a parallel port device driver

you may want to try configuring the port to the other

interrupt level.

"NOTE: Data port has stuck bits!"

The rotating bit test has detected bits in the data

port which cannot be set or cleared properly.

You have probably inserted the Loopback incorrectly.

Of course if there is no Loopback wire present, then

something else must be happening! It could be that an

active device is forcing some of the data lines to a

fixed level. It could also mean that your port exists,

but some of the data bits are not working, which is a

polite way of saying that your port may be busted.

Parallel may report a false Port Type in this case.

Make sure that the loopback, if used, is correct. Make

sure that no devices are plugged into the port.

"IRQ style is INVERTED"

PARALLEL.DOC                                                             Page 17

Parallel Port Information Utility  ������������������������������  User's Manual

The port's IRQ line functions in an opposite sense from

the PC standard. A significant amount of software will

fail to work with such ports. There is no remedy for

this condition in general; the best we can advise is to

use a different port if you have a bus system or try

other CMOS settings on a laptop if that is possible.

You might try contacting your vendor.

Parallel may falsely report inverted IRQ if there is

an external device connected to the port.

"Possible IRQ Conflict"

Two or more ports in your system are using the same IRQ

level. If you are not using interrupt-driven parallel

port peripherals or printing software then you might

simply ignore the warning if you are not experiencing

any problems. However, you should know that some very

strange results can occur when two parallel ports are

trying to drive the same IRQ line. Sometimes one or both

ports can falsely appear to have INVERTED IRQ.

The best solution is to move one of the ports to another

IRQ level, if that is possible.

"BIOS Data doesn't match hardware"

The contents of the BIOS Data Area LPT Table does not

correspond to the expected initial state at boot time.

This is probably due to a parallel port device driver

or a SWAPLPT program having fiddled with the BIOS data,

in which case it may be considered normal operation.

This information is useful if you are having problems

with print redirection. Also, some parallel port device

drivers do not examine the hardware directly to determine

which ports exist, but rather only look at the BIOS.

If you have run some other parallel port software which

has changed the BIOS in a non-standard way, then these

device drivers may fail to find any parallel ports, or

may try to access the wrong port. This can often be

solved by changing the order of loading or by manually

configuring the failing driver.

PARALLEL.DOC                                                             Page 18

Parallel Port Information Utility  ������������������������������  User's Manual

---------------------------

Automatic IRQ detection

---------------------------

Parallel uses two methods for automatic detection of a port's IRQ

level: port float or the external loopback connector.

Understanding Port Float

------------------------

Many parallel ports will "float" in a unstable logic state when

connected to an unpowered external device such as a printer. This

unstable logic state may cause the port to generate a limitless stream

of apparent interrupts to the CPU if the port's interrupt line is

enabled. This peculiar behavior is used to advantage by Parallel.

If you have a unpowered (powered off) printer, or other device,

attached to a port, then Parallel may be able to detect the interrupt

level using this floating port behavior. In this case, Parallel will

display "IRQ determined by port float". You should treat the IRQ

determined in this way as a "best guess". To be sure of the IRQ,

you need to disconnect the floating device and use loopback.

In some cases, attaching a unpowered device to a port will enable

Parallel to detect IRQ by loopback, since the combination of port and

device appears indistinguishable from a loopback connector to Parallel.

FLOAT NOTE: Powered on devices usually hold the interrupt line in a

stable state and prevent IRQ autodetection. It is best to disconnect

your parallel port device(s) and use the loopback connector when you

need the absolute best results from Parallel.

EXAMPLE: On my home system, an unpowered HP LaserJet II printer on an

8bit port "floats" and Parallel reliably reports IRQ5. Attached to a

4bit port on the same system, the LaserJet is indistinguishable from a

loopback connector.  So turning the LaserJet off converts it into an

expensive, but handy, paperclip. Turning it on prevents IRQ detection.

TECHNICAL NOTE: Whether or not a given device and port combination will

float, emulate loopback, or be invisible to Parallel is determined by

the pull-ups on each end, CMOS vs TTL logic behavior, and the power

available at the port to a unpowered device. In the above example, I

suspect that enough power is provided by the 8bit TTL port to drive the

unpowered CMOS logic in the LaserJet into an unstable oscillating

state, which appears to the system as a frenzy of level 5 IRQs.

Port float is a handy effect, but use Loopback for best results.

Loopback

--------

The most reliable way to determine IRQ is to use an external

loopback connection. Loopback is described in the next section.

PARALLEL.DOC                                                             Page 19

Parallel Port Information Utility  ������������������������������  User's Manual

--------------------------------------------

IRQ detection using the LoopBack Feature

--------------------------------------------

******************

* IMPORTANT NOTE *

******************

First, DECIDE NOT TO SCREW UP YOUR PORT by crudely jamming wires into

the wrong places with too much force.  REMEMBER that the female DB-25

connector is likely to be made out of CHEAP, BREAKABLE PLASTIC, so

decide right now not to break it. WE ARE NOT LIABLE FOR ANY DAMAGE YOU

MAY DO FOR WHATEVER REASON.

The danger is that you will push too hard when inserting the loopback

wire and force the female connector pin out of its position in the DB-

25 shell. That is why WE SUGGEST MAKING A LOOPBACK PLUG using a male

DB-25 connector as described below. If you cannot make a loopback plug,

then use a parallel port extension cable instead. This protects your

port from physical damage and also makes it a lot easier to work with.

The port is supposed to be electrically designed to tolerate the

loopback perfectly well. However there is a DANGER OF DAMAGE BY STATIC

DISCHARGE so make sure that you GROUND YOURSELF TO THE PORT before

inserting the loopback wire. It would be terrible if you blew out a

$25 parallel port on a $1000 486 motherboard simply because you failed

to PAY ATTENTION TO THESE PRECAUTIONS. PROCEED AT YOUR OWN RISK.

The other danger is that you will apply the loopback to a port which is

not really a parallel port at all. DON'T DO THIS. If you are not sure

of what you are doing STOP RIGHT NOW. We have no control over what you

might damage if you try this with a port other than a parallel port.

Whew... on with the instructions.

1.  Find the parallel port(s) you want to test. They will be female

DB-25 connectors. MAKE SURE THAT YOU HAVE CORRECTLY IDENTIFIED

THE PARALLEL PORT. Disconnect any devices (printers, network

adapters, external disks, etc) which might be attached to other

parallel ports on your system.

2.  Make sure that the connector IS NOT SOME OTHER TYPE OF CONNECTOR

like an external SCSI connector or a COM port.

SCSI ports can be either female DB-25s or a larger connector,

like a 37, 40, or 50-pin connector. COM ports are usually male

DB-25 or small male DB-9 connectors, but this is not guaranteed.

Look in your manual if there is any doubt. DON'T PROCEED unless

you are sure you have identified the parallel port(s) correctly.

3. Connect the Loopback Plug and go to Step 7.

If you do not have a Loopback Plug, use a straight through M-F

parallel port extension cable to prevent damage to your port.

Connect the male end of the extension cable to your port and

place the loopback wire into the female end of the cable.

PARALLEL.DOC                                                             Page 20

Parallel Port Information Utility  ������������������������������  User's Manual

4.  Identify Pin 10 and Pin 9 on the female end of the extension

cable. Refer to the illustration below. Notice that a DB-25

connector is wider on one side.  The pins you want are the 4th

and 5th from the left on the wider edge.

USE A FLASHLIGHT TO FIND THE MARKINGS IF YOU HAVE ANY DOUBT.

Front View         -------- Pin 10 - 4th from left on wider edge

Female DB-25       |   ---- Pin  9 - 5th from left on wider edge

Connector          |   |

v   v

____________________________________________________________

/                                                            \

\     13  12  11  10   9   8   7   6   5   4   3   2   1     /

\                                                          /

\     25  24  23  22  21  20  19  18  17  16  15  14     /

\______________________________________________________/

Pin 10 is the interrupt line (the ACK line)

Pin  9 is the high order data line (the 80h bit)

4.  Create a loopback wire. A standard paperclip works fine. Just

straighten it out and bend it into a "U" shape with the ends 1/8

inch apart. Make sure the ends of the paperclip are SMOOTH and

have NO ROUGH EDGES that might damage the plastic connector.

5.  GROUND YOURSELF to the PC and to the parallel port by touching

your hand and the loopback wire first to the PC's metal chassis

near the parallel port connector and then to one of the connector

nuts or the connector metal housing itself. Do this to DISCHARGE

ANY STATIC ELECTRICITY you may have about you (created for

example by scooting across the rug in a silk negligee on the

way to your PC or by stroking your cat with woolen gloves).

Parallel ports are usually resistant to static discharge, but not

all ports are created equally, so don't ask for trouble.

6.  Carefully insert the paperclip into the extension cable's DB-25

connector so that Pin 9 is connected to Pin 10. DON'T FORCE IT.

It will insert at most about 1 quarter of an inch (about .5 cm)

and there should be absolutely no physical resistance.

7.  Now run Parallel. It will automatically use the loopback to

determine which IRQ level the port is actually using. You may

loopback more than one, or all, ports at the same time.

8.  Remove the loopback when you are done.

9.  Congratulate yourself on using reasonable handling precautions

when working with expensive equipment.

PARALLEL.DOC                                                             Page 21

Parallel Port Information Utility  ������������������������������  User's Manual

--------------------------------------

Making Your Own Loopback Connector

--------------------------------------

We strongly advise that you make a loopback connector, inside of using

the expedient paperclip approach.

Simply obtain a male DB-25 connector and solder a permanent connection

bewteen Pin 9 to Pin 10 on the back of the connector.

Male DB-25 connectors are inexpensive and commonly available in

electronics parts stores.

This will prevent any physical damage to your port, which is the major

source of risk with this procedure, and the reason for our incessant

nagging about precautions. This sort of loopback connector will be no

more dangerous to your system than a printer, and is a handy accessory

if you frequently use Parallel.

Connect Pin 10 to Pin 9

Back View          -----    Pin 10 - 4th from left on wider edge

Male DB-25         |   |    Pin  9 - 5th from left on wider edge

Connector          |   |

v   v

____________________________________________________________

/                                                            \

\     13  12  11  10   9   8   7   6   5   4   3   2   1     /

\                                                          /

\     25  24  23  22  21  20  19  18  17  16  15  14     /

\______________________________________________________/

Pin 10 is the interrupt line (the ACK line)

Pin  9 is the high order data line (the 80h bit)

PARALLEL.DOC                                                             Page 22

Parallel Port Information Utility  ������������������������������  User's Manual

--------------------------------------------

Semi-Bidirectional Ports, A Special Case

--------------------------------------------

A port is semi-bidirectional when it is capable of performing 8BIT

input when connected to a device which has strong drivers. So this

semi-bidirectionality comes about as the relationship between the

port and the device it is connected to.

This type of port will appear to be a 4BIT port with most parallel

port external peripherals, since low power CMOS drivers are by far

the most common in these external devices.

A few peripherals have stronger TTL drivers which can achieve 8BIT

operation by overdriving the port. There are even sub-classes of

ports which can achieve the effect depending on what data is used.

Some vendors describe their ports as bidirectional when actually

they are only semi-bidirectional and will only function as 4BIT ports

with most peripherals. True 8BIT ports are preferable because they

have a significant speed advantage with most external peripherals.

Indeed, some parallel port peripherals require true 8BIT capability.

Identifying semi-bidirectional ports

------------------------------------

Step 0  =>   Read the cautions and disclaimers applying to loopback

Step 1  =>   Select the port to test. The test only applies to 4BIT ports.

Step 2  =>   Attach a parallel port extension cable to the port

Step 3  =>   At the female DB-25 end, loopback Pin 8 to Pin 21 (GND).

Step 4  =>   Run Parallel

Step 5  =>   If Parallel reports "Data port has stuck bits" then the

port is likely to have semi-bidirectional capability.

We will probably build this test into the next version of Parallel.

PARALLEL.DOC                                                             Page 23

Parallel Port Information Utility  ������������������������������  User's Manual

-----------------------------------------------------------------

Using Parallel to Manually Configure Parallel Port Peripherals

-----------------------------------------------------------------

With the information that Parallel reports, you are equipped

to perform manual configuration of network adapters and other

parallel port peripheral devices, should they require such manual

configuration. Here are a few examples of how Parallel's IO address

and IRQ level are used in popular network software configuration

files when them for use with parallel port adapters.

Note that these examples also apply to our Pipx and Pndis drivers

which allow Netware Lite, Windows for Workgoups, and Lan Manager

networking using simple parallel port cables between two systems.

TestDrive versions of Pipx and Pndis are available on Compuserve,

or you can buy the commercial versions directly from us. These

drivers allow you to use your favorite network software between

two systems through the parallel port, without network adapters.

This is a handy capability for laptops, and in other situations

where you need truely portable and universal networking at low cost.

Pipx and Pndis support several different types of cables, ranging

from the low-performance "laplink" type cable you may already

have up to our fabulous Universal High-Speed Turbo Cable (wow!)

which can do parallel port networking at 200 Kbytes/sec or more.

In the following examples, assume Parallel has reported a parallel

port at IO address 3BC, IRQ 7.

These examples are not meant to be cookbook instructions, but

are offered to give you a feel for how to apply the information

displayed by Parallel. Other manual configuration parameters may or not

not be supported by specific vendors as custom keywords. Examples

are: IOMODE, IRQMODE, CABLETYPE, PORTTYPE, BITMODE, and so forth.

Netware NET.CFG file for ODI drivers

------------------------------------

PORT 3BC

INT  7

Lan Manager or Windows Workgroups PROTOCOL.INI file

---------------------------------------------------

IOBASE = 0x3BC

IRQ = 7

Other device configuration files and programs

---------------------------------------------

If you are having trouble installing a parallel port peripheral, be

prepared to run Parallel when calling your vendor for technical support.

They will be better able to help you use their custom features if you

can tell them your exact parallel port configuration.

PARALLEL.DOC                                                             Page 24

Parallel Port Information Utility  ������������������������������  User's Manual

---------------------

Technical Support

---------------------

Parallel is Freeware, and comes with Freeware-style support. It has

been extensively tested with a wide range of systems from original PCs

to 486s and works with all parallel port hardware we've seen.

We are willing to offer friendly assistance on an as-available basis,

but only via Compuserve mail (CIS 76640,203) or in the IBMHW forum.

Alas, we do not offer free telephone assistance to the general public.

If you are in dire need of technical support in solving parallel port

related problems, you may open a support account with us by contacting:

Mailing Address                           Shipping Address

---------------------------               ---------------------------

Parallel Technologies, Inc.               Parallel Technologies, Inc.

PO Box 7                                  18080 NE 68th, # A-100

Redmond, Wa 98073-0007                    Redmond, Wa 98052

(206) 869-1136

(206) 869-9767 (fax)                      ( for UPS, FEDEX, etc )

Compuserve ID# 76640,203

InterNet ID# 76640.203@COMPUSERVE.COM

The Parallel software itself, and other parallel port software

utilities and developer's libraries, example drivers, hardware

prototyping tools, contract development services, and a wide range of

OEM parallel port products are available from Parallel Technologies.

Custom versions of the Parallel Port Information Utility are available.

Please call us if your firm would benefit from a custom version of the

Parallel Port Information Utility.

Vendors of parallel port hardware are invited and requested to contact us

so that we may ensure that Parallel is compatible with their products.

We offer consulting services in the design of parallel port hardware,

parallel port peripheral devices, and DOS and Windows software.

Permission is given to distribute and use Parallel without charge,

provided you adhere to the terms and conditions of the license

agreement given below. Specifically, you may not charge anyone for

for sharing or using Parallel, and you may not modify it or change it.

Nor may you use it as part of a commercial product or service without

our written permission.

Vendors may freely distribute Parallel on an individual basis, without

charge, by electronic BBS or other means.  However, you may not include

it in or with your product without our written permission.

We welcome your comments and suggestions by Email, phone, or fax.

If you have particular features you would like to see supported by

Parallel, we'd be more than glad to hear from you.

PARALLEL.DOC                                                             Page 25

Parallel Port Information Utility  ������������������������������  User's Manual

---------------------------------------------

Other Products from Parallel Technologies

---------------------------------------------

We at Parallel Technologies' have been creating innovative parallel

port devices and software since 1989. Here are a few highlights:

o  1st ... network adapter for the parallel port

o  1st ... built-in passthrough printing port

o  1st ... automatic self-configuring drivers

o  1st ... universal support for all parallel ports

o  1st ... network adapters with multiple media support

o  1st ... Windows 3.1 multimedia driver for parallel adapter

o  1st ... Multiple Parallel Device API for shared devices

o  1st ... intelligent parasitic power for self-powered devices

o  1st ... daisychained parallel port peripherals

o  1st ... multiplexed parallel port peripherals

o  1st ... Parallel Port Expansion System

o  1st ... product development tools for OEMs and developers

o  PC MAG Editor's Choice for Ethernet parallel port NIC

o  most total network adapters  --  6 and counting...

o  most total parallel port peripherals  --  12 and counting...

We have in-depth experience with the wonderful and sometimes bizarre

world of the PC parallel port and serve on both the EPP and IEEE 1284

standards committees. Having created more than a dozen parallel port

devices with their associated drivers and software utilities, we are

the most experienced source for off-the-shelf technology, developer

tools and components, and contract services.

Here are a few of our current and forthcoming products that you may

find interesting.

Parallel -- the Parallel Port Information Utility

-------------------------------------------------

A handy utility which examines your system's parallel ports and

reports the Port Type, IO address, IRQ level, BIOS name, and an

assortment of informative notes and warnings in a compact and easy

to read display. The output may be redirected to a file for tech

support purposes. Parallel uses very sophisticated techniques for

port and IRQ detection and is aware of a broad range of quirky port

features.

Parallel is the definitive parallel port Tech Support tool.

Parallel is Freeware, subject to a restricted license agreement.

It may be used free of charge for any personal, non-commercial

purpose. Vendors should contact us regarding commercial licenses.

A forthcoming commercial version of Parallel will use an active

loopback connector enabling many new features including detailed

resolution of port IO behavior and effective throughput testing.

PARALLEL.DOC                                                             Page 26

Parallel Port Information Utility  ������������������������������  User's Manual

Parallel Port Networking

------------------------

Now you can network two PCs using your favorite network software

with just a parallel port cable between the systems.

Our family of parallel port network drivers (PIPX, PNDIS, PPACKET,

and others TBA) work with standard network software to turn your

parallel port into a network adapter.  All types of ports are

supported with a very few specific exceptions.

Supports: Netware Lite, Workgroups for Windows, MS Lan Manager,

various TCP/IP systems, and other network operating systems that

use the ODI, NDIS, or packet driver interfaces.

Performance is better than you might expect, ranging from a low

end of 40 KBps between 4BIT ports all the way up to an effective

200 KBps between EPP ports using our TURBO cable. This upper range

is better than many ethernet ISA cards.

The drivers are available with or without cables, and existing cable

types typically used with laplink type products are also supported.

The MultiPort Parallel Port Expansion System

--------------------------------------------

The MultiPort 4-port multiplexor turns your parallel port into

a general purpose expansion system, capable of supporting up to

four simultaneously operating parallel port peripherals including

network adapters, portable hard drives, tape backup units, CDROMs,

floppies, and so forth. For example, you can use the MultiPort Mux

to attach a network adapter, a hard disk, a CD-ROM, and a printer,

all operating together through a single parallel port.

Now your laptop (and desktop) can enjoy a full range of peripherals

with all the portability, sharability, and convenience of parallel

port devices.

Parallel Port Developer's Kit

-----------------------------

Pipx was constructed with Parallel Technologies' advanced parallel

port software, which has been used in over 10 parallel port devices

including network adapters, parallel port disks, parallel port

wireless IR adapters, parallel port MIDI interfaces, and so forth.

This software includes support for all types of ports, IO code

that is extremely resistant to port idiosyncrasies, and full

autodetection of all port hardware features. The code is very well

documented. A selection of sample drivers for parallel port devices

is also available.

PARALLEL.DOC                                                             Page 27

Parallel Port Information Utility  ������������������������������  User's Manual

OEM Products

------------

Most of our products are available for OEM licensing, including

network adapters, EPP-capable hard disks, parallel-SCSI interface,

parallel MIDI adapter, and the MultiPort parallel port expander.

Development Services

--------------------

We specialize in contract development of parallel port peripherals

and associated software including DOS and Windows device drivers.

Our broad and deep experience with parallel port devices and our

commitment to specialization allows us to design, implement, test,

and support them very efficiently.

You might be interested in our 30/90 development offer. We propose

to convert any ISA peripheral into a parallel port device in only

30 days to prototype, 90 days to production.

We also offer more limited development support, such as developing

additional devices drivers for existing peripherals.

We are quite experienced with EPP ports and are ready to upgrade

your existing parallel port devices to full EPP performance.

Consulting and Training

-----------------------

In support of in-house development, you will find our consulting

and training services invaluable in avoiding the pitfalls of

parallel port development while achieving the highest possible

product performance and quality.

PARALLEL.DOC                                                             Page 28

Parallel Port Information Utility  ������������������������������  User's Manual

------------------------------

Software License Agreement

------------------------------

IMPORTANT

Read this agreement before installing or using the Software. Installing

and/or using the Software on your computer indicates your acceptance of

these terms. If you do not agree to these terms and conditions, delete the

Software and all it's associated files from your computer. This Software

License Agreement gives you ONLY RESTRICTED RIGHTS to use the Software (and

all it's associated files and documentation) as follows:

YOU MAY

1.  Use the Software without charge on as many systems as you wish,

2.  Share the Software with others for their personal use, but only

if you share it intact, without any changes or modifications of

any kind, and complete with all it's documentation and associated

files. You must pass it on exactly in the same form as we release

it, or not pass it on at all.

3.  Make backup copies of the Software, so long as you place our

copyright notice on the backup copies,

4.  Request and receive limited technical support as provided in the

Freeware Product Support Policy below.

5.  Vendors may distribute the Software free of charge by electronic

or other means for the purpose of technical support, but may not

include the Software in their products without our written permission.

YOU MAY NOT

1.  Use the Software in any way except as provided in this agreement,

2.  Modify, alter, copy, transfer, or reproduce the Software in any

way, except as expressly allowed in this agreement,

3.  Translate, de-assemble, de-compile, reverse program, or reverse

engineer the Software or any of its components,

4.  Charge any kind of fee for distributing the Software without our

written permission,

5.  Use the Software for commercial purposes without our permission,

6.  Sublicense, lease, rent, sell, distribute, or otherwise dispose

of the Software without our written permission.

This license is effective from the date of first use of the Software and

shall remain in force until terminated. You may terminate this agreement by

destroying any and all copies of the Software and its documentation.

PARALLEL.DOC                                                             Page 29

Parallel Port Information Utility  ������������������������������  User's Manual

Parallel Technologies, Inc. may terminate this agreement if you fail to

comply with any of the terms and conditions of it. You agree to destroy any

and all copies of the Software and its documentation, in whatever form, if

we terminate this license agreement, and you agree to provide us written

certification of such destruction upon our request.

Statement of Exclusion of Warranty

ALL PRODUCTS DELIVERED PURSUANT TO THIS AGREEMENT ARE DELIVERED ON AN

"AS-IS" BASIS AND PARALLEL TECHNOLOGIES, ITS DEALERS AND DISTRIBUTORS,

EXPRESSLY DISCLAIM ANY AND ALL WARRANTIES OR CONDITIONS OF MERCHANTABLE

QUALITY OR FITNESS FOR A PARTICULAR PURPOSE.

Limitation of Liability

PARALLEL TECHNOLOGIES DISCLAIMS ALL WARRANTIES RELATING TO THIS SOFTWARE,

WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, AND ALL

SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY DISCLAIMED.  NEITHER

PARALLEL TECHNOLOGIES NOR ANYONE ELSE WHO HAS BEEN INVOLVED IN THE

CREATION, PRODUCTION, OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY

INDIRECT, CONSEQUENTIAL, SPECIAL, PUNITIVE, OR INCIDENTAL DAMAGES OR

DAMAGES DUE TO LOST PROFITS, DATA, OR INFORMATION ARISING OUT OF THE USE OR

INABILITY TO USE SUCH SOFTWARE EVEN IF PARALLEL TECHNOLOGIES HAS BEEN

ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS.  IN NO EVENT SHALL

PARALLEL TECHNOLOGIES' LIABILITY FOR ANY DAMAGES EVER EXCEED THE PRICE PAID

FOR THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM OF CLAIM.  THE

PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND PERFORMANCE

OF THE SOFTWARE.  This warranty gives you specific rights and you may have

other rights.

This agreement shall be governed by the laws of the State of Washington and

shall inure to the benefit of Parallel Technologies, Inc. and any

successors, administrators, heirs and assigns.  Any action or proceeding

brought by either party against the other arising out of or related to this

agreement shall be brought only in a STATE or FEDERAL COURT of competent

jurisdiction located in King County, Washington.

=================================================================

=                        FBI WARNING                            =

=       Failure to observe the restrictions of this license     =

=                     may be a violation                        =

=        of 17 United States Code sections 101 through 810      =

=      which carries severe personal and corporate penalties.   =

=================================================================

PARALLEL.DOC                                                             Page 30