A N I M

An IFF Format For CEL Animations

prepared by:

Sparta Inc.

23041 de la Carlota

Laguna Hills, Calif 92653

714) 768-8161

contact: Gary Bonham

also by:

Aegis Development Co.

2115 Pico Blvd.

Santa Monica, Calif 90405

213) 392-9972

1.0 Introduction

The ANIM IFF format was developed at Sparta originally for the

production of animated video sequences on the Amiga computer.  The

intent was to be able to store, and play back, sequences of frames

and to minimize both the storage space on disk (through compression)

and playback time (through efficient de-compression algorithms).

It was desired to maintain maximum compatibility with existing

IFF formats and to be able to display the initial frame as a normal

still IFF picture.

The basic ANIM format described here has been in use for over a year

in-house at Sparta with the XOR mode.  The delta mode is a recent,

and very effective, addition/improvement.

1.1 IFF File Format Overview

The general philosophy of ANIMs is to present the initial frame

as a normal, run-length-encoded, IFF picture.  Subsequent

frames are then described by listing only their differences

from a previous frame.  Normally, the "previous" frame is two

frames back as that is the frame remaining in the hidden

screen buffer when double-buffering is used.  To better

understand this, suppose one has two screens, called A and B,

and the ability to instantly switch the display from one to

the other.  The normal playback mode is to load the initial

frame into A and duplicate it into B.  Then frame A is displayed

on the screen.  Then the differences for frame 2 are used to

alter screen B and it is displayed.  Then the differences for

frame 3 are used to alter screen A and it is displayed, and so

on.  Note that frame 2 is stored as differences from frame 1,

but all other frames are stored as differences from two frames

back.

ANIM is an IFF FORM and its basic format is as follows (this

assumes the reader has a basic understanding of IFF format

files):

FORM ANIM

. FORM ILBM         first frame

. . BMHD                normal type IFF data

. . CMAP

. . BODY

. FORM ILBM         frame 2

. . ANHD                animation header chunk

. . DLTA                delta mode data

. FORM ILBM         frame 3

. . ANHD

. . DLTA

...

The initial FORM ILBM can contain all the normal ILBM chunks,

such as CRNG, etc.  The BODY will normally be a standard

run-length-encoded data chunk (but may be any other legal

compression mode as indicated by the BMHD).

The subsequent FORMs ILBM contain an ANHD, instead of a BMHD,

which duplicates some of BMHD and has additional parameters

pertaining to the animation frame.  The DLTA chunk contains

the data for the two available delta compression modes.  If

the older XOR compression mode is used, then a BODY chunk

will be here.  In addition, other chunks may be placed in each

of these as deemed necessary (and as code is placed in player

programs to utilize them).  A good example would be CMAP chunks

to alter the color palette.  A basic assumption in ANIMs is

that the size of the bitmap, and the display mode (e.g. HAM)

will not change through the animation.

1.2 Recording ANIMs

To record an ANIM will require three bitmaps - one for

creation of the next frame, and two more for a "history" of the

previous two frames for performing the compression calculations

(e.g. the delta mode calculations).

There are three frame-to-frame compression methods currently

defined:

1.2.1 XOR mode

This mode is the original and is included here for

compatibility with some programs which still can output

this mode.  In general, the delta modes are far superior.

The creation of XOR mode is quite simple.  One simply

performs an exclusive-or (XOR) between all corresponding

bytes of the new frame and two frames back.  This results

in a new bitmap with 0 bits wherever the two frames were

identical, and 1 bits where they are different.  Then this

new bitmap is saved using run-length-encoding.  A major

obstacle of this mode is in the time consumed in performing

the XOR upon reconstructing the image.

1.2.2 Long Delta mode

This mode stores the actual new frame long-words which are

different, along with the offset in the bitmap.  The

exact format is shown and discussed in section 2 below.

Each plane is handled separately, with no data being saved

if no changes take place in a given plane.  Strings of

2 or more long-words in a row which change can be run

together so offsets do not have to be saved for each one.

Constructing this data chunk usually consists of having

a buffer to hold the data, and calculating the data as

one compares the new frame, long-word by long-word, with

two frames back.

1.2.3 Short Delta mode

This mode is identical to the Long Delta mode except that

short-words are saved instead of long-words.  In most

instances, this mode results in a smaller DLTA chunk.

The Long Delta mode is mainly of interest in improving

the playback speed when used on a 32-bit 68020 Turbo Amiga.

1.3 Playing ANIMs

Playback of ANIMs will usually require two buffers, as mentioned

above, and double-buffering between them.  The frame data from

the ANIM file is used to modify the hidden frame to the next

frame to be shown.  When using the XOR mode, the usual run-

length-decoding routine can be easily modified to do the

exclusive-or operation required.  Note that runs of zero bytes,

which will be very common, can be ignored, as an exclusive or

of any byte value to a byte of zero will not alter the original

byte value.

2.0 Chunk Formats

2.1 ANHD Chunk

The ANHD chunk consists of the following data structure:

UBYTE operation (=0 set directly,

=1 XOR mode,

=2 Long Delta mode,

=3 Short Delta mode)

UBYTE mask      (XOR mode only - plane mask where each

bit is set =1 if there is data and =0

if not.)

UWORD w,h       (XOR mode only - width and height of the

area represented by the BODY to eliminate

unnecessary un-changed data)

WORD  x,y       (XOR mode only - position of rectangular

area representd by the BODY)

ULONG abstime   (currently unused - timing for a frame

relative to the time the first frame

was displayed - in jiffies (1/60 sec))

ULONG reltime   (timing for frame relative to time

previous frame was displayed - in

jiffies (1/60 sec))

UBYTE interleave (unused so far - indicates how may frames

back this data is to modify.  =0 defaults

to indicate two frames back (for double

buffering). =n indicates n frames back.

The main intent here is to allow values

of =1 for special applications where

frame data would modify the immediately

previous frame)

UBYTE pad[21]    (this is a pad for future use for future

compression modes.  Some of these - maybe

16 - are intentionally provided for use

by Jim Kent for operation codes for each

plane and other ancillary data he

requested)

2.2 DLTA Chunk

This chunk is a basic data chunk used to hold the delta

compression data.  The minimum size of this chunk is 32 bytes

as the first 8 long-words are byte pointers into the chunk for

the data for each of up to 8 bitplanes.  The pointer for the

plane data starting immediately following these 8 pointers will

have a value of 32 as the data starts in the 33-rd byte of the

chunk (indes value of 32 due to zero-base indexing).

The data for a given plane consists of groups of data words.  In

Long Delta mode, these groups consist of both short and long

words - short words for offsets and numbers, and long words for

the actual data.  In Short Delta mode, the groups are identical

except data words are also shorts so all data is short words.

Each group consists of a starting word which is an offset.  If

the offset is positive then it indicates the increment in long

or short words (whichever is appropriate) through the bitplane.

In other words, if you were reconstructing the plane, you would

start a pointer (to shorts or longs depending on the mode) to

point to the first word of the bitplane.  Then the offset would

be added to it and the following data word would be placed at

that position.  Then the next offset would be added to the

pointer and the following data word would be placed at that

position.  And so on...  The data terminates with an offset

equal to 0xFFFF.

A second interpretation is given if the offset is negative.  In

that case, the absolute value is the offset+2.  Then the

following short-word indicates the number of data words that

follow.  Following that is the indicated number of contiguous

data words (longs or shorts depending on mode) which are to

be placed in contiguous locations of the bitplane.

If there are no changed words in a given plane, then the pointer

in the first 32 bytes of the chunk is =0.