DMP32.DOC

Documentation for the Dual Module Player/32 v2.90

Copyright (C) 1994 Otto Chrons

DMP32.EXE is  a program  demonstrating the  capabilities of the DSMI/32

programming interface (DSMI stands for Digital Sound & Music

Interface). It's a standalone module  player running in 32-bit

protected mode with  which  you can play STM, S3M, 669, FAR and MTM

files  as well  as Amiga modules (MOD or NST). DMP32 is cardware so you

can freely distribute  it and I hope you did, as long as you include

this help file with the executable. If you like the product, please

send me a postcard to the address shown on the endscreen of DMP32!!

Here is the command line syntax:

DMP32 [options] [modulename] [@listfile] [options]

Where:

------

modulename : name of a module (wildcards *,? allowed)

listfile   : name of a file containing list of module names

options  : -Sxxxx          set sampling rate 4000-44100

alternatively 4-44

-H or -?        show help screen

-Ix             interrupt number

-Pxxx           port address (use 42 for PC speaker)

-Dx             DMA channel

-O              Scramble module order

-L              Inhibit looping

-M              Mono mode (SB16 & PAS)

-Q              Quality mode (on 8-bit cards only)

-Cx             Desired card where x is:

1 for Sound Blaster

2 for Sound Blaster Pro

3 for PAS+

4 for PAS16

5 for SB16

8 for Windows Sound System & compat.

(like AudioTrix Pro)

9 for Gravis Ultrasound

-Txxx           Buffer size xxx (0-32000)

-E              Disable extended tempos (MOD only)

-Axxx           Amplify by xxx, 31 is normal volume

-B              Disable EMS usage

-8              Force an 16-bit card into 8-bit

mode (works with Aria, PAS16 and WSS)

-Nxx            Set default panning to xx (0-63,100)

100 = Surround sound

-W[command]     Automatic DOS shell, executes optional

'command'.

-Zxx	      Use 25/50 row display

-X              Quiet screen mode. DMP32 writes nothing

on the screen

-G              Use non-DMA download mode for GUS

If you use some options all the time, you can put them into an

environment variable DMP32:

Example:

You always want to use quality mode and sampleing rate of 22kHz and

mono mode, put following line into your AUTOEXEC.BAT

SET DMP32=-q -s22 -m

NOTE!  You cannot override single letter options (-l,-o,-m,-q) with

command line parameters. All other options can be overriden.

Alternatively you can use the CmdLine parameter in DMP32.INI for same

purpose.

�������������������������������������������������������������������������Ŀ

�You may have to  give your soundcard's IRQ  number and/or port address   �

�if DMP32 fails to find them. If you don't provide both values then DMP32 �

�will assume 220h for port address, and 7 for interrupt number. Giving    �

�wrong address  will hang  your machine,  but wrong interrupt number      �

�only stops the playing after a second. The "/C" option is useful if you  �

�want to use some other card, than the one DMP32 automatically detects.   �

���������������������������������������������������������������������������

You don't  have  to add  any extension  to  the name,  because the

program will try  extensions MTM, AMF, STM, 669, S3M, FAR, MOD and NST

if it doesn't find the file. Note that you can supply many modulenames

and list files.

Example:

DMP32 beyond -s20 *.NST @goodmods

goodmods is a text file containing following lines:

occ-san

stars

intro3

DMP32  will use  the  sampling  rate  of  20KHz  and  play following

modules: BEYOND.MOD,  OCC-SAN.STM, STARS.MOD, INTRO3.MOD  and all

.NST-files in the current directory.

DMP32 has also a config file called DMP32.INI. It should be placed in the

same directory with DMP32.EXE but DMP32 will also find it if it's somewhere

in the PATH. See the file itself for explanations on config switches

and strings.

You can also play modules inside archives by specifying in DMP32.INI how

to extract files from the archive.

How to use DMP32

After DMP32 has  loaded the first  module, it starts  to play it and

shows some information about  it. DMP32 displays module's  name, its file

size, the amount of  memory used for module and sampling rate.  Because

of the AMP's Intelligent Module Loader  (IML) modules  usually take

10-30% less  space. I've  seen a 244Kb module shrink into 151Kb when I

loaded it with DMP32. While playing, DMP32 updates fields showing current

pattern, row, elapsed time, volume and sampling rate. Under these

fields are four lines showing the current status of the corresponding

tracks. Here is an example:

�������������������������������������������������������������������������

bassdrum            � F#2 � 64 �               � ���������������������

strings7            � D-3 � 20 � ARPEGGIO      � ���������������

guitar42            � E-2 � 12 � PORT.NOTE     � �����������

guitar44            � G-2 � 57 � FINE VOL      � ������������������

�������������������������������������������������������������������������

The first  field shows  the instrument  currently playing  on that

track. Second is the note value, third its volume and fourth indicates

what special command is running. The bar simulates a VU  meter on music

equipment, but you shouldn't take it  as  an exact  representation of

the music  level, it's there because it looks nice!

The rest of the screen is filled with instrument names. The numbers before an

instrument name indicate the channels that instrument is playing on.

Pressing 'H'  brings up a  quick-help showing  command line options and

keys used to control the program.

Keys :

��������

P            pause module, any key resumes

1-9          turn track xx on/off

0            turn selected track on/off

D            DOS shell (write EXIT to get back)

N            load next module (if you have supplied more than one)

S            stereo mode on/off (SB Pro), default is on

F            filter on/off (SB Pro), default is off

left/right   Jump to next/previous pattern

up/down      select track

F1-F10,+,-   set volume

[,]          Set new speed

{,}          Set new tempo

V            Real/fake volume bars

L,M,R,U      Panning full left/mid/right/surround

, .          Panning left/right

Z		Switch between 25- and 50-row display

H            Help screen

ESC          exit

Try this module player  with all the modules  you have, and if you

encounter problems with any modules, inform the author. Have fun.

�����������������������

Contacting the author

�����������������������

���������������������������������������������������������������������Ŀ

�                                                                     �

�   By mail:                                                          �

�   ��������                                                          �

�                                                                     �

�            Otto Chrons                                              �

�            Vaajakatu 5 K 199                                        �

�            FIN-33720 Tampere                                        �

�            FINLAND                                                  �

�                                                                     �

�   InterNet:                                                         �

�   ����������                                                        �

�            otto.chrons@cc.tut.fi                                    �

�                                                                     �

�����������������������������������������������������������������������

Newest version is always available at:

������������������������������������������������Ŀ

� R.A. LAW         +358-37-5849007           24h �

� ftp.wustl.edu    /pub/MSDOS_UPLOADS/modplayers �

��������������������������������������������������

See CABINET.DMP for a list of DMP/DMP32 support sites in Finland.

See DMP32.REV for revision history.