TIMESET-UK (tm)

Version 7.20uk

Accurate Time for Your Computer by Modem

Copyright 1993, Life Sciences Software

Stanwood, Washington, U.S.A.

Introduction

TimeSet-UK is a specially designed program for getting accurate time for your

computer by modem from a commercial digital time source serving serving the

British Isles.  This service updates its own system clock frequently and

regularly from international atomic clock time services and uses internal

clock stabilisation to ensure the highest possible clock accuracy for your

computer.

The program features automatic reconfiguration for Summer Time and Winter

Time, using the current British time season rules (Summer Time begins on the

last Sunday in March, Winter Time the day after the third Saturday in October,

both at 02:00:00).

Once you have configured TimeSet-UK, using it is very simple.  All you have

to do is press a function key to initiate the call.

The program will work with any computer from a PC on, but maximum performance

can be expected only with a 286 computer or later.

Configuring TimeSet-UK

This is probably the most important section of this manual.  Please read it

carefully and refer to it again if you have any problem running TimeSet-UK.

In almost every case, problems with running TimeSet-UK are due to improper

configuration.

If your copy of TimeSet-UK has never been used before, it will automatically

present the configuration screen when you first run it.  If you have

downloaded your copy from a BBS, there is a good chance that someone has

already configured the program for his own system.  In that case, the program

present the main screen when you load it.  That screen is recognizable by the

big running digital time display.  In that happens, you should press F3 to

get to the configuration screen.  When the configuration screen appears,

press E to get into editing mode, so you can change each configuration item

as needed.

The configuration consists of six lines of data that appear at the tope of

the configuration screen, as in this example:

Time season:  WINTER TIME

Port configuration:  COM1    3F8    IRQ4

Modem speed:  1200

Dialing method:  TONE

Modem initialization:  &C1&D2X4

Phone number prefix:

Press E to get into editing mode.  This will cause the data fields to the

right of the labels to be highlighted and will move the cursor to the start

of the first entry, which in this example is WINTER TIME.  You can move to

each line to make changes by pressing the up-arrow or down-arrow key.  On any

given line, you can use the right-arrow, left-arrow, HOME and END keys to

position the cursor anywhere on that line where you wish to make a change.

To remove a character at the location of the cursor use the DELETE key.  To

insert a character between existing characters, simply move to the correct

location on the line and type it (the configuration editor opens in insert

mode by default).  If you wish to replace one character with another, press

the INSERT key once to get into strikeover mode, then move to the desired

location and type the character.  To replace an entire line, move the cursor

to the far left and press Delete repeatedly until the existing line is empty,

then type in your new data.An easy configuration option is now available via

the CTRL-I option,you simply enter CTRL-I option at the relevant field you

will then see a list a valid options to choose from,this is the recommended

way of configuring timeset.

As you scroll from line to line while in the editor, a help window will

automatically appear on your screen to list options for that line.

Each configuration line is discussed in the following sections.

Time Season

One of three options must be entered on this line: SUMMER TIME, WINTER TIME,

or UNIVERSAL TIME.  (The "TIME" part is optional.)  Summer Time is the period

between late March and late October when our clocks are set ahead one hour.

Winter Time is the period between late October of the current year and late

March of the following year, when our clocks are set normally.  If what you

see on the Time Season line does not match the current time season, simply

delete what is there and type in the correct season.

If you aren't sure what season it is (which is not uncommon near the

transition times), do not leave the field empty, make a guess.  TimeSet-UK

automatically calculates the correct starting and ending dates for Summer

Time and Winter Time for any given year each time you run it, and if you have

guessed wrong about the current season, the program will automatically

reconfigure itself for the right season and adjust the clock accordingly the

first time you place a call to the telephone time service.  As soon as

TimeSet-UK captures the time and date information it compares the captured

date with what you entered as the time season and makes any necessary

adjustment.  The same mechanism operates when the seasons actually change,

so it will never again be necessary for you to manually reconfigure the Time

Season; the program will reconfigure itself automatically the first time it

is run after any season change.

Not many users will need to select UNIVERSAL TIME.  Universal Time is another

name for Greenwich Mean Time.  It is an international reference time (the

time at 0 degrees longitude) and it has no seasons; that is, it is not

advanced by an hour in early spring and set back an hour in autumn.  This

option is for people who want their computer clocks set to universal time for

astronomical or other special purposes.  Airlines, for example, operate

internally on Universal Time (though this is not reflected in the arrival and

departure schedules they provide the public, which are in civil time and do

follow time seasons).

Port Configuration

This line consists of three groups of characters as follows:

COMx      yyy     IRQz

where x, yyy, and z are numbers.   x is the number of the serial port to

which your modem is attached.   yyy is the address (in hexadecimal notation)

that your system uses for that port.   z is the number of the IRQ (Interrupt

Request line) your system uses for that port.

You need to be very careful when you edit this line, particularly with the

port address and IRQ (yyy and z).  Refer to the help window, which opens

automatically when you reach this line.  The window is a table of standard

addresses and IRQs for COM1 to COM8, with the column on the left for standard

computers and the one on the right for PS/2 computers.  Scan the column that's

appropriate for your kind of machine until you see the COM port to which your

modem is attached, then edit the Port Configuration line to match it.  If you

are using a nonstandard IRQ number for a particular port, you will need to

put that number immediately after "IRQ" instead of the number that's listed

in the table.

Modem Speed

Since the time signal generator system you'll be calling transmits at 1200

bps, this is the preferred modem speed setting for TimeSet-UK.  It is

possible with some modems to lock the local speed high, at 38400 bps for

example, while the line speed drops to 1200 bps to match the remote after

connection is made, but nothing is gained by it.  It's best to set your modem

speed to 1200 in the first place and be done with it.  If you do opt to

maintain a high local speed, you'll need to put appropriate commands in

TimeSet-UK's modem initialization string, which is discussed in a later

section.  Depending on the quality of your modem, a high local speed can

interfere with proper functioning in TimeSet-UK.

Dialing Method

You have two choices:  TONE dialing or PULSE dialing.  If you type in TONE,

TimeSet-UK will put ATDT in front of the phone number.  If you type in PULSE,

the program will use ATDP.

Modem Initialization

If your modem is a simple asynchronous modem that can accept software

configuration commands and lacks such refinements as error correction, data

compression, and protocol negotiation with the remote modem, the only

initialization commands you'll need are &C1 (wait for a carrier), &D2 (hang

up when DTR is turned off) and X4 (use a result code set that includes BUSY,

CONNECT, NO DIALTONE, and NO CARRIER).  There is no need to separate any

modem commands with spaces; they can be run together:  &C1&D2X4.

If you have an old modem that is configured with switches rather than with

software commands, you'll only need X4 on the modem initialization line, but

you'll have to flip appropriate switches in the modem to force it always to

wait for a carrier and to hang up when the DTR is turned off.

Whether done with software commands or with switches, TimeSet-UK will not

work correctly unless the modem is configured to wait for a carrier and to

hang up when DTR is turned off.  This means that &C1&D2 should be part of any

TimeSet-UK initialization string for a modem that accepts software commands,

unless they are default conditions in a particular modem (which is the case

with certain brands).

Configuring for Data Compression/Error-Checking Modems

As modem prices continue to drop, more and more people are buying new high-

speed modems capable of using data compression and error checking protocols.

All of these nice features should be turned off when you're trying to get

time by modem.  Your new modem should be configured to run in what is called

"basic" or "direct" asynchronous mode, in other words just like an

old-fashioned modem.  Here is where things can get a bit complicated, because

different modem manufacturers use different commands to achieve that state.

Many of them use &Q0, but some use some other command.  You'll need to check

your own modem manual and, if necessary, call the modem manufacturer's

technical support department to find out how to turn off all the frills and

get your modem into basic asynchronous mode.

In addition to a command for basic asynchronous mode, you'll need a command

to defeat negotiation with the remote modem.  Fortunately, the command for

that doesn't seem to vary among manufacturers.  It's N0 (N zero).  Next,

you'll need to set a speed register to force connection at 1200 bps.  The

command for that is S37=5 for most of the newer modems.

Putting it all together, an initialization string that allows TimeSet-UK to

work with several brands of high-speed protocol modems is:

&Q0N0S37=5&C1&D2X4

If this string doesn't work with your modem, consult your modem manual for

equivalent commands for basic asynchronous mode, no negotiation, and forced

connection at 1200 bps.  &C1&D2X4 means the same thing in all Hayes-compatible

modems and should not be changed.  It's what goes in front of that group that

can vary by modem brand.

Note that "AT" should not be in your modem initialization string since

TimeSet-UK supplies the necessary attention codes itself.

Please note that Timeset(UK) will support error correction MNP levels

2-4,these protocols inevitably introduce slight delays in time string

processing.So to an extent there is a tradeoff between accuracy and

precision.

Phone Number Prefix

The time service phone number is already coded into TimeSet-UK, so you never

have to think about it.  There are situations, however, where an actual phone

number has to be preceded by some other code.  For example, in an office

environment you may need to enter a number to get an outside line before you

can dial the actual phone number.  The Phone Number Prefix line is where you

should place that code, using commas to create any needed pauses.

Saving the Configuration

Before saving the configuration, check your work to make sure there are no

errors, paying special attention to the Port Configuration and Modem

Initialization lines.  When you're satisfied that everything is correct,

press F10 to save the configuration to TimeSet-UK's source file (TSETUK.COM).

As soon as the data are saved, the program will switch to the opening screen,

ready to make a call to set your computer's clock.  Whenever you wish to

inspect or change your configuration, simply press F3 at the opening screen.

If you're just checking, you can get back to the opening screen by pressing

F3 again.  If you want to make make changes press E to edit.  To save your

revised configuration, press F10.  If you change your mind and don't want to

save your revisions, press F3, and you'll be returned to the opening screen

with the older values preserved.

Calling for Time

Initiating the Call

To call press F1 at the opening screen.

the software should then receive time data strings from the time service.

Strings are displayed on your monitor as they are received, one every second.  Several

strings are "wasted" to provide time for modem "jitter" to abate.  If there

is no corruption by line noise, the 10th string is captured for parsing and

time setting.

Your switchboard may have call barring against 0891 prefix calls if so ask

your system manager to remove it for your extension, many newer boards will

allow you to make calls to a specific number but not numbers of a general

prefix eg you can call 0891 616717 but not other 0891 numbers.

The Summary Screen

The final screen summarizes the results of the call.  The top half displays

the string that was captured and the data that was extracted and calculated

from it.  The lower half displays the time values put into the computer at

the instant of time setting:  the computer clock time before and after, the

date before and after, and the time season.  The final screen also shows when

the next time season change will occur.  If TimeSet-UK is configured for

Winter Time, the date and hour for the next switch to Summer Time are shown.

If the program is configured for Summer Time, the date and hour for the next

switch to Winter Time are shown.  If TimeSet-UK is configured for Universal

Time, both dates are shown for the current year, but only for reference;

since Universal Time has no seasons TimeSet-UK will not make any automatic

reconfiguration on those dates.Please note the date appears in ISO format

but will not affect the way in which time/date is shown by DOS,Windows or

your application.

Your options at the final screen are to press the Backspace key to return to

the opening screen, press the Spacebar to redial, or press ESC to exit to DOS.

These same keys function the same way on any TimeSet-UK screen when a call is

in progress.  On most screens the desired action will occur immediately.

If you press them while line delay measurement is in progress, however, it

may take several seconds before you see a response.  This is because the

program is devoting most of its attention to measuring the delay accurately,

and checks the keyboard only occasionally.

Running TimeSet-UK from a Batch File

The description so far has been on how to run TimeSet-UK by loading it and

pressing a key to initiate a call for time.  The program can also be run

automatically from an AUTOEXEC.BAT file by including putting tsetuk /A

the command line./A means call automatically without line delay measurement.

In an automatic run,the program loads, immediately dials the time service,

sets the clock, and exits to DOS or back to the AUTOEXEC file.

Putting TimeSet-UK in your AUTOEXEC file will cause a call to the time service

each time you boot your computer.  This may be more frequent than you desire,

or more frequent than necessary if RighTime is installed and has learned its

clock correction factor.  If so, you can use the batch file enhancer program

If2Day to limit automatic calls to a certain day of the week or month.

If2Day is a free program by Gary Miller, who has kindly granted permission to

include his program in the TimeSet package.

If you wanted TimeSet-UK to call the time service on bootup every Wednesday

morning, you could put the following line in your AUTOEXEC file:

If2Day is_Wednesday aft_8:00 B4_9:00 run TSETUK /A

which translates to, "If today is Wednesday and the time is after 8 am and

before 9 am, run TimeSet-UK automatically with line delay measurement."

The documentation that accompanies If2Day also shows how to specify a run

between specific hours on a particular day of the month.

Summary of Command Line Switches

Several command line switch options are available for TimeSet-UK.

They include:

/M -- Force a monochrome display.  This may be useful in certain

laptop computers that might not show sufficient contrast between

TimeSet-UK's screen colors.

/P -- Generate a pulse at the parallel port at the instant the clock is

set.  The pulse generated when /P is invoked occurs at pin 1 of the

parallel port, where the voltage at the instant of the pulse drops from

the normal +5 volts to zero (with respect to ground, pin 25) for about

55 msec.  The pulse can be inverted, shaped, or amplified to trigger an

external electronic device.  The default port for this action is LPT1,

but another command is available if you wish to use another parallel

port (see next item).  This function should not be invoked if a printer

is attached to the selected port.

/LPTn -- Select a printer port other than the default LPT1 for pulse

generation.

/0P -- Generate a pulse at the default or selected parallel port after

the computer clock has been set, but wait until seconds = 0, that is,

until the start of the next minute.  This is useful for calibrating

certain electronic clocks that can be preset to the next minute,

inactivated, then triggered into activity with an electrical pulse.

When this switch is used, a countdown to zero seconds is displayed

after the computer clock is set.  At zero seconds the pulse is generated

at the selected parallel port.

/0C --  This is similar to /0P, except that a pulse is not generated

when the countdown reaches zero.  It's mainly for those who want a

countdown to synchronize their watch or clock, although this can be

done readily with the large clock display at the opening screen,

especially if one-second audible ticks are toggled by pressing the

Delete key.

/LF --  Open and maintain a log file for timesetting action.  The file

created, which can viewed with any editor or with the TYPE

command in DOS, displays relevant before/after clock information

that can reveal how much your computer clock drifts.  It also includes

data on RighTime's warm and cold correction rates, if you have

RighTime 2.54 or later installed.

/FS --  Stands for "Fast Screen."  This command eliminates the

"parting curtains" effect when TimeSet-UK changes displays a new

screen, causing them to snap into view instead.

TimeSet-UK and Windows

It is possible to run TimeSet-UK under Microsoft Windows.It's best to run it

in DOS either before you start Windows or after you have shut it down.  The

reason it's not a good idea is that Windows is so busy with the system that it

sets the new time when it gets around to it.  As a result, clock settings made

under Windows are always a fraction of a second slow.  If that is acceptable,

then go ahead and use TimeSet-UK in Windows, although the timesetting delay,

which may be variable, will probably prevent RighTime from developing a

stable correction for your computer's clocks.

If you run TimeSet-UK in Windows 386 Enhanced Mode, you'll need to edit

TimeSet-UK's PIF to give the program maximum priority (10000) and to let no

other program share its idle time.  You'll also need to run the program in

full text mode, not in a graphics window, because painting a graphics window

slows things too much.  It will also help to load TimeSet-UK with the

fast-screen (/FS) switch.There is also an icon included in the package

called TSETUK.ICO

The package Clock manager from Graphical Dynamics (Seattle,Washington) is

included in the professional package.This enable you to launch applications

at a given time and date.For you convenience two batch files are included

ONEPC.BAT will call Timeset(UK) for a single PC while NETPC.BAT will call

Timeset(UK) and then transfer this time to the server (note the calling

workstation must have sufficient privelege to transfer time for more

details see Novell networks section below)

Timeset(UK) and Novell Networks

A PD utility called servtime has been included in the package to allow a

workstation to transfer its date/time to the server.Note the calling

workstation must be logged onto the network with supervisor privelege.A

batch file is included (NETPC.BAT) to call Timeset(UK) and then update

the server , PC's then logging onto the system will then pick up the

correct date/time.

If you wish to lock any PC's on a network to the server use the utility

NOVSYNC.SYS on each PC you wish to lock (further details in NOVSYNC.DOC),

this will prevent users or their software altering the time on their PC.

NOVSYNC.SYS is only included in the professional release.

If there are any PC's that you wish not to get their time from the

server (eg If you've got Righttime installed on the master clock PC) the

procedure is as follows

- The login shell must be level 3.70 or above.

- Dosup7 should be installed.

- You put "SET_STATION_TIME OFF" in users login script.

When this user logs into Novell there time/date will not reflect the servers

time/date.

An ideal way to lock an entire network of PC's to the correct date/time for

only 20 pence per call is to do the following:

- Nominate one PC as the master clock with this PC having console

priveleges,Righttime loaded , no novsync.sys and server time/date

update disabled.

- All other PC's pick up time/date from server and if desired have

Novsync loaded.

- The master time pc has a program launcher loaded (for windows clock

manager is recommended) this then initiates (say) weekly calls to

Timeset(UK).

Timeset(UK) and Windows for Workgroups

The software Netsync from David Warren(Pleasant Hills,California) is included

in the professional package.As at the time of writing (October 1993) There are

few sites in the UK using Workgroups for Windows I feel it best you refer

directly to the documentation in NETSYNC.WRI.

Using AUTOTSET.COM

AutoTset is a memory resident program that can trigger dialing by TimeSet-UK

at a specific time every day or as long as you keep it memory-resident.  This

is most useful for people who never turn off their computers.  With AutoTset

preloaded, TimeSet-UK will wait at its opening screen until the appointed

time, when it will be triggered by AutoTset to dial, with or without line

delay measurement depending on how AutoTset is loaded.

Instructions for loading AutoTset are available just by typing AutoTset at

the DOS prompt:

autotset hh:mm

This triggers dialing without line delay measurement at the appointed time

(hh:mm).  In setting the dial time, follow the DOS convention, where midnight is 00:00

and 1:00 pm is 13:00.  If you want the call to occur on the hour, it's not

necessary to add :00 for minutes;  the program will accept 3 as equivalent

to 03:00.In order to avoid congestion if you you use autodial please put in

an irregular time eg 02:27 and not exact hours or quarter hours.

To remove AutoTset from memory, type AutoTset /U.