SIMPLE SIMON SOFTWARE

POB 926

BEDFORD, TX  76095

(817) 267-6773 VOICE

(817) 283-5871 FAX

SSFAXER is a program with a one line command for faxing.  What could

be more simple than to just type in the com port, phone number and the file

name.

ABOUT SSFAXER

At the C:\SSFAXER prompt type:

SSFAXER

The following will appear on the screen:

Simple Simon Faxer Version UNREGISTERED 3.00

(c) 1992, 1993 M. Bates (817) 267-6773

Usage is  :  SSFAXER PORT PHONENUMBER FILENAME

PORT         - 1...4 Valid Com/Fax Port Number

PHONENUMBER  - T or P (Touchtone or Pulse + Phone Number)

FILENAME     - Complete Path to (1...5) ASCII or FAX G3 file(s)

i.e.  SSFAXER 4 T8172835871 C:\AUTOEXEC.BAT

------------------------------------------------------------------------------

If the fax phone number is local, the command line will be as follows:

C:\SSFAXER>SSFAXER 4 T2835871 C:\AUTOEXEC.BAT

If the fax phone number is long distance, the command line will be as follows:

C:\SSFAXER>SSFAXER 4 T18172835871 C:\AUTOEXEC.BAT

If there is more than one file to fax, the command line will be as follows:

C:\SSFAXER>SSFAXER 4 T18172835871 C:\AUTOEXEC.BAT C:\WORK>TEST.DOC

Five files can be faxed during one fax transmission.

RETURNS AFTER EXECUTION OF SSFAXER

Example:  Okay is a return.

After SSFAXER has been successfully executed, the following will appear on the

screen:

C:\SSFAXER>SSFAXER 4 T8172835871 C:\AUTOEXEC.BAT

Simple Simon Faxer Version UNREGISTERED 3.00

(c) 1992, 1993 M. Bates (817) 267-6773

Dialing

Phone Answered

Exchanging Info

Sending Document

Phone Hang Up

Okay

If the fax number called is busy, the following will appear on the screen:

C:\SSFAXER>SSFAXER 4 T8172835871 C:\AUTOEXEC.BAT

Simple Simon Faxer Version UNREGISTERED 3.00

(c) 1992, 1993 M. Bates (817) 267-6773

Dialing

Line Busy

If the fax number called is not answered by the remote fax or if an telephone

operator message intercepts the call, the following will appear on the screen:

C:\SSFAXER>SSFAXER 4 T8172835871 C:\AUTOEXEC.BAT

Simple Simon Faxer Version UNREGISTERED 3.00

(c) 1992, 1993 M. Bates (817) 267-6773

Dialing

NO Answer, BAD Answer

If any return message other than those shown above appear on the screen under

the command line it will most likely appear as follows:

C:\SSFAXER>SSFAXER 4 T8172835871 C:\AUTOEXEC.BAT

Simple Simon Faxer Version UNREGISTERED 3.00

(c) 1992, 1993 M. Bates (817) 267-6773

Dialing

Phone Answered

Sending Document

Unknown ERROR  (and a number)

If Unknown ERROR is returned when using SSFAXER, fax us a screen print of the

error with the number and we will fax information back to you.