TERMINAL PROGRAM FOR USE WITH THE KANTRONICS UTU INTERFACE

THIS PROGRAM IS FULLY OPERATIONAL FOR ANY ONE USING A KANTRONICS UTU.

IT CAN BE RUN FOR DEMONSTRATION PURPOSES OR PRACTICE WITHOUT A TU.

BEFORE GOING ON THE AIR, HOWEVER, THE FOLLOWING PROGRAM MODIFICATIONS

MUST BE MADE:

1 UNDER THE PROGRAM LINE SENDCALL: ENTER "DE  UR CALL SIGN " FOR d$

2 CHANGE LINES 10 THRU 90 TO ENTER MESSAGES DESIRED

EVEN IF YOU LET THE ORIGINAL MESSAGES STAND YOU MUST ENTER

YOUR CALL ON LINES 80 AND 90 AND OF COURSE YOUR OWN HANDLE AND QTH

ON LINE 10

3 NEW DISK STORED MESSAGES CAN BE ENTERED FROM WITHIN THE PROGRAM

BY PRESSING DOWN ARROW AND SELECTING ITEM 2 FROM THE MENU

A BRIEF DESCRIPTION OF PROGRAM OPERATION FOLLOWS:

RECEIVE MODE:

(THIS PROGRAM WILL RECEIVE BAUDOT,CW,ASCII AND AMTOR)

IT IS NECESSARY FOR THE UTU TO BE TURNED ON FIRST BEFORE RUNNINTG THE

PROGRAM SINCE THE PROGRAM INITIATES THE UTU IN THE RTTY 60 WPM MODE AND

TOGGLES THE UTU'S  F E X  OPTIONS. (ie. turns (F)ull menu and (E)cho "off"

AND (X)ON "on")

AFTER INITIATION AND A BRIEF WAIT, THREE WINDOWS ARE OPENED UP FOR DISPLAY.

THE LARGE WINDOW COMPRISING THE UPPER 3/4 OF THE SCREEN IS CALLED THE

RECEIVE/TRANSMIT WINDOW AND ALL RECEIVED TEXT IS DISPLAYED THERE. THE

SMALLER, BLACK SCREEN IS THE TYPE AHEAD BUFFER.  WHILE IN THE RECEIVE MODE

ALL KEYSTROKES ARE DISPLAYED IN THIS WINDOW FOR LATER TRANSMISSION.

CORRECTIONS CAN BE MADE WHEN TYPING AHEAD BY USING THE BACK SPACE KEY.

THE NARROW WINDOW RUNNING VERTICALLY ON THE RIGHT HAND SIDE OF THE

SCREEN CAN BE OPENED WHILE EITHER IN THE RECEIVE OR TRANSMIT MODE BY

PRESSING THE F9 FUNCTION KEY.  WHEN THIS IS DONE, AN ABBREVIATED

DESCRIPTION OF THE VARIOUS CANNED MESSAGES AVAILABLE ARE DISPLAYED.

PRESSING THIS KEY ALSO DISPLAYS A "*" IN THE TYPE AHEAD BUFFER.  AT THIS

POINT YOU MAY PRESS ANY KEY FROM 1 THRU 9 OR F1 THRU F6 TO SELECT THE

DESIRED MESSAGE.  THIS SELECTION IS ALSO DISPLAYED IN THE BUFFER WINDOW

IMMEDIATELY FOLLOWING THE ASTERISK.

MESSAGE HANDLING IS DESIGNED SO THAT CERTAIN MESSAGES SUCH AS HANDLE,

STATION DESCRIPTION ETC. CAN ONLY BE SENT ONCE PER QSO.  THEY ARE ALSO

ERASED FROM THE DISPLAY WINDOW AFTER BEING SENT, SO ONE CAN SEE AT A GLANCE

AFTER PRESSING THE F9 KEY WHICH MESSAGES ARE STILL AVAILABLE FOR

TRANSMISSION.  ONCE A NEW CALL HAS BEEN INPUT AND STORED (NEW QSO) THEN

ALL MESSAGES ARE REACTIVATED.

STORING THE CALL SIGN OF THE STATION YOU WISH TO CALL OR ESTABLISH A QSO

WITH IS DONE BY PRESSING THE UP ARROW.  A PROMPT THEN ASKS YOU TO ENTER THE

CALL SIGN.  FIRST, HOWEVER, A PROMPT ASKS IF YOU WANT TO ERASE FROM MEMORY

ALL PREVIOUS TEXT.  IF YOU DO NOT RESPOND BY PRESSING "E" THEN ALL PREVIOUS

TEXT REMAINS IN MEMORY.  THIS PROGRAM CAN STORE 500 LINES (SEVERAL HOURS)

OF TEXT INCLUDING THAT WHICH HAS BEEN TRANS-MITTED AS WELL AS RECEIVED.

LATER THIS TEXT MAY BE STORED TO DISK OR PRINTED IF DESIRED.  IT SHOULD BE

NOTED, HOWEVER, THAT MEMORY STORAGE OF RECEIVED OR TRANSMITTED TEXT ONLY

OCCURS WHEN AN INPUT IS ENTERED AND STORED FROM THE UP ARROW PROMPT. IF FOR

EXAMPLE, THE RETURN KEY IS PRESSED INSTEAD OF ENTERING  A VALUE FOR THE

CALL SIGN, THEN NO FURTHER MEMORY STORAGE OF TEXT OCCURS UNTIL THE UP ARROW

IS AGAIN PRESSED AND AN INPUT IS MADE IN RESPONSE TO CALL SIGN PROMPT.

FROM THE RECEIVE MODE YOU CAN CHANGE THE OPERATING MODE BY PRESSING THE

DEL KEY WHICH DISPLAYS A MODE OPTION MENU TO SELECT FROM.  THIS PRESENTS

CHOICES OF BAUDOT, ASCII, CW, AMTOR B OR AMTOR L.  ANOTHER CHOICE IS SHOWN

ON THIS MENU WHICH MAY BE USED ON RARE OCCASIONS.  THIS CHOICE IS SELECTED

BY PRESSING 'O'.  ASSUMING A UTU IS CONNECTED TO THE SERIAL PORT OF THE

COMPUTER, THE UTU RESPONDS BY DISPLAYING A LIST OF SET UP OPTIONS ON THE

SCREEN.  IT IS  IMPORTANT FOR THE CORRECT OPERATION OF THIS PROGRAM TO MAKE

SURE THE UTU'S XON OPTION IS ON AND ECHO IS OFF.  IT IS NOT NECESSARY TO

CHANGE ANY OTHER OPTIONS FROM THEIR DEFAULT VALUES, HOWEVER, IT IS

RECOMMENDED TO HAVE THE FULL MENU OPTION OFF.  AFTER COMPLETING ANY UTU SET

UP CHANGES, THE PROGRAM GOES BACK TO THE MODE OPTION MENU FOR RE-SELECTION

OF THE OPERATING MODE.

IN THE RECEIVE MODE YOU CAN ALSO INPUT A CHANGE IN SPEED BY PRESSING

THE RIGHT ARROW AND THEN SELECTING THE DESIRED SPEED FROM THE DISPLAY.

PRESSING THE LEFT ARROW INITIATES A PROMPT TO INPUT A TEMPORARY MESSAGE.

THIS ALLOWS THE STORAGE OF A MESSAGE IN THE PROGRAM MEMORY AND MAKES IT

HANDY FOR MAKING COMMENTS REGARDING THE DAY'S WEATHER ETC.  YOU WILL SEE

THIS MESSAGE SYMBOL DISPLAYED IN THE MESSAGE WINDOW WHEN THE F9 KEY IS

PRESSED. IT IS MESSAGE 4 AND IS DESIGNATED AS TMPY.

MESSAGES 1 THRU 3 AND 5 THRU 9 ARE PERMANENT AND CAN ONLY BE CHANGED

BY CHANGING THE APPROPRIATE PROGRAM LINES.  MESSAGES F1 THRU F6 ARE

SAVED ON DISK. THESE CAN BE CHANGED  FROM THE MENU OBTAINED WHEN THE

DOWN ARROW IS PRESSED.

PRESSING THE DOWN ARROW CLOSES THE 3 WINDOWS AND RESULTS IN THE DISPLAY

OF A SPECIAL MENU USED FOR MAKING UP AND STORING NEW DISK MESSAGES,

SAVING QSO'S TO DISK, PRINTING QSO'S IN MEMORY OR FROM DISK, DISPLAYING

BRIEF PROGRAM INSTRUCTIONS OR QUITTING THE PROGRAM.

IF YOU SELECT THE OPTION TO PRINT QSO'S THAT HAVE BEEN PREVIOUSLY STORED

TO DISK, A PROMPT THEN ASKS FOR THE INPUT OF FILE NAME.  THIS IS THE NAME

WHICH WAS ASSIGNED WHEN TEXT WAS SAVED TO DISK.  ANOTHER PROMPT THEN

REQUESTS THE INPUT OF A CALL SIGN. THIS IS THE CALL SIGN THAT WAS ENTERED

PREVIOUSLY IN RESPONSE TO THE UP ARROW PROMPT.  THE RESULT IS A SELECTIVE

PRINT OUT OF A PARTICULAR QSO RATHER THAN PRINTING THE ENTIRE TEXT FILE.

PRESSING THE ESCAPE KEY TOGGLES THE PROGRAM FROM RECIEVE TO TRANSMIT

AND VICE VERSA.

TRANSMIT MODE:

(NOTE THIS PROGRAM IS NOT DESIGNED FOR TRANSMITTING AMTOR A 'MASTER'

AMTOR A CAN BE MONITORED BY SELECTING AMTOR L FROM THE MENU)

WHEN FIRST ENTERING THE TRANSMIT MODE YOU HAVE ONLY TWO OPTIONS. YOU CAN

PRESS ESCAPE AND RETURN TO RECEIVE OR YOU CAN PRESS THE F8 FUNCTION KEY.

ASSUMING A CALL HAS BEEN INPUT FOR STORAGE, THIS FORCES AN ID.  IF NO CALL

HAS BEEN STORED THEN ONLY THE OPERATORS CALL IS TRANSMITTED.  REMEMBER, THE

PROGRAM WILL RESPOND TO ONLY EITHER OF THESE TWO KEYS AT THIS POINT.

ASSUMING THE F8 KEY WAS PRESSED AND THE ID SENT, THEN THE PROGRAM IS NOW

IN THE IMMEDIATE MODE.  ANY KEY REPRESENTING BAUDOT CHARACTERS WHICH IS

PRESSED WILL CAUSE IMMEDIATE TRANSMISSION OF THAT CHARACTER.  OF COURSE THE

ESCAPE KEY WILL TOGGLE THE PROGRAM BACK TO THE RECEIVE MODE.

( NOTE: AT THIS POINT CANNED MESSAGES CANNOT BE SENT FROM THE IMMEDIATE

MODE IF THERE IS ANY TEXT STORED IN THE TYPE AHEAD BUFFER. )

NOW ASSUMING NOTHING MORE IS DESIRED TO BE TRANSMITTED FROM THE

IMMEDIATE MODE, THEN TRANSMISSION OF BUFFER STORED TEXT OR MESSAGES CAN BE

INITIATED BY PRESSING THE F10 FUNCTION KEY.  OF COURSE WHILE THE STORED

TEXT IS BEING SENT, ADDITIONAL TEXT MAY BE ENTERED INTO THE TYPE AHEAD

BUFFER, HOWEVER THIS BUFFER WILL STORE ONLY EIGHT LINES AT ANY ONE TIME.

AS THE TEXT SCROLLS ADDITIONAL LINES MAY BE ADDED SO THAT THE OPERATOR

CAN THEORETICALLY BE TYPING 7 LINES AHEAD CONTINUOUSLY.

WHEN A CANNED MESSAGE IS BEING SENT FROM THE BUFFER AND THE OPERATOR

DECIDES HE WOULD LIKE TO ABORT THE MESSAGE HE CAN DO SO BY PRESSING

THE F10 KEY.  THE MESSAGE IS THEN TERMINATED AND THE TRANSMISSION THEN

SKIPS TO THE NEXT BUFFER CHARACTER AMD CONTINUES ON.

IF THE TEXT BEING SENT FROM THE BUFFER FINALLY CATCHES UP TO THE TEXT

BEING INPUT FROM THE KEYBOARD THEN THE PROGRAM IS AGAIN AUTOMATICALLY

RESTORED TO THE IMMEDIATE MODE.  IT SHOULD BE NOTED THAT CERTAIN

MESSAGES SUCH AS CQ AUTOMATICALLY RETURNS THE PROGRAM TO THE RECEIVE

MODE UPON COMPLETION OF THE MESSAGE TRANSMISSION.  ALSO NOTE THAT IF

THE F8 FUNCTION KEY IS PRESSED WHILE IN THE TYPE AHEAD MODE A RED "i"

IS STORED IN THE BUFFER WINDOW.  WHEN THE TRANSMISSION REACHES THIS

POINT THE ID IS SENT AND THE PROGRAM AUTOMATICALLY GOES TO RECEIVE.

ANOTHER HELPFUL FEATURE BECOMES APPARENT WHEN TRANSMITTING.  AS TEXT IS

SENT FROM THE TYPE AHEAD BUFFER IT IS ERASED FROM THIS WINDOW AND DISPLAYED

IN THE TRANSMIT WINDOW.  THE OPERATOR CAN ALWAYS TELL BY GLANCING AT THE

SCREEN THE STATUS OF THE TYPE AHEAD BUFFER.

PROGRAM HINTS:

ON OCCASSION AN UNSOLICITED APPEARANCE OF A MENU GENERATED BY THE UTU MAY

BE PRINTED ON THE RECEIVE SCREEN.  THIS IS CAUSED BY A FAILURE OF

COMMUNICATION BETWEEN THE COMPUTER AND UTU WHEN INITIATING A MODE

SELECTION.  IF IT SHOULD OCCUR THEN SIMPLY PRESS THE DEL KEY AND RE-SELECT

THE DESIRED MODE.  IN STUBBORN CASES RESULTING FROM THE UTU BEING OUT OF

SYNC, TURN THE UTU OFF AND THEN BACK ON.  NOW SELECT THE 'O' OPTION AND

WHEN THE UTU SENDS THE OPTION MENU PRESS F E X.

IF THE PROGRAM SHOULD BE ABORTED AND THEN RE-RUN FROM THE BEGINNING WITH-

OUT TURNING THE UTU OFF AND BACK ON, THEN XON AND ECHO WILL BE TOGGLED TO

THEIR ORIGINAL DEFAULT VALUES.  THIS CAN BE CORRECTED BY PRESSING THE DEL

KEY, SELECTING THE 'O' OPTION AND PRESSING 'E','X'.

THERE HAS BEEN OCCASSIONS WHEN THE PROGRAM HAS HUNG UP WHEN MAKING A

SPEED SELECTION WHILE PRINTING COPY ON THE SCREEN.  THE ONLY OPTION IS TO

REBOOT.  THIS IS NOT A SOFTWARE PROBLEM AND CAN BE AVOIDED BY REDUCING

THE VOLUME OF RECEIVED SIGNALS TO THE POINT WHERE NO COPY IS MADE DURING

THE INTERVAL OF SPEED SELECTION.

WHEN BREAKING THE PROGRAM TO MAKE MODIFICATIONS OR CHANGES IT IS BEST TO

RE-ENTER BY TYPING GOTO 4.  THIS DISPAYS THE MENU WHICH ALLOWS PROGRAM

CONTINUATION.  IF YOU INSTEAD TYPE RUN, YOU MAY GET AN OUT OF MEMORY

MESSAGE AFTER DOING THIS A NUMBER OF TIMES.

SUMMARY OF SPECIAL PROGRAM KEYS AND THEIR USAGE:

RECEIVE MODE:

UP ARROW -   ALLOWS INPUT OF CALL SIGN OF OTHER STATION FOR STORAGE.

ESC KEY  -   TOGGLES PROGRAM BETWEEN RECEIVE AND TRANSMIT MODES.

F9 KEY   -   DISPLAYS MESSAGE MENU AND PRINTS A '*" IN THE BUFFER WINDOW

ANY VALID MESS# IMMEDIATELY FOLLOWING THE * WILL TRIGGER

AUTOMATIC TRANSMISSION OF THAT MESSAGE IN TURN

DOWN ARROW-  SENDS PROGRAM TO A SPECIAL MENU

LEFT ARROW-  REQUESTS AN INPUT OF A MESSAGE FOR TEMPORARY STORAGE

RIGHT ARRW-  REQUESTS AN INPUT OF SPEED FOR RTTY,ASCII, OR CW MODE.

DEL KEY   -  INPUTS UTU MENU FOR MODE CHANGE

TRANSMIT MODE:

PROGRAM NOW ONLY RESPONDS TO ESC KEY OR F8 KEY

F8 KEY    -  SENDS IDENTIFYING CALL SIGNS ( WHEN F8 IS SENT FROM BUFFER

A 'K' IS ADDED TO THE END AND THE PROGRAM GOES TO RCV MODE.

F10 KEY   -  (ALL KYBD ENTRIES ARE NOW IMMEDIATELY TRANSMITTED UNTIL F10

IS PRESSED) PRESSING F10 CAUSES TRANSMISSION OF STORED TEXT

IN TYPE AHEAD BUFFER.  ALL KEY STROKES ARE NOW STORED IN

THE TYPE AHEAD BUFFER.  WHEN BUFFER IS EXHAUSTED THE

PROGRAM RETURNS TO THE IMMEDIATE MODE.

F9 KEY    -  OPENS MESSAGE WINDOW FOR DISPLAY OF AVAILABLE CANNED

MESSAGES, BY NUMBER, FOR TRANSMISSION.

OF COURSE A HARD COPY OF THESE INSTRUCTIONS CAN BE MADE BY LLISTING rtty.doc

NOW JUST SIMPLY RUN THE PROGRAM AND PLAY WITH IT.  A UTU IS NOT NECESSARY

TO EXPLORE THE PROGRAM FEATURES.

END OF TEXT FILE