A Report on Technology and Standards for Digitizing and Compressing Audio Data

By

Dr. Gunnar Moan

e-mail:	gmo@sognhs.no

telefax:	+47 56 76 190

Western Norway Research Centre

P.B. 142, 5801 Sogndal, Norway

28th of May 1993

Abstract

This report provides a survey of sound codecs which can be used to support on-line transmission of audio using ISDN.  The implementation of such sound codecs has recently been made possible due to the development of powerful signal processors.

The author has, within the budget allocated this study, tried to give an accurate description of existing products, and development trends which may affect future products.  Sound codecs represent technology which is still under development.  The author would therefore recommend readers to check details concerning products of interest with the appropriate manifacturer or vendor of these products listed at the end of the report.

Acknowledgment

The author would like to express his thanks to:

Birte Christensen-Dalsgaard, UNI-C

Yannick Mahieux, France Telecom

Anton Thimet, FHG IIS

for their useful comments and help in making this report.

Table of Contents

1  Background	1

2  Compressing digital sound	2

2.1  Compression algorithms	2

2.2  The MPEG/ISO standard for digital sound	3

2.3  Codecs for telematic access to digital audio	6

2.3.1  Audio Export	7

2.3.2  Philips	9

2.3.3  Digigram	10

2.3.4  ARCOMIS	12

2.3.5  AETA	13

2.3.6  Alcatel Telettra	14

2.3.7  Corporate Computer Systems (CCS)	15

2.4	Comparison of available codecs	17

Bibliogaphy	22

List of acronyms used in the report	23

Appendix A	24

tc "1  Background"§1  Background

JUKEBOX is a project supported by the Commission of the European Communities Libraries Program.  The partners in the project are: State and University Library (Denmark), Discoteca di Stato (Italy), National Sound Archive (UK), and Western Norway Research Centre (Norway).  The project proposes to extend existing services offered by libraries to include on-line access to sound archives.  Such archives contain sound recordings of invaluable importance in complementing written documentation currently held in libraries.

Libraries represent a well established information structure with the required capabilities to secure copy-right legislations attached to audio recordings.  Many libraries have started to build their own collections of commercially available recordings.  Enabling libraries to become public access points to sound archives will remove the need for each library to keep duplicate copies of audio recordings, and increase the amount of audio material available to users of libraries.

The proposed service will consist of the following two systems: The first of these systems will be a multimedia terminal for presenting sound recordings.  This terminal will be placed in libraries and use ISDN to enable the public to search in catalogue databases and retrieve copies of sound recordings stored in archives.  The second system will be placed in each archive, and provides an interface to bibliographic data and a collection of digital copies of selected sound recordings.  This system will translate user queries to the search language used by catalogue databases in archives.  The system will communicate access to entries in the collection of audio recordings identified in the result from a bibliographic search.

The aim of the JUKEBOX project is to evaluate a service providing telematic access to collections of sound recordings. The delivery of this service will require technology for compressing digital audio.  The purpose of this report is to provide a basis for selecting such technology for the development of a pilot service.  The report also contains views on technology trends which can influence the future developments of a service providing telematic access to sound archives.

The project proposal describes a pilot service which in this report has been used to formulate requirements to technology for the project.  The key features of the proposed service is the need to provide on-line access to audio recordings of high fidelity, and that these recordings will be made available in a compressed format.  The telematic network selected for the implementation of the service is ISDN.  The information for this survey has been collected by contacting institutes conducting research on compressing sound, and manifacturers of audio codecs.  These bodies were contacted by mail and telephone.  In addition, subscribers to a number of relevant conferences on NEWS have been informed about the project and invited to send information on technology and application areas.  The appendix at the end the report gives a listing of the sources used in the survey.

tc "2  Compressing digital sound"§2  Compressing digital sound

The most common form of digital audio is CD Digital Audio.  The audio quality of this format is achived by sampling sound at 41.1 KHz, where the samples from each mono-channel are represented with 16 bit precision.  One minute of audio in this format represents approximately 10 Mb (megabytes) of data.  This amount of data is expensive to store, and to distribute using digital networks.

The project proposes a pilot service delivering on-line telematic access to collections of compressed audio.  A previous survey conducted in the project recommends using ISDN (Integrated Services Digital Network) based on a total assessment of transmission capacity, national and international availability, and cost related to expected usage of the network in the proposed service.  Transferring CD Digital Audio in real time on ISDN requires a compression rate of 1:12.  The development of DSP (Digital Signal Processor) chips has made it possible to implement algorithms which meet this requirement.

The algorithms used by codecs capable of on-line transmission of digital audio are not reversible.  This means that it is not possible to make an identical copy of the source of a sound based on the compressed format.  This term also means that it is not possible to transform compressed audio into the format used by other types of compression algorithms, since the information removed in the compression process will differ between algorithms.  The need for defining an international standard has therefore been of particular importance to compressed audio.

The two main factors determining the choice of codec suitable for the JUKEBOX project is; the format used for compressed audio and the interface to telematic networks.  The section below gives an introduction to alternative compression techniques which are used by the codecs discussed in this report.  This is followed by a section describing the MPEG/ISO standard now specified for compressed audio.  The next part of the chapter presents the codecs considered in this survey.  The last part of the chapter contains a listing of the requirements used in the evaluation of codecs, and make recommendations for the future process of selecting a codec for the project.

tc "2.1  Compression algorithms"\c §2.1  Compression algorithms

The development of most sound codecs has been aiming to achieve on-line transmission of audio.  This is still a new research area where the number of commercially available codecs are increasing.  The algorithms used by existing codecs apply psychoacoustic models to remove information describing sounds which the human ear cannot recognize.  The most effective algorithms can reduce the amount of data represented by CD Digital Audio by 92% with little or no audible loss of sound quality.

Algorithms with a high compression rate usually fall into two main categories: subband coders and transform coders.  A brief description of these two approaches is given below based on the advantages and disadvantages listed by Ernst Eberlein, Heinz Gerhäuser and Stefan Krägeloh[EBER90].

Subband coders are usually implemented as a tree of filters splitting the sound into subbands adapted to the properties of the human ear.  The coded signal is compressed by masking noise in the individual subbands.  One advantage of this method is the ability to take into consideration the spectral selectivity of the ear THEI87].  It is easier to detect noise in the lower bandwidths, and more bits can be allocated to the coding of this part of the audio signal.  The reconstruction of audio is achieved by first decoding individual subbands.  A tree of filters inverse to the filters used to produce the subbands are here used to recreate the broadband signal.

Even though the development of digital filters has made it easier to implement this approach, Eberlein et. al[EBER90]. claims that such complex filters produce a large delay.  A second class of subband codecs using polyphase filterbanks has relatively short delays, moderate complexity with a good resolution in the time domain.  Both classes of codecs, however, have a low frequency resolution which is hardly adequate for calculating a concise masking level.

The second category of codecs are transform coders.  According to Eberlein et. al., this group has a low to medium complexity with the potential for high compression rates, and an excellent frequency resolution which is important when applying psychoacoustic properties.  One group of transform coders use TDAC (Time Domain Aliasing Cancellation).  With this method, blocks of audio are sampled over time and tranformed to the frequency domain where the size of the blocks determine the resolution of frequency and time.

One technique belonging to this group, which has produced good results particulary for low bitrates, is OCF (Optimum Coding in the Frequency domain).  This technique transforms samples of sound into the frequency domain using MDCT (Modified Discrete Cosine Transformation).  A psychoacoustic module is used in an interactive process to mask all quantization noise.  The good results obtained with this technique has a cost in terms of computational requirements and a relatively long delay when coding audio.  Decoding audio, however, requires much less processing.

tc "2.2  The MPEG/ISO standard for digital sound"\c §2.2  The MPEG/ISO standard for digital sound

The introduction of DSP processors, such as the Motorola DSP56000 and the DSP32C by AT&T, made it possible to implement algorithms of the types described above for real-time encoding and decoding of digital audio.  Such codecs are commercially available as 19” racks or as processor-boards for personal computers.

In parallel to the development of audio codecs, MPEG (Moving Picture Expert Group) took the initiative to a working group with the objective to define a standard for encoding moving pictures stored on digital storage media.  The work of the group was later extended to include audio, and Swedish Broadcasting Coorporation was in 1989 asked to conduct audio tests of available codecs[BERG90].

In these tests the sound quality was measured for stereo audio where each channel was compressed to 128- and 96 kbps (kilobits per second), and for audio in mono compressed to 64 kbps.  The listening tests involved 50 subjects and used a method known as “double blind test with hidden reference” where the subjects had access to a fixed source of reference.  The same method was used to evaluate the robustness of codecs.  A random event generator was here used to introduce bit errors of the rate 10-4.  The objective of this test was to measure the ability of codecs to operate with error rates which can be experienced on digital telematic networks.

Only two pilot codecs met the requirements of the tests.  One of these codecs was ASPEC (Adaptive Spectral Perceptual Entropy Coding) developed by Fraunhofer-Institut für Integrierte Schaltungen (FHG IIS), Germany in collaboration with Thomson-Brandt (Germany), CNET (France) and AT&T (USA).  The second codec was MUSICAM (Masking pattern Universal Sub-band Integrated Coding And Multiplexing) developed by Institut für Rundfunketechnik (IRT), Germany in collaboration with CCETT (France) and Philips (Netherlands).  These two codecs were later subjected to new tests at the Swedish Broadcasting Coorporation.  These tests formed the basis for the MPEG/ISO standard described below which contains elements from both MUSICAM and ASPEC.

Sound codecs conforming to the standard must allow input and output of digital audio sampled with 16 bit resolution for the sampling rates of 32 KHz, 44.1 KHz, and 48 KHz.  The bitrates supported by encoder and decoder are 64, 96, 128 and 192 kbps.  The delay accepted for encoding and decoding of audio sampled at 48 KHz is 80 ms. when transmitted in stereo at a bitrate of 128 kbit per mono channel.  The standard also address storage of audio, where the access time to a unit of encoded sound must be less than 100 ms.

The MPEG/ISO standard focuses on the decoding of digital audio, and only defines the format of the data delivered to this process.  In this way, the standard allows for the development of impoved encoders which can be used in existing applications without needing to replace the decoders[EBER93].  The standard defines the format of the output from the encoder in terms of the following three layers with increasing sophistication and performance[MPEG90].

Layer I:	Simplified version of MUSICAM

Layer II:	Practically identical to MUSICAM

Layer III:	A merge of ASPEC and MUSICAM

The first and second layer (layer I and II) are based on MUSICAM and specify the mapping of digital audio into 32 subbands and the framing to be used.  The third layer (layer III) conforms to ASPEC by transforming subband samples.  The frequency resolution is increased by using 576 subbands, which allows for a more effective bit allocation.  Layer III reduces redundancy by exploiting the statistical properties of the coded data stream, and can use unnecessary data bits to code critical segments of the audio signal.  The free choice of any data rate under layer III makes it possible to tune the codec to the particular requirements of an application.

The higher layers defined under the MPEG/ISO standards can be described as extensions to layer I where layer III represents the most sophisticated layer.  Codecs conforming to a layer of the standard can therefore decode audio compressed by codecs belonging to the same or lower layers.  Codecs conforming  to layer III will be able to decode audio compressed by all codecs conforming to the standard.

There now exists CCIR recommendations for the use of low bitrate audio codecs in broadcast applications.  These recommendations were made on the basis of extensive tests conducted between January and July 1992 of nearly all available low bitrate codecs.  In these tests, the codecs conforming to layer II and III of the MPEG/ISO standard achieved the over all best results.  The requirements for the tests are summerized by Eberlein et. al[EBER93]. as follows:

- Contribution links with a bitrate of 180 kbps per audio channel.  20 KHz bandwidth, no audible impairments when using a chain of 5 codecs.

- Distribution links with a bitrate of 128 kbps per audio channel.  20 KHz bandwidth, no audible impairments when using a chain of 3 codecs.

- Emission links with a bitrate of 128 kbps per audio channel or 96 kbps per audio channel when using joint stereo.  20 KHz bandwidth, a sound quality higher than 4.0 on the CCIR impairment scale.

- Commentary links with a bitrate of 60 kbps per audio channel.  15 KHz bandwidth and a sound quality equivalent to linear 14 bit PCM format.

The preliminary results[CCIR92] recommends the use of layer II for contribution, distribution and emission links, while layer III is recommended for commentary links.

The measurements of audio quality obtained in tests of codecs at low bitrates can be used to define a "target bitrate" for which a codec will produce audio of "good" quality.  The notion of "good" quality describes audio where degradation of the signal may be perceptible, but not annoying.  In many applications this quality will be indistinguishable from CD digital audio.  The following "target bitrates" are suggested for the different layers of the MPEG/ISO standard:

layer I:	target bitrate of 192 kbps per audio channel

layer II:	target bitrate of 128 kbps per audio channel

layer III:	target bitrate of 64 kbps per audio channel

Codecs conforming to a layer of the MPEG/ISO standard will be able to decode audio compressed by codecs adhering to the same, or lower layers.  The quality of sound delivered, however, is not guaranteed, since the standard does not specify how audio is to be encoded.  According to M. Mahieux at CNET (Centre National d'Etudes des Télécommunications), there may be a noticeable difference between the sound quality of the codecs that took part in the tests conducted when defining the MPEG/ISO standard and codecs commercially available today, because the psychoacoustical models used by these codecs are different.

There exists other standards for compressed audio besides the MPEG/ISO standard.  The description of codecs in this report will make reference to the G.722 standard developed by CCITT (Comité Consultatif Internationale de Télégraphic et Téléphonique).  Most codecs conforming to this standard supports a frequency bandwidth of 7.5 KHz, which is often used in high quality speach application.  The recommendations of this standard has also been used to support a bandwidth of 15 KHz.

The "target bitrates" defined above represent a mark for the sound quality of codecs conforming to the different layers of the MPEG/ISO standard.  The audio quality of codecs can not be "guaranteed" for bitrates below the "target bitrate", and will greatly depend on the implementation of the codec.  It is therefore recommended that the choice of codec is based on listening tests if the quality of sound is important, particularly for applications where the codec is to be used for data rates below the "target bitrates" listed.

tc "2.3  Codecs for telematic access to digital audio"\c §2.3  Codecs for telematic access to digital audio

The codecs described in this survey conforming to the MPEG/ISO standard are implementations of the ASPEC or the MUSICAM algorithms.  The figure below provides an overview of the codecs described later in this section.

Figur 1.  Overview of codecs considered in this report

Below is a description of the codecs considered in this survey under the heading of the companies which produces them.  The prices listed are in ECU calculated via NOK using the following rates obtained for the last quarter of April 1993:

1 ECU:	8,25 NOK

1 ECU:	1,95 DEM

1 ECU:	6,58 FRF

tc "2.3.1  Audio Export"\c §2.3.1  Audio Export

This company currently offers two models of codecs developed by Fraunhofer-Institut für Integrierte Schaltungen (FHG IIS).  The institute conducted the implementation of the ASPEC algorithm used in specifying Layer III of the MPEG/ISO standard.  This is the most sophisticated layer providing the best audio quality at low bitrates.  The most advanced of the two models is housed in a 19 inch rack, and is a modernized implementation of the pilot codec used in the specification of the MPEG/ISO standard.  The other model is an implementation of the ASPEC algorithm on a PC card.

The ASPEC encoder/decoder: is implemented as two seperate units placed in 19 inch racks.  These units represents a modernised version of the original ASPEC codec.  The system has digital and analogue input for two channel audio sampled at 32, 44.1 and 48 KHz.  The user can select between 16 different bitrates per audio channel ranging from 32 up to 192 kbps.  These bitrates corresponds to audio bandwidths ranging from 8,8 up to 20,1 KHz for audio sampled at professional quality.   Two X.21 connections are used for telematic transmission of mono and sterio audio using one or two B-channels.

Price:	Encoder:	6964 ECU

Decoder:	6231 ECU

Features:	Two channel audio processing conforming to Layer III of the

MEPG/ISO standard.

Variable bitrates ranging from 32 to 192 kbps per channel.

AES/EBU and SPDIF interface for digital audio.

XLR interface for analogue audio.

2 x X.21 interface for synchronised transmission of audio

RS485 data interface.

The ASPEC encoder and decoder are separate units which can be integrated in a computer system using the RS485 interface.  There is no software library,  however, supporting such use of the codecs.  The encoder and decoder represents the high-end of codecs for transmitting sound at low bitrates.  The "target bitrate" for the codec is 64 kbps which enables the codec to deliver joint stereo audio of that quality using one ISDN connection.  Listening tests considering user requirements to sound quality and the type of recordings to be made available in the proposed service, will determine if it is acceptable to encode sound at the bitrate of 32 kbps.  This solution will make it possible to send stereo audio using only one B-channel and thereby half the expected transmission costs of the project.

PC- Layer III card: is a full size PC card based on DSP 32C (AT&T) and DSP56001 (Motorola) providing realtime audio processing according to layer III of the MPEG/ISO standard.  The encoder is configured to code single- or two channel audio for bitrates from 32 to 320 kbps.  It is possible to load new software into the card to adapt to the requirements of application areas, or to install improvements of the encoder being used.  The card can send and receive compressed audio from an external storage device or by using the X.21 interface.  The PC software consists of a C-library for configuring the codec and supporting the development of applications.

Price:	3587 ECU

Features:	Two channel audio processing card designed for PC/AT.

The codec conforms to layer III and supports variable

compression rates from 320 kbps to 32 kbps per mono channel.

Sampling frequencies in KHz: 48, 44.1 and 32.

Balanced input/output with digital interface to AES/EBU.

2 x X.21 interface for communicating encoded audio.

The decoder will automatically recover the sampling rate.

The output of the decoder can be synchronised to a studio

master clock.

C-library available for developing applications and uploading

DSP software for configuring the codec.

The PC Layer III card uses the same algorithm as the ASPEC codec, and has the same "target bitrate" of 64 kbps.  The codec is a full size PC card which is designed to be integrated in computer based systems providing on-line access to stereo audio using only one ISDN connection.  The codec is not scheduled for sale, however, before the third quarter of 1993.

FHG IIS can recommend the PC card for the JUKEBOX project to evaluate a pilot service.  The card, however, contains both the encoder and decoder.  The encoder is the most expensive part of the codec utilizing 4 DSP processors.  The decoding of audio is a much simpler process which can be achieved with only one DSP processor.  There is only need for the decoder in the service proposed in the JUKEBOX project.  FHG IIS has already developed a configuration of the card where the encoder and decoder is implemented on two separate cards.  This pair of cards was ordered by NSM Aktiengesellschaft and used to implement a service providing ISDN access to a music databank.  The cards provided by NSM therefore represent an option better tailored to the hardware requirements of the service suggested in the project proposal.  The price for the decoder or the encoder card should be less than the PC Layer III card described above.

tc "2.3.2  Philips"\c §2.3.2  Philips

This company has developed the MAGIC (Multi Audio system with Groupable Interfaces and Codecs), which is a codec designed for high sound quality at low bitrates.  The intended application for this system is the transmission of one or more audio channels on different networks and communication links including ISDN.  The codec supports a number of interfaces for transmission of encoded audio, including CCITT H.221, H.242 and G704.  Philips claims to be able to integrate several standards for data reduction, such as G.711, G.722 by CCITT and ISO/MPEG Layer II and III.

The MAGIC system is housed in a 19 inch rack which can be configured to provide a combination of the plug-in units described below.  The base system contains the network interfaces listed underneath.

Price:	Price not fixed.

Features:	The codec supports two channel audio processing.  Each system

can contain a combination of the following plug-in units

connected by an internal bus.

- Layer III with digital AES/EBU interface.

- Layer II or Layer III with XLR interface for analogue audio.

- Layer II with digital interface to AES/EBU and XLR

interface for analogue audio.

The processing of layer II and III each contain the following

features:

Audio codec Layer II for two mono channels or

one stereo bidirectional channel.

bitrates for mono are: 64, 128, and 192 kbps.

bitrates for stereo are: 128, 192, 256 and 384 kbps.

Sampling frequencies in KHz: 32 or 48.

Audio codec Layer III for two mono channels or

one stereo bidirectional channel.

bitrates for mono are: 64, 128, and 192 kbps.

bitrates for stereo are: 128, 192, 256 and 384 kbps.

Sampling frequencies in KHz: 32, 44.1 or 48.

The base system contains an interface for access to variable

network.  The encoded audio signal from up to 6 plug-in units

can be multiplexed by the interface unit.

network interfaces:

- 1 x X.21 (64 - 1929 kbps).

- 3 x X.21 (64 or 128 kbps).

- RS 499 (64 - 1929 kbps).

- G.703, G704 (2 Mbps interfaces).

- 1.544 Mbps (T1).

- RS 232.

- RS 485.

tc "2.3.3  Digigram"\c §2.3.3  Digigram

This company was one of the first to produce a PC-card implementation of a codec for commercial sale.  One of their models was used to develop a system for archiving radio programs at the Norwegian Broadcasting Corporation where data is stored on 1/4 inch catridges.  Digigram has provided information on two of their codecs, and a kit enabling real-time transmission on ISDN.

PCX5: is a half size PC/AT card based on the Motorola 56000 DSP range.  The codec offers real-time data compression under MUSICAM conforming to layer I and II of the ISO/MPEG audio standard.  The card has an interface for two-channel digital and analogue audio.  Application development is facilitated by an OEM software kit running under both WINDOWS and MSDOS.  This kit is optional.

Price:	3723 ECU (5927 ECU including OEM tool kit)

Features:	Two channel audio processing card.

Sampling frequencies in KHz: 48, 44.1, 32, 22.05, 16, 11.025.

The analogue audio specifications are made for uncompressed

sound.  No available data describing the quality of compressed

audio.

Compression rates from 1:4 to 1:24. Two compression types:

MPEG/ISO layer I and II.

Compatible with WB48SBC.

Digital input/output in AES/EBU format.

Analogue input/output with 16 bit A/D conversion.

Internal synchronisation of several PCX5 cards.

SMPTE time code in.

PCX7: is a new model scheduled for sale from the end of April 1993.  This card is fully compatible with the PCX5 using the same format for encoding sound.  Technological improvements has made it possible to produce this card at almost half the price of PCX5.  The PCX5, however, produces audio of a better quality since it has balanced output.  The PCX5 also has digital input/output for AES/EBU as a standard feature.  This interface is optional for the PCX7.  Another option is the OEM development kit under WINDOWS or MSDOS.

Price:	2052 ECU (4286 ECU including OEM tool kit)

Features:	Two channel audio processing card.

Sampling frequencies in KHz:

48, 44.1, 32 , 22.05 , 16, 11.025, 8

The analogue audio specifications are made for uncompressed

audio.

No available data describing the quality of compressed audio.

Compression rates from 1:4 to 1:24.

Two compression types:

MPEG/ISO layer I and II

Compatible with WB48SBC

Analogue input/output with 16 bit A/D conversion

Internal synchronisation of several PCX7 cards

Both MIDI IN/OUT and SMPTE time code input for

synchronisation.

Transkit: consists of a PCXC card and a driver and provides an interface between a PCX5 or PCX7 card and a standard ISDN card.  The kit will be available for sale at the end of April 1993, and will then make it possible to integrate on-line transmission of MPEG/ISO audio into new and existing applications.

Price:	4088 ECU

Features:	Transmission rates in kbps: 64, 128, 192, 256, 320, 384.

for digital or analogue sources sampled at 32, 44.1 or 48 KHz.

Supports applications running on:

PC SNET (OST)

MR 300 (XCOM)

DEVISE S (SCII)

Conform to ISDN signalling levels 1, 2 and 3 according to VN2.

Function calls in transkit (see below) are independent of type of

ISDN card.

- Call

- Init answer

- Answer

- Frame emission

- Frame reception

- End communication

Transmission delay is 250 ms. for audio sampled in mono at

48KHz and compressed to 128 kbps (2 B channels)

Connection delay is 8 sec.

Synchronisation delay is 300 ms.

Both PCX5 and PCX7 can compress stereo audio sampled at any rate up to 48 KHz to 64 kbps per mono channel.  The encoded data can then be transmitted across two B channels and decoded as a stereo signal.  This approach is below the "target bitrate" of codecs conforming to Layer II, and must be considered in terms of the requirements of the users and the type of sound recordings to be made available.  Digigram expects their codecs to meet requirements to sound quality for compression rates down to 128 kbps per audio channel.  This bitrate requires one ISDN connection to transmit audio in mono.  Transmission of stereo audio can be achieved by using two ISDN connections.

Digigram has sales offices in most European countries including all the countries participating in JUKEBOX.  The company has promised to assist the project by adjusting the driver in the transmission kit to the ISDN card used in these countries.  The PCXC card is used to synchronise the internal clock signal when presenting sound and is therefore only required when receiving compressed audio.  The proposed service will require the OEM software kit for developing the user system and the archive system.  The software kit is not required for other installations of these systems.

tc "2.3.4  ARCOMIS"\c §2.3.4  ARCOMIS

This is the only company known to this reports which has implemented their codec on an extention-card for Macintosh.  The compreSound-AD1 card was developed in collaboration with Digigram and use the same algorithm for processing audio.   The card is therefore an implementation of MUSICAM supporting Layer I and II of the MPEG/ISO standard.  The use of two Motorola 56001 processors provide real-time processing of two channel audio with a "target bitrate" of 128 kbps per audio channel.  The card has an interface for sampling analogue sound at 32, 44.1 and 48 KHz.  The UER/AES interface is for storing and retrieving both compressed and uncompressed digital audio from  a number of storage devices, such as magnetic disk, DAT and CD.  Input/output for SMPTE and MIDI time code enables video and audio synchronisation.

Price:	3800 ECU

Features:	PC card for Macintosh for  real-time processing of

two channel audio.

Sampling rates in kbps include: 64, 96, 102, 128

Interface to analogue audio sampled at 32, 44.1 and 48 KHz

UER/AES interface for both

compressed and uncompressed digital audio.

ARCOMIS has several software tools supporting the development of systems for on-line transmission of audio using ISDN.  One tool is fileNet for transferring files of compressed audio using from 1 to 4 B-channels.  This software supports interconnection between networks using LOCALTALK or ETHERTALK.  Another tool is etherSound for implementing a server for on-line transmission of audio.  The tool has an interface to ETHERTALK where up to 8 users can simultaneously receive compressed audio.

A number of existing systems has been developed by ARCOMIS for the compreSound-AD1.  One important application area is the radio industry where compreSound is used for editing programs and for remote transmission using ISDN.  It will be useful for the JUKEBOX project to be able to base the implementation of the pilot service on components from these systems.

The managing director of ARCOMIS, mr. Lapalus,  proposes the development of a service using the modified MPEG/ISO Layer I.  The compreSound-AD1 will for this format produce a bitrate of 102 kbps for stereo audio sampled at 32 KHz.  The use of this format will enable on-line transmission of stereo audio.  Mr. Lapalus has predicted the bandwidth of such a transmission to 10 KHz.

Mr. Lapalus has also specified a possible system configuration for the pilotservice.  A Macintosh Quadra 950 is suggested used in the archives as a server supporting multi-user access via several ISDN communication cards.  A Macintosh II VX 5/80 could be used as the basis for the user terminal.  The price for this computer, equipped with a compreSound-AD1 card and ISDN card, is by mr. Lapalus estimated to approximately 5319 ECU.

The "cyclone" and "tempest" machines are examples on the development trend for the future.  Mr. Lapalus has been informed that these machines will be released by Apple Computer inc. at the end of the year, and will be equipped with a DSP 3201 processor capable of decompressing MPEG/ISO audio.

tc "2.3.5  AETA"\c §2.3.5  AETA

AETA has developed codecs which has been used in applications for transmission of audio before the MPEG/ISO standard was defined.  They are now also developing codecs which conform to this standard, and have a range of products which can be used to implement the proposed service.

Plextel is a family of codecs built as stand-alone units.  The different configurations of this codec now includes a version supporting Layer II of the MPEG/ISO standard for two-channel bidirectional audio.  The bitrates supported by the codec is designed for using a multiple of B-channels where the "target bitrate" is 128 kbps.  The encoder and decoder of this codec is available as seperate units which can be supplied with an ISDN interface or with an interface to terminal adapters for ISDN.

Price:	Encoder:	2926 ECU

Decoder:	2926 ECU

Features:	Bidirectional transmission of audio processed according to

Layer II of the MPEG/ISO standard.

Target bitrate of 128 kbps per audio channel

15 KHz at 56/64 kbps per audio channel

7.5 KHz at 32 kbps per audio channel

V11, V35 or buildt-in interface for ISDN

Input/output for two channel analogue audio.

RS232 serial data interface

The model in the Plextel range of codecs described above represents the best choice for on-line transmission of stereo audio with a bandwidth of 15 KHz or more using one or two ISDN connections.  This model can also be used to transmit 7 KHz stereo audio using only one B-channel.  AETA plans to conduct tests in Norway later this year with a view to using the codec during the Olympic games in Lillehammer for on-line transmission of audio on ISDN.

The development of products at AETA is conducted in close cooperation with France Telecom.  One of the products is a PC card which is planned to be extended to support bidirectional transmission of one channel audio with a bandwidth of 7,5 KHz for a bitrate of 64 kbps.  AETA are also looking at developing codecs supporting Layer III by using a DSP processor developed by Texas Instruments inc.  This is one of the most powerful processors on the market which can make it possible to implement such a codec using only four processors.

tc "2.3.6  Alcatel Telettra"\c §2.3.6  Alcatel Telettra

This company has developed a prototype card designed for audio transmission in HDTV (High Definition TeleVision) applications.  The current version has AES/EBU interface for two channel digital audio sampled at 32, 44.1 and 48 KHz.  The board contains 4 Motorola DSP 56002 processors providing a real-time implementation of Layer I and II of the MPEG/ISO standard.  The codec has a 1 Mb memory bank which can be used to configure the codec to support other standards for digital audio, such as CCITT.

This codec will be available in the beginning of the third quarter of 1993.  By the end of the year, the telematic interface will consist of two X.21 connections and firmware for synchronising transmission of audio on two B channels.

Price:	Not yet decided

Features:	Two channel audio processing card conforming to

the MPEG/ISO Layer II standard.  Programmable to support

other standards for audio.

AES/EBU interface for digital audio sampled at

32, 44.1 and 48 KHz.

An analogue interface is planned for 1994.

This PC-card is still under development, and cannot be described in detail in this report.  The specification of the development work planned for 1993 suggests that this card will be able to support on-line telematic access to digital audio.  The compliance with Layer II of the MPEG/ISO standard indicates support for bitrates from 64 to 384 kbps.  The planned dual X.21 interface will then make it possible to use two B-channels for synchronised transmission of one or two channel audio.  The flexibility of the design can be used to employ other standards for digital audio for applications which does not require the sound quality of the MPEG/ISO Layer II.  Mr. Parladori at Alcatel informs that the card will have interface to a serial line or a data bus.  This will enable software control of the card and storage of sound on the PC.

tc "2.3.7  Corporate Computer Systems (CCS)"\c §2.3.7  Corporate Computer Systems (CCS)

This company has developed three stand-alone codecs, and a PC-card implementation available in different configurations is expected later in 1993.  The codecs have an interface to many communication standards, including both the European and the American standard for ISDN.  They therefore support bitrates of both 56 and 64 kbps for one B-channel, or 112 and 128 kbps  for one ISDN connection.  The codecs use an implementation of MUSICAM and support either the MPEG/ISO Layer II or CCITT G.722.  One of the models supports both of these standards.

CCS CDQ 2000: use an implementation of MUSICAM conforming to the Layer II of the ISO/MPEG standard, and is developed for transmission of audio on ISDN or satelite.  The codec offers bitrates from 56 to 384 kbps.  When using two B channels, the codec automatically compensates for any time delay between two channels following different transmission paths.  Transmission failure when using one B channel will cause an automatic switch to the other B channel without any downtime.  Terminal adapters from a number of manifactures can be used for bitrates above 128 kbps to synchronise up to 6 B channels.

The CDQ 2000 is available in three different configurations.

Encoder:  The basic version has analogue XLR inputs for stereo audio.  The signal is sampled at 48 KHz with 16 bit resolution.  A digital AES/EBU interface is an option.  CCS Europe informs that there has been developed versions of this model supporting other sampling rates such as 44.1 and 32 KHz.  These are sold for the same price as the standard model.

Price:  3636 ECU (optional AES/EBU interface: 485 ECU)

Decoder:  The decoder has the same interface as the encoder, and reconstructs audio into a mono or stereo signal depending on the encoded data.

Price:  3272 ECU (optional AES/EBU interface: 485 ECU)

Encoder/Decoder: This configuration enables on-line bidirectional communication of compressed audio.

Price:  6788 ECU (optional AES/EBU interface: 606 ECU)

Features:	All configurations of the CDQ 2000 contains the same features.

The bitrates per channel include:

32, 48, 56, 64, 112, 128, 129, 256, 384 kbps.

Sampling rates for analogue sound is 48 KHz.

AES/EBU interface is optional.

Custom designed models available for other sampling rates.

Transmission interfaces: V.35, X.21, and RS422A

CCS CDQ 1000:  is developed for transmitting compressed audio in mono on ISDN, and claims to be the only codec enabling the user to select between two standards for encoding digital audio.  The MPEG/ISO Layer II standard is used for audio sampled at 48 or 24 KHz.  The use of 48 KHz enables the CDQ 1000 to communicate with the CDQ 2000.  The 24 KHz mode can only be interpreted by this model.  The second standard supported by the CDQ 1000 is the CCITT G.722 providing a sound quality of 7.5 KHz for any of the two sampling rates above.  This mode meet the requirements of high quality speech applications and is compatible to the CCS Micro series described below.

Price:  	3030 ECU

Features:	Encode mono audio in the formats of Layer II and G.722.

Produce bitrates of 56 and 64 kbps.

One XLR connection for analogue input/output.

Digital input/output in AES/EBU format.

Transmission interfaces: V.35, X.21, and RS422A

Micro 66i: is the latest model in the CCS Micro series of codecs.  The codec is designed for field work in the broadcast industry, and therefore comes in a transportable model which is easy to use.  The model prized below is housed in a 19 inch rack.  The Micro 66i is a bidirectional codec with XLR connection for input and output of one channel audio.  The codec use a sampling frequency of 7.5 KHz and process audio according to the CCITT G.722 for the two bitrates of 56 and 64 kbps.

Price:	2303 ECU

Features:	Process one channel audio sampled at 7.5 KHz.

XLR interface to one channel analogue audio.

Conforms to the CCITT G.722 standard for audio

sampled at 7 KHz.

Produce bitrates of 56 and 64 kbps.

Interface to X.21 and V.35 for audio transmission.

CCS PC-card: is designed for IBM AT computers and provide on-line two channel audio processing using a format meeting the requirements of the MPEG/ISO Layer II standard.  The card has input/output for both analogue and digital sound.  The card is intended for applications where compressed audio is stored and distributed on a storage device such as magnetic or optical disks.  CCS informs that the card will not be available before July 1993.  The company has not decided on the prices for the configurations listed below.  There has also been expressed a demand for a card with a telematic interface.  New configurations of this card can therefore be expected during 1993.

Price:	Not yet decided

Features:	Two channel audio processing card based on MUSICAM and

conforming to MPEG/ISO Layer II

bitrates supported: 56, 64, 06, 112, 128, 192, 256, 384 kbps

XLR connections to stereo audio and AES/EBU interface

for digital audio.

Sampling rates: Not specified, 44.1 used as example

The following configurations will be available:

Decoder only with AES/EBU option.

Encoder only with AES/EBU option.

Encoder/decoder with AES/EBU option.

CCS produces a range of codecs designed mainly for the radio industry.  The models described above support the different bitrates for ISDN in Europe and USA, and are therefore suited for on-line transmission of audio between these two continents.  CCS has developed codecs for two audio standards where the CDQ 1000 is the bridge supporting both the MPEG/ISO Layer II and the CCITT G.722.  The MPEG/ISO Layer II is the most advanced of the two standards with a "target bitrate" of 128 kbps per audio channel.  The implementation of the CCITT G.722 has a bandwith of 7 KHz which is particularly useful in high quality speach applications.  The CCS PC-card will not be commercially available before July 1993.  The card is initially designed for editing digital audio stored on magnetic disks.

None of the codecs listed above is supported by a toolkit for developing applications providing on-line access to compressed audio.  This can be achieved by using the RS422 interface to the stand-alone units.  The company informs that there has been expressed demands for a telematic interface to the PC-card and that it is possible that CCS will implement a configuration of the card meeting this requirement.  It will not be possible to get all details concerning the PC-card before sometime into the third quarter of 1993.

tc "2.4	Comparison of available codecs "\c §2.4Comparison of available codecs

It has been the objective of this report to present as complete a picture as possible of codecs available on the market.  The figur below provides an overview of the attributes of these codecs in terms of the requiements listed above.

Figur 2.  Overview of attributes of available codecs

All codecs described in this report support bitrates required for using ISDN for on-line transmission of digital audio.  Not all codecs, however, are currently suitable for implementing the service proposed in the JUKEBOX project.  Most codecs are designed for the radio industry, and are often manifactured as separate units with little ability to be connected to computer-based systems.  The report also describes PC cards which are designed for hard-disk recording and editing of audio.  These cards can not be used for telematic transmission of audio unless supported by additional hardware.

One of the most important factors affecting the choice of codecs for the JUKEBOX project is the standard used for digital audio.  The only way to transfer compressed audio to a new format is by first decompressing the data.  The compression of audio is not reversible, which means that data will be lost during the decoding process.  The effect of this loss may not be audible for compression rates equal or above the "target bitrate" of the codec.

The standard supported by the codec selected for the project will determine the ability to later make direct use of other collections of compressed audio.  Layer III of the MPEG/ISO standard is supported by the most advanced of codecs which can also interpret encoded audio complying to lower layers of the standard.  The use of such a codec in the project will therefore increase the chances to make use of compressed audio generated by other projects.  As an effect, however, selecting a codec complying to Layer III will stop owners of codecs supporting lower layers of the MPEG/ISO standard to use audio compressed in the JUKEBOX project.

There are currently more codecs on the market supporting Layer I and II than codecs supporting Layer III.  The main reason for this difference is that Layer II was standardized in the spring of 1991, while Layer III was not standardized before a year later.  Most codecs have found application areas in the radio industry, where layer II codecs are being used to archive audio.  The same radio stations may want to deliver compressed audio to the archives taking part in the JUKEBOX project, as well as wanting to access the collections of compressed audio to be developed by these archives.

The description of codecs in this survey, however, suggests a future move towards the implementation of codecs supporting Layer III of the MPEG/ISO standard.  The introduction of new and more powerful DSP processors will reduce the cost of developing Layer III codecs.  Services, such as JUKEBOX, need inexpensive decoders.  This can be achieved for Layer III since the process of decoding audio can be accomplished by using only one processor.  There are now being developed models of personal computers equipped with DSP processors which can be programmed to perform this task.

The "target bitrate" of 64 kbps is a further reason for the JUKEBOX project to select a codec supporting Layer III.  This bitrate makes it possible for such codecs to transfer joint stereo audio using one ISDN connection.  Codecs supporting Layer II has a "target bitrate" of 128 kbps which enables the transmission of one audio channel on one ISDN connection.  The transmission cost for the project can be divided by two if Layer III codecs are to be used for transmitting one audio channel.

It is important to be aware of that the MPEG/ISO standard does not specify the encoding of audio.  The audio quality of codecs conforming to the different layers of the standard can therefore differ.  Such differences are most likely to be noticable for bit rates below the recommended "target bitrate".  The choice of codec should therefore be based on the results from listening tests, particularly when considering using bitrates below the "target bitrate".

It is also important to note that audio codecs represent new technology and that producers of codecs are trying to find new markets for their products.  New codecs are therefore being developed, and existing codecs adapted to new applications.  Reader of this report are therefore recommended to use the list of contacts at the end of this report to check details concerning codecs of interest.

tc "Bibliogaphy"§Bibliogaphy

[BERG90]	S. Bergman, C. Grewin, T. Rydén. MPEG/Audio Subjective Assessments. MPEG 90/196. July 1990.

[CCIR92]	CCIR Task Group 10-2. Draft new recommendation - low bit-rate audio coding. Revision 3 to Document 10-2/TEMP/7-E. October 8 1992

[EBER90]	E. Eberlein, H.Gerhäuser and S. Krägeloh. Audio codec for 64 kbit/sec (ISDN Channel) - Requirements and Results. ICASSP. 1990

[EBER93]	E. Eberlain, H. Popp, B. Grill, J. Herre. Layer III A Flexible Coding Standard. conf. proc. AES Berlin. March 16-19 1993

[MPEG90]	International Organization for Standardization. Committe Draft for MPEG Audio Coding Standard. MPEG 90/December 1990.

[THEI87]	G. Theile, M. Link and G. Stoll. Low-bit rate coding of high quality audio signals. conf. proc. AES London. March 10-13 1987

List of acronyms used in the report

AES/EBU		Audio Enginering Society/European Broadcast Union

ASPEC		Adaptive Spectral Perceptual Entropy Coding

CCITT		Comité Consultatif Internationale de

Télégraphic et Téléphonique

CD			Compact Disk

CNET		Centre National d'Etudes des Télécommunications

DSP			Digital Signal Processor

Gb			Gigabytes

HDTV		High Definition TeleVision

ISDN		Integrated Services Digital Network

ISO			International Standards Organization

kbps			kilobits per second

Mb			Megabytes

MDCT		Modified Discrete Cosine Trabsformation

MIDI			Musical Instrument Digital Interface

MPEG		Moving Picture Expert Group

MUSICAM		Masking pattern Universal Sub-band Integrated

Coding And Multiplexing

OCF			Optimum Coding in the Frequency domain

SMPTE		Society of Motion Picture and Television Enginers

TDAC		Time Domain Aliasing Cancellation

tc "Appendix A"§Appendix A

This appendix presents the list of institutions contacted by mail to obtain information for the survey.  The type of work conducted by these institutions range from developing multimedia systems to manifacturing hardware.  Some institutions are involved in several fields.  The last part of the appendix presents a list of conferences on NEWS where subscribers were invited to supply information to the survey.

List of addresses:

Apple Computer, Inc

20525 Mariani Ave.

Cupertino

CA 95014

USA

Alcatel Italia - Telettra

Central R&D - DSP Laboratory (C10)

att: G. Parladori

Via Trento, 30

20059 Vimercate (MI) - Italy

ARCOMIS

att: J. -François Lapalus

CITTN - 121, rue Chanzy

59260 LILLE-HELLEMMES

FRANCE

ARIEL

att: Marc Lindahl

433 River Road

Highland Park, NJ 08904

USA

Audio Processing Technology Limited

att: Charlie Day

Begbroke

Oxford OX5 1RU

GREAT BRITAIN

AVICOM

Infologics AB

att:  Lasse Hellquist

Box 91, S-19122 SOLLENTUNA

SWEDEN

BARCO-EMT GmbH

att: Gerhard Möller

Postfach 1520

D-7630 LAHR

GERMANY

CNET

Centre National D’Études Des Télécommunications

att: Yannick Mahieux

IAA-TSS, 2, rue de Trégastel - BP 40

F-22301 LANNION CEDEX

FRANCE

Commodore Business Machines, Inc.

1200 Wilson Dr.

West Chester

PA 19380

USA

Corporate Computer Systems

att: Dr. Hinderks

33 West Main Street

HOLNDEL, NJ 07733

USA

CREOTEC Corporation

att: Scott Lamb

Office at Las Colinas,

122 West Carpenter FWY

Suite 440, LB17 IRVING,

TEXAS 75039,

USA

DataCity Communications Oy

att: J-P Paalassalo

Iso-Heikkiläntie 14

20200 TURKU

FINLAND

DIC Digital

att: Pierre Vulliez

13 rue de la Brasserie

27500 Pont Audemer

FRANCE

Digidesign Inc.

M3 Sweatt

1360 Willow Road, Suite 101

Menlo Park, CA 94025

USA

Digigram

att: Patrick Hucher

Parc de Pré Milliet

38330 Montbonnot

FRANCE

23 M^S MTV 23/41

DSG Advanced Developments

2400 East Bayshore Parkway

Mountain View, CA 94043

USA

FHG IIS

att: Anton Thimet

Wetterkreuz 13

D-8520 Erlangen

GERMANY

Gothenburg University

Dep. of education and educational research

att: Berner Lindström

Frölundagatan 118,  Box 1010

S-43126 MÖLNDAL,

SWEDEN

IBM Corp.

Old Orchard Rd.

Armonk

NY 10504

USA

Ideal Harware Ltd

Tolworth Tower

Surbiton

Surrey KT6 7EL

GREAT BRITAIN

Intel Corp.

3065 Bowers Ave.

Santa Clara

CA 95051

USA

MENNESKE OG MEDIA

att: Thor Erik Sem

Prinsessealléen 32

N-0275 OSLO,

NORWAY

Morgenstierne & Co. a/s

v/ Tor Andreasen

Po.Box 15 BOGERUD

0621 OSLO,

NORWAY

Multimedia Technologies Ireland Ltd

att. John Boyle

Robert Schuman Building

University of Limerick

Plassey, Limerick, IRELAND

NSM Aktiengesellschaft

p.b. 1564

Saarlandstr. 240

6530 Bingen am Rhein

GERMANY

OSC

att: Josh Rosen

2901 23rd st.

SF CA 94110

USA

OTARI

att: Mike Winter, Unit 13,

Elder Way, Waterside Drive, Langly

Slough Berkshire SL3 6EP

GREAT BRITAIN

Oxxi

1339 East 28th St.

Long Beach

CA 90806

USA

Sony Corp. of America

Sony Dr.

Park Ridge

NJ 07656

USA

Tandberg Data A/S

v/ Ståle Bondestad

Kjelsåsveien 161

P.O. Box 9 KORSVOLL

N-0808 OSLO

NORWAY

The Voyager Company

att: Aleen Stein

1351 Pacific Coast Hwy.

Santa Monica,

CA 90401, USA

XIS

att: Pierre Keiflin

5, rue de Vallat

34000 Montpellier

FRANCE

Conferences on NEWS:

COMMUNICATION/COMPRESSION

comp.dcom.telecom

alt.dcom.telecom

comp.dcom.isdn

biz.comp.telebit

comp.dcom.servers

comp.protocols.iso

comp.protocols.iso.dev-environ

comp.sys.mac.comm

comp.sys.mac.comm

comp.unix.solaris

COMPRESSION

comp.archives.admin

comp.compression.research

comp.sys.ibm.pc.soundcard

COMPRESSION/STORAGE

comp.multimedia

comp.sys.mac

comp.arch

comp.sys.sun.hardware

GENERAL

comp.sys.mac.hardware

comp.sys.ibm.pc

comp.infosystems