SCSI DEVICES

The current driver works with most things SCSI, although we haven't

tried everything. Most SCSI hard drives have been tested -

SEAGATE

QUANTUM         best performer so far

CONNER

FUJITSU         also very fast

MAXTOR

NEC

We haven't found any drive not to work, but be wary of old devices. I

don't think ADAPTEC ST-506 to SCSI ACB4000 controllers work, but later

ESDI controllers may work. If anyone has access to such boards, I would

like to hear how they go.

Other SCSI devices will rely on SCSI direct (implemented in SCRAM 500

driver). This has been tested with a Toshiba CD ROM using the CD-ROM

File System and works OK. I think SCSI tapes will work with appropriate

software. Let me or Will know if you encounter problems and we will

attempt to resolve them.

CHEAP SCSI

For those of you with huge amounts of cash, there are lots of SCSI boxes

available. Mount a SCSI hard drive (1.8 Gbyte at least) in one and

connect to the SCRAM 500 using a MAC 25->50 SCSI cable - simple.

If you are on a budget, the following method is for you.

Buy an IBM power supply cube from a PC supplier. This will have an AC

connector and several hard drive 4 pin power connectors. If your 500

power brick ever shuffles off its mortal coil, wire up the funny power

connector to the IBM power supply (standard trick).

Mounting the SCSI drive is an exercise in ingenuity. At least use a shoe

box to keep the dust off. Given the ridiculous price of IBM clone boxes,

it might be economical to buy a mini tower and put the SCSI stuff in

there. It might be possible (I haven't tried it) to actually strip down

your A500 and put it inside as well. Use the keyboard bare, or get a

2000 keyboard.

Make a SCSI cable by using 50 way ribbon and a header (to plug into the

back of the drive). At the other end of the 50 way ribbon put either a

50 IDC Centronics socket, or solder the 50 way cable wires to a DB25

direct.

The only thing to watch here is NOT to use 25 way ribbon to carry SCSI.

The whole idea of 50 way ribbon cable for SCSI is that every signal is

interleaved with GND. This guards against cross-talk and is a good idea.

The MAC cable (25 wires) is not proper SCSI - even though it is the

industry standard. It works by scrambling the wires to cancel out

cross-talk. Fast SCSI devices have been known to behave erratically with

MAC cables even on MACs, so be wary.

A nice solution would be a small board which converts a DB25 to a 50 way

header strip allowing the use of cheap 50 way ribbon with mass

termination headers at both ends. This could plug onto the SCRAM 500

directly. Such a board could make use of +5 on pin 25 to power some

diagnostic LEDs (to monitor BSY, REQ, ACK, SEL and possibly DATA).

For the serious SCSI hacker you can make a SCSI monitor with some cheap

hardware as follows:-

solder 2 DB25 (1 male 1 female) connectors back to back - this gives

you a 25 way pass-through SCSI plug with all signals exposed

solder pin 1 of a 1K SIP to pin 25 (+5 volts).

solder -ve (short wire) of various LEDS (different colour, shape) to

signals of interest on SCSI - REQ, ACK, BSY, SEL etc

solder +ve (long wire) of LEDS to SIP pins

The LEDs will be sort of 3 state indicators:- dim    = undriven

bright = active low

off    = active high

NJJ