CUSTOM CHIPS

============

You will notice that the design of the SCRAM 500 centers on four custom

GAL (Generic Array Logic) chips which do just about everything. Each

chip packs in the equivalent of 5 - 10 TTL devices, so you might guess

that a few years ago this card would probably be as big as a dinner

plate and need a cooling fan. So it is with all computer gear now -

custom logic is the only way to fly.

In addition to the space savings achieved by custom logic, there are

other advantages -

lower parts count improves fixability

increased chip functionality simplifies troubleshooting

design errors can be rectified in firmware

new features can be added by chip swapping

variations such as alternate address mapping possible

A functional description of the SCRAM 500 Chips is detailed below.

BERTIE

BERTIE controls the AutoConfigure logic of the SCRAM 500. This chip

describes the RAM and IO sections to the Amiga and sets RAM size and

base offsets.

1       AB\     Enables AutoBoot code in EPROM to be mounted

2       SEL\    Chip Select AutoConfig space

3       RES\    Reset

4       RW\     Active write cycle

5       A1      Address 1

6       A5      Address 5

7       A4      Address 4

8       A6      Address 6

9       A2      Address 2

10      GND     GND

11      A3      Address 3

12      D12     Data 12 IO

13      D13     Data 13 IO

14      D14     Data 14 IO

15      IOEN\   Latch IO space (SCSI)

16      MEMEN\  Latch memory space (Fast RAM)

17      SIZ1    RAM size option

18      SIZ0    RAM size option

19      D15     Data 15 IO

20      VCC     +5 volts

CYRIL 8

CYRIL is the SCSI AutoBoot controller chip. This chip manages data

transfers to the 8490V SCSI controller, Block DMA, interrupts, bus

synchronization and disk LED.

1       DRQ     DMA data request from SCSI chip

2       IRQ     Interrupt request from SCSI chip

3       IOEN\   IO space configured OK

4       SLCT\   IO space selected

5       RES\    Reset

6       RW\     Write enable

7       UDS\    Upper Data Strobe

8       A15     Address 15

9       A14     Address 14

10      GND     GND

11      A13     Address 13

12      XRDY    Amiga bus signal to insert wait states

13      INT2\   Amiga bus INT2

14      DISK\   LED indicator (active low)

15      ROM\    Select AutoBoot ROM

16      DATA\   SCSI DMA select

17      SCSI\   SCSI registers select

18      IOR\    Read strobe to SCSI chip

19      IOW\    Write strobe to SCSI chip

20      VCC     +5 volts

GRISWOLD

GRISWOLD is the master timing controller for the DRAMs. This chip

generates RAS and CAS, refreshes the RAMs and synchronises with the

68000.

1       CLK     14\ Meg clock generated from Amiga bus

2       CLK     14\ Meg clock

3       RW\     Write strobe

4       QC      Refresh counter bit 2

5       QD      Refresh counter bit 3

6       MSEL\   RAM space select

7       MEMEN\  RAM space configured

8       UDS\    Upper Data Strobe

9       LDS\    Lower Data Strobe

10      GND     GND

11      OE\     Enable output signals

12      SEL     Mux control low selects CAS\

13      MRAS\   RAM access RAS signal

14      RFRAS\  RAM refresh RAS signal

15      CASL\   CAS lower byte

16      CASU\   CAS upper byte

17      NC      no connect

18      RDY     Ready for 68000 (synchronisation for refresh)

19      CLR     Clear refresh counter

20      VCC     +5 volts

HUMPHREY

HUMPHREY controls RAM operation including address decoding, bank

selection, 1M/4M selection and XRDY operation.

1       RFRAS\  RAM refresh RAS cycle

2       MRAS\   RAM access RAS signal

3       RDY     RAM cycle ready for data transfer

4       A19     Address 19

5       A20     Address 20

6       RW\     Write cycle

7       A21     Address 21

8       A22     Address 22

9       A23     Address 23

10      GND     GND

11      AS\     Address Strobe

12      RAS3\   RAS Bank 3

13      MSEL\   RAM space decode

14      RAS2\   RAS Bank 2

15      RAS1\   RAS Bank 1

16      BRW\    Buffered WE\ signal to RAM array

17      4MEG\   Low signal selects 1M x 4 devices

18      XRDY    XRDY to Amiga bus to hold off DTACK\

19      RAS0\   RAS Bank 0

20      VCC     +5 volts

NJJ