Most of the components on the SCRAM 500 are readily sourced - the parts

list specifies a JDR Microdevices catalogue number where possible, however a

couple of parts may need a bit of explanation.

86 PIN EDGE CONNECTOR

This size of connector is almost unknown outside of Amiga circles,

so you might have trouble sourcing one. The type you want is the one

WITH mounting lugs - important If you are putting the SCRAM in a case

and want to fully secure the extender card. Try to find a Taiwanese

part if you want cheap.

MINI TOGGLE SWITCH

I can't find a part number for this, but it is a right angle PCB

mount unit .2" wide, .4" high and about .5" long. If you want, you

could just put jumper pins in and option for AutoBoot.

ZIP SOCKETS

Yes, there is a low cost plastic 20 pin ZIP socket (out of Taiwan),

but no, I can't tell you where to get it. Try some major socket

suppliers. If you have no joy then just get machine pin strips and

break them and solder them in place. Masking tape is useful to hold

these strips while you tack them in place.

DB25 SCSI SOCKET

This RA DB25 should be easy to source, but make sure you get the

SHORT style. There are three lengths available, but only the short

one will do.

SCSI TERMINATORS

These SIP resistor paks are 8 pin 220/330 ohm nets commonned at either

end. Not as common as other resistor paks, but fairly easy to get.

SCSI CHIP

The DP8490V is a National Semiconductor part. It is an enhanced

version of the venerable NCR 53C80 SCSI Controller - which lives in

just about every Macintosh and most Amiga SCSI boards. It is pin

compatible with the 53C80 and with apropriate driver software could

be replaced by the 53C80. Any supplier who stocks NS components

should be able to supply the DP8490V. For data, see NS MASS STORAGE

HANDBOOK.

DRAMS

The SCRAM 500 (in fact all my designs) uses ZIP DRAMS. The ZIP

package is a SIP with Zig-Zag pins. The A3000 uses ZIPs. The reason

for ZIPs is their compactness - 8Meg in an area 1" x 3.25"! The

other reason for using ZIPs is they give you a wide range of

configurations. You can use either 256Kx4 (1Meg chips) or 1Mx4 (4Meg

chips). Thus you can have 2Megs with 1M chips  - or 2Meg, 4Meg, 8Meg

by using 4M chips. Here is a list of suitable 4Meg devices:-

MICRON          MT4C4001Z-80    1Mx4

TOSHIBA         TC514400Z-80    1Mx4

FUJITSU         MB814400Z-80    1Mx4

NEC            uPD424400V-80    1Mx4

OKI            MSM514400Z-80    1Mx4

HITACHI         HM514400Z-80    1Mx4

For the sake of confusion, some DRAMs (used in the A3000) are called

Static Column RAMs - or SCRAMs for short. These DRAMs should work in

the SCRAM 500, but will not improve the performance. They are better

off in an Amiga 3000 where they will do some good! The SCRAM 500

derives its name from SCsi RAM and has no other connection with any

other Static Column RAM chip living or dead. Do not be confused!

People who upgrade their A3000 machines to Static Column DRAMs may

be a co-operative source of 4 Meg ZIPs for a SCRAM 500! They have to pull

the Page mode chips out anyway - so seek them out.

NJJ