SCRAMPREP

SCRAMPREP is a user friendly utility for partitioning SCSI hard drives.

It has an automatic mode which divides the disk into equal size

partitions and sets up the RDBs. As well you can partition, prep,

low-level format, and verify manually. I think you will find this

utility useful and easy to use. One caution - SCRAMPREP does not

AmigaFormat the partitions. After using SCRAMPREP you must reboot (to

have the RDB read) and then Format each partition.

Note that when writing RBD info to a boot partition SCRAMPREP will look

for FastFileSystem in l: - so put later versions here if you like.

FastFileSystem is built into WB 2.

ScramAutoMount

This program can be run from the Startup-Sequence and will mount a

PREPed Hard Drive (with RDB) avoiding the need for a MountList entry.

This is for people with KickStart 1.2 or who for some other reason want

to use a .device driver from the devs: directory. If you have KickStart

1.2, why don't you upgrade so you have AutoBoot?

DRIVERS

There are 4 drivers supplied on this disk. The .bin files are binaries

for ROMing. The .device files are for use with a mountlist - where they

must be called SCRAM8.device.

SCRAM_8.bin             standard driver for ROMing

SCRAM_8.device          standard driver for devs:

SCRAM_8_movep.bin       faster driver for ROMing using optimized transfer

SCRAM_8_movep.device    faster driver for devs:

The reason for including both versions is because that the faster

version MAY exhibit data errors when used with a noisy cable on very

fast SCSI drives. If you think you are having cable problems, try the

slow version (turn AutoBoot switch off copy it to devs:). Typical

diskspeeds for a Quantum are 460K B/s v. 540K B/s for the two drivers.

SCRAM_8 currently supports removeable media, SCSI direct, and some SCSI

CD ROM drives (tested with Toshiba). Coming soon is Floptical support

and AMAX support.

AUTHOR

All the SCRAM 500 software and drivers were written by:-

Will McGovern

PO Box 247

NEW LABTON

NSW     2289

Australia

EMAIL  c8841520@jupiter.newcastle.edu.au

Any requests for source code or assistance in extending the drivers

should be directed to Will.

Watch for updates.