TI 940

The TI940 emulation supports the commands as described  by  the

Texas Instruments Model 940 EVT Programmer's Reference Guide (No.

2213828-9701,	Rev.  C).

To Get		     Press Key on

TI 940 Key    Transmitted     IBM Keypad

=========================================

NUMLOK - Off

Arrows:

Up		ESC A		 Up

Down		ESC B		Down

Right 	ESC C		Right

Left		ESC D		Left

Other Commands:

INS CHAR	ESC P		Ins

DEL CHAR	ESC Q		Del

Functions:

F1	       ESC i 1		 F1

F2	       ESC i 2		 F2

Switches available on EMulate command:

EM /<switch_name>=<value>[,<switch_name>=<value>]

FKEYS=ON   (Default)  The function keys are as specified above.

OFF	      The function keys transmit the string they

have been assigned in MIRROR.