VIP7200 KEYBOARD MAP

The following table describes the special function keys of the

VIP7200  and shows the transmitted sequences.  It also shows the key

or  key sequence required to produce this function on the  IBM-PC

keyboard.  The VIP7200 has 14 function keys, four arrow keys,

To Get VIP    Characters     Press Key on

7200 Key     Transmitted     IBM Keypad

=========================================

Arrows:

Up		 ESC A		 Up

Down		 ESC B		 Down

Right 	 ESC C		 Right

Left		 ESC D		 Left

Other Commands:

Home		ESC H		Home

EOL		ESC K		PgDn

EOP		ESC J		Ctrl-PgDn

Function keys:

F1		 ESC 0		 F1

F2		 ESC 2		 F2

F3		 ESC 6		 F3

F4		 ESC 8		 F4

F5		 ESC :		 F5

F6		 ESC <		 F6

F7		 ESC >		 F7

FS1		 ESC 1		 Shift-F1

FS2		 ESC 5		 Shift-F2

FS3		 ESC 7		 Shift-F3

FS4		 ESC 9		 Shift-F4

FS5		 ESC ;		 Shift-F5

FS6		 ESC =		 Shift-F6

FS7		 ESC ?		 Shift-F7

HOME		 ESC H		 Home

CLR		 ESC `		 Keypad Minus

ERASE EOL	 ESC K		 PgDn

ERASE EOP	 ESC J		 Control-PgDn

XMIT		 ESC i		 Keypad Plus

Switches available on EMulate command:

EM /<switch_name>=<value>[,<switch_name>=<value>]

FKEYS=ON   (Default)  The function keys are as specified above.

OFF	      The function keys transmit the string they

have been assigned in MIRROR.