CDDARIP.DOC     DOCUMENTATION FOR CDDARIP

Version 0.90 on May 12, 1993

Cddarip is a new program that does the previously impossible.  It allows

one to extract audio samples directly from an audio CD (CDDA).  Until the latest

generation of drives (these expensive double speed drives), it was impossible to

read audio samples through a CD-ROM player.  Most units had provisions to play

the discs when it is not being used for computer data, and would often furnish

computer programs to emulate the buttons on a standard audio CD player.  However,

the units had their own DACs and audio was taken from a headphone or line output

connected to the drive.

This program is intended for the Toshiba XM-3401B drive.  It MAY work on

similar models by Toshiba, and it is rumored the Apple CD300 is a similar drive

inside.  CDDARIP has not been tested thereon.  It will not likely work on other

manufacturers units, thought the latest Sony and NEC drives could have this

capability.

This program came to being out of curiosity.  I had obtained the Toshiba

SCSI command manual to solve another problem(see fix_xa.lzh) and I discovered

command capability to retrieve CDDA digital audio.  I wrote a short program to

grab an arbitrary sample and save it as a raw 8 bit file.  I played back the

result and low and behold, it sounded something like the original.

To make it usable, an interface had to be generated, and this was 70% of

the programming time.  Plus everything is in BCD and some code had to be written

to handle this.  The result is basically a CDDA control program (CDDARIP can

be used for this just fine).  This allows the user to browse his CD to set the

proper times to sample.  Then he can actually sample the data to disk.

Data from this can be used with other programs, as it puts it out in the

standard IFF 8SVX format.  Think of it, your favorite guitar licks can come up

when there is a system message with 2.0 or above.

Cddarip was compiled with SAS/C  6.02.  It is designed to work with all

68000 family processors.

REQIREMENTS:

TOSHIBA XM-3401-B or similar 3400 series drive.  Apple CD300 MAY work not tested.

SCSI Interface that supports SCSI direct.  Tested on Amiga 3000.

Headphones or amplifier/speakers on 3401's audio output.

WorkBench/Kickstart (TM Commodore-Amiga inc) 2.04 or greater.

Note: It should NOT require any CD ROM filesystem as it accesses SCSI directly.

Hard disk for saving larger samples.

USING CDDARIP:

There is no special installation procedure.  Copy this file to any directory

though c: is a good place.  And this doc file where you can get to it easily if

you have questions later.

This program is run from CLI or shell.  Type "CDDARIP" Unit [device] <CR>

where unit number is the SCSI number(0-6).  Device is entered if NOT using

a Commodore 2091 card or A3000.  If you have either of these, do not enter

a device.  For a GVP controller on unit 3, you would type "cddarip 3 gvpscsi.device""'

and so on.

A window will pop up in Workbench screen.  At the top is fifty squares with

numbers in them.  These select a track on the disc.  If you have a CDDA disc

inserted when starting, the valid tracks are highlighted.  Click on one of these

with the mouse and it will start playing.  Just like my Sony player.  Below

that is a rectangle with a status line in it.  Watch this space, the program may

be telling you something.  During play, it will give the status, the track number,

then the time from disc start, and time into track.  If you make an error, it

will tell you.  If the drive is not connected correctly or wrong type of disc in

you will get a CMD Sense Error.  This is the drive saying it cannot complete the

command given it.  This is not a fatal condition, correcting the problem and

retrying will remove this.  The program also queries the CDROM every half second

or so to update status.  This will come up when changed.

Next is a row of transport buttons.  These work like their counterparts on

an audio player and do not need much explanation.   There is start of track,

reverse scan, pause, play, forward scan, and next track.  Note there is no stop

button,  this is not in the command set for some reason, use pause instead.

At the bottom, there are  four slightly longer buttons.  The  PLAY SEL

will play the portion selected to be sampled as a preview.  It and all playing

is done thru the Toshiba audio, NOT the Amigas.  When you are sure of what you

want to sample, press CONVERT.  This will create the IFF file.  In this mode

the double speed is disabled, and it takes whatever time the sample is to

convert, plus a little for overhead.  If you change your mind during a long

sample, hit ABORT.  Keep in mind the partial result may not be usable.

Keep in mind at the default 22KHz, that a five second sample is over 100K.

make sure there is enough room available.  I do not recommend saving directly

to floppies.  RAM may be used but be sure you have enough room.

CHK DISC will update the track table AND SHOULD BE SELECTED ONCE AFTER THE

CD IS CHANGED OR INSERTED.   Future versions may autosense this.

On the right side are some parameters.  First is the Output Rate, this sets

the sample rate.  Default is 22KHz or 1/2 the CD frequency.  11KHz can be selected

to save RAM in non critical applications but quality is poor.  44KHZ or full

rate can also be used but most all Amigas cannot play back this fast so pitches

will be flat.   Just below that is Output Mode.  This selects IFF MONO, which is

a mono mix of both channels.  Also, left only or right only can be selected.

Stereo was deleted because of format problems that require ALL left channel THEN

all right channel data instead of interleaving them.  It is possible to take a

left and a right sample and stitch them together in another program.

The next two string gadgets set the path and the file name.  This determines

what the file created will be called.  This is pretty crude now and it is up to

the user to set this and not write over any previous work.

The bottem right contains two string and two button gadgets.  Start time

is the time from start of disc that sampling will start at.  It can be set three

ways.  Hitting the button called 'NOW' to the right will set it to the current

play time in the status window.  Keep in mind this is not real accurate and will

usually be 0.5-1 second later.  The next way is using keys to fine adjust.  Using

the up and down arrow will add and subtract one frame (1/75 sec).  The PgUp and

PgDn will add or subtract one second.  Holding the shift key during all this

will multiply these by 8.  The third way is direct entry, click the mouse on

either the min, sec or frame and change it.  If an invalid combo is entered,

a BCD error message will appear and the previous value returned.  The sample

duration is set one of two ways.  First hitting the END button will set the

current play position to be the end point IF it is greater than the start time

else it is ignored.  This can be directly entered as well.  No keys have been

set to this yet, but may be in the future.

The BCD format is:  MM:SS:FF     Where,

MM= minutes, can be 00-74.

SS= seconds, can be 00-59.

FF= frames , can be 00-74.

To exit, hit the close button in the upper left corner.  No requesters will

appear, and no one will bug you about shareware fee.  This is not shareware at

this point and would only become so if more complex features are added.  This

has been an exploration and mastry of SCSI.  Note that the CD will continue

to play even when stopped.  It may be desirable to hit pause first.

LEGAL:

CDDARip is  1993 Jim Boros, all rights reserved.

This softaware is not public domain but may be freely distributed provided

that:

1)  The program and documentation are not modified and are kept intact.

2)  This is not sold for any cost other than connect time or resonable

disk charge (<$6).

3)  It is not included in any commercial product.

No warranties of any type, expressed or implied are granted.  I will

not be liable for any damage to software or hardware, or any patent or copyright

infringement arising out of this product.    This program is able to copy commercial music, and is supplied for test and

educational purposes only.  Using samples from CD may require consent from the

copyright holder.  Especially for commercial or publicly visible application.

The music industry probably has concerns about perfect duplication of CD and

other digital audio.  This will not happen with CDDArip since the quality is

degraded to 8 bits, less than a good analogue copy.   Also the cost of media and

drives for that magnitude of data would be many times the cost of the CD in a

record store.

This may be distributed by Fred Fish, and by any BBS, and any redistributable

software library.  And it may be distributed anywhere in the world.

TO DO:

Support for other audio formats, IE AIFF.  With slightly higher resolution.

More likely as higher quality sound becomes available for Amiga.

Clean up interface and make it a little easier to use.

Fix any bugs.

CONTACT:

I'm on BIX, the handle is jboros.  Leave any bug reports or suggestions

there.