****    BTNtape:  an AmigaDOS handler for SCSI tape drives

****       Version 2.0   3/28/91

****       Freeware by Bob Rethemeyer   (DrBob@cup.portal.com)

****

****    (c) Copyright 1990, 1991  Robert Rethemeyer.

****      This software may be freely distributed and redistributed,

****      for non-commercial purposes, provided this notice is included.

****      All original files must be included in the distribution.

BTNtape is a "Better Than Nothing" SCSI tape device handler.  It provides

flat-file access to a SCSI tape drive from application programs using

simple DOS calls to Read() and Write().  It can also be used with the Amiga

TAR utility for disk backups.  It requires a "SCSI-direct" compatible

SCSI bus adapter.  This is version 2.0, update to version 1.0.  It now

supports many more tape drives, and includes some new features.

Some advantages of BTN over other handlers and dedicated tape backup programs:

* Includes a monitor program that keeps you informed of what's happening

* Supports many standard SCSI tape drives and Amiga SCSI bus adapters

* Supports the 3M MCD-40, a non-standard SCSI tape drive

* Supports any size of tape block, and multiple-block accesses

* Supports tape files that span multiple tape volumes

* Double buffering improves streaming performance

* Includes an interface to send SCSI control commands to the drive

* Includes an interface to set drive-specific Mode Select parameters

* TAR tapes can be exchanged with Un*x systems

=================== ******> R E A D   T H I S <****** ======================

DISCLAIMER:

"This software is provided 'as-is', without warranty of any kind,

either expressed or implied.  In no event will I, Robert Rethemeyer,

be liable for direct, indirect, incidental or consequential damages or

data loss resulting from the use or application of this software.  The entire

risk as to the results and performance of this software is assumed by you."

I cannot guarantee that this software is compatible with any combinations

of drives or adapters other than those I have specifically tested.

If you do not agree with this disclaimer,  I recommend you not use this

software;  wait for commercial tape drivers.

============================================================================

This handler is probably not the all-purpose tape driver you have been waiting

for.  It won't work with any backup program which requires Exec devices.

I claim only that it works with the Amiga TAR program ported by Jonathan Hue.

But until we see good commercial or shareware tape software, you might

find this handler to be "better than nothing".

I will try to provide support for this handler, especially for bug fixes,

as my time and resources allow.  But I cannot commit to adding all the

enhancements or special features requested by users.  If you have something

in mind, ask, but I can't guarantee anything.

Although this documentation refers to alleged problems with some

manufacturers' driver software, that should not be construed as a statement

about the quality of their products.  In fact, there is still a possibility

that the problems may be in my handler software or the equipment I used.

My apologies to those people who tried to use version 1.0 and didn't have

a 3M tape drive.  At that time I naively thought that all SCSI tape drives

must look the same, and wrote as though that were true.  Wrongo...

So this version should handle many more types of drives, but there is

no way I can test all the combinations of drives and SCSI adapters.

==============================================================================

DISTRIBUTION:

The distribution should contain the following files:

tape-handler          handler binary

tape.c                source for main handler

tapeio.c              source for interface to SCSI-direct

pktstuff.c            source for packet handling, by Phillip Lindsay

tape.h                local include file

tplink.h              local include file for handler/monitor structure

lmkfile               SAS/Lattice make file for handler

makefile              Manx make file for handler

tapemon               directory

tapemon               monitor binary

tapemon.c             monitor source

tapemon.doc           documentation for the monitor program

tapemon.lmk           SAS/Lattice make file for tapemon

mountlist             example mountlist entry

changes.doc           a brief history of BTNtape

tape.doc              you're reading it

COMPATIBILITY:

You must have a SCSI adapter and software that honors the SCSI-direct

protocol as defined in the "devices/scsidisk.h" include file.  The list

below shows the manufacturers I know about that support SCSI-direct

(but this list I'm sure is incomplete).

This handler has been tested on the following equipment (but not in

all the combinations):

SCSI adapters:

* Commodore A2091

* Commodore A590

* Commodore A3000 (built-in), including DOS 2.0

* GVP Series II

* HardFrame

* Nexus

* Commodore 2090A- works *if* the hard drive is ST506.

* Supra 4x4 (Amiga 1000)- works, but see Supra user notes.

* Trumpcard- with 2.0F software, 4.0 ROMs, Interrupt 2 jumper.

Tape drives:

* Archive 2150S (Viper)

* Teac MT-2ST Cassette

* Exabyte EXB-8200 8mm Cartridge Tape Subsystem

* Emulex MT-02 SCSI/QIC-36 tape adapter

* 3M MCD-40  40MB cartridge tape drive (direct access)

Even if your hardware is not listed here, there is a good chance it

will still work if it adheres to the standards (SCSI-direct and SCSI).

The handler treats a drive as one of two types: sequential or direct access.

The 3M drive is the only direct access drive I currently know about.

Everything else is usually sequential.  The handler will adjust itself

for either type.  If you get it to work with some drive model I haven't

mentioned here, I would like to hear about it.

INSTALLATION:

1.  Copy the TAPE-HANDLER file to L:TAPE-HANDLER.

Copy the TAPEMON program to C: or a directory appropriate for you.

2.  Create an entry in DEVS:MOUNTLIST something like this:

TAPE: Handler = L:tape-handler

Priority = 5         /* use 11 for Supra */

Mount = 1            /* optional */

Stacksize = 4000

GlobVec = -1

Startup = "yourscsi.device/AA-x/BB-x/..." /* EXPLAINED BELOW */

#

There is nothing magic about the name TAPE:   You may use some

other valid device name if you like.

3.  Install the handler in DOS using the CLI command "MOUNT TAPE:"

Do not use the ARP version of MOUNT !!

If you get a write-protect requester, click Cancel on it

(this is a side effect of MOUNT- it tries to open TAPE in write mode).

Unfortunately, if the handler fails due to a mountlist or OpenDevice

problem, there is no visible error indication- MOUNT still works.

[does anyone know how I can make MOUNT fail?]

The device will show up in "ASSIGN LIST", but no handler process exists.

Type "ASSIGN TAPE: REMOVE", fix the problem, and try again.

THE STARTUP PARAMETER

The mountlist Startup parameter is used by the handler to determine how

to access your drive.  FAILURE TO ENTER PROPER INFORMATION IN STARTUP

MAY RESULT IN PROBLEMS INCLUDING HANGS, VISITS FROM THE GURU,

OR DATA LOSS.  This is the messiest part of setting up.  Pay attention.

*NOTE*  The Startup string format has been changed from v1.0

of the handler.  You must change to the new format.  It is more

readable, more flexible, and can be more easily expanded in the future.

*NOTE*  The new Startup string format will NOT work with the

ARP (AmigaDOS Replacement Project) version of the MOUNT command.

You MUST use the Commodore version of MOUNT.  To tell the difference,

ARP 1.3 MOUNT is 2372 bytes long, and CBM MOUNT (1.3.2) is 5604 bytes.

ARP MOUNT inexplicably truncates Startup at 40 bytes.  There is no need

to completely abandon ARP- you can still use the CBM MOUNT.

Just give the two MOUNT commands different names in your C: directory.

The Startup string must be in the form:

Startup = "devicename/keywd-num/keywd-num/..."

It must not have any embedded blanks and must be enclosed in quotes.

The pieces must be separated by slashes.

The first parameter must be the name of the device driver which provides

the SCSI-direct command interface. Do not include any path information.

This name is defined by the manufacturer of your SCSI adapter.

(v1.0 users- DO NOT use the old circumvention characters: $ or + ).

Note letter cases.  The names for the adapters I know about are:

A3000,A2091,A590 -> scsi.device

A2090A           -> hddisk.device

Supra            -> supradirect.device

GVP              -> gvpscsi.device

HardFrame        -> HardFrame.device

Nexus            -> Nexus.device  (NOT NexusTape.device!)

Trumpcard        -> IVS_SCSI.device

ICD Advantage    -> icddisk.device

The name parameter MUST be the first item in the string.  All other

parameters may be in any order, but the name must be first.

The remaining items consist of pairs of two-letter keywords and a number

separated by a dash (minus sign).  The keywords are explained below.

You may omit any of them (except UNit), and a default value will be

assumed.  The numbers may be decimal, or hex in the "0xNN" format.

Keyword-default

---------------

UN       UNit:  The unit number of your tape drive, as it will be

addressed by your hard disk driver.  Usually you can

just specify the SCSI bus ID, example: "UN-0" for 0.

More complex setups may require the form "UN-0xBLI",

where B=board#, L=logical unit#, I=SCSI bus ID#.

Be sure you have set the drive ID switches or jumpers

to a value that is unique on the SCSI bus.

2090A owners- add 3 to the ID number.

LU-0     Logical Unit:  another number that may be required to address

your drive.  Most drives are hardwired for 0, so this

parameter may be omitted.  It is provided here for the

few exceptions that are probably out there.

BS-512   Block Size: the number of bytes in a SCSI logical block for

your tape drive.  Consult your drive documentation.

Some drives allow you to specify different

block sizes, within a legal range;  the handler will

command the drive to use the size you specify.

Others can use only one fixed size.  If you can vary

the block size, you must use the same size when reading

a tape as you did when writing it.

NB-1     Number of Blocks:  the number of SCSI logical blocks to write

or read in each tape operation.  See below.

BT-0     BufmemType:  the type of memory to allocate for the tape

buffers, same as BufMemType for mountlists.  Use

0 or 1 for "any", 2 or 3 for "chip", 4 or 5 for "fast".

FM-1     FileMarks:  specifies number of file marks to write when

closing at the end of a write, or the number to skip

for each file when opening for read.

Allowed range is 0-255.  Sequential drives only.

VB-0     Variable Block flag:  the handler normally operates sequential

drives in fixed block mode.  Specifying VB-1 will force

operation in variable block mode instead.  Use this

if your drive does not support fixed block mode or

you for some reason need variable block mode.

RB-0     Reserved Blocks:  for direct access drives only.  Sets how

many blocks to skip at the beginning of the tape.

OV-1     OVerlapped I/O Flag:  the handler normally attempts to overlap

tape drive I/O and DOS I/O to increase throughput.

Specifying OV-0 disables overlapping and double buffering

for systems that can't walk and chew gum at the same time.

DF-0     Device Flags:  equivalent to the Flags parameter in mountlists.

It is passed in the OpenDevice() call to open the SCSI-

direct driver. If you don't know what you would use

this parameter for, then you probably don't need it.

SU-0     SUpra Flag: activates the circumvention for the Supra 4x4

v1.10 problem.  Specify SU-1 to activate this.

See below. (Replaces "+" in handler v1.0)

C9-0     CBM 2090A Flag:  activates the circumvention for the Commodore

2090A problem.  Specify C9-1 to activate this.

See below. (Replaces "$" in handler v1.0)

Here are some examples of valid Startup strings:

Startup = "scsi.device/UN-7"   /* the minimum info required */

Startup = "scsi.device/BS-0x400/NB-16/LU-3/UN-0x102"

Startup = "supradirect.device/un-4/bs-8192/nb-8/bt-1/su-1"

A note about the NB parameter:  Some drives may be mechanically more

efficient writing multiple blocks.  You may want to tweak this

parameter to find out.  The handler will wait until NB blocks have been

accumulated before starting a write.   Buffered drives may also have a

limit on the number of blocks they can hold at once, so the size of NB

relative to that number may have an effect.  Consult your drive docs.

In general, the more blocks the better.

This parameter also determines how much memory is needed for

buffers.  The handler uses double buffering, so it will need

2 * BS * NB bytes when you Open() it.

For example, if BS=8192 and NB=8, then 8192*8 = 64K,

for a total of 128K.

2090A USERS- NOTE

Apparently (I don't own a 2090A) there is a software bug in the

CBM 2090A SCSI-direct driver that causes problems with tape

writes.  Bob Mitchell figured out that the problem can be

circumvented by asserting data address bit 24 for writes.

This handler can optionally invoke that circumvention.

To do so, specify "C9-1" in the Startup string.

The circumvention is NOT necessary for other adapters.

A worse problem- the 2090A cannot seem to handle both the disk

and the tape drive on the same SCSI bus.  Simple reads or writes

to the tape work fine, but intermixed tape reads and disk writes

(such as during a restore) will cause the 2090A to get confused.

This does not happen if the hard drive is on the ST506 bus.

It also does not happen if you restore the files to RAM or floppy.

Make sure the tape is out of the drive when you boot

AmigaDOS;  otherwise, DOS never comes up.

When you mount TAPE:,  make sure there is no cartridge in the

tape drive.  For some reason I do not understand, the 2090A

causes a reset to the tape drive when OpenDevice is called.

If a cartridge is in the drive at that time, the system

eventually enters an unusable state.  I recommend using

"Mount = 1" in your MountList entry for TAPE: so when TAPE:

is mounted, it will put up a requester because the drive is

empty;  click on cancel, then everything will be happy.

SUPRA USERS- NOTE

Apparently (I DO own one) there is a problem with the Supra v1.10

SCSI-direct software that causes the host adapter to hang when a

non-zero byte count is supplied in the command (SCSICmd.scsi_Length).

It works fine when a data length of zero is used.  The handler can also

circumvent this problem.  Specify "SU-1" in the Startup parameter.

I have reported this to Supra, and they promised to look into it.

You MUST run the tape handler at a higher priority than the

Supra driver task.  You do this by changing the "Priority ="

line in the mountlist entry.  For Supra v1.10, the task runs

at priority 10.  Therefore, the tape handler should run at

"Priority = 11".  If some other Supra version runs at a different

priority, change the mountlist priority accordingly.

Use a utility such as Xoper or TaskX to find task priorities.

If the cartridge is in the drive at the time you boot AmigaDOS,

SupraMount will hang the system if it touches the tape drive.

To avoid this, use the selective mount feature of SupraMount

to mount only your disk unit.  Example: "SupraMount 6".

HOW TO ACCESS TAPE DATA

Now that the handler is installed, how do you talk to it?

You can use TAR, or your own application program, including ARexx.

You can write data to the tape and later read it back in a manner

similar to other files, using DOS calls to Open, Read, Write, and Close.

The handler makes the tape data look like a sequential AmigaDOS file.

There is NO FILE SYSTEM on the tape, but you may place multiple files

on a tape by positioning the tape to specific file marks or blocks.

The handler does not support mixed reads and writes.  That is, you may

do only Reads or only Writes, but not both.  Seeks are not allowed.

You cannot Open the tape in "append" mode.

Tape data is written in quantities of "NB*BS" bytes.  No information

about the exact length of the file is written.  If a tape file

ends before filling a block, the remainder of the block is filled with

zeroes.  Therefore, any program reading the tape must know when to

stop reading data, otherwise it will read extraneous zeroes past

true end of the file.  For instance, this will not work, because the

second copy won't know the size of the file it is copying:

COPY filename TAPE:

COPY TAPE: newfile    ; will read too much data

TAR works because it essentially writes and interprets the tape data

as its own little file system that includes the length of each file.

When a tape file is closed in write mode, one or more filemarks

(as determined by "FM") are written at the end of the file to

delineate the file from subsequent files. (Sequential drives only).

If there is no tape in the drive at the time TAPE: is

opened, DOS will put up a requester asking for volume "TAPE"

"in any drive".  Insert the tape (in the tape drive, not

a floppy drive :-)  and click on retry after tape motion stops.

Refer to the tape with one of the following file name forms:

TAPE:     Rewinds and opens the tape at the first available block.

TAPE:*    Opens the tape at the current position, without rewinding.

TAPE:num  Opens the tape at the position specified by the number.

For sequential drives, the number indicates the file position.

The 0th file is at the beginning of the tape, file 1 follows it,

etc.  Note, the files must have been previously written on the tape.

Files are separated by one or more filemarks as specified by the

FM startup parameter.  For the 3M direct access drive, the number

specifies an absolute SCSI block number on the tape.  You must

remember (write down) the block number of the beginning of each

file on the tape, as reported by TAPEMON.

TAPE:RAWCMD   A special mode for issuing a raw SCSI command, not for

accessing the tape medium.  See "RAW COMMAND INTERFACE" below.

TAPE:MODESEL  A special mode for writing mode-select data, not for

accessing the tape medium.  See "USER-SPECIFIED MODE SELECT" below.

A QUICK FUNCTIONALITY TEST

Before jumping into disk backups and restores, try the following

simple test to see if your setup is basically working.

Write a large ASCII text file, such as TAPE.DOC, to the tape

and see if you can read it back intact.  This can be done with the

CLI TYPE command:

TYPE tape.doc TO TAPE:     ; writes the file to tape

then... TYPE TAPE:                 ; reads the file back

The second TYPE should legibly print the ASCII file on your screen.

When you have seen enough, use control-C to abort it.

USING TAR FOR BACKUPS AND RESTORES

For backups, I use the version of TAR ported by Jonathan Hue.

It is 33860 bytes long.   Each TAR archive is written to the tape

as a single file, terminated by a file mark (on sequential drives).

NOTE: the version of TAR distributed with Amiga UUCP apparently

does NOT work with BTNtape.  I don't know why.   Other similar

programs, like PAX, may also work, but I haven't tried them.

TAR will backup all directories, files, and subdirectories in the

directory you specify, including the date, protection, and filenote

attributes.  The easiest way to do a backup is to CD to the desired

directory or partition (e.g. "cd DH0:"), then...

To make a backup archive with TAR  :   tar -cv -f tape: ""

(note the null string)

To add another archive file        :   tar -cv -f tape:* ""

To restore files from archive      :   tar -xv -f tape: *

To restore a specific file         :   tar -xv -f tape: path/filename

To restore from archive at file 6  :   tar -xv -f tape:6 *

To list the archive on the tape    :   tar -t  -f tape:

(this also tests archive integrity)

To make an archive log             :   tar >logfile -t -f tape:

To look at raw tape data           :   type tape: opt h  ... cntl-C

TAR can also make backups using a list of files from another file.

When you restore an individual file from an archive, provide the

exact path and file name;  these names are case sensitive.

Refer to the TAR documentation for more information.

END-OF-TAPE HANDLING

If the application reaches the end of the tape, the handler will put up

a requester asking you to insert the next tape and click "Continue",

or click "Abort".  The continue option allows the application program

to access a file across multiple tape volumes without interruption.

WAIT FOR TAPE MOTION TO STOP on the new tape before clicking Continue.

If you click Abort, an I/O error is returned to the application.

*NOTE*  EOT handling is invoked when any of the blocks in the group of "NB"

blocks gets an EOM or Volume Overflow error.  When you click Continue,

the entire group of blocks is rewritten/reread on the new tape.

Therefore, to use the multiple-volume capability of the handler, you must

use the same blocking factor (value of NB) when reading the tape as

when it was written.

*NOTE*  EOT handling for the 3M drive will work only if the ROM version

supports the READ_CAPACITY SCSI command.  If you see no "Capacity"

message from TAPEMON, then EOT handling is unavailable.

ERROR HANDLING

This version of the handler does not deal with tape errors.  If the

drive detects a medium error, the current operation will be aborted

by returning -1 for Read() or Write().  Some drives insulate the

user from bad blocks by automatically correcting them or remapping

to a good block, but other drives may not do this.

3M DRIVE USERS- NOTE

The following applies only to the 3M MCD-40 direct access drive.

The 3M drive does not use file marks.  When you reference the

tape as "TAPE:num", num is the SCSI block number, not the file number.

When writing multiple files to a tape, you should write down the

block numbers where each file starts (unless you have a good memory).

Use the TAPEMON program to see the block numbers.

The RB parameter in Startup may be used to skip over a number of

blocks at the beginning of the tape, if there is something there

(like volume information) you wish to preserve.

Be sure your cartridge is formatted.  If you try to use an

unformatted tape, results will be non-optimal.  You can buy

preformatted tapes, or you can format your own- see the section on

raw commands below. Cartridges may be formatted with an interleave

factor. Interleave is the number of physical blocks separating adjacent

logical blocks.  The 3M unit defaults to interleave 2, but I find

for my Supra that 3 improves performance slightly.

The raw command to format a tape is:  04 80 20 00 00 80

which denotes the format command, immediate completion, precondition

the tape, default interleave, and retry on errors.

A note about the hardware.  If you haven't noticed it yet, the

3M drive is EXTREMELY sensitive to electrical noise.  It is

*essenstial* that the drive be properly grounded.  The ground

in the power connector is insufficient.  You should run a thick

wire or braided strap from the metal side-plate to a good ground,

preferrably the power supply ground.  I'm not kidding.

HELPFUL HINTS:

Label your tapes.  Include the file number or block number

of each file on a multi-file tape.

Try backing up and restoring a floppy before doing the

same with your hard drive, to make sure the process works for you.

For the same reason, make a HD backup to floppies first.  Then

if the tape restore fails, you have something to fall back on.

This is obvious, right?  Make sure you have a copy of the handler and

TAR on an easily-accessible floppy.  If the only copy you have is

on your hard drive, you might find it difficult to restore

from a tape backup after a disk wipe-out.

THE MONITOR PROGRAM

Included in with the handler is the TAPEMON program.  TAPEMON is

a program that runs under a separate CLI and prints one-liner

messages from the handler.  The messages include the current

tape operation and block number, and error and sense codes.

The handler will run with or without TAPEMON; i.e. TAPEMON is optional.

TAPEMON must be started up AFTER the handler is mounted and activated.

Read TAPEMON.DOC for more information.

RAW COMMAND INTERFACE

This handler provides a means to send your own SCSI control commands

to the tape drive, without the need for a special program.

Some control commands you might want to send it would be REWIND,

UNLOAD, ERASE, FORMAT, LOCK, etc.  Only control commands may be sent-

there is no provision for any data transfer; don't try it.

(As you might guess, raw commands can be dangerous.)

The raw command mode is invoked by writing data to the file name

"TAPE:RAWCMD".  (v1.0 users- the dollar signs are no longer used).

This magic word is how the handler knows you want to do a raw command.

There are two ways to send data via RAWCMD using CLI commands:

1) use the ECHO command:     ECHO >TAPE:RAWCMD "hex command bytes..."

2) use the TYPE command:     TYPE filename TO TAPE:RAWCMD

The ECHO command is useful to place in a script or create an alias

for the command.  You may also place the data in a file and use TYPE;

line 1 of the file contains the bytes to be sent as the raw command.

The exact value of the data bytes is determined by the byte format of

the SCSI command you want to send. (Refer to your drive documentation.)

The data must contain ONLY hex digits and spaces terminated, by a newline.

Each byte of the raw command should be specified as a pair of hex

digits;  separate bytes by spaces.  Up to 10 bytes may be specified.

The data for TYPE must be on the first line of the file.

You can use any following lines as comments, but the total size of

the file must be less than the size of BS*NB.

Command examples:

Rewind:     01 00 00 00 00 00

Erase:      19 00 00 00 00 00

Lock:       1E 00 00 00 01 00

USER-SPECIFIED MODE SELECT

In recognition of the fact that it will never be able to provide

switches for all possible modes of all tape drives, BTNtape

allows you to set the modes of the drive directly.

These modes might include:  buffering, speed, auto-loading,

enable disconnect, medium type, etc.

*NOTE* Do not use this feature unless you know what you are doing.

This feature is provided for the convenience of "power users".

Most drives can get by with defaults if you do not set the modes,

so it is unlikely that not setting a mode will cause problems.

Drive modes are set using the SCSI command MODE_SELECT (0x15).

BTNtape will send its own MODE_SELECT to the drive if necessary

to set the drive to fixed block mode and set the block length.

But you may also send this command to the drive at other times,

using any data you wish.

The command is sent in a manner very similar to the RAWCMD

method described above, but the magic word MODESEL is

used instead.  The handler provides the SCSI command,

and you provide the data.  The data is specified as pairs of

hex digits separated by spaces, in a file for TYPE, or directly

in an ECHO command.

ECHO >TAPE:MODESEL "hex data bytes..."

or   TYPE filename TO TAPE:MODESEL

Since each drive has different things that are affected by MODE_SELECT,

you MUST refer to the programming documentation for your drive.

But in general, the mode select data consists of a parameter

list, a block descriptor, and vendor unique parameters.

*NOTE*  If you specify a block descriptor, it will behoove you to

specify the same block length as specified in the mountlist entry

when using fixed block mode, or zero if using variable block mode.

The handler counts the number of bytes you specify (up to 64),

and uses that for the SCSI command data length.  The drive may

be fussy about the length, so specify exactly the number of bytes

as described in your docs.

Mode Select data example:

--parmlist-   ----block-descriptor---   vendorparms

00 10 00 08   03 00 00 00 00 00 04 00   11 22 33 44

|     |    |                 || ||

Buff mode  BD Len  Density         Blk Len

FINE TUNING

There are a few factors you can tweak to get good performance

on your particular system.  Good performance means the tape tends

to run continuously for long periods of time and seldom has

to stop and back up to get a running start at the next block.

The tape may have to stop while TAR gets more data from the

hard disk, but you can minimize it.

The handler's NB parameter can be increased from 1 to

some larger number as your memory allows.  With more handler

blocks, a large quantity of data is available to dump to the tape,

but there is probably a point of diminishing return.

The TAR program also has a block parameter.  It controls how much

DOS data TAR collects before sending it to the handler.  If you

use many TAR blocks, it makes no sense to also use many handler

blocks,  since the data has to go through both buffers

anyway.  You can increase the TAR blocks if your SCSI-

direct driver does not support multi-block operations.

Using the TAPEMON program may slow down the handler slightly

when it prints messages, but it's not really a problem.

If you absolutely need maximum throughput, shut down TAPEMON.

HOW IT WORKS

Two buffers are maintained so that one buffer can be used by DOS

while the other is exchanging data with the drive.  With the

multitasking nature of the Amiga, this should help keep the tape

in motion, but it may not help much with non-DMA SCSI adapters.

In write mode, the handler collects DOS data in one buffer while

the previously filled buffer is being written to tape.  In read mode,

the handler reads ahead on the tape to fill one buffer while

DOS gets data from the previously read buffer.

The handler does not write (or expect) anything special on

the tape for itself.  That is to say, the data written on the

tape is your raw data, nothing else.  If I were to use parts

of the tape for file system information, I would be

introducing a sort of standard-- I don't think that is a good

idea right now.  Raw data provides a lowest common denominator

which should help compatibility with other things.  For instance,

a TAR tape created on a non-Amiga system is readable using

this handler and Amiga TAR.

BUGS

The variable block mode of operation (VB-1) has not been tested.

The handler has not been tested with AmigaDOS 1.2.

The handler does not multi-task well, but I believe that to be

a characteristic of the SCSI-direct drivers, not the handler.

Don't expect to do much else while a backup is in progress.

In fact, other things going on can reduce tape drive performance.

AUTHOR:

Email:  DrBob@cup.portal.com   OR   ...!sun!portal!cup.portal.com!DrBob

USmail: Robert Rethemeyer

979-4 Belmont Terrace

Sunnyvale, CA  94086

USA

If you want to contact me about a problem, please try to include

with your mail the following:

*  a copy of your mountlist entry for TAPE:

*  a capture of the output from TAPEMON.  Just redirect its

output to a file:     TAPEMON >filename

*  a verbal description of what happens to the drive and/or Amiga

(e.g.  tape doesn't move, smoke issues from Amiga, etc.)

*  any other info that may help me figure out what's going on.

Remember, I'm not there to see what is happening, and I

probably don't have your drive model available to recreate

the problem.  I have only your description to go by.

MANY THANKS GO TO:

Bob Mitchell for the SCSI-direct programs I used to get started,

and the 2090A circumvention.

Beta testers Bill Seymour, Don Phillips, Steve Goldstein,

Dick Wood, Don Camp, George Armhold, Gary Walborn.

Phillip Lindsay for the my.handler example code.

Rene' Vega for advice and Manxification.