PROGRAM

Set040  (2nd Release)

The command syntax of this new release of Set040

is not compatible with the older V1.xx releases of

Set040.  Please read this entire document file so you

can become familiar with the new syntax.

SOURCE

Written in Assembly Language using Macro68 V3.170

AUTHOR

Nic Wilson

Nic Wilson Software

P.O. Box 1164

TOOWOOMBA QUEENSLAND 4350

Australia

EMAIL  cbmaus!cbmozq!wilson!nicw@cbmvax.commodore.com

CBMNET nicw@wilson.adsp.sub.org

TELEPHONE/FAX +61 76 358539 After Hours Australian Time

DISTRIBUTION

This release of Set040 is shareware and freely distributable.

Electronic distribution is allowed.

DISTRIBUTION

Permission is hereby granted to distribute this program on the

condition that no profit is gained gained from its distribution.

All document files must accompany it and remain unchanged.  If the

document files are crunched, then a suitable de-cruncher must also

be supplied with them that works on ALL CPU version Amigas.

It MAY NOT be distributed on those so called 'copyright' non

distributable PD disks.  It MAY be distributed on a BBS as long as

that BBS does NOT have individual charges for downloading of files.

Please send any bug reports or ideas to the address at the top of

this file.

Commercial companies may also distribute the program without

permission, but I would appreciate being notified so I can

inform you of updates.

* * * DONATIONS * * *

---------------------

TO DATE THE DISTRIBUTION AND USAGE HAS BEEN LARGE, BASED ON THE

AMOUNT OF EMAIL AND UPGRADE ENQUIRIES I HAVE RECEIVED,  AND YET I

HAVE RECEIVED VERY FEW CONTRIBUTIONS.  COME ON GUYS, I SPENT 12

MONTHS WRITING THIS PROGRAM AND I AM STILL UPDATING AND ENHANCING

IT.

A dontation of $25.00 American or equivalent is suggested, OR if

you cannot afford that, any amount will be gratefully accepted.

Overseas users, please DO NOT send personal or EuroChecks.  They

are very difficult and expensive to cash.  Please send an overseas

bank draft in Australian Dollars drawn on any Australian bank.  Your

local bank should be able to help you with this.

Any cash currency is acceptable as it is very easy to convert to

Australian.

The following credit cards are accepted.

OVERSEAS  -> MASTERCARD, VISA.

AUSTRALIA -> BANKCARD, MASTERCARD, VISA.

These cards can be accepted by mail, email or phone.  Confidentiality

is guaranteed.  If using credit card, please include the card number,

the expiry date and the name as it appears on the card.

Donations in the form of software or hardware is also acceptable.  I

write many reviews here in Australia and quite often give free plugs

to companies that have been generous enough to donate to my work.

COPYRIGHT

This program, and its source code & document files are

Copyright (C) Nic Wilson Software 1991-1993.

FUNCTION

1. To remap Kickstart V2.04 or greater from ROM into

FAST RAM on an Amiga equipped with a 68040 CPU, using

the MMU, with options for various functions.

This remapped kickstart gives a vast improvement in

the speed the 68040 can access kickstart, and faster than

any hardware re-mapping of Kickstart.

2. To remap Kickstart V2.04 or greater on an Amiga equipped

with an A3640 card such as the A4000 and to survive a

reboot without using the MMU.

3. FASTROM can be done in a default memory efficient mode,

or a full map addressing mode.

4. To allow the FASTROM remap to be removed and

all resources reclaimed.

5. To load and remap, a different Kickstart from disk or file

and use the MMU to boot into this Kickstart.  Kickstarts

1.2, 1.3. 2.04, 3.0 or greater are supported.  An option

to completely survive a reboot is supported.

6. To manipulate both the 68040 caches and the various modes.

7. To display brief or verbose information regarding the

setting of the MMU remap, and 68040 registers and modes.

8. To switch between the 68040 and 68030 on any Progressive

Peripherals & Software 68040 card.

9. To sum up, this program gives the user of a 68040 the

facilities he lost when ROMS were fitted, plus some

extra ones and with greater flexibility.

Page 1

CONTENTS

Disclaimer.............................................. 2

Acknowledgements........................................ 2

Archive................................................. 2

Requirements............................................ 2

Other info.............................................. 2

Usage................................................... 3

Default................................................. 3

Option availability..................................... 3

A4000 MAPROM............................................ 3

Example of A4000 startup-sequence entry................. 3

Full definitions........................................ 4

FULLMAP OPTION.......................................... 6

Kickstart logical and physical Write-Protection......... 7

Kickstart 1.2 and 1.3................................... 7

Kickstart 2.0x.......................................... 8

KICKROM to FASTROM...................................... 8

Reset survival.......................................... 8

Program examples........................................ 9

Example of installing FASTROM........................... 9

Example of installing KICKROM........................... 9

Example of converting a KICKROM to FASTROM.............. 9

Example of converting a KICKROM to FASTROM FULLMAP...... 9

Example of installing the reset handler................. 9

Example of startup-sequence entry....................... 10

Example of removing the reset handler................... 10

Example of Cache manipulation........................... 10

Example of removing a FASTROM........................... 10

Example of switching from 68040 to 68030................ 11

Improvements............................................ 11

Technical notes......................................... 12

The DEFAULT setup....................................... 13

The FULLMAP setup....................................... 14

Trademarks and References............................... 14

DISCLAIMER

Although I beleive this software performs the functions

described in this document file, the program is supplied

"as is" without any warranty on performance of any kind,

either implied or expressed.  Nic Wilson Software will not

be liable for any damages, including lost profits, lost

savings or other incidental or consequential damages arising

from the use or inability to use such software, even if

Nic Wilson Software has been advised of the possibility

of such damages, or for any claim by any other party.

This disclaimer supercedes any other document or disclaimer

on previous versions either written or verbal.

ACKNOWLEDGEMENTS

Thank you to Dave Haynie and Michael Sinz for their answers

to my questions regarding the 68040 over EMAIL.

Thank you to Progressive and Peripherals and Software for

their generosity with the A3000/68040 cards.

Thank you to my friend Paul Coward (Digisoft) for a brilliant

assembler, Macro68 V3.170.  Set040 assembles with an average

pass time of 0.41 secs to executable, and without it 68040

code in new syntax would have been difficult.

Thank you to William J Coldwell for his enhancement suggestions

and for information on errata documents on the 68040 CPU.

REQUIREMENTS

Depending on the function, the requirements to launch

particular functions are different and are as follows:-

FASTROM    AmigaDOS V2.04 or greater, 68040 CPU, 1MB FAST RAM minimum

KICKROM    AmigaDOS V2,04 or greater, 68040 CPU, 1MB CHIP RAM minimum

KICKROM

to FASTROM AmigaDOS V1.2  or greater, 68040 CPU, 1MB FAST RAM minimum

MAPROM     AmigaDOS V2.04 or greater and an A3640 card or A4000.

TOGGLECPU  AmigaDOS V2.04 or greater, with any PP&S 68040 card *

* If an A3000 card, the A3000 must be a 25MHZ model, NOT 16MHZ.

OTHER INFO

This program was written and tested on an Amiga A3000 with a

Progressive Peripherals & Software 68040 25MHZ card and with a

PP&S Mercury card.  More recent versions are written and tested on

the Amiga 4000.  It is currently working on, and being regularly

tested on a CSA 40/4 Magnum Pro.  If it works on your card please

let me know.  If it doesn't, let me know too and maybe we can rectify

it for yours and other 68040 cards.

Page 2

USAGE

Set040 [FASTROM option option ...]

[KICKROM [path|DFx: FORCE]]

[MAPROM path/file FORCE]]

[option option ...]

FULLMAP [NO]REVERSE, [NO]DCACHE, [NO]ICACHE, [NO]CACHE [NO]ZCACHE,

[NO]RESET, [NO]COPYBACK, [NO]ATCCBACK, [NO]DTT0CBACK, [NO]DTT1CBACK,

NOCLICK, NOTITLE, TOGGLECPU, NOFASTROM, VERBOSE, HELP

All options can be supplied in either upper or lower case.

Command line examples are given on Page 9.

Some options can only be used with certain settings and are

shown below.

DEFAULT:

If no options are supplied, Set040 will display its default

brief information on the current settings.  Copyback modes are

not shown in this display.  They are shown in full with the

'VERBOSE' option.

OPTION AVAILABILITY

These options are only valid as sub-options to FASTROM or KICKROM

NOCLICK, NOTITLE, [NO]RESET

This option is only valid as a sub-option to KICKROM

FORCE

These options are only valid as sub-options to FASTROM

FULLMAP [NO]REVERSE

These options are available as sub-options to FASTROM or KICKROM,

or separately after a FASTROM or KICKROM installation to modify

current settings.

[NO]DCACHE, [NO]ICACHE, [NO]CACHE [NO]ZCACHE, [NO]COPYBACK,

[NO]ATCCBACK, [NO]DTT0CBACK, [NO]DTT1CBACK

These options are only valid separately.

TOGGLECPU, HELP, VERBOSE

This option is only valid after ROM-FASTROM installation.

NOFASTROM

Page 3

DEFINITIONS

FASTROM           - Remaps the current ROM or KICKROM kickstart to FAST

RAM.  Without any further options the default

configuration of CACHE NOZCACHE COPYBACK will be

installed.  If other options are used, they must

follow this option.  The setup can be either the

default mapping for FULLMAP mapping.

FULLMAP		  - Installs complete mapping for the entire 4 Gig space

If you are using 68040.library 37.10 or greater, this

option is redundant and will just waste memory.  Use a

default setup when using these versions of the library.

See the 'FULLMAP OPTION' heading on Page 6 for more

information on this option.

NOFASTROM	  - Remove the FASTROM setup and free all resources.

This option is not valid for KICKROM-FASTROM.

[NO]REVERSE       - Allocate memory for FASTROM from either the top

(REVERSE) of the memory map or from the bottom

(NOREVERSE) of the map.  If this option is not

specified, it will default to REVERSE.  Always use

the default first, unless you have trouble surviving

a reset, and then try the NOREVERSE option.

[NO]DCACHE        - Enable or disable the DATA CACHE.

[NO]ICACHE        - Enable or disable the INSTRUCTION CACHE.

[NO]CACHE         - Enable or disable both the INSTRUCTION & DATA CACHES.

[NO]ZCACHE        - Enable or disable caching the ZorroII address area.

[NO]RESET         - Install or remove the reset handler.

[NO]COPYBACK      - Enable or disable Copyback globally (DTT0, DTT1 & ATC).

[NO]ATCCBACK      - Enable or disable Copyback for:

default - $01000000-$03FFFFFF.

FULLMAP - $01000000-$3FFFFFFF.

[NO]DTT0CBACK     - Enable or disable Copyback for:

default - $08000000-$0FFFFFFF.

FULLMAP - $40000000-$7FFFFFFF.

[NO]DTT1CBACK     - Enable or disable Copyback for:

default - $04000000-$07FFFFFF.

FULLMAP - $80000000-$FFFFFFFF.

NOCLICK           - Stop the floppy drives from clicking.  This is a

better way than the various noclick programs that

are around (including my own), as experience shows

that changing the flags in the TDU_ structure actually

slows the response time of the drive seeing a diskchange.

NOTITLE           - Disable changing the Workbench title.

TOGGLECPU         - Toggle the 030 & 040 modes (PP&S 040 CARD ONLY).

HELP or ?         - Displays brief help on various options.

VERBOSE           - Verbose listing of current settings.  This option will

display the current settings, the type of caching for

memory areas, some of the 68040 registers and the

kickstart mapping addresses.

FORCE		  - This is an option to KICKROM, it forces a new KICKROM

to be installed, when a KICKROM is already installed.

This saves the long process that used to be required to

do this.  This has been done as an option, so that the

kickrom line in your startup-sequence will not take

effect on subsequent resets.

MAPROM		    Takes a single parameter, of the kickstart file to be

loaded.  This function requires an A3640 card or A4000.

If no parameter is specified then a default of

'DEVS:kickstart' will be loaded.  If the current version

of kickstart is the same as the file and error will result

unless the FORCE option is specified.

NOTE. The MMU *must* be off for this option to

work, boot with no startup-sequence before attempting

it, or place is conditionally before 'setpatch' in

the startup-sequence.

EXAMPLE OF STARTUP-SEQUENCE ENTRY

IF EXISTS C:Set040

Set040 MAPROM DEVS:kickstart

ENDIF

SetPatch QUIET

Page 4

KICKROM [path|DFx:] -

Loads and remaps a kickstart to CHIP RAM then reboots into the

new kickstart.  This option is always valid, except, when KICKROM

is already installed the FORCE option must be specified.  After

installation, it may be moved to fast ram by using the FASTROM

option.  If you have a board that has hardware fastrom ability,

this must be disabled before any attempt to use the KICKROM option.

The KICKROM option is capable of remapping a kickstart to any

address.  This is transparent to the user as the program checks the

kickstart and extracts the address it is destined for, and maps it

to that address.  You do not need memory at this address, it will

be mapped to that address with the MMU.  Set040 has the ability to

use the kickstart files that come supplied on the "Kickfile" disk,

although if these are used on an A3000 the hard disk will not boot

and your fast memory will not be available.  A3000 users should make

kickstart files from superkickstart disks, using the 'Update2.x'

program.

This function requires an Amiga with at least 1MB of chip memory

for it to work.  It will attempt to allocate about 576k of

memory within the chip memory area as high as possible.  The

allocated chip memory from this allocated block to the end of

the next one megabyte boundary, will be hidden from exec after

the reboot.  When Set040 is used again, to convert KICKROM to

FASTROM, this hidden memory will be added to the free pool.  If

your chip memory usage is large, Set040 may not be able to obtain

the required memory.  If this happens I suggest closing as many

programs as possible or re-booting with a minimum set-up so that

Set040 can allocate the memory it requires.

Even though we are about to re-boot, into a new kickstart, Set040

will NOT just take the ram required, as this may cause a crash.

If the memory allocated, falls within the first megabyte on an Amiga

with two megabytes of chip ram, the second megabyte will be lost

completely unless added in with the 'addmem' command.  This should

not happen under normal circumstances, as Exec normally allocates

memory from lower addresses first.  This means that the memory

allocated by Set040 should always fall within the highest one

megabyte block.  Although when this KICKROM is converted to FASTROM,

all the chip memory will be added back to the free pool, including

the top megabyte.

When installing the kickstart, a screen showing a 68040 CPU picture

informing you of the loading will be shown , then it will inform you

that the kickstart is being patched to prevent the MMU from being

disabled, and then, that system will be rebooted.  During  this reboot,

there will be a 3 second delay and then the power light will flash

rapidly to inform you that this function is operating successfully.

See 'FORCE' option also.

path  =

If a path/filename or DFx: is not given, then Set040 will attempt

to load a default kickstart file 'DEVS:kickstart'.

Full path and filename to a Kickstart file.

The Kickstart file can be a 256K file (1.2 or 1.3) or a

512k file (2.0x) as saved by the "Update2.x" or "Update1.3"

program supplied with your A3000.

DFx:  =

Instead of a filename, the kickstart can be loaded from an

A3000 SuperKickstart disk or an A1000 Kickstart disk.  If a

SuperKickstart is used a requester window will open and prompt you

for the Kickstart version you require.  These choices will be "V1.3"

or "V2.0".  If the drive you specified does not have a disk in it,

you will be prompted for a disk via a requester.

Page 5

FULLMAP OPTION

This option required more explanation than could be given in the

definitions above.  The 'FULLMAP' option is valid as a sub-option

to the FASTROM option, for a ROM remap or a KICKROM remap.

*********************************************************************

If you are using 68040.library 37.10 or greater, this option is

redundant and a default setup should be used.  If this option is used

on these version of the library, it will only waste memory, as the

68040.library replaces Set040's MMU setup and does the entire mapping

itself.

*********************************************************************

The default mapping is limited and was a compromise as explained

in the Technical notes on page 12.  The FULLMAP option allows for

a full mapping of the entire 4 Gigabyte address space of the 68040

processor.  I suggest you read the Technical notes so you may

understand the compromises that had to be made for a default setup.

The FULLMAP option has addressed the mapping in an entirely

different way to the default setup.  This full setup will require

just over one megabyte of fast memory in order to install the

FASTROM.  This megabyte includes the room required for the

Kickstart image, and will be allocated from the highest address

possible, (unless the NOREVERSE option is given) with the highest

priority memory in the map.  This memory however must be 'LOCAL'

memory.  If not enough LOCAL memory is available, the attempt will

fail.

LOCAL memory is that type of memory that does not "disappear" upon

a reset, and gets it name because it is usually local to the CPU.

Set040 tries to allocate this type of memory by adding the

MEMF_LOCAL bit to the attributes when calling AllocMem.

One quarter of the memory map is controlled by the ATC, and these

addresses are $0 to $3FFFFFFF.  These addresses include those locations

known as 24 Bit address space, Commodore Reserved areas, A3000 Mother

board Fast ram, CoProcessor Slot Expansion, and a portion of the

ZorroIII Expansion area.  This means that the ATC is now contolling

all areas that the default setup allowed altogether, plus a portion of

the map that was invalid in the default setup.

The rest of the ZorroIII Expansion area, from address $40000000 to

$7FFFFFFF is controlled by the DTT0/ITT0 registers.  The rest of the

address space from addresses $80000000 to $FFFFFFFF are controlled by

the DTT1/ITT1 registers and this area is known as 'Reserved by

Commodore'.

All areas above the 24 Bit address space will default to

cachable copyback unless other options are specified.  Remember

that the options to control ATC, DTT0 and DTT1 copyback are

still valid with this option, but they will be controlling

different areas of the map as explained above.  Normally these

options will not be required, because the default settings on boot-up

will be the same as Kickstart normally uses for the 68040.  If these

options are given after the installation, the 'VERBOSE' option

will show the changes that will take affect on a certain area

of the map.  After a reset, all modes will be restored to standard,

so these will require changing again, if you do not use default

values.

The FULLMAP option allows the Kickstart image to be protected from

writes, in the physical location as well as the logical location

as in the default mode, but due to technical difficulties the physical

area is only writeprotected, after each reset, when the FASTROM command

is given.  Even though FASTROM is already installed, when it is

launched again in the startup, Set040 will recognise that the FASTROM

is still installed and will make certain tests.  One of these tests

includes setting the physical location as writeprotected.  The

VERBOSE option of Set040 shows the logical and physical area settings

for you to check.  See below (Page 7) for more on this subject.

* * * IMPORTANT * * *

You MUST read the document file for the SetPatchPatch program supplied.

Page 6

KICKSTART WRITE-PROTECTION

The logical address of kickstart will always be marked as write-

protected and the error handler will trap any write accesses.  The

logical addresses will be marked as write-protected on a FASTROM

FULLMAP	installation.  The default mapping mode does not support

write-protection of the physical area.  During a reset, this write-

protection must be removed, otherwise an exception would occur during

the Kickstart memory test and SCRAM test routines.  The removal of

the write-protection is transparent to the user, and is handled by

the reset handler. The write-protection can be re-installed by Set040

if the FASTROM option is given.  This would normally happen in the

startup-sequence (See Page 10).  When this option is given, Set040

will recognise that a FASTROM is already installed and will check to

see if it is write-protected at its physical location.  If not, the

MMU setup will be modified to reflect the write-protected state.  If

your startup-sequence entries are similar to the example on Page 10,

this process will be automatic and transparent to the user.

KICKSTART 1.2 - 1.3

************************************************************

** USE 1.3 or 1.2 WITH CAUTION, THEY ARE NOT FULLY COMPATIBLE **

************************************************************

Kickstart 1.2 and 1.3 should not be used on the 68040 without

extreme caution.  These kickstart were not designed for the

special cache modes of the 68040.  The ability to install them

in Set040 has been supplied for testing purposes, incase a

programmer does not have a 1.3 Kickstart Amiga to test on.

Watch out for 1.3 'SetClock' commands, they will guru the

A3000/68040 every time.  Make sure you delete them or replace

them with your A3000 'SetClock' command.

If the new Kickstart is 1.2 or 1.3,  only the remaining chip

memory will be available after booting.  Any FAST RAM on the

A3000 mother board will not be available.  This memory must be

added to the system with the "addmem" command.  It has been done

this way incase the fast memory is not required for some reason.

The usage for the 'addmem' command for different memory sizes is

shown below.

When the FAST ram has been added, you may run Set040 again with

optional arguments (See KICKROM to FASTROM).

AddMem usage for A3000 32bit wide fast ram are as follows:

A3000 4MB  32 bit ram = AddMem 7C00000 7FFFFFF

A3000 8MB  32 bit ram = AddMem 7800000 7FFFFFF

A3000 12MB 32 bit ram = AddMem 7400000 7FFFFFF

A3000 16MB 32 bit ram = AddMem 7000000 7FFFFFF

Page 7

KICKSTART 2.0x

If the new Kickstart is 2.0x (V37+) your FASTRAM will still be

available and your hard disk would have booted.  The addmem

command will not be required.  This set-up will still be in

CHIP ram though, and can be moved to fast memory by running Set040

with the FASTROM and other options.

NOTE:

FOR COMPATIBILITY REASONS, WHEN THE KICKROM to FASTRAM CONVERSION

IS PERFORMED, THE DATA CACHE WILL ONLY BE ENABLED FOR V2.0X

KICKSTARTS, AND WILL BE DISABLED FOR OTHER KICKSTARTS.  IF YOU

WISH TO CHANGE THIS, OR MANIPULATE THE OTHER CACHES, USE THE

CACHE OPTIONS.

ONCE THIS KICKROM KICKSTART IS INSTALLED IT CANNOT BE REMOVED WITH

THE NOFASTROM OPTION. ANY ATTEMPT TO DO SO WILL GIVE AN ERROR.

SEE 'NORESET' OR 'FORCE' OPTIONS, OR SEE 'RESET REMOVAL' BELOW.

KICKROM to FASTROM

When a KICKROM set up is first installed, the kickstart

will be in chip memory.  In this mode you have three options.

Firstly, may leave it as it is, and it will be lost on the next

reboot.  Secondly you may install the reset handler, and the

kickstart will remain in CHIP memory and survive a reboot, or

thirdly you may use the FASTROM option(s) to move it to FAST

memory with a default, or FULLMAP setting, and optionally survive

a reboot.

RESET SURVIVAL

The KICKROM function, by default will not survive a hardware

(keyboard) reset.  This can however be installed by using the

RESET option.

Separate installation allows you to install it should you forget,

when doing a KICKROM to FASTROM conversion, or allows you make

a CHIP ram KICKROM setup survive a reboot, without ever converting

to FAST ram.  This is useful in situations where enough FAST ram is

not available or the FAST memory does not come on-line early enough

in the reset-boot sequence.  If you find that the KICKROM-FASTROM

does not survive a reboot or crashes, then try a CHIP ram KICKROM

setup.

Once installed the current KICKROM kickstart will survive every

reboot until removed with the NORESET option, or changed with the

FORCE option.

PLEASE NOTE THAT THIS HANDLER USES THE COLDCAPTURE VECTOR, AND THE

COOLCAPTURE VECTOR DURING THE RESET PROCESS.  SOME ANTI-VIRUS

PROGRAMS (INCLUDING MY OWN NoVirus, PRIOR TO BRAINFILE 066) MAY SEE

SOME POSSIBLE NEW VIRUS  IN MEMORY,  BUT DO NOT WORRY, IT IS ONLY

THIS RESET HANDLER.  THESE VECTORS WERE CHOSEN SO THAT RAD: TYPE

DEVICES MAY ALSO SURVIVE A REBOOT AND NOT CONFLICT WITH Set040.

THE ACCESS FAULT EXCEPTION VECTOR IS ALSO MODIFIED, TO PREVENT

ATTEMPTED WRITES TO THE KICKSTART FROM CAUSING A SOFTWARE FAILURE.

THE FASTROM KICKSTART IS WRITEPROTECTED WITHIN THE MMU TABLES.

For other anti-virus authors, it is easy to identify this handler

by fetching the pointer stored in coldcapture, subtracting four from

this address pointer and testing for the following value at that

memory location:

Hex value=$52535448 = RSTH in ascii

Page 8

PROGRAM EXAMPLES

*************************************************************************

***********************************************************************

NOTE    IN ALL THE FOLLOWING EXAMPLES, IF YOU ARE USING 68040.LIBRARY

37.10 OR GREATER, THEN THE SETPATCH COMMAND MUST COME AFTER THE

SET040 ENTRIES, AND THE SETPATCHPATCH COMMAND MUST BE DELETED AND

NOT USED AT ALL.

CHECK THE VERSION OF 68040.LIBRARY YOU ARE USING AND MAKE SURE

YOU INSTALL CORRECTLY, OR IT WILL NOT WORK.

EXAMPLE OF INSTALLING A FASTROM (68040.library 37.10 or greater)

To remap the rom, to fast ram with noclick drives:

Set040 fastrom noclick

setpatch QUIET		<----Commodore's setpatch must come after

<----Delete the setpatchpatch entry

***********************************************************************

*************************************************************************

EXAMPLE OF INSTALLING A FASTROM (Kickstarts up to and beyond V2.04)

To remap the rom, to fast ram with noclick drives:

Set040 fastrom noclick

SetPatchPatch            <-- the standard 'SetPatch' must already

be installed.

EXAMPLE OF INSTALLING A KICKROM

To install a KICKROM setup from a default file "DEVS:kickstart".

Set040 kickrom

To install a KICKROM as above, when KICKROM already installed.

Set040 kickrom force

To install a KICKROM as above, when KICKROM already installed.

Set040 kickrom devs:kick force

To install a kickrom option from a kickstart disk in df1:

Set040 kickrom df1:

EXAMPLE OF CONVERTING KICKROM TO FASTROM

To convert the KICKROM to FASTROM, with reset handler, and

noclick drives.

Set040 fastrom reset noclick

SetPatchPatch

EXAMPLE OF CONVERTING KICKROM TO FASTROM WITH THE FULLMAP OPTION

To convert the KICKROM to FASTROM, with reset handler, and

noclick drives and install a FULLMAP type remap. This option

will require 1067570 bytes of LOCAL memory.

Set040 fastrom fullmap reset noclick

SetPatchPatch

EXAMPLE OF INSTALLING RESET HANDLER SEPERATELY

The reset survival trap handler can be installed without

performing the KICKROM to FASTROM conversion by using RESET option.

If this option is given, the kickstart will remain in CHIP memory

and be able to survive a reboot.

Set040 reset

NOTE, This option cannot be done after installation if you

are using kickstart 3.0, 68040.library 37.10 or greater.

The reset option must be given when installing Set040.

Page 9

EXAMPLE OF KICKROMMING IN THE STARTUP-SEQUENCE

If you want to use a different kickstart in the startup-

sequence and have this survive a reset and be in fast memory,

all that is required is three lines.

The first line will only load the kickstart once, After this is

done the next boot will cause the first line to fail, as the output

goes to nil: it is transparent to the user. The second line will

then map it to fast ram and install the reset handler and no-click

patch once.  After this both lines will transparently fail each boot.

This saves you having to use IF/WARN/ENDIF statements.

For KICKROM-FASTROM survival you could use:-

SetPatch quiet			    <--- Normal Setpatch command

Set040 >nil: kickrom kickstartfile

Set040 >nil: fastrom fullmap reset noclick

SetPatchPatch >nil:

For CHIP ram KICKROM survival you could use:-

SetPatch quiet			    <--- Normal Setpatch command

Set040 >nil: kickrom kickstartfile

Set040 >nil: reset

SetPatchPatch >nil:		    <--- MUST be after Setpatch and

AFTER Set040

EXAMPLE OF RESET REMOVAL

Set040 noreset

Removes the reset handler so that the next boot will

return to your ROM based kickstart.

EXAMPLE OF CACHE MANIPULTION

Disable both caches                      - Set040 nocache

Enable  both caches                      - Set040 cache

Disable Data Cache                       - Set040 nodcache

Enable  Data Cache                       - Set040 dcache

Enable  Both Caches and disable Copyback - Set040 cache nocopyback

Enable  Both Caches and enable  Copyback - Set040 cache copyback

Disable Both Caches and disable Copyback - Set040 nocache nocopyback

EXAMPLE OF REMOVING FASTROM

SET040 nofastrom

NOTE:	This function is not possible on a KICKROM or KICKROM-FASTROM

setup.

Page 10

EXAMPLE OF SWITCHING FROM 68030-68040 OR VICE-VERSA

Set040 togglecpu

One of the following cards must be installed.

PP&S A3000-68040 card.

PP&S A3000 Mercury card.

*	PP&S A2000-68040 card.

*	PP&S A2000 Zeus card.

*	TO DATE, THIS FUNCTION HAS ONLY BE TESTED ON THE FIRST TWO ENTRIES.

THIS SWITCH IS NOT PERMITTED ON 16MHZ A3000'S, AND SHOULD NOT

BE ATTEMPTED.

TOGGLECPU is the only option that is permitted on a 68030 CPU.  All

other options  will display an error.  When this option is given,

a requester will open and will prompt you to either 'CHANGE' or

'ABORT' via two gadgets.  If aborted, no further action takes

place and the program will exit.  If 'CHANGE' is selected, another

requester will open and prompt you to reboot the computer.  This

requester will have a single 'ABORT' gadget, giving you a final

chance to abort.  The actual reboot must be done by hand, using

the keyboard, because the PP&S 68040 card requires a hardware reset

to switch modes.  This cannot be generated with software.  (NO,

the reset instruction will not work!) :-(

IMPROVEMENTS

All suggestions will be carefully considered.

All bugs will be rectified A.S.A.P.

I will attempt to overcome all incompatibilities

if sufficient information on your particular board

is supplied ie. amount of onboard ram, address of such

ram, A3000 or A2000 board, full description of other

hardware you use, full description of any plug in cards,

full description of your Amiga, OS version numbers, etc!!

Page 11

TECHNICAL NOTES

The average user may not completely understand the following

notes, suffice to say an understanding of how this program

works is not required.  Information given here is mainly

for the benefit of the experienced user or interested programmer.

These notes may be of some help if you experience problems

with certain 68040 cards.

NOTE: The termination for certain address has been taken from

the latest information I have available on the memory map

for the A3000 series of computers and is dated July 1991.

I wrote this program for a few reasons.  To gain knowledge

in programming the 68040 processor in assembler and to discover

the differences from its older brothers.  I wanted to test Macro68

assembler in 68040 instructions.  When I ran a test of the difference

in speed between reading the ROMS and reading FAST RAM, I realized a

68040 MMU program was a necessity.  I found that while reading the

ROMS the 68040 was only marginally faster than the 68030, yet in

FAST RAM the 68040 leaves the 68030 standing.

The MMU set-up remaps the kickstart ROMS to FAST RAM, and allows

for instruction caching of the address areas the same as for the

data areas mentioned below.  This set-up protects the new kickstart

by marking this area of memory as write-protected in the ATC table

entries.  This remap is faster than the hardware FASTROM that

is being done on some 68040 cards.  Testing has shown that a MMU

remap is between two and three times the speed of a hardware map.

Re-mapping kickstart to FAST RAM poses a few problems because of

the design of the 68040 MMU.  Most of the first 16MB of the Amiga

memory map must be set for non-cachable serialized.  The rest of

the 4 gigabyte space can be set for cachable copyback,  but because

of limitations of the 68040 MMU, programming this is not easy.  The

standard V2.04 or PP&S set-up manages this by using both of the Data

Transparent Translation registers.  This works fine but does not

allow for re-mapping of the kickstart.  The 68040 MMU looks at

Transparent Translation registers first, and if an address matches,

the ATC tables are not accessed.   Therefore a remap of the kickstart

would not be seen if we used the same values in the transparent

registers as PP&S, or Commodore uses.

The 68040 has a simple 3 level ATC table and page sizes of either

4k or 8k.  Our set-up discussed here uses the 8k page size.

If the ATC is used, each entry in the first table covers 32MB.

This means that we must at least have entries in this table

for that address space.  If we want this entire 32MB to be valid

we must have complete entries in the second and third tables to

cater for it.  This is not a large problem but does use around

17k of memory for each 32MB space covered in the table.  The

problem is enhanced due to the fact the 68040 does not support

'early termination descriptors' as the 68030 did.

The next problem exists because, two of each data and instruction

transparent translation registers are not enough to control the

rest of the space.  We could put all entries in the ATC tables,

but this would mean very large tables and make it difficult to

control the copyback mode over a large area.  A mode similar to

this has been catered for by Set040 as an option and still allows.

for compatibility with the CPU040 program from PP&S, CPU from

Commodore and any other 68040 cache manipulation program that uses

the OS routines for manipulation of the caches and modes.

Page 12

Set040 allows for two different setups.  These allow for either a

smaller memory efficient setup, or a full setup that requires just

over one megabyte of memory.

THE DEFAULT SETUP

The first 16MB is set in the tables as non-cachable serialized.

Copyback mode for the second, third and fourth 16MB spaces can be

conrolled globally between copyback or writethrough before or after

installation, by command line options.  The addresses of this  48MB

space is $1000000 - $3FFFFFF.

The next 64MB address space ($4000000 - $7FFFFFF) is controlled

by one each of the two data and instruction transparent translation

registers for the data and instruction spaces respectively.  The

data register is set for cachable copyback unless the NODTT1CBACK

option is used at installation.  This can be changed after

installation with the various copyback options.  The instruction

registers are set for cachable.

The above caters for the first 16MB and the area defined as

'A3000 mother board space'.  Above this area we have a 128MB

space which is defined as "CoProcessor Slot Expansion".  This is

controlled by the second data and instruction transparent translation

registers and can be optionally switched between copyback or

writethrough before or after installation by command line options.

This area will default to copyback at installation unless NODTT0CBACK

option is given.

All the ATC entries except for the kickstart remap entries actually

get transparently translated.  The 68040 users guide states that if

the physical address equals the logical address in a page entry, then

that entry is transparently translated.  Transparent translation means

that there is no CPU overhead to do the translation.  The kickstart

remap routine makes use of the new 'MOVE16' instruction to copy the ROM

image into ram.  This allows for a very fast copy and saves having to

call OS routines to flush the caches after the copy, as the Move16

instruction does not permit caching of the data copied and invalidates

any entries in the cache for the destination addresses.

The rest of the map which ranges from $10000000-$FFFFFFFF has been

set as invalid, simply to conserve memory.  The default set-up should

allow for the good compatibility between different 68040 cards with

the least use of memory.

Page 13

THE FULLMAP SETUP

This mode allows for mapping of the entire 4 gigabyte space with

memory usage of a little over a megabyte, including the Kickstart

image.

The first one gigabyte of address space is controlled by the ATC.

The second gigabyte is controlled by the DTT0 and ITT0 transparent

translation registers, and the third and fourth gigabytes are

controlled by the DTT1 and ITT1 registers.  Caches for the first

16MB of the address space is set the same as the default setup.

All of the remaining address space is set for cachable copyback,

but this can be somewhat controlled by various command line options.

For further information see the FULLMAP description on page 6.

When programming these setups, I attempted to keep them fully

compatible with the standard programs from Commodore and PP&S.

This was done in the way of a stand alone program called SetPatchPatch.

This 'patch' allows the CPU, CPUSet, CPU040 and other correctly

written programs to be fully aware of the MMU setup that Set040 has

installed.  In cases where the program that was supplied with your

card is not written correctly, I suggest you either use Set040 or one

of the above programs for cache manipulation.  Yours can be tested

by using one of the above, and taking note of the effect it has

when the 'Set040 verbose' command is issued.  Your program should

have EXACTLY the same effect as one of the above or Set040 does,

otherwise it is not correctly written.

TRADEMARKS

AmigaDOS, Amiga and Kickstart are trademarks of Commodore Amiga Inc.

PP&S is an abbreviation for Progressive Peripherals and Software.

MC68030, MC68040 and Motorola are trademarks of Motorola Inc.

REFERENCES

M68000PM/AD  Programmers Reference Manual (inc. CPU32)

Published by Motorola Literature Distribution USA.

M68000UM/AD  8-/16-/32-Bit MicroProcessor User's Manual Eighth Edition.

Published by PRENTICE HALL, Englewood Cliffs, N.J. 07632

MC68040UM/AD 32-Bit MicroProcessor User's Manual

Published by Motorola Literature Distribution USA.

MC68030UM/AD 32-Bit MicroProcessor User's Manual

Published by Motorola Literature Distribution USA.

MC68040 Errata notes supplied by William J Coldwell.

Page 14