# TABLE OF CONTENTS

TABLE OF CONTENTS	1

1. Voodoo Banshee Features	2

1.1 Voodoo Banshee Board Outline	3

SP3800 using SGRAM	3

SP3800C using SGRAM	3

2. S3 Savage3D (391) Features:	4

2.1 S3 Savage3D Board Outline	5

SP391ESD using SDRAM	5

SP391ESG using SGRAM	5

3. Hardware Installation	6

3.1 Package Contents	6

3.2 Installing the Card	6

3.3 Resolutions and colors supported:	7

3.3.1Voodoo Banshee	7

3.3.2 S3 Savage 3D (391)	8

4. Smart Installation	9

5. Manually install Windows 95 & Windows 98 drivers	10

5.1 Installation and Setup	10

5.1.1 Windows95	10

5.1.2 Windows98	10

5.2 How to Change Color Depth and Resolution	11

5.3 How to Change Refresh Rate	11

6. Windows NT 4.0	13

FCC Statement	14

Warning:	14

Trademark Acknowledgments	14

This manual is for SP3800 (Voodoo Banshee) and SP391 (Savage3D)

# 1. Voodoo Banshee Features

General Features

 Fully integrated 128-bit VGA/2D/3D/Video Accelerator

 Ultimate 3D experience with 100 Mpixels/sec and 4 Million triangles/sec

 No-compromise 3D image quality at frame rates

 Optimized for software DVD acceleration (Optional)

 Full VMI interface (including a host port) for optional:

Full hardware DVD decoding, Video capturing, TV tuner support

 High-resolution 1600x1200 85Hz with a 230MHz RAMDAC

 PC97 and PC98 rev 1.0 compliant

 VESA DDC2B support

2D Acceleration

 Full featured 128-bit BitBlt Engine & Windows GUI Acceleration

 Source and Destination Chroma-keying for DirectDraw

3D Acceleration

 Full hardware setup of triangle parameters

 16-bit integer and floating-point Z-buffering with biasing

 Transparency and chroma-key with dedicated color mask

 Alpha blending on source and destination pixels

 Sub-pixel and sub-texel correction to 0.4x0.4 resolution

 24-bit color dithering to native 16-bit RGB

 Per-pixel atmospheric fog with programmable fog zones

 Polygon edge anti-aliasing

 Perspective correct (true divide-per-pixel) 3D texture mapping

 True per-pixel, LOD MIP mapping with biasing and clamping

 High performance bilinear and trilinear filtering

 RGB modulation/addition/blending combines textures and shaded pixels

 Texture compositing for multi-texture special effects

 Support for 14 texture map formats

 8-bit paletted textures with full bilinear filtering

 Texture compression through narrow-channel YAB format

Video Acceleration

 Multiple video window support

 Bilinear horizontal and vertical filtering

 YUV 4:2:2, and YUV 4:2:0 planar support

Host Interface

 High performance AGP interface including optimized support for

sideband addressing and pipelining

 FIFO optimized for high speed bursting of geometry and texture data

 Optimized for Pentium II IO architecture

Memory System

 Advanced architecture with 1.6 GB/sec memory bandwidth

 Support 8/16 MB SGRAM Frame buffer

 Software support Windows95/98 & NT4.0

## 1.1 Voodoo Banshee Board Outline

### SP3800 using SGRAM

16MB on board, board size: 161 x 108 mm.

512X32

512X32

VMI

512X32

512X32

512X32

512X32

DB15

14.318MHz

Voodoo

Banshee

BIOS

512X32

512X32

AGP 2.0

### SP3800C using SGRAM

16MB on board, board size: 151 x 88 mm. (another 4 pieces SGRAM on solder side)

VMI

512X32

512X32

DB15

512X32

14.318MHz

Voodoo

Banshee

BIOS

512X32

AGP 2.0

# 2. S3 Savage 3D(391) Features:

-  BUS TYPE: 64bit AGP 2X Side bands 133MHz BUS

 CHIP SET: S3 Savage 3D

 Memory Size: 8MB SGRAM / SDRAM

2D / 3D Graphics Accelerator

 Enhanced 128 bit Graphics Engine.

 Integrated 250MHZ RAMDAC, supports resolutions and refresh rates up to

1600 x 1200 (85HZ)

 Leading 3D Performance with the highest image quality.

 Triangle setup engine delivers 5M triangles / sec

 Peak performance rates of 125M pixels / sec with all features enabled

Complete 3D feature set

 Triangle Setup Engine.			 Palletized Textures

 Single pass tri-linear Filtering		 8KB Texture Cache

 S3 Texture Compression			 Edge Anti-Alisaing

 True Color Rendering			 Perspective Correction

 Void & Cluster Dithering			 Alpha Test

 Spectlar & Diffuse Shading		 Multiple Textures

 Alpha Blending				 Procedural Morphing

 16/24 bit Z buffering			 Texture Morphing

 Vertex and Table Fog			 Reflection Mapping

 MPEG-2 Video Textures			 Shadows

Optimized S/W DVD Solution(Optional)

 Mpeg-2 acceleration features reduce CPU utilization and provide high frame rates.

 Motion Compensation

 Sub-picture blending and highlights

 Planar to Packed Conversion

 Support image quality

 Filtered down scaling of host data

Complete Video Feature Set

 YUV Planar to Packed Format Conversion

 HW Sub-picture Blending and Highlights

 Motion Compensation

 Front-end Scalar with high quality Downscaling

 Advanced Pixel Formatter for Multiple Video Windows

 Advanced Streams Processor with Enhanced Scaling Algorithm

 Bus Mastering of IDCT Data for SW DVD Playback

 De-Interlacing Filter for Bob & Weave.

 Independent Color Adjustment

 3-tap Programmable Flicker Filter & Vertical Overscan Compensation

 Integrated NTSC/PAL Encoder with S-Video, AV connector Support

## 2.1 S3 Savage3D Board Outline

### SP391ESD using SDRAM

8MB on board, Board size: 160 x 83 mm.

BIOS

1MX16

FC

DB15

1MX16

14.318MHz

S3

Savage3D

1MX16

S-Video

1MX16

AV(RCA)

AGP 2.0

### SP391ESG using SGRAM

8MB on board, Board size: 160 x 83 mm. (another 2 pieces SGRAM on solder side)

BIOS

FC

512X32

DB15

14.318MHz

S3

Savage3D

S-Video

512X32

AV(RCA)

AGP 2.0

# 3. Hardware Installation

## 3.1 Package Contents

- Voodoo Banshee or S3 Savage3D Accelerator card.

- CD title or Software diskettes

- This manual.

## 3.2 Installing the Card

- Turn off your computer.

- Remove the cover of the computer per the owner’s manual.

- Install the card in the AGP bus slots.

- Replace the cover.

## 3.3 Resolutions and colors supported:

### 3.3.1Voodoo Banshee

Voodoo Banshee with 16MB are fully compatible VGA with the addition of Hi-Color and True Color modes depending on the amount of video memory stalled. The amount of memory needed to display various resolutions is shown below.

Voodoo Banshee

| Resolution & Colors | BPP | Refresh Rate (HZ) |
| --- | --- | --- |
| 640x480-256c | 8 | 60/75/85/100/120 |
| 640x480-64Kc | 16 | 60/75/85/100/120 |
| 640x480-16Mc | 24 | 60/75/85/100/120 |
| 640x480-16Mc | 32 | 60/75/85/100/120 |
| 800x600-256c | 8 | 60/75/85/100 |
| 800x600-64Kc | 16 | 60/75/85/100 |
| 800x600-16Mc | 24 | 60/75/85/100 |
| 800x600-16Mc | 32 | 60/75/85/100 |
| 1024x768-256c | 8 | 60/70/75 |
| 1024x768-64Kc | 16 | 60/70/75 |
| 1024x768-16Mc | 24 | 60/70/75 |
| 1024x768-16Mc | 32 | 60/70/75 |
| 1280x1024-256c | 8 | 60 |
| 1280x1024-64Kc | 16 | 60 |
| 1280x1024-16Mc | 24 | 60 |
| 1280x1024-16Mc | 32 | 60 |

### 3.3.2 S3 Savage 3D (391)

S3 Savage3D AGP bus VGA cards are fully compatible VGA with the addition of Hi-Color and True Color modes depending on the amount of video memory stalled. The amount of memory needed to display various resolutions is shown below.

S3 Savage3D

| Resolution & Colors | BPP | Refresh Rate (HZ) | 8MB |
| --- | --- | --- | --- |
| 640x480-256c | 8 | 60/72/75/85 |  |
| 640x480-64Kc | 16 | 60/72/75/85 |  |
| 640x480-True | 32 | 60/72/75/85 |  |
| 720x576-256c | 8 | 60/72/75 |  |
| 720x576-64Kc | 16 | 60/72/75 |  |
| 720x576-True | 32 | 60/72/75 |  |
| 800x600-256c | 8 | 56/60/72/75/85 |  |
| 800x600-64Kc | 16 | 56/60/72/75/85 |  |
| 800x600-True | 32 | 56/60//72/75/85 |  |
| 1024x768-256c | 8 | 60/70/75/85 |  |
| 1024x768-64Kc | 16 | 60/70/75/85 |  |
| 1024x768-True | 32 | 60/70/75/85 |  |
| 1280x1024-256c | 8 | 60 |  |
| 1280x1024-64Kc | 16 | 60 |  |
| 1280x1024-True | 32 | 60 |  |

( (I):interlaced  :Support  x: Not Support )

# 4. Smart Installation

For Windows 95 OSR2, Windows 98, WindowsNT4.0 SP3

4.1 Put the DRIVERS CD in your CD-ROM

4.2 Click the Driver installation

4.3 If everything fine, screen will show Confirm Installation window.

Then Select the OK

4.4 Now appears the Driver Confirmation window

Select the Install

(Here the picture shows Voodoo Banshee for example, if your are using

S3 Savage3D, Driver Confirmation window will show S3 Savage drivers)

4.5 Then restart your computer.

# 5. Manually install Windows 95 & Windows 98 drivers

## 5.1 Installation and Setup

### 5.1.1 Windows95

The Windows setup program installs and modifies all of the necessary files. Follow these steps to install the Windows 95 drivers.

- Insert the DISC (CD Title Driver).

- Select Control Panel from My Computer group.

- Select the Display icon

- Double-click on the Display icon in the Setting -> Control Panel folder. You can also click the right mouse button anywhere on the desktop and select the Properties option form the pop-out menu.

- Click the Settings tab.

- Click the change Display Type button.

- Click the Have Disk button.

- Change directories and Select banshee.inf.(s3savage.inf)

- Disc (CD Title) is E:\drivers\3dfx\3800\WIN95 (E:\dreivers\s3\391\win95)

- Note: “E:”CD Drive  “3800”VGA model name

- Select AGP driver form the list and click the OK button. If a message stating that one or more driver files is older than the files on the system respond YES to overwrite the files.

- Click the Close button and Apply button, and restart your computer.

### 5.1.2 Windows98

The Windows setup program installs and modifies all of the necessary files. Follow these steps to install the Windows 98 drivers.

- Insert the DISC (CD Title Driver).

- Select Control Panel from My Computer group.

- Select the Display icon

- Double-click on the Display icon in the Setting -> Control Panel folder. You can also click the right mouse button anywhere on the desktop and select the Properties option form the pop-out menu.

- Click the Settings tab.

- Then show  [Unknown Device.] Properties tab

Select Adapter

- Then show Standard PCI Graphics…. tab

Select Change

- Then show Update Device …. tab

Select Next

- Then show Update Device …. tab

Select Search for a better……

Then select Next.

- Then show Update Device …. tab

If the Specify a location is wrong

Please select Brows

- Then show Browse for Folder tab

Select E:\drivers\3dfx\3800\win98 (E:\drivers\s3\391\win98)

- Disc (CD Title) is E:\drivers\3dfx\3800\WIN95 (E:\dreivers\s3\391\win95)

- Note: “E:”CD Drive  “3800”VGA model name

Then select OK

- Then show Update Device Driver Wizard tab

Select CD-ROM driver

Click the Specify a locator, if right

Then select Next

- Then show Update Device Driver Wizard tab

Select Next

- Then show Update Device Driver Wizard tab

Select Finish

- Then show System Settings Change tab

Select OK

## 5.2 How to Change Color Depth and Resolution

- Make sure that you have installed windows95/98 Driver.

- Click on the Start box in the lower left corner and proceed to Control Panel.

- Inside the Control panel group , click on Display icon to open the Display Properties folder and select the Setting table

- Click on the pull-down arrow from the Color palette area to select color depth or adjust the sliding bar to either Less or More from the Desktop area

- Select OK to restart WINDOWS95 and new color depth or the new resolution takes effect.

## 5.3 How to Change Refresh Rate

- Make sure that you have installed windows95/98 Driver. Click on the Start box in the lower left corner and proceed to Setting, Control Panel

- Inside the Control panel group , click on Display icon to open the Display Properties folder and select the S3 Refresh table

- Click on the Change Configuration to change refresh rate

- Select OK and new refresh rate takes effect

- Here the picture shows Voodoo Banshee for example.

- Refresh Rate default is Optimal.

- If you change Refresh Rate from Optimal to another , you must be to restart WINDOWS 95

- Be sure your monitor can support DDC functions(or it is Plug & Play monitor),

otherwise you can not select refresh rate.

# 6. Windows NT 4.0

The following steps describe how to install Windows NT4.0 display drivers

- Select Control Panel from the Main group.

- Select the Display icon.

- Select Change Display Type.

- Select Change from the Adapter Type area.

- Select Other.

- Place the Disc (CD Title) into Driver. Click OK.

- Select Install and click “YES” when the Installing Driver dialog box appears.

- When the Windows NT Setup dialog box appears select CDROM, and click ”Continue”.

A message appears stating that drivers were successfully installed. Click OK. Another message appears stating that the driver could not be restarted dynamically. Restart Windows NT to run the new driver. Click OK.

# FCC Statement

This device complies with part 15 of the FCC Rules. Operation is subject to the following two condition.(1).this device may not cause harmful interference. (2).this device must accept any interference received, including interference that may cause undesired operation

## Warning:

Note: This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, used and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference in to radiate communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different form that to which the receiver in connected.

- Consult the dealer or an experienced radio/TV technician for help.

Notice:

(1) An Unshielded-type power cord is required in order to meet FCC emission limits and also to prevent interference to the nearby radio and television reception. It is essential that only the supplied power cord by used.

(2) Use only shielded cables to connect I/O devices to this equipment.3

(3) Changes or modifications not expressly approved by the party responsible for compliance could void the use’s authority to operate the equipment.

# Trademark Acknowledgments

- All brand names and trademarks are the property of their owners.