TN210-3.DOC

NETWORKING OVERVIEW

***********************

There are two TNC-2 based amateur network concepts in use today.  These are

the  STATIC  and  the  DYNAMIC  routing  concepts.    Each  has  advantages and

disadvantages.  An  example of  the  static routing concept is ROSE,  developed

by Tom Moulton,  W2VY.    With Static routing, a configuration file is prepared

listing  all of the nodes  with appropriate routes and  loaded into each static

node.  If available, secondary  routing  information  is  also  listed in  case

primary routes fail.

The  advantage  of  static  routing is it does not require system overhead,

due to  all  routes being  preprogrammed.    This is considered advantageous as

system overhead adds to circuit loading,  thus depriving users a certain amount

of  channel  capacity.     Another advantage of static routing is it forces the

system  providers into  MANAGING  their  networks.    Here we are assuming this

will  lead to efficient network performance.    Addition of  new nodes into the

system  can only happen following coordination and, upon consent of the network

manager.

With  dynamic  routing, each node  has a  means of becoming  "automatically

aware" of other nodes,  and of their relative position in the network.    To be

truly "dynamic," it's desirable routing  information be kept reasonably current

with  updates  on new nodes entering the system and the deletion of  nodes that

have failed.

TheNet accomplishes this by each  node  periodically  broadcasting  its own

existence plus a listing of other  nodes  heard during  broadcasts  by neighbor

nodes.    This information propagates  from node-to-node throughout the network

in a domino-like fashion.   Included in  the broadcasts is  routing information

based on a theoretical concept termed  "path quality".    This concept says the

path quality between  any two nodes is assigned a numerical value as determined

by factors relating to the type of  path and it's baud rate.   For instance, we

can  easily  recognize  the path quality over a full-duplex  wire-link directly

connected between two TNC's at 9600 baud is MUCH better than is an interference

ridden HF channel at 300 baud.   Path quality then can be considered a function

of  received  DATA versus TIME  (throughput), with larger  amounts  equating to

better path quality and lesser amounts to poorer quality.

In a system there may be several alternative routes to any one destination.

As  used in  TheNet systems, path quality is important since it's the means for

the node to select the best route to a  destination node.    Figure 1 lists the

recommended path quality values for various types of circuits.

TYPE OF PATH                       PATH QUALITY     RELIABILITY

------------------------------------------------------------------------------

9600 baud RS-232 wire line link (2 TNC's).       255              99%

9600 baud RS-232 satellite link (2 TNC's).       252              98%

9600 baud RS-232 diode matrix (3 TNC's).         248              97%

9600 baud Hidden transmitter-free radio link.    240              94%

1200 baud Hidden transmitter-free radio link.    224              88%

1200 baud Multi-user radio link.                 192              75%

1200 baud Multi-user WAN HF radio link.          180              70%

300 baud Multi-user HF radio link.               128              50%

------------------------------------------------------------------------------

FIGURE 1

The  path  quality  approximations above are for "ideal conditions," and are

the values typically assigned by individual  NodeOps  within the network.   The

following example illustrates the algorithm used in selecting routing qualities

to destination nodes.

1200 baud simplex multi-user network

------------------------------------

CASTRO   <192>   BELVU   <192>   CORNWL   <192>   DEAL

192/256     X    192/256    X     192/256  =  108/256

On  this  system,  the  route between  each neighbor node has a path quality

value of 192.     Using the formula:  Path Quality  divided  by  256 times Path

Quality divided by 256 for each of the three hops yields a circuit path quality

value of 108 between CASTRO and DEAL.

9600 baud hidden transmitter-free network

---------------------------------------------------

MILTY   <240>   ACME   <240>   JUSTIN   <240>   DEAL2

240/256     X     240/256   X    240/256  =  211/256

MILTY's  routing table would list the destination path to DEAL2 as a "211."

A  user requesting a connect to DEAL2 would be routed via the above path unless

a higher path quality route existed.

New nodes  in the network  broadcast their existence to their neighbors and

are  added  TO  the routing tables.    In order to scrub failed  nodes FROM the

routing tables, every TheNet node has a  "node keep alive"  counter.  The first

time  a neighbor node  broadcast is received,  the node assigns an initializing

number  value  to  the  received  node.      When  a  broadcast  is missed, the

initializing number is decremented by one (1).    Should a series of broadcasts

by  that node be missed and the counter decrements to zero, that node's listing

is  deleted  from the routing table and is no longer included  in  future  node

broadcasts.    However if, before  reaching  zero a  broadcast is received, the

initializing counter is reset.

An  advantage of  TheNet  is,  once setup, the NodeOp can pretty well trust

the automatic  features  to  take  care of  networking changes.   The node will

require  little  or  no  intervention  unless  a  catastrophic  failure occurs.

Attention  paid  to  initial  node setup will reduce  overhead  and allow it to

perform  as  a  "good neighbor"  within  the  networking  community.    Another

advantage   TheNet  has  for  some  is  it  broadcasts  the  NodeOp's  callsign

throughout the system.  This is a recognition reward for the time and money the

NodeOp has invested.